3GPP TSG-RAN WG2 #113bis-e	Tdoc R2-2103943

Electronic meeting, 12th – 20th April 2021

Agenda Item:	8.13.2.3

Source:	Ericsson

Title:	Other WID related SON features

Document for:	Discussion, Decision

# 1	Introduction

In RAN#88e a new WID on enhancement of data collection for SON/MDT in NR was approved [1]. In the following, we will discuss the objectives of the WID related to SON aspects. In particular, we will outline the scope and the requirements that RAN2 should address in Rel.17 when it comes to SON features.

# 2	Discussion

The new Rel.17 WID on enhancement of data collection for SON/MDT [1] contains various objectives that calls for enhancements to the existing SON framework. Specifically, the following was captured in the new WID:

- Support of data collection for SON features, including CCO, inter-system inter-RAT energy saving, inter-system load balancing, 2-step RACH optimization, mobility enhancement optimization, and leftovers of Rel-16 SON/MDT WI (PCI selection, energy efficiency (OAM requirements), Successful Handovers Reports, UE history information in EN-DC, load balancing enhancement, MRO for SN change failure, RACH Optimisation enhancements) [RAN3, RAN2]

- Specification of the UE reporting necessary to enhance the network configuration [RAN2].

- Specification of the inter-node information exchange, including possible enhancements to S1/NG, X2/Xn, and F1/E1 interfaces [RAN3]

Besides the above main objectives, the WID also mentions the following topic to be treated with lower priority:

Depending on the progress of the work, the following objective may be discussed in the later part of the WI:

  - NR-U related SON/MDT optimization which aims to reuse e.g. the existing NR-U measurements [RAN3, RAN2]

In this paper, we focus on the following topics:

- Enhancements to SCGFailureInformation reporting

- Mobility History Information Reporting

- RACH Optimization

- Possible NR-U impacts

## 2.1 Enhancements to SCGFailureInformation reporting

As highlighted in our contribution [2], the SCGFailureInformation reporting lacks some information that instead are present in the RLF report. 
In particular, in SCGFailureInformationNR message and SCGFailureInformation message, the perRAInfoList field is not included and thus MN or SN is unaware of any RA related issues that led to the RLF on SCG. We also note that the following fields are missing in the SCGFailureInformationEUTRA message, SCGFailureInformationNR message and SCGFailureInformation message: previousPSCellID, failedPSCellID, connectionFailureType, timeConnFailure fields. Those fields can aid the SN initiated SN change procedure, therefore it is beneficial to include them.

RAN2 should discuss whether these missing pieces of information should be directly included in the SCGFailureInformationEUTRA/SCGFailureInformationNR/SCGFailureInformation or in separate messages. In our understanding adding such information in the SCG failure information messages may significantly increases the size of this report substantially which is not desired, especially because the SCGFailureInformation report should be delivered quickly reliably in order to minimize the service interruptions. Such concern should not be neglected, especially e.g. for URLLC type of scenarios.

We will therefore propose to include such MRO information in a separate message that the network can inquiry after the SCG failure at any more suitable time when the link is stable, similar to the RLF report.

Include perRAInfoList field related to NR SCG failures in a separate message, rather than in the SCGFailureInformationNR/ SCGFailureInformation message.

Include previousPSCellID, failedPSCellID, connectionFailureType and timeConnFailure related to NR/EUTRA SCG failures in a separate message, rather than in the SCGFailureInformationEUTRA/SCGFailureInformationNR/SCGFailureInformation message.

The UE transmits the above message when the network requests it, similar to the RLF-report.

## 2.2 Mobility History Information Reporting

In email discussion 852 [3], companies shared their views on some aspects related to MHI. Companies discussed whether PSCell information should be nested inside of PCell MHI report or should be a separate list. It seems that most companies agree that the solution is to have PSCell information nested inside PCell MHI report, so that the association between PCell and visited PSCell while connected to this PCell will be given implicitly.

PSCell MHI should be nested within the PCell MHI.

In our view also the LTE specification should be enhanced to include PSCell MHI. Additionally, according to 36.331, UEs only include E-UTRAN related PCell information in the MHI report, and the time spent outside E-UTRAN. As a consequence, the UE will not include the list of NR visited cells in the LTE MHI, so that it will not be possible for an LTE cell to retrieve the list of NR visited cells, and also to distinguish whether during the time spent outside EUTRA, the UE had coverage issues or it was simply visiting NR cells.

Current TS36.331 does not allow the UE to report the list of NR visited cell. Therefore, it will not be possible for the network to retrieve from the LTE MHI, the NR visited cell and hence also to determine whether during the time spent outside E-UTRA the UE was out-of-coverage or visiting NR cells.

Given the above observation, it is proposed to enhance the MHI in TS 36.331 so that it includes the list of visited NR PCells and PSCells. This would be imply aligning the MHI structure of TS 38.331 and TS 36.331.

Include the list of visited NR PCells and PSCells in the VisitedCellInfoList of TS 36.331 i.e., RAN2 to align the MHI of TS 38.331 and TS 36.331.

Another important issue discussed in discussion 852 [3] were whether PSCell MHI information is to be reported to both PCell (MN) and PSCell (SN) or only to PCell.

In our view, only sending the PSCell MHI to the PCell implies the SN should rely on the MN requesting the information related to the SN changes.

Furthermore, there might be deployment scenarios, e.g. in EN-DC in which the MN is a Rel-15 eNB and SN is Rel-17 gNB. Hence then MN (and PCell) doesn’t have the capability to fetch MHI information. Although the UE has full MHI information logged including PCell and PSCell, such restriction hinders network to collect the report from UE.

In EN-DC scenario, sending MHI information only to PCell may result in loss of information from the UE.

It would be more efficient if the SN can fetch autonomously the MHI report from the UE and use the information if supported by the UE. For example, the UE may indicate MHI availability in the RRCReconfigurationComplete via SRB3 to the SN, to which the SN may reply with a mobilityHistoryReportReq transmitted via SRB3. Or the UE may indicate MHI availability to the SN via the MN in SRB1, in which case the mobilityHistoryReportReq is transmitted by the SN via SRB1.

The mobility history information needs to be sent to both MN (PCell) and SN (PSCell).

A discussion on further enhancements of current MHI report was also discussed and all companies agree that at least PSCell ID (CGI or PCI+frequency) and time UE stayed in each PSCell to be added into PSCell MHI. While this is important but not sufficient in our view.

Figure 1: Timeline of a UE switching between DC and non-DC connectivity

Consider a scenario where UE switches between DC and stand-alone configuration for the same PCell; such a timeline is shown in Figure 1. Here, the total time for the UE in a given PCell is T. However, it has some period where no PSCell is configured (Tw1,Tw2,Tw3) and some period with PSCells (Ts1,Ts2,Ts3). If UE only includes the times it stayed in PSCells (Ts1,Ts2,Ts3)in its MHI report, it is unclear to network when it didn’t have any DC connectivity. However, this information is important in network as it may provide some indication regarding coverage, as well as potential ping-pong between the PSCells.

UE includes time without PSCell in the PSCell MHI report.

In addition, information on the deployment characteristics of the cell will give an indication of the size of the cell and can further help the understanding of the UE mobility patterns. One example could be the carrier frequency of the cell which today is only included in the mobility history information when the Physical Cell ID is reported and not the globally unique ID. However, even when knowing the globally unique ID of the cell, it is hard to know the coverage area of that cell, which is more correlated with the carrier frequency of the cell.

RAN2 to consider including carrier frequency of the cell in the mobility history report.

## 2.3 RACH Optimization

The WID mentions also other “RACH optimization” as possible Rel.16 leftover.

A first leftover is related to the inclusion of SCell in RA-Report which was agreed in Rel.16. That is because, a random access procedure can also be triggered in the SCell, e.g. upon reception of PDCCH order.
In particular, in the current specification, the UE can either choose to report the CGI of the PCell or the physical cell identity and the carrier frequency of the SCell. Not both. However, in case the random access happens in the SCell, it is preferable if the UE can report both the SCell (in which RA occurred) and the PCell. In this way, if the RA-report is received by a gNB different than the gNB in which RA occurred, it is possible for the network to forward the RA-Report to the intended gNB in which RA was performed.

Enhance the existing ASN.1 of the RA-Report, in order to allow the UE to include the PCell also in case the RA occurred in an SCell.

Regarding this, we note that the existing RA-Report-r16 unlike the RLF report, does not contain any location information or radio measurements. One may argue that such information is not needed since the RA report only refers to successful RA attempts. However, even if a random access attempt was successful, the reasons for which the UE triggered the random access procedure may be due to radio issues that might be beneficial for the network to know. For example, if the UE triggered random access for the sake of beam failure recovery or SR failure issues, including the location information and some radio measurements may bring value to the SON framework. In other cases, e.g. access related RA or HO related RA, such information does not seem to be needed.
This information can also be beneficial to understand UL/DL mismatch issues, that were discussed at last RAN2#111 meeting.

The existing RA report does not include location information and radio measurements, even if the RA procedure, in some cases, may be triggered by radio issues. Including this information can be beneficial for the network to detect UL/DL mismatches.

RAN2 to include the location information and the radio measurement in the RA report depending on the raPurpose, e.g. in case of SR failure, beam recovery failure, UL synchronization issues.

Additionally, related to the RA-Purpose, we note that in some cases RLF might be due to random access problems. The RLF report contains the RA-InformationCommon, but not the raPurpose. Hence, from the RLF report it is not possible to know for which reason the UE triggered the random access that eventually resulted into an RLF.

Include in the RLF-Report the raPurpose whenever the RLF cause is randomAccessProblem.

## 2.5 Enhancements related to NR-U

NR-U functionality has been introduced in Rel.16, giving the possibility to the UE to operate in the unlicensed spectrum. Unlike LAA in LTE, in which case only user plane data could be transmitted in the unlicensed spectrum, in NR-U any type of transmission (hence including L1/L2 and RRC control signalling) can be transmitted in the unlicensed spectrum. Additionally, with NR-U, also the SpCell can operate over the unlicensed spectrum (not only the SCells), thereby opening the possibility for standalone unlicensed operations.

NR-U implies some new set of features that both the UE and the network should comply in order to perform NR-U operations. In particular, before transmitting on the unlicensed spectrum, both the UE and the gNB should perform LBT and sense the wireless channel, in order to ensure that the medium is not occupied by other transmissions which could be generated by non-3GPP technologies such as WiFi. Additionally, also new types of measurements representing the RSSI and channel occupancy have been introduced to reflect the characteristics of the unlicensed channel.

In the SON/MDT WID, this topic has been assigned lower priority, but we believe that some changes can be addressed with relatively small effort during the WI phase. 
In the current SON framework, only the RLF-report considers the NR-U system, i.e. the RLF cause can be set to “lbtFailure” in current legacy. However, no other information representing LBT statistics and RSSI/channel occupancy measurements are considered in the RA/RLF report. Moreover, as per current specification, no RA-report is even included if the UE performed RA due to consistent LBT failures. Therefore, RAN2 can for instance start enhancing the existing RA/RLF report and consider the above aspects.

RAN2 aims at some basic enhancements to the current SON framework to address the NR-U system, e.g. enhancements to the existing RA/RLF report.

# Conclusion

In the previous sections we made the following observations:

Observation 1	Current TS36.331 does not allow the UE to report the list of NR visited cell. Therefore, it will not be possible for the network to retrieve from the LTE MHI, the NR visited cell and hence also to determine whether during the time spent outside E-UTRA the UE was out-of-coverage or visiting NR cells.

Observation 2	In EN-DC scenario, sending MHI information only to PCell may result in loss of information from the UE.

Observation 3	The existing RA report does not include location information and radio measurements, even if the RA procedure, in some cases, may be triggered by radio issues. Including this information can be beneficial for the network to detect UL/DL mismatches.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Include perRAInfoList field related to NR SCG failures in a separate message, rather than in the SCGFailureInformationNR/ SCGFailureInformation message.

Proposal 2	Include previousPSCellID, failedPSCellID, connectionFailureType and timeConnFailure related to NR/EUTRA SCG failures in a separate message, rather than in the SCGFailureInformationEUTRA/SCGFailureInformationNR/SCGFailureInformation message.

Proposal 3	The UE transmits the above message when the network requests it, similar to the RLF-report.

Proposal 4	PSCell MHI should be nested within the PCell MHI.

Proposal 5	Include the list of visited NR PCells and PSCells in the VisitedCellInfoList of TS 36.331 i.e., RAN2 to align the MHI of TS 38.331 and TS 36.331.

Proposal 6	The mobility history information needs to be sent to both MN (PCell) and SN (PSCell).

Proposal 7	UE includes time without PSCell in the PSCell MHI report.

Proposal 8	RAN2 to consider including carrier frequency of the cell in the mobility history report.

Proposal 9	Enhance the existing ASN.1 of the RA-Report, in order to allow the UE to include the PCell also in case the RA occurred in an SCell.

Proposal 10	RAN2 to include the location information and the radio measurement in the RA report depending on the raPurpose, e.g. in case of SR failure, beam recovery failure, UL synchronization issues.

Proposal 11	Include in the RLF-Report the raPurpose whenever the RLF cause is randomAccessProblem.

Proposal 12	RAN2 aims at some basic enhancements to the current SON framework to address the NR-U system, e.g. enhancements to the existing RA/RLF report.

# References

RP-201281, New WID on enhancement of data collection for SON/MDT in NR, 3GPP TSG RAN meeting #88-e

R2-2001118, SCGFailureInformation message content alignment with RLFReport, Ericsson, RAN2#109-e

R2-21xxxxx, [Post113-e][852][NR R17 SON/MDT]  2 step RA and other SON changes (CATT), CATT (Rapporteur), RAN2#113-bis