3GPP TSG-RAN WG2 Meeting #113bis-e electronic	R2-2104005

Online, 12 – 20 April 2021

Source: 	Huawei, HiSilicon

Title: 	Discussion on slice based RACH configuration

Agenda Item:	8.8.3	RACH

Document for:	Discussion and decision

- Introduction

During the RAN#91-e, a new WID for RAN Slicing [1] was approved. For slice based RACH configuration, detailed objectives are given as follows,

- Support slice based RACH configuration, specify mechanisms and signalling including, for Mobile Originating cases [RAN2]

- Configure separated PRACH configuration (e.g., transmission occasions of time-frequency domain and preambles) for slice or slice group

- Configure RACH parameters prioritization (e.g., scalingFactorBI and powerRampingStepHighPriority) for slice or slice group

- Determine how this works with existing functionality, which may include how to perform RACH type selection (e.g., 2-step and 4-step), support of RACH fall-back cases, handling of simultaneous configuration with similar functions such as legacy RA prioritization (e.g., MPS and MCS UEs).

In this contribution, the discussions are mainly about the technical details of the slice grouping design for slice based PRACH configuration, RACH parameters prioritization, and RACH fallback.

- Discussion

  - Slice grouping

For similar service requirements, multiple S-NSSAIs may share the same RACH configuration. If the RACH configuration is broadcast for each S-NSSAI individually or for a list that includes all relevant S-NSSAIs, the SIB overhead would be significant. Therefore, a straightforward approach is to divide the S-NSSAIs into different slice groups indicated by different slice group IDs. S-NSSAIs within the same slice group share the same configuration. And RAN can broadcast a separate RACH configuration for each slice group ID.

The mapping of slice group ID to S-NSSAIs contains slice group ID values and the corresponding mapped S-NSSAI values. The mapping relationship can be maintained by RAN or CN. For details of slice grouping, it can be found in section 2.2 in [4].

  - Slice based RACH configuration

In this section, technical details of slice based RACH configuration are discussed, including PRACH, RACH parameters, and RACH fallback.

    - Slice based PRACH configuration

According to TS 38.321 [3], for the random access resource selection of 2-step and 4-step RACH, if the association between the RO/Preamble and the SSB is configured, an SSB is selected from the SSBs with SS-RSRP above (msgA-)rsrp-ThresholdSSB. If no SSB’s SS-RSRP is greater than (msgA-)rsrp-ThresholdSSB, the UE selects any SSB and randomly selects a Preamble from the Preambles corresponding to the SSB. Otherwise, the Preamble is randomly selected. Then, the next available RO corresponding to the selected SSB is selected.

Therefore, for 2-step and 4-step RACH, to achieve slice-specific RACH resource isolation, separate ROs, and/or Preambles can be configured per slice group.

Proposal 1: It is proposed that RAN2 agree on separate ROs, and/or Preambles for each slice group in 2-step RACH and 4-step RACH.

    - Slice based RACH parameters prioritization

According to TS 38.321 [3], existing prioritizations for RACH parameters are applied to MPS and MCS services. Upon receiving the access identity associated with MPS or MCS from the NAS layer, the AS layer selects relevant prioritized RACH parameters for the RACH procedure. Related parameters are powerRampingStepHighPriority and scalingFactorBI given in the RA-Prioritization IE.

Furthermore, the above two parameters are common to MPS and MCS services. To achieve slice based RACH prioritization, one possible approach is to reuse these two parameters for certain slice groups.

Proposal 2: It is proposed that RAN2 agree to reuse the existing RACH parameter prioritization for certain slice groups, i.e., powerRampingStepHighPriority and scalingFactorBI.

However, these parameters may not be sufficient to meet the various requirements of different slices. To achieve RACH prioritization among different slices, slice-specific RACH parameters should be introduced. For example, a shorter contention resolution timer (i.e., ra-ContentionResolutionTimer) can satisfy the delay-sensitive requirement of URLLC slices. Other possible RACH parameters include, for example, preambleTransMax, ra-ResponseWindow, etc.

Proposal 3: It is proposed that RAN2 agree to support slice-specific RACH parameters for different slice groups, e.g., ra-ContentionResolutionTimer, preambleTransMax, and ra-ResponseWindow, etc.

Regarding the possible collision between the legacy RA prioritization (e.g., MPS/MCS) and the slice-specific RA prioritization, the solution varies case by case,

- Case #1: slice-specific and legacy RA prioritizations share the same configuration, for example, the same set of powerRampingStepHighPriority and scalingFactorBI.

Solution: No collision for the same RA prioritization configuration.

- Case #2: slice-specific and legacy RA prioritization configurations do not overlap, for example, slice-specific ra-ContentionResolutionTimer, preambleTransMax, ra-ResponseWindow, etc.

Solution: No collision, and the two RA prioritization configurations can work together.

- Case #3: slice-specific and legacy RA prioritizations have the same RA prioritization parameter type, but different values, e.g., slice-specific powerRampingStepHighPriority and scalingFactorBI.

Solution: Collision, and it is suggested that slice-specific RA prioritization overrides the legacy RA prioritization.

Proposal 4: It is proposed that RAN2 agree that the slice-specific RA prioritization overrides the legacy RA prioritization in case of conflict.

    - Slice based RACH fallback

Currently, the trigger of the fallback from 2-step to 4-step RA is that the number of the RACH preamble transmissions (i.e., msgA-TransMax) for 2-step RA reaches the maximum.

Based on the above proposals, we may have the following 4 RACH combinations:

- 2-step RA with slice-specific configurations (e.g., resource and parameters);

- 2-step RA with legacy configurations;

- 4-step RA with slice-specific configurations;

- 4-step RA with legacy configurations;

Following the above combinations, there may be some fallback cases, e.g.:

- From 1) to 3)

- From 2) to 4)

- From 1) to 2), or from 3) to 4)

We see some benefits of some fallback cases, and some existing means could be considered, e.g., msgA-TransMax is the trigger for the fallback from 2-step RA to 4-step RA. However, the fallback mechanisms may be complicated and details may need to be further investigated.

Proposal 5: For the slice based RACH fallback, the benefits, combinations, and complexities of solutions should be carefully justified.

- Conclusion

In this contribution, we mainly discuss the technical details of the slice grouping design for slice based RACH configuration. It is proposed:

Proposal 1: It is proposed that RAN2 agree on separate ROs, and/or Preambles for each slice group in 2-step RACH and 4-step RACH.

Proposal 2: It is proposed that RAN2 agree to reuse the existing RACH parameter prioritization for certain slice groups, i.e., powerRampingStepHighPriority and scalingFactorBI.

Proposal 3: It is proposed that RAN2 agree to support slice-specific RACH parameters for different slice groups, e.g., ra-ContentionResolutionTimer, preambleTransMax, and ra-ResponseWindow, etc.

Proposal 4: It is proposed that RAN2 agree that the slice-specific RA prioritization overrides the legacy RA prioritization in case of conflict.

Proposal 5: For the slice based RACH fallback, the benefits, combinations, and complexities of solutions should be carefully justified.

- References

- RP-210912, New WID on enhancement of RAN Slicing for NR.

- 3GPP TR 38.832 V0.5.0, Study on enhancement of Radio Access Network (RAN) slicing.

- 3GPP TS 38.321 V16.3.0, Medium Access Control (MAC) protocol specification.

- R2-2104004, Discussion on slice based cell reselection under network control, Huawei, HiSilicon