3GPP TSG-RAN WG2 Meeting #113bis electronic	R2-2104032

Online, April 12 – April 20, 2021

Source:	CATT

Title:	Discussion on slice based Cell Reselection

Agenda Item:	8.8.2

Document for:	Discussion and Decision

- Introduction

In the enhancement on RAN support of network slicing WID [1], detailed objectives of the work item are:

The work item aims to standardize the enhancement on RAN support of network slicing. Detailed objectives of the work item are:

- Support slice based cell reselection, specify mechanisms and signalling including [RAN2]

- To assist cell reselection, broadcast the supported slice info of the current cell and neighbour cells, and cell reselection priority per slice in system information message.

- To assist cell reselection, include slice info (with similar information as in SI message) in RRCRelease message.

- Support slice based RACH configuration, specify mechanisms and signalling including, for Mobile Originating cases [RAN2]

- Configure separated PRACH configuration (e.g., transmission occasions of time-frequency domain and preambles) for slice or slice group

- Configure RACH parameters prioritization (e.g., scalingFactorBI and powerRampingStepHighPriority) for slice or slice group

- Determine how this works with existing functionality, which may include how to perform RACH type selection (e.g., 2-step and 4-step), support of RACH fall-back cases, handling of simultaneous configuration with similar functions such as legacy RA prioritization (e.g., MPS and MCS UEs).

Note: The use of Rel-17 RAN slicing enhancements in given cells shall not prevent from accessibility for Rel-15 and Rel-16 UEs.

Note: RAN3 objectives to be added after RAN3 study phase is finished.

Note: This work item should take SA2 output on slicing enhancement into consideration if RAN impacts are identified, e.g. the relation between Tracking Area and S-NSSAI is expected to impact the solution for slice based cell reselection.

In this contribution, we’d like to discuss the item 1 support slice based cell reselection.

- Discussion

From the objective description, the slice based cell reselection is included in the normative work. The specifying work includes two sub items, one is for broadcasting in SI and one is for the similar in RRC release. In broadcasting, it includes broadcast the slice information and per slice priority. In this contribution, we will discuss from these three aspects.

Before we go to the detail discussion on the specifying, we need consider the some assumption of the slice based cell reselection like as the last Note in the objective. e.g. the relation between Tracking Area and S-NSSAI is expected to impact the solution for slice based cell reselection. The SA2 sent the liaison to the other groups for the confirmation on whether the cells in one TA/RA support the same slice in R15/16 spec.  Almost groups answer YES firmly except RAN2. So the SA2 assume that all cells of a tracking area support the same S-NSSAI(s) and the S-NSSAI(s) of the Allowed NSSAI are supported by all tracking areas in a registration area. If we obey this assumption, the slice based cell reselection may not be one reasonable case if the UE move within the RA because the UE always knows supported slice of neighbour cell. But if the UE move cross the RA, the slice based cell reselection can help UE to fast access the intended slice.

The assumption that all cells of a tracking area support the same S-NSSAI(s) is clarified in R16 and before. If we would have more flexible deployment on supported slice, the cell supported slice in one tracking area may be different in the future release. So we should not couple the slice supporting in one TA and slice based cell reselection.

Observation 1: The slice based cell reselection can be performed at least when the UE move across the RA

Observation 2: The cell supported different slice in one tracking area may defined in future release

Proposal 1: Decouple the specifying of slice based cell reselection and the relation between Tracking Area and slice supporting

In the study phase, the slice based cell reselection was discussed and the scenarios, solutions and open issues are captured in the solution 4 in TR38.832 [2].

Solution 4: Slice related information for cell reselection is provided in the system information or RRCRelease message.

Solution 4 can address the issue 1, issue 2, issue 3 and issue 4. There is benefit to broadcast slice related cell reselection info in SIB. The slice info (with similar information as agreed slice info in SI message) can be added in RRC release message. Details can be discussed in WI phase.

The concerns on security and SIB payload size for broadcasting slice related cell reselection info need to be resolved in WI phase (e.g., providing only SST, on-demand SIB, SIB segmentation, slice grouping or slice associated UAC information).

To assist cell reselection, RAN can broadcast the supported slice info of the current cell and neighbour cells, and cell reselection priority per slice. The slice info may be: providing only SST, on-demand SIB, SIB segmentation, slice grouping (if any), or slice associated UAC information where other solutions are not precluded. Details can be discussed in WI phase. And,  how to ensure UE doesn't lose coverage due to slice prioritization can be considered in WI phase.

The solution of adding the intended slice for MT access in slice specific cell reselection is not pursued.

Slice group is supported for both solution 3 and solution 4. Whether to define a new grouping mechanism or reusing UAC access category is left to WI phase.

- Supported Slice information

The particular slice identified by S-NSSAI. The S-NSSAI comprising is descripted in 23.501 as below.

An S-NSSAI is comprised of:

-	A Slice/Service type (SST), which refers to the expected Network Slice behaviour in terms of features and services;

-	A Slice Differentiator (SD), which is optional information that complements the Slice/Service type(s) to differentiate amongst multiple Network Slices of the same Slice/Service type.

An S-NSSAI can have standard values (i.e. such S-NSSAI is only comprised of an SST with a standardised SST value, see clause 5.15.2.2, and no SD) or non-standard values (i.e. such S-NSSAI is comprised of either both an SST and an SD or only an SST without a standardised SST value and no SD). An S-NSSAI with a non-standard value identifies a single Network Slice within the PLMN with which it is associated. An S-NSSAI with a non-standard value shall not be used by the UE in access stratum procedures in any PLMN other than the one to which the S-NSSAI is associated.

The S-NSSAI may include both SST and SD, the length of SST is 8bits and the length of SD is 24bits. Totally the maximum length of S-NSSAI is 32bits. As we discussed in study phase, the payload size should be a concern if the NSSAI broadcasted in system information.  Also from SA3 required security point view, including NSSAI related information in unprotected RRC signaling should be avoided when possible and shall be avoided when privacy of such information (e.g., when UE is accessing privacy sensitive slices) is required. So exposing the completed S-NSSAI in System Information does not make sense.

In the TR of study, we identify several solutions for the slice information broadcasting, include providing only SST, on-demand SIB, SIB segmentation, slice grouping or slice associated UAC information.

- Providing only SST

This solution may significant reduce the size of slice information. But the slice with different SD and the same SST would not be distinguished when the UE perform cell reselection. The UE may reselect to not intended slice only based on SST information. As above mentioned the comprising of S-NSSAI, only the standard values S-NSSAI only includes SST and some non-standard values S-NSSAI also only include SST and no SD.  From this point view providing only SST may fulfil our requirement. So we should study how the deployment slices with SD. Another issue is whether exposing only SST is permitted from security point view. If these two issues are solved, we don’t need introduce any new concept for slice information broadcasting. This solution is best choice.

Proposal 2: Check with SA3 whether the SST of Slice can be broadcasted in system information

Proposal 3: Study the scenario of cell reselection to the slice with SD

- slice grouping or slice associated UAC information

These two solutions use similar mechanism. The S-NSSAI will not be exposed in system information. These solutions can fulfil the security requirement.  The issue about size of slice information can be solved.

The UAC is existing feature. The Operator-defined access categories can include slice information and Operator-defined access category definitions can be signalled to the UE using NAS signalling. But it cannot provide the slice information of neighbour cell.

Slice group definition is new concept. SA2 don’t have this definition and not use it in their specification. We may just define the slice group for radio resource and RAN used.  How to inform UE the relation between the slice group and slices should be considered.  One solution is via the NAS signalling as UAC. One solution is via RRC signalling. If transfer this information via NAS, the slice group information should be transferred from RAN to Core Network and then to UE via NAS. This solution will impact RAN3/SA2/CT1 specification. To simply the slice group solution, the information transfer to UE via RRC is more reasonable.

Proposal 4: Transfer the relation information between slice group and slice to UE via RRC if the slice group solution is adopted

Consider the slice based specific RACH defined may use slice group concept; we may align these two items about slice group definition introduced.  If the slice group is used in slice based RACH, the slice group solution is better than UAC solution.  The UAC cannot used for identified the slice based RACH.

- SIB segmentation. In this solution, we suppose slice information include the whole S-NSSAI. With this solution, the delay may be introduced and size of the information cannot be reduced.

- On-demand SIB can be used any above solution for the system information acquisition. But  it cannot address the issue on size and security

Comprehensive above discussion, we prefer go to the providing only SST if the issues are solved.  And the slice group solution is second high priority.

Proposal 5: The solution about slice group or providing only SST should be prioritized

- Cell reselection priority per slice

Introduce the cell reselection priority per slice may help use to fast access the intended slice. In current NR spec, the cell reselection priority in system information is defined in Serving Frequency and inter-Frequency. If the cell reselection priority per slice is introduced, it should be signification complex. The UE should consider the priority of both slices and frequency. As above discussion, the S-NSSAI size is too long to include the broadcasting. To reduce the size and complexity, the slice group or just SST can be used for slice based cell reselection priority designing.

Proposal 6: slice group or only SST can be used for slice based cell reselection priority

The UE can support at most 8 requested and allowed slice. Even though the slice based cell reselection priority defined, how the UE perform the cell reselection base on the slice based priority is still one issue. Which slice UE will use for cell reselection if more than one slices included intended slice. The priority of the intended slice may be defined for solving this issue.

Proposal 7: study the mechanism about how UE perform the slice based cell reselection if two or more slices in the intended slice

- Include slice info (with similar information as in SI message) in RRCRelease message.

We discuss the slice information in SI as above, include slice support information and slice based cell reselection priority. In current spec, the cell reselection priority for the given UE is configured based on the RFSP in which the allowed slice already is considered. The UE performing the next access may include more slices in the intended slice except the allowed slice. So the cell reselection priority for slice supported in neighbour cell can be carried in the RRC release message for the UE cell reselection.

Proposal 8: slice base cell reselection priority can be carried in RRCRelease for the cell reselection

- Conclusion

In conclusion, we propose the followings:

Observation 1: The slice based cell reselection can be performed at least when the UE move across the RA

Observation 2: The cell supported different slice in one tracking area may defined in future release

Proposal 1: Decouple the specifying of slice based cell reselection and the relation between Tracking Area and slice supporting

Proposal 2: Check with SA3 whether the SST of Slice can be broadcasted in system information

Proposal 3: Study the scenario of cell reselection to the slice with SD

Proposal 4: Transfer the relation information between slice group and slice to UE via RRC if the slice group solution is adopted

Proposal 5: The solution about slice group or providing only SST should be prioritized

Proposal 6: slice group or only SST can be used for slice based cell reselection priority

Proposal 7: study the mechanism about how UE perform the slice based cell reselection if two or more slices in the intended slice

Proposal 8: slice base cell reselection priority can be carried in RRCRelease for the cell reselection

- References

[1] RP-210912 New WID on enhancement of RAN Slicing for NR, CMCC

[2] TR38.832 Study on enhancement of Radio Access Network (RAN) slicing