3GPP TSG-RAN WG2 Meeting #113bis-e                               R2-2104059

Electronic, 12th April – 20th April 2021

Agenda Item:	8.12.3.1

Source: 	Huawei, HiSilicon

Title:	eDRX for RedCap UE

Document for:	Discussion and Decision

Introduction

In the approved WID [1] on RedCap, there are the following objectives on eDRX:

- Specify support for the following Extended DRX enhancements for RedCap UEs [RAN2, RAN3, RAN4]: - Extended DRX for RRC Inactive and Idle with eDRX cycles up to 10.24 s, without using PTW and PH, and with common design (e.g. common set of eDRX values) between RRC Inactive and Idle - Extended DRX for RRC Inactive and Idle with eDRX cycles up to 10485.76 s; the details of mechanisms and feasibility regarding maximum length of the extended DRX cycles for RRC Inactive and Idle need to be checked by SA2, CT1 and/or RAN4. - RAN2 to decide which Node(s) configure eDRX in RRC_Idle and RRC_Inactive.

In this contribution, we continue to discuss eDRX mechanisms for RedCap UE.

Discussion on eDRX

The length of eDRX cycle

As agreed in last RAN2 meeting, it should be possible for (at least some) RedCap UEs to receive emergency broadcast services. Different options allowing RedCap UEs to receive emergency broadcast services and/or reduce paging power consumption (and corresponding recommended eDRX lower boundary) are captured in [2] as follows.

Option 1: eDRX supports a lower bound of 2.56s.

Option 2: For RedCap UEs, if the NAS configures the UE with a 2.56 DRX cycle, the RedCap UE follows this DRX even when the RAN paging cycle is shorter. eDRX lower bound can be kept to baseline 5.12s.

Option 3: gNB can configure 2.56s default broadcasted DRX cycle for those RedCap Ues that need to receive emergency broadcast services and a shorter UE-specific RAN paging cycle for UEs with tighter latency requirements (e.g. smartphones). eDRX lower bound can be kept to baseline 5.12s.

Option 4: RedCap Ues that need to receive emergency broadcast services are not expected to request to be configured with eDRX, and no specific handling/configuration is required for those UEs. eDRX lower bound can be kept to baseline 5.12s.

Option 5: RedCap UE can request an eDRX configuration while still monitoring in between for ETWS and CMAS. eDRX lower bound can be kept to baseline 5.12s.

Among them, Option 1 and 2 (i.e. Solution 1 in TR38.875 [3]) assume that RedCap UEs do not need to monitor paging according to the default paging cycle, which may result in a potential risk of UE missing SI change indicator. Additional efforts have to be made to deal with the associated issues. Option 3 (i.e. Solution 2 in TR38.875) is based on the assumption that gNB are configured with a 2.56 second default paging cycle, which is seldom used in existing deployments. Option 4 assumes that RedCap UEs should decide between benefiting from eDRX power saving or supporting emergency broadcast services. Hence, we prefer option 5, wherein RedCap UE can request an eDRX configuration while still monitoring in between for ETWS and CMAS. eDRX lower boundary can be kept to the baseline 5.12s.

- RedCap UE can request an eDRX configuration while still monitoring in between for ETWS and CMAS. eDRX lower bound can be kept to the baseline 5.12s. (Option 5)

The longest eDRX cycle that can be supported by a 10 bit H-SFN is 10485.76s. There is no technical reasons to limit artificially to a lower value for RedCap UE from RAN2 point of view. Then, it is proposed that the eDRX cycle values for both RRC_IDLE and RRC_INACTIVE range from 5.12s to 10485.76s

- eDRX cycle values for both RRC_IDLE and RRC_Inactive range from 5.12s to 10485.76s.

eDRX configuration for RRC_INACTIVE

As discussed in study phase, there are two options for which node decides the eDRX configuration for RRC_INACTIVE.

Option 1: CN decides the eDRX parameters for RRC_INACTIVE.

Option 2: RAN decides the eDRX parameters for RRC_INACTIVE

In R16 eMTC connected to 5GC, the ng-RAN decides the eDRX cycle for RRC_INACTIVE, based on the idle mode eDRX cycle configuration provided by the AMF. For RedCap UE, it would be beneficial for the gNB to be able to configure shorter eDRX cycle in RRC_INACTIVE, for example to limit the amount of buffering required for the DL data, depending on the gNB capability. In addition, it is common understanding that RAN is responsible for the configuration in RRC_INACTIVE.  Therefore, it is proposed that the same principle applied to RedCap UE

- RAN decides the eDRX parameters for RRC_INACTIVE.

The introduction of eDRX cycle exceeding 10.24s for RRC_INACTIVE may have an impact the current NAS retransmission mechanism. Therefore, if it is agreed that RAN decides the eDRX parameters for RRC_INACTIVE, some coordination mechanism between the AMF and RAN is needed to handle this issue. The following are two potential solutions.

Solution 1: AMF provides information to the RAN for the setting of the eDRX cycle in RRC_INACTIVE

Solution 2: RAN provides information to the AMF for the setting of NAS retransmission timers

In solution 1, considering that different CNs may support different NAS retransmission mechanisms, e.g. minimum retransmission interval and/or maximum number of retransmissions are decided by network implementation, the CN provides a maximum value of eDRX cycle in RRC_INACTIVE to RAN for reference, and RAN can configure the eDRX cycle in RRC_INACTIVE accordingly.

In solution 2, the RAN decides the eDRX cycle in RRC_INACTIVE and provides the value to the AMF. The AMF sets the values of the NAS retransmission timers so that they are long enough for the eDRX cycle in RRC_INACTIVE not to interfere with the NAS retransmission mechanism.

- To deal with the NAS retransmission issues when introducing longer eDRX cycle in RRC_INACTIVE, two potential coordination methods between the AMF and RAN can be considered.
Solution 1: AMF provides information to the RAN for the setting of the eDRX cycle in RRC_INACTIVE;
Solution 2: RAN provides information to the AMF for the setting of NAS retransmission timers.

Considering that both the eDRX value range and the eDRX configuration have impacts on SA2/CT1/RAN4, it is proposed to send a LS to SA2/CT1/RAN4 about RAN2 conclusion on eDRX range and eDRX configuration mechanism.

- Send a LS to SA2/CT1/RAN4 on the RAN2 conclusion for eDRX: including the eDRX cycle value range and eDRX configuration mechanism.

SI acquisition in eDRX mechanism

In non-eDRX scenario, when the network changes (some of the) system information, it first notifies the UEs about this change throughout a modification period, and transmits the updated system information in the following modification period. The length of modification period equals modificationPeriodCoeff *defaultPagingCycle and the maximum value of modificationPeriodCoeff is 4. When eDRX mechanism is introduced in NR, the eDRX cycle may be larger than the SI modification period. Then, the UE may miss the SI change notification. In LTE, an eDRX SI modification indication and an eDRX acquisition period were introduced. Then, it is proposed that the LTE eDRX SI acquisition mechanism is considered as baseline.

- The LTE eDRX SI acquisition mechanism is considered as the baseline (i.e. an eDRX specific SI update indication and eDRX SI modification boundary) for the case where eDRX cycle is larger than the SI modification period.

Conclusion and Proposals

In this contribution, eDRX for reduced capability UEs was discussed with the following and proposals:

- RedCap UE can request an eDRX configuration while still monitoring in between for ETWS and CMAS. eDRX lower bound can be kept to the baseline 5.12s. (Option 5)

- eDRX cycle values for both RRC_IDLE and RRC_Inactive range from 5.12s to 10485.76s.

- RAN decides the eDRX parameters for RRC_INACTIVE.

- To deal with the NAS retransmission issues when introducing longer eDRX cycle in RRC_INACTIVE, two potential coordination methods between the AMF and RAN can be considered.
Solution 1: AMF provides information to the RAN for the setting of the eDRX cycle in RRC_INACTIVE;
Solution 2: RAN provides information to the AMF for the setting of NAS retransmission timers.

- Send a LS to SA2/CT1/RAN4 on the RAN2 conclusion for eDRX: including the eDRX cycle value range and eDRX configuration mechanism.

- The LTE eDRX SI acquisition mechanism is considered as the baseline (i.e. an eDRX specific SI update indication and eDRX SI modification boundary) for the case where eDRX cycle is larger than the SI modification period.

References

- RP-210918, Revised WID on support of reduced capability NR devices, Nokia, Ericsson, RAN #91e.

- R2-2102019 Report of 3GPP TSG RAN WG2 meeting #113-e Online.

- TR38.875 V1.1.0: Study on support of reduced capability NR devices.