3GPP TSG-RAN WG2 Meeting #113bis-e	R2-2104119

# Electronic meeting, 12 – 20 April 2021

Agenda item:		8.1.3

Source:	Intel Corporation

Title:	MBS support for delivery mode 2

Document for:		Discussion and Decision

# Introduction

In RAN2#113-e meeting, following was agreed:

⇒ Both idle/inactive UEs and connected mode UEs can receive MBS services transmitted by NR MBS delivery mode 2 (Broadcast service as already agreed, TBD other). The ability for connected mode UEs to receive this may depend on the network provisioning of the service (e.g. which freq), UE connected mode configuration and UE capabilities. ⇒ The two-step based approach (i.e. BCCH and MCCH) as adopted by LTE SC-PTM is reused for the transmission of PTM configuration for NR MBS delivery mode 2. ⇒ Assume it is possible to reuse LTE SC-PTM mechanism for the CONNECTED UEs to receive the PTM configuration for NR MBS delivery mode 2, i.e. broadcast based manner. ⇒ Assume that MCCH change notification mechanism is used to notify the changes of MCCH configuration due to session start for delivery mode 2 of NR MBS (other cases FFS, if any). ⇒ Assume that MBS Interest Indication is supported for UEs in connected mode for Broadcast service (assume that as usual there is no mandatory network requirement, network action is up to network). ⇒ MBS Interest Indication is NOT supported for UEs in idle/inactive mode for NR MBS delivery mode 2. ⇒ Assume that some information for purpose of service continuity can be provided for NR MBS delivery mode 2. (FFS what - need to be revisited, e.g. based on progress in other groups, e.g. USD, SAI/TMGI etc) ⇒ FFS whether support UE awareness of MBS services on frequency basis for service continuity for NR MBS delivery mode 2 (i.e. Reuse LTE SC-PTM mechanism). ⇒ FFS Support frequency prioritization during cell reselection for service continuity for NR MBS delivery mode 2 (i.e. Reuse LTE SC-PTM mechanism). ⇒ P2: Whether UEs that receive Multicast can be released to RRC Inactive / Idle and continue receiving Multicast is Postponed. Should limit to RRC inactive in future discussions

Email discussion “[Post112-e][069][MBS] Delivery mode 2” was initiated to discuss MCCH aspects.

In this contribution, we discuss MBS support for delivery mode 2.

# Discussion

## Applicability of delivery mode 2 to multicast session

In RAN2#112-e meeting, following was agreed: “R2 assumes that delivery mode 2 is used for broadcast sessions. The applicability of delivery mode 2 to multicast sessions is FFS.” In RAN2#113-e meeting, following was captured in Chairman notes “P2: Whether UEs that receive Multicast can be released to RRC Inactive / Idle and continue receiving Multicast is Postponed. Should limit to RRC inactive in future discussions”.

SA2 TR 23.757 [1] defines the following regarding multicast session:

- Active multicast session: Established multicast session in active state. Multicast data are transmitted to UEs that joined the multicast session. 5GC resources for the multicast session are reserved. Corresponding Radio resources are reserved depending on participating UE locations. UEs that joined the multicast session are in CM CONNECTED state. UEs are allowed to join the multicast session (subject to authorization check)

It can be seen that from SA2 perspective, UEs receiving multicast data should be in CM-CONNECTED state.

Observation 1: From SA2 perspective, UEs receiving multicast data should be in CM-CONNECTED state.

Given that CM-CONNECTED state corresponds to RRC_INACTIVE and RRC_CONNECTED states, and multicast session can be received in RRC_CONNECTED, the question is whether multicast session can be received in RRC_INACTIVE as well.

From QoS perspective, there should be no limitation of receiving multicast session in RRC_CONNECTED only. As concluded in TR 23.757 [1] clause 8.5 (Key issue #4: QoS level support for Multicast and Broadcast communication services), the 5G QoS model and parameters as defined in TS23.501 clause 5.7 also apply to MBS service. According to TS23.501 Clause 5.7, there are some services have relatively lower QoS requirement, which are suitable for delivery mode 2. For instance, MCPTT requires longer latency (75ms packet delay budget) and lower reliability (10-2 packet error rate). It can be seen that there are multicast services having similar QoS requirements as broadcast services. Given that delivery mode 2 is used for broadcast services, there is no reason to exclude multicast services for delivery mode 2.

Moreover, multicast services such as group communication and IoT services to massive devices requiring low power consumption can also benefit from receiving multicast session provisioned over delivery mode 2 in RRC_INACTIVE state.

Proposal 1: Multicast session can be provisioned over delivery mode 2, and can be received in RRC_INACTIVE state.

## MCCH

There are several open issues regarding MCCH design.

Area specific MCCH

One issue is whether MCCH can be area specific, similar to area specific SIB introduced in Rel-15. In typical cases, MCCH is cell specific regarding ongoing MBS sessions, therefore it is unlikely that neighboring cells share the same MCCH content. Introduction of area specific MCCH requires that the version of the MCCH (similar to valueTag) as well as area ID (similar to systemInformationAreaID) are signaled in MBS SIB. The reason not to reuse systemInformationAreaID in SIB1 is that the area for SIB and MCCH can be different. Given that version of MCCH is signalled in MBS SIB, the MBS SIB should be updated whenever MCCH content changes. This makes two-step MBS configuration approach not useful at all. In addition, this approach cannot work if MCCH should be changed faster than 640 ms (minimum BCCH modification period). Given the increased overhead, unclear benefit, and potential issues discussed above, it is proposed to not consider area specific MCCH.

Proposal 2: Area specific MCCH is not supported.

On-demand MCCH

There were proposals regarding on-demand MCCH to minimize the overhead of MCCH. On-demand MCCH increases latency especially in consideration of service continuity. Delivery mode 2 is similar to LTE SC-PTM, which supports MCCH modification period as short as 20 ms. Although delivery mode 2 is for “low” QoS services, it is expected that delivery mode 2 can support MCCH modification period in a similar value range as that in LTE SC-PTM. Therefore the additional delay due to on-demand MCCH can be significant in some cases.

Proposal 3: On-demand MCCH is not supported.

Single vs. multiple MCCHs

In LTE SC-PTM, there is a single SC-MCCH. There were proposals to have multiple MCCHs for NR MBS design, with the aim to have flexible configuration for different type of services. However, there are also potential drawbacks for multiple MCCH approach, e.g. more power consumption for UEs monitoring multiple MCCHs. In addition, there are increased complexity and more discussion is needed on multiple MCCH design, e.g. how UE can know which subset of MCCHs to monitor, MCCH notification, and DRX for multiple MCCH monitoring. It is therefore proposed to reuse LTE SC-PTM design of single MCCH.

Proposal 4: PTM configuration is transmitted by a single MCCH within a cell.

MCCH change notification

In RAN2#113-e meeting, following was agreed: “Assume that MCCH change notification mechanism is used to notify the changes of MCCH configuration due to session start for delivery mode 2 of NR MBS (other cases FFS, if any).” However, following is indicated in SA2 LS S2-2102077 [2]:

SA2 agreed to remove session start/stop procedures towards RAN and only have activation/inactivation procedures for multicast. The following states were defined: - Active multicast session: Established multicast session in active state. Multicast data are transmitted to UEs that joined the multicast session. 5GC resources for the multicast session are reserved. Corresponding Radio resources are reserved depending on participating UE locations. UEs that joined the multicast session are in CM CONNECTED state. UEs are allowed to join the multicast session (subject to authorization check). - Inactive multicast session: Established multicast session in inactive state. No multicast data are transmitted. UEs that joined the multicast session may be in CM CONNECTED or CM IDLE state. UEs are allowed to join the multicast session (subject to authorization check). For broadcast, only session start/stop are applicable.

To align with SA2 terminology, RAN2 agreement should be modified to “MCCH change notification mechanism is used to notify the changes of MCCH configuration due to session start (for broadcast session) or session activation (for multicast session) for delivery mode 2 of NR MBS”.

Proposal 5: MCCH change notification mechanism is used to notify the changes of MCCH configuration due to session start (for broadcast session) or session activation (for multicast session) for delivery mode 2 of NR MBS.

## MBS interest indication

In RAN2#113-e meeting, following was agreed: “Assume that MBS Interest Indication is supported for UEs in connected mode for Broadcast service (assume that as usual there is no mandatory network requirement, network action is up to network).” The reason not to consider interest indication for multicast service is that gNB is aware of the multicast sessions UE has joined based on information provided by CN.

In LTE, MBMS interest indication conveys the following information:

- The frequencies which provide the service(s) that the UE is receiving or is interested to receive simultaneously, and which can be received simultaneously in accordance with the UE capabilities;

- A single bit whether UE prioritizes MBMS reception over unicast.

LTE MBMS interest indication can be reused for NR MBS. For multicast service, although gNB is aware of the multicast sessions UE has joined, gNB is not aware of the multicast sessions UE can receive simultaneously. In multicast session procedure (TR 23.757 [1] Figure 8.2.3-1), UE might join a multicast session before gNB allocates resource for the multicast session. Therefore providing information regarding MBS frequencies UE can simultaneously receive can help gNB configure UE (e.g. CA / DC) properly taking into account UE’s preference and capabilities. In addition, the preference bit regarding MBS vs. unicast can be also helpful for gNB scheduler.

Proposal 6: As in LTE, MBS interest indication sent in RRC_CONNECTED conveys: 1) the frequencies which provide the MBS services (including both broadcast and multicast services) that the UE is receiving or is interested to receive simultaneously, and which can be received simultaneously in accordance with the UE capabilities; 2) a single bit whether UE prioritizes MBMS reception over unicast.

## Frequency prioritization

In RAN2#113-e discussion, there were two FFSes related to frequency prioritization:

⇒ FFS whether support UE awareness of MBS services on frequency basis for service continuity for NR MBS delivery mode 2 (i.e. Reuse LTE SC-PTM mechanism).

⇒ FFS Support frequency prioritization during cell reselection for service continuity for NR MBS delivery mode 2 (i.e. Reuse LTE SC-PTM mechanism).

In LTE MBMS, frequency prioritization rule is defined in TS 36.304 as follows:

If the UE is capable either of MBMS Service Continuity or of SC-PTM reception and is receiving or interested to receive an MBMS service and can only receive this MBMS service while camping on a frequency on which it is provided, the UE may consider that frequency to be the highest priority during the MBMS session TS 36.300 [2] as long as the two following conditions are fulfilled: …

The frequency prioritization rule is helpful for UE to receive MBS service in RRC_IDLE/INACTIVE and it is preferred to be supported in NR as well. To facilitate the frequency prioritization, a SIB similar to LTE SIB15 can be introduced to provide MBS service information.

Proposal 7: As in LTE SC-PTM, UE is aware of MBS services on frequency basis for service continuity for NR MBS delivery mode 2 (i.e. reuse LTE SC-PTM mechanism), and frequency prioritization during cell reselection for service continuity is supported for NR MBS delivery mode 2.

# Conclusion

In this contribution, we discuss MBS support for delivery mode 2, and have the following observations:

Observation 1: From SA2 perspective, UEs receiving multicast data should be in CM-CONNECTED state.

We propose the following:

Proposal 1: Multicast session can be provisioned over delivery mode 2, and can be received in RRC_INACTIVE state.

Proposal 2: Area specific MCCH is not supported.

Proposal 3: On-demand MCCH is not supported.

Proposal 4: PTM configuration is transmitted by a single MCCH within a cell.

Proposal 5: MCCH change notification mechanism is used to notify the changes of MCCH configuration due to session start (for broadcast session) or session activation (for multicast session) for delivery mode 2 of NR MBS.

Proposal 6: As in LTE, MBS interest indication sent in RRC_CONNECTED conveys: 1) the frequencies which provide the MBS services (including both broadcast and multicast services) that the UE is receiving or is interested to receive simultaneously, and which can be received simultaneously in accordance with the UE capabilities; 2) a single bit whether UE prioritizes MBMS reception over unicast.

Proposal 7: As in LTE SC-PTM, UE is aware of MBS services on frequency basis for service continuity for NR MBS delivery mode 2 (i.e. reuse LTE SC-PTM mechanism), and frequency prioritization during cell reselection for service continuity is supported for NR MBS delivery mode 2.

# References

[1] 3GPP TR 23.757, "Study on architectural enhancement for 5G multicast-broadcast services"

[2] S2-2102077, SA2 LS, "Reply LS on 5MBS progress and issues to address"