3GPP TSG-RAN WG2 Meeting #113bis-e                  	R2-2104171

E-Meeting, April 12 – April 20, 2021

Source:	Xiaomi

Title:	Discussion of the paging cause support for MUSIM

Agenda Item:	8.3.4

Document for:	Discussion and Decision

- Introduction

During the RAN#91e meeting, MUSIM SID was further updated in RP-210316 [1] described with the following objectives:

- Specify, if necessary, enhancement(s) to address the collision due to reception of paging when the UE is in IDLE/INACTIVE mode in both the networks associated with respective SIMs [RAN2] - RAT Concurrency: Network A can be NR or LTE. Network B can either be LTE or NR. - Applicable UE architecture: Single-Rx/Single-Tx. - Specify mechanism for UE to notify Network A of its switch from Network A (for MUSIM purpose) [RAN2]: - RAT Concurrency: Network A is NR. Network B can either be LTE or NR. - Applicable UE architecture: Single-Rx/Single-Tx, Dual-Rx/Single-Tx - Unless SA2 find an alternative solution or decides otherwise, specify mechanism for an incoming page to indicate to the UE whether the service is voLTE/VoNR. [ RAN2] - RAT Concurrency: Network A is either LTE or NR. Network B is either LTE or NR. - Applicable UE architecture: Single-Rx/Dual-Rx/Single-Tx

In this paper, we discuss the problem indicated in the objective 3 based on the latest progress of the SA2 MUSIM project and make proposals accordingly.

- Discussion

SA approved a study about MUSIM in SP-200091 [2], and the conclusion about the study item is included in section 8 in TR 23.761 [3]. The key issue 1 listed in TR 23.761 is related to the paging cause issue:

- Key Issue 1: Handling of Mobile Terminated service with Multi-USIM device

The latest conclusions about KI#1 in TR 23.761 are:

8.2	Conclusions for Key Issue #2: Enabling Paging Reception for Multi-USIM Device

The following conclusions are agreed for the baseline functionality:

-	For both EPS and 5GS, only the voice is supported to be indicated as the paging cause to the UE. The indicated Paging Cause as voice can be determined based on the PPI related to voice, or APN and DSCP value related to voice.

-	The network, applies the Paging Cause only for UEs that have provided indication they operate in MUSIM mode.

-	The Paging Cause Solution shall ensure that a UE can detect when the feature is not supported in the cell where it is camping (either because the RAN or CN does not support the feature).NOTE 3:	"voice" refers to MMTel voice (5GS and EPS) and CS domain voice (EPS only).

-	If Multi-USIM device received paging by Network-A in RRC_Idle mode and the device decides to accept the paging, UE shall perform as existing procedure (send the Service Request message).

-	If Multi-USIM device received paging by Network-A in RRC_Idle mode and the device decides not to accept the paging, a UE supporting NAS BUSY indication attempts to send a BUSY Indication via the SERVICE REQUEST message to network unless it is unable to do so e.g. due to UE implementation constraints.

-	As part of the SERVICE REQUEST message carrying the BUSY indication the UE may provide assistance information to the network regarding MT data/signalling handling, as described in clause 8.3.

-	The UE MMI shall not require input from the user in order for the UE to decide whether to respond to paging.

As we can see from the conclusion from SA2 about paging cause problem, only the voice is supported to be indicated as the paging cause to the UE for both EPS and 5GS.

Also according to the last SA MUSIM WID in SP-210091 [4] in the SA#91e plenary meeting, the objectives are:

The objective is to specify enhancements to 5GS and EPS functionality and procedures for the following aspects:

-	A single paging cause with the meaning of “voice” for both EPS and 5GS according to the conclusions in TR 23.761 clause 8.1.

-	NAS Busy Indication for both EPS and 5GS in RRC_IDLE mode according to the conclusions in TR 23.761 clause 8.1.

-	Enabling paging reception for EPS according to the conclusions in TR 23.761 clause 8.2.

-	NAS-level leaving procedure for EPS according to the conclusions in TR 23.761 clause 8.3.

-	Leaving procedure for 5GS.

NOTE 1:	The decision to support NAS-based leaving or RRC-based leaving or both will be determined taking into account feedback from RAN2.

NOTE 2:	The WID objectives will be further updated based on progress of work in other WGs and on open issues identified in the SA2.

Clearly SA2 will try to specify enhancements to 5GS and EPS functionality and procedures to enable a single paging cause with the meaning of “voice” according to the conclusions in TR 23.761 clause 8.1.

Proposal 1: Start the work in RAN2 now to introduce a single paging cause with the meaning of “voice” in the paging message.

- Conclusion

This paper discusses the problem indicated in the objective 3 and proposes:

Proposal 1: Start the work in RAN2 now to introduce a single paging cause with the meaning of “voice” in the paging message.

- References

[1] RP-210316 Revised WID: Core part: Support for Multi-SIM devices for LTE/NR

[2] SP-200091 Revised SID: Study on system enablers for multi-SIM devices.

[3] TR 23.761 V1.2.0 Study on system enablers for devices having multiple Universal Subscriber Identity Modules (USIM).

[4] SP-210091 Revised WID on system enablers for Multi-USIM devices.