3GPP TSG-RAN WG2 Meeting #113bis-e                          R2-2104174

Online, April 12 – April 20 2021

Agenda item:	8.3.3

Source:	Samsung

Title:	Open issues on network switching for Multi-USIM devices

Document for:	Discussion & Decision

Introduction

RAN2 made some progress on network switching scenarios and their procedures for multi-USIM devices, but actual signalling details are still FFS. This contribution discusses the remaining open issues.

Discussion

Long-time switching procedure

At RAN2#113-e meeting, RAN2 reached the following agreement on long-time switching procedure:

- Agreements

- 1	Switching procedure can be used to notify network A that the UE has a preference to leave RRC_CONNECTED state in network A.

It is FFS whether RRC or NAS signalling to be used for long-time switching procedure. In [1], SA2 agreed to introduce NAS based leaving procedure for EPS i.e. UE can provide a leaving/release request indication and paging restriction information via TAU request or Service request message.

Observation 1: SA2 agreed to introduce NAS based leaving procedure for EPS i.e. UE can provide a leaving/release request indication and paging restriction information via TAU request or Service request message.

It seems not desirable to support separate long-time switching procedures for EPC and 5GS. It only entails specification complexity with marginal benefits for the same purpose. One can say that RRC signalling enables to include additional UE assistance information e.g. preferred RRC state and achieve low latency compared with NAS based solution. But, preferred RRC state is not essential information and if needed, it can be also included in NAS message. In addition, CN needs to be informed anyway so that latency is not a critical issue. Thus, it is proposed to support long-time switching procedure for 5GS via NAS signalling only.

Proposal 1: For 5GS, only NAS signalling is used to support the switching procedure to leave RRC_CONNECTED state in network A.

Short-time switching procedure

At RAN2#113-e meeting, RAN2 reached the following agreement on short-time switching procedure:

- Agreements

- 2	The switching procedure can be used to notify network A that the UE has a preference to be kept in RRC_CONNECTED state in network A while temporarily switching to network B.

Unlike long-time switching procedure, CN does not need to be involved for short-time switching procedure. Thus, it is proposed to support short-time switching via AS signalling.

Proposal 2: AS signalling is used to support the switching procedure to be kept in RRC_CONNECTED state in network A while temporarily switching to network B.

Network A has no idea when and how long UE switches to network B as there is no coordination between two networks involved. A switching gap configuration should be negotiated between network A and UE. Existing measurement gap configuration allows a single periodic measurement gap pattern either per FR or per UE solely designed for network A and its value range is quite limited. It is cleaner to introduce new switching gap configuration for multi-USIM purpose to handle any kind of activities on network B.

Proposal 3: New switching gap configuration is introduced to support the switching procedure to be kept in RRC_CONNECTED state in network A while temporarily switching to network B.

Each required activity on network B can be in one-shot (e.g. RNAU, SI request etc) or with some periodicities (e.g. paging monitoring, serving cell/neighbour cell measurements). In order to handle any kind of activities on network B, a generic and common switching gap configuration should be considered. Thus, it is desirable that UE requests its preferred short time gap pattern(s) to network A, and network A (re-)configures it accordingly. In addition, some configured switching gaps may not be needed any more as required activities in network B may change over time. Release of configured switching gap(s) requested by UE should be supported.

Proposal 4: UE requests its preferred short time gap pattern(s) to network A for performing certain activities in network B, and network A (re-)configures switching gap(s) accordingly. Release of configured switching gap(s) can be requested to network A by UE.

Determination for short-time and long-time switching procedure

It is pertinent question as to what activities on network B be determined for short-time and long-time switching procedures by the UE. It won’t be a good idea to have diversity in UE implementations with this regard lest it should hamper network scheduling efficiency and operation in general. An efficient and deterministic approach can be that the network configures a gap duration threshold as a reference for the UEs to determine activities either as short-time switching or long-time switching e.g. activities that may be performed in less than gap duration threshold, are considered for short-time switching procedures.

Proposal 5: Network can configure a gap duration threshold (e.g. a max gap duration for short time switching gap configuration) as a reference for the UE to determine activities as short-time switching or long-time switching.

Busy indication in RRC_INACTIVE state

At RAN2#112-e meeting, RAN2 reached the following agreement on busy indication:

From RAN2 point of view, it is feasible that the busy indication is sent as an RRC message with security for RRC_INACTIVE. FFS how this works.

Note that SA2 [2] agreed to support busy indication in RRC_IDLE state.

Observation 2: SA2 agreed to support busy indication in RRC_IDLE state.

For RRC_INACTIVE state, it was left up to RAN whether/how to support busy indication. We think that busy indication can be supported for RRC_INACTIVE state as well.

Proposal 6: Busy indication is supported in RRC_INACTIVE state.

It is desirable to send busy indication on other network as much as possible to minimize the on-going service interruption in current network. As UE calculates resumeMAC-I using the integrity algorithm (NIA or EIA) in the stored AS context and includes it in RRCResumeRequest or RRCResumeRequest1, there seems no security issue even though its message is not integrity/ciphering protected. It is more or less the same as RNA update except that new resume cause for indicating busy indication is used.

Proposal 7: Introduce new resume cause "busy indication" and RRCResumeRequest or RRCResumeRequest1 is used to support busy indication in RRC_INACTIVE state.

Open issue on network switching scenario

At RAN2#112-e meeting, the following agreements were made:

Agreements

2: The Sub-Case 3-2, i.e. Dual-RX/Single-TX UE stays in RRC_CONNECTED mode in NW A while performing reception and transmission in NW B(in RRC_ CONNECTED or during RRC setup/resume period ), is not considered in the WI from RAN2 viewpoint. Scheduling gap is not excluded.

It is not clear whether UE is allowed to stay in RRC_CONNECTED state in network A while performing reception and transmission in network B during configured switching gaps. As busy indication is supported in RRC_IDLE state, we think it is beneficial to perform RRC connection establishment/resume procedure in network B during configured switching gaps. Otherwise, UE needs to initiate the switching procedure to leave RRC_CONNECTED state in network A for merely sending a busy indication to network B.

Proposal 8: RAN2 to discuss whether UE can perform transmission and reception in network B during configured short time gaps while staying in RRC_CONNECTED state in network A, regardless of applicable UE architecture (e.g. Single-Rx/Single-Tx, Dual-Rx/Single-Tx).

Conclusion

In section 2, we made the following observations:

Observation 1: SA2 agreed to introduce NAS based leaving procedure for EPS i.e. UE can provide a leaving/release request indication and paging restriction information via TAU request or Service request message.

Observation 2: SA2 agreed to support busy indication in RRC_IDLE state.

Based on the above, RAN2 is requested to discuss and if possible agree on the following proposals:

Proposal 1: For 5GS, only NAS signalling is used to support the switching procedure to leave RRC_CONNECTED state in network A.

Proposal 2: AS signalling is used to support the switching procedure to be kept in RRC_CONNECTED state in network A while temporarily switching to network B.

Proposal 3: New switching gap configuration is introduced to support the switching procedure to be kept in RRC_CONNECTED state in network A while temporarily switching to network B.

Proposal 4: UE requests its preferred short time gap pattern(s) to network A for performing certain activities in network B, and network A (re-)configures switching gap(s) accordingly. Release of configured switching gap(s) can be requested to network A by UE.

Proposal 5: Network can configure a gap duration threshold (e.g. a max gap duration for short time switching gap configuration) as a reference for the UE to determine activities as short-time switching or long-time switching.

Proposal 6: Busy indication is supported in RRC_INACTIVE state.

Proposal 7: Introduce new resume cause "busy indication" and RRCResumeRequest or RRCResumeRequest1 is used to support busy indication in RRC_INACTIVE state.

Proposal 8: RAN2 to discuss whether UE can perform transmission and reception in network B during configured short time gaps while staying in RRC_CONNECTED state in network A, regardless of applicable UE architecture (e.g. Single-Rx/Single-Tx, Dual-Rx/Single-Tx).

References

[1] S2-2101102, "Introduction of Leaving procedure – SR based."

[2] TR 23.761, "Study on system enablers for devices having multiple Universal Subscriber Identity Modules (USIM)."