3GPP TSG-RAN WG2 Meeting #113bis-e	R2-2104231

Electronic Meeting, April 12th – 20th, 2021

Agenda item:	8.2.2.3

Source: 	Intel Corporation

Title: 	Considerations on reactivating SCG

Document for:	Discussion and Decision

# Introduction

One important aspect of this WI is to reduce delay during SCG state transition. Otherwise, there would be not much benefit compared to the Rel-16 SCG release/add feature. In general, there is a trade-off between delay and power saving and of course power saving is also an important aspect, but we already developed an ultimate solution for power saving, i.e. Rel-16 SCG release/add. In this Rel-17 WI, it is better to focus reducing delay while activating or deactivating SCG.

With respect to NW activating a deactivated SCG, the last RAN2-113 made some good progress based on email discussion [1] (relevant parts highlighted):

- 1	NW-triggered SCG activation is indicated to the UE via the MCG.

- 9	NW-triggered SCG deactivation can be indicated to the UE via the MCG. FFS via SCG.

- 2	The UE behaviour when the SCG activation is indicated to the UE via the MCG is one or more of the following options:

- option 1)	similar to reconfiguration with sync, i.e. the UE always initiates random access to the PSCell.

- option 2)	in certain cases:

- -	the UE does not initiate random access and monitors PDCCH on the PSCell (at the latest after the specified processing time).

- -	the SCG can schedule data transmission on the PDCCH

- The UE decides not to perform random access (one option to be selected):

- option 2a) if the TA timer is still running and possibly other conditions (FFS how TAT starts)

- option 2b) based on the contents of the SCG activation indication

- FFS for option 2a): in the SCG deactivated state, the UE monitors some DL beams (FFS if the same as BFD or RLM) and, if the UE sees that the beams are not good enough (details FFS), the UE either (one of the options to be selected):

- -	will perform random access upon reception of the next SCG activation indication from the MCG

- -	reports measurement results (details FFS) via the MCG and wait for reconfiguration.

- 7	Further discuss the format and content of the SCG activation indication from the MCG to the UE after there is more progress on solution 2.

- 5	Continue to discuss whether some kind of beam monitoring (similar to RLM/BFD) should be supported when the SCG is deactivated. FFS if this only applies to when TAT is running.

- 6	Clarify the meaning of "the UE maintains DL sync while the SCG is deactivated" (e.g. whether that is a consequence of doing RRM measurements of the PSCell or something more is needed).

- 8	Further discuss the comparison between

- -  	define a mechanism for SCG activation upon UL data arrival on SCG bearers

- -	use split bearer with primary path on MCG (network sees UL data and can initiate activation)

- 11	It is FFS whether the UE can provide some assistance information for deactivation of the SCG (but there is no proposal so far).

FFS if in absence of PDCCH monitoring and UL transmission, and it is possible to assume that TA is valid when the TA timer has not expired.

- 1a 	SCG activation can be requested by MN/SN/UE. FFS on how to accept/reject the procedure. FFS which signalling is used.

- 1b 	SCG deactivation can be requested by MN/SN. FFS whether UE can request deactivation. FFS on how to accept/reject the procedure. FFS which signalling is used.

- 3 	RRC signalling is defined for the interaction between UE/MN and MN/SN in SCG activation/deactivation. FFS if lower-layer signalling is needed.

In this contribution, we share our views on some agreements/FFSes related to the UE behaviour when SCG is activated by NW.

# Discussion

When the UE receives an indication to reactivate SCG, the UE should be able to monitor PDCCH to start data activities on PSCell. From the email discussion [1], there are several aspects we need to consider:

- Beam quality is an important factor, since poor beam quality may result in the UE missing some PDCCHs and aggravate the activation.

- TA (timing alignment) validity is also an important factor. Since there is no PDCCH monitoring while SCG is deactivated, TA cannot be adjusted by MAC CE. And no UL transmission is possible while SCG is deactivated for NW to evaluate TA validity. TAT (TA timer) is to guarantee the validity of TA (which can be up to 10sec considering DRX cycle). If TAT was expired while deactivated, the UE should initiate RACH upon receiving activation indication as in legacy.

- Regarding RACH, NW should be able to configure the UE to initiate RACH upon receiving activation indication. RACH is the main source of delay and there can be some criterions to skip RACH when reactivating SCG, but at least it cannot be solely left up to the UE to decide.

From the above observations, first, we believe that the RAN2 agreement selecting either the option 2a or the option 2b should be re-visited. Upon receiving SCG activation indication, the UE should be able to perform RACH if TAT has been expired. The UE should also be able to perform RACH if NW indicates the UE to do so via SCG activation indication. Both should be allowed upon receiving SCG activation indication.

Proposal 1: The RAN2 agreement selecting either option 2a or option 2b should be re-visited. Upon receiving SCG activation indication, the UE should be able to perform RACH if TAT has been expired or NW configures the UE to do so.

In case TAT has not been expired nor running, there is a room to skip RACH and reduce activation delay. In this case, beam quality becomes the primary factor, since, as mentioned above, poor beam quality may result in the UE missing some PDCCHs and aggravate the activation. The UE should be able to check whether beams are good enough to start monitoring PDCCH.

The UE may use beam information given by RRC (SCG activation) signalling, which could be possible if NW configured periodic RRC measurements with beam information. Another possibility could be by Beam Failure Detection (BFD) mechanism. If BFD is allowed while SCG is deactivated, then the UE measures beam quality, and upon beam failure detection, the UE may declare new good beams via BFR MAC CE [2] through MCG, for which NW can use to configure to the UE when activating SCG.

In a nutshell, we believe that measuring beam quality at least on PSCell while SCG is deactivated (e.g. via RRC configured measurements) is beneficial to skip RACH and quickly allow the UE to monitor PDCCH.

Proposal 2: Measuring beam quality at least on PSCell while SCG is deactivated (e.g. via RRC configured measurements) is beneficial to identify the desirable beam(s) upon receiving SCG activation indication to start monitoring PDCCH without RACH.

However, in legacy, beam failure detection (BFD) triggers beam failure recovery procedure (BFR) which involves RACH for SpCell [3], for which there is no need to initiate while SCG is deactivated and TAT has not been expired (nor running). If beam measurement is allowed on PSCell while SCG is deactivated, RACH should not be initiated upon BFD. Only BFR MAC CE (including candidate beams) is useful, but this needs to be transferred through MCG.

Proposal 3: If RAN2 agrees to allow beam measurements while SCG is deactivated, do not initiate RACH upon beam failure detection while SCG is deactivated. Only BFR MAC CE (including candidate beams) is useful through MCG.

Moreover, in case beams are not good enough when receiving SCG activation indication, the current RAN2 agreement seeks for choosing one of the following options:

  - The UE performs RACH upon reception of the next SCG activation indication from MCG

  - The UE reports measurement results (details FFS) via MCG and wait for reconfiguration.

However, we think two options are not mutually exclusive. First, the first option is not clear when the next SCG activation indication would happen for RACH and based on what.

In addition, given that beams are turned out not good enough to monitor PDCCH, calibrating beams again (e.g. based on measurement report and RRC reconfiguration as in the second option through MCG) could take more time than simply triggering RACH over SCG. As proposed above, beam management at least on PSCell while SCG is deactivated is beneficial to identify the desirable beam(s) upon receiving SCG activation indication to start monitoring PDCCH without RACH. If all these efforts of maintaining good beams while SCG was deactivated turned out to be failed and not enough to monitor PDCCH upon receiving SCG activation indication, then it could be better to do RACH over SCG directly, rather than trying to recalibrate beams again through MCG (involving MN-SN interaction). And NW can configure contention-free RA resources when sending SCG activation indication, to be used for the case when beam(s) turns out not good enough to monitor PDCCH, thus making RACH over SCG swift. If so, the UE may not have to wait for the next SCG activation indication to perform RACH as in the first option.

Proposal 4: When receiving SCG activation indication, if beams are turned out not good enough to monitor PDCCH, the UE should be able to perform RACH immediately over SCG without wating for the next SCG activation indication.

# Conclusion

Based on the discussion in the present contribution and the observations above we propose:

Proposal 1: The RAN2 agreement selecting either option 2a or option 2b should be re-visited. Upon receiving SCG activation indication, the UE should be able to perform RACH if TAT has been expired or NW configures the UE to do so.

Proposal 2: Measuring beam quality at least on PSCell while SCG is deactivated (e.g. via RRC configured measurements) is beneficial to identify the desirable beam(s) upon receiving SCG activation indication to start monitoring PDCCH without RACH.

Proposal 3: If RAN2 agrees to allow beam measurements while SCG is deactivated, do not initiate RACH upon beam failure detection while SCG is deactivated. Only BFR MAC CE (including candidate beams) is useful through MCG.

Proposal 4: When receiving SCG activation indication, if beams are turned out not good enough to monitor PDCCH, the UE should be able to perform RACH immediately over SCG without wating for the next SCG activation indication.

# Reference

[1] R2-2101966, “[AT113-e][230][eDCCA] Solution alternatives for SCG activation and deactivation” Huawei

[2] TS 38.321, “NR; Medium Access Control (MAC) protocol specification”, RAN2

[3] TS 38.300, “NR; NR and NG-RAN Overall Description; Stage 2”, RAN2