3GPP TSG-RAN WG2 Meeting #113bis-e	R2-2104309

e-Meeting, 12 Apr - 20 Apr 2021

Agenda item:	6.1.3.1

Source:	Apple

Title:	Treatment of overlapping SR/Data

Document for:	Discussion / Decision

# Introduction

In RAN2#112e, RAN2 discussed the treatment of SR and Data for the case when the priority of the logical channel that triggered SR is higher than the priority of the uplink grant, and the SR and the UL grant are of the same L1 priority [3]. RAN2 sent an LS to RAN1 to confirm the intended behavior [1]. RAN1 provided in R1-2102244 [2] a Reply LS for overlapping SR/Data, which will be discussed by RAN2 in RAN2#113bis-e.

In connection with the RAN1 understanding in [2], this contribution looks at overlapping SR/Data while considering what happens when a) MAC does not know the result of UCI multiplexing or b) MAC would know the result of the UCI multiplexing in PHY.

Our companion paper in [6] contains an analysis of overlapping SR/Data cases identified by RAN1 in [2].  This paper provides more detail on aspects related with UCI multiplexing with a view to a potential enhancement of the interface to the physical layer. Note that our RAN1 contribution in [7] contains further information from a physical layer perspective.

# Background

MAC procedures for handling a Scheduling Request (SR) overlapping with an UL grant are specified in sections 5.4.1 and 5.4.4 of TS 38.321. We assume that MAC knows the RRC configured PUCCH resource for SR while the final PUCCH resource (selected after completion of the UCI multiplexing in the PHY layer) is not currently known to MAC. This is related to the following part in the MAC spec.

TS 38.321, sub-clause 5.4.4

The MAC entity may be configured with zero, one, or more SR configurations. An SR configuration consists of a set of PUCCH resources for SR across different BWPs and cells.

…

3>	if the PUCCH resource for the SR transmission occasion overlaps with neither a UL-SCH resource nor an SL-SCH resource; or   …

3>	if the MAC entity is configured with lch-basedPrioritization, and the PUCCH resource for the SR transmission occasion does not overlap with the PUSCH duration of an uplink grant received in a Random Access Response or with the PUSCH duration of an uplink grant addressed to Temporary C-RNTI or with the PUSCH duration of a MSGA payload, and the PUCCH resource for the SR transmission occasion for the pending SR triggered as specfied in clause 5.4.5 overlaps with any other UL-SCH resource(s), and the physical layer can signal the SR on one valid PUCCH resource for SR, and the priority of the logical channel that triggered SR is higher than the priority of the uplink grant(s) for any UL-SCH resource(s) where the uplink grant was not already de-prioritized, and the priority of the uplink grant is determined as specified in clause 5.4.1; or

# Discussion

As per the LS from RAN1 in [2] and following the analysis in [6], there are two different paths of understanding as to how much information is available to MAC about the outcome of UCI multiplexing.

- Option A: MAC is not aware of the final result of the UCI multiplexing in PHY. This is supported by the current MAC spec.

- Option B: MAC is aware of the final result of the UCI multiplexing in PHY. This would require that MAC is (made) aware of the final PUCCH resource from PHY.

We would like to explore if interfaces could be enhanced to align the result of the UCI multiplexing in PHY with the selection of SR vs Data in MAC. Especially in the case when both UL skipping and lch-basedPrioritization are enabled, we think that a slightly updated behavior can be beneficial.

Let’s first recap the current procedure. For overlapping SR/PUSCH, when MAC prioritizes SR transmission the UE adheres to the following behavior.

For SR/PUSCH overlapping in time and when SR has higher LCH priority, if MAC has not yet delivered a MAC PDU for the PUSCH to PHY, MAC instructs the physical layer to only signal SR. Otherwise, if a MAC PDU has already been delivered to PHY, MAC still signals SR and PHY drops either the PUSCH or SR.

In case MAC delivered both SR and PUSCH to PHY, the dropping behavior is based on L1 priority. If the L1 priority of SR and PUSCH is different and the UE supports intra-UE multiplexing/prioritization of overlapping PUCCH/PUSCH with two priority levels in the physical layer (UE capability ul-IntraUE-Mux-r16), PHY will drop the transmission with the lower L1 priority level. Otherwise, or if SR and PUSCH have the same L1 priority and overlap in time, the PHY behavior is defined as SR is dropped and PUSCH is transmitted. This is following Rel-15 behavior.

The only exception is for PUSCH without UL-SCH where according to TS 38.213, the PUSCH is dropped. This is because a PUSCH without UL-SCH is transparent to MAC and such a PUSCH can never include BSR. Therefore, SR is given a higher precedence in the PHY procedures here.

Observation 1: If MAC delivers both SR and PUSCH and they overlap in time, PHY drops either SR or PUSCH. For same L1 priority, as PHY does not have a notion of LCH priority, it can happen that the transmission of higher LCH priority gets dropped by PHY. This is not intended by the RAN2 procedures.

Proposal 1: For SR and PUSCH of equal L1 priority overlapping in time, in order to facilitate an ability to drop the transmission of lower LCH priority, MAC delivers to the physical layer only SR or PUSCH but not both.

Table 2 summarizes the overlapping SR/Data cases from two different angles in accordance with the perspectives taken by RAN1 in [2]. We list some key aspects for consideration and try to explore the benefits of each of the options. Option A is based on the understanding that MAC is not aware of the UCI multiplexing in PHY, as per the MAC behavior currently specified in Rel-16. Option B looks at what might happen if MAC had insight into the outcome of the UCI multiplexing in PHY.

Table 2: Summary of overlapping SR/Data options

| Cases identified by RAN1 | Option A (current MAC) MAC is not aware of the UCI multiplexing in PHY | Option B (enhancement) MAC is made aware of the UCI multiplexing in PHY |
| --- | --- | --- |
| Case 1: only SR overlaps with PUSCH of equal L1 priority | Both MAC and PHY select the same PUCCH resource, therefore, there is no need for MAC to know the outcome of the UCI multiplexing in PHY. | There should not be any difference between RRC configured valid SR PUCCH resource and final PUCCH resource for SR. PUCCH transmission timing can assist MAC to select either PUSCH or SR. |
| Case 2-1: the final PUCCH resource after UCI multiplexing does not overlap with PUSCH | Current Rel-16 MAC layer behavior aligns with RAN1 understanding 1. Either SR or PUSCH is left out although PHY can transmit both. MAC delives either SR or PUSCH, however, as the final PUCCH resource does not have an overlap, both SR and PUSCH could be sent, but one of them is unnecessarily dropped (or not considered). | Both SR and PUSCH can be transmitted. Pro: MAC is able to identify there is no overlap, hence both SR and PUSCH can be delivered to PHY. Con: Complexity to be evaluated. Note there is going to be a different UCI multiplexing outcome depending on whether the SR is positive or negative. This affects both UL skipping and SR. Case 2-1(a): If SR is positive as in the figure then there is no overlap. But if SR is negative, AN/CSI comes as part of PUSCH. Case 2-1(b): If SR is positive as in the figure then there is no overlap. And if SR is negative, assuming sufficient timeline AN/CSI got transmitted earlier and it is not part of the PUSCH either. |
| Case 2-2: the final PUCCH resource after UCI multiplexing overlaps with PUSCH | Current Rel-16 MAC layer behavior aligns with RAN1 understanding 2. MAC selects either SR or PUSCH. Con: MAC delivers either SR or PUSCH while in case 2-2(b), if SR is not present, PHY may actually transmit AN/CSI and PUSCH separately. If MAC sends both a MAC PDU and SR while expecting SR to be prioritized due to LCH based prioritization, SR gets dropped in PHY according to Rel-15 behavior, which applies for equal L1 priorities. | MAC knowledge of the final PUCCH resource may not lead to a different outcome for the selection of SR vs PUSCH in MAC, as anyway either SR or PUSCH can be sent. If the final PUCCH resource was known in advance, may, in some cases and when SR is negative, allow AN/CSI over a separate PUCCH even before the PUSCH, however there is a timing constraint related with the evaluation of SR status. |
| Case 3: other UCI(s) overlaps with a PUSCH, SR overlaps with the PUSCH, SR does not overlap with other UCI(s) | Current Rel-16 MAC layer behavior aligns with RAN1 understanding 2. MAC selects either SR or PUSCH. | No immediate advantage, since the final PUCCH resource does not change. However, extra knowledge can assist MAC in taking the decision. MAC selects either SR or PUSCH. |
| Case 4: other UCI(s) overlaps with SR of an equal L1 priority, but SR does not overlap with the PUSCH of an equal L1 priority | Current Rel-16 MAC layer behavior aligns with RAN1 understanding 1. MAC assumes both SR and PUSCH can be transmitted by the physical layer. In reality, PHY multiplexes SR with AN/CSI and selects either the transmission with SR or the PUSCH. Con: PHY may happen to drop the transmission of higher LCH priority, which is not intended from RAN2 perspective. | Pro: Complexity reduction. If the final PUCCH resource was known to MAC, either SR or PUSCH can be delivered. The transmission with the higher LCH priority can be selected for transmission, no accidential dropping. |

As can be seen from the table above, when lch-basedPrioritization is configured and assuming MAC is unaware of the final PUCCH resource following UCI multiplexing in PHY, the UE may end up with unnecessary SR/PUSCH dropping (when the final PUCCH isn’t overlapping with PUSCH) or unexpected SR dropping (when the final PUCCH is overlapping with PUSCH).

Observation 2: When MAC is unaware of the final PUCCH resource the UE may end up with unnecessary SR/PUSCH dropping or unexpected SR dropping.

The MAC layer is required to instruct the physical layer to signal SR on a valid PUCCH resource. In our understanding, ‘valid PUCCH resource’ refers the RRC configured PUCCH resource for SR, as this is the only PUCCH resource that is known to MAC. If as a result of the UCI multiplexing in PHY the final SR-PUCCH resource used by the physical layer changes, then either MAC cannot signal the SR according to the blue text (as quoted in section 2 above) or MAC signals the SR on a wrong PUCCH resource. Obviously neither of these two interpretations reflects the intented behavior.

Proposal 2: RAN2 to discuss the understanding of ‘valid PUCCH resource’ in TS 38.321 with respect to its relation to the final PUCCH after UCI multiplexing in PHY.

Even though MAC does not consider the L1 priority as part of its process to select SR or PUSCH, the MAC layer could still gain insight into how PHY resolves the UCI multiplexing for overlapping SR/PUSCH. One way to achieve this is to provide MAC with the final PUCCH resource Z. Then if the outcome of UCI multiplexing is known to Layer2 upfront, MAC can provide either SR or PUSCH to PHY and not both, which allows for complexity reduction. This especially applies when UL skipping is configured together with lch-basedPrioritization.

As shown in our RAN1 contribution in [7], the combination of UCI multiplexing and UL skipping can be complicated, and a few additional input variables need to be considered. To reduce the amount of uncertainties between PHY and MAC for UCI multiplexing, SR/PUSCH and UL skipping decisions, MAC can provide its SR status to PHY beforehand and receive an UCI indication in return. This approach is based on the solution in section 5 of [7]. The solution involves a form of a ‘provisional' or ‘early’ SR status indication from MAC to PHY, e.g., when SR is triggered after BSR as a first step. The real instruction to signal SR still comes later, after LCH based prioritization, as already present in the current specification (in a final step).

More specifically, MAC may send an “early” SR status to PHY (e.g., right after a scheduling request is triggered) indicating its intention to send a positive SR. The PHY layer uses the SR status indication as an input to calculate the final PUCCH, which is then provided to MAC in a UCI indication. The UCI indication may describe not only the possibility to send SR but can also indicate the presence of HARQ-ACK/CSI in the final PUCCH resource Z. In a next step, MAC uses the UCI indication as input to LCH-based prioritization and UL skipping. If, as a result of LCH-based prioritization a MAC PDU is created then SR is not selected and the PHY proceeds with the PUSCH transmission, which can include multiplexed UCI for HARQ-ACK/CSI. Alternatively, MAC may choose to prioritize SR over PUSCH. Note, that the actual instruction to signal SR (or to trigger an UL transmission for PUSCH) only happens after MAC has been provided with the UCI indication (for example, in a grant indication).

In summary, this enhancement can enable better use of radio resources and reduce complexity when UL skipping and intra-UE prioritization are both required.

Proposal 3: The UE utilizes additional information from PHY which assists MAC in the SR vs PUSCH selection process. Whether or not MAC can instruct the physical layer to signal SR is guided by a new UCI indication from PHY to MAC that provides timing and UCI contents (SR, HARQ-ACK, CSI) based on the final PUCCH resource Z selected during UCI multiplexing in PHY.

Proposal 4: To assist UCI multiplexing when UL skipping is configured, MAC provides an early SR status indication to PHY.

The functionality to handle both UL skipping and intra-UE prioritization through an early SR status and a UCI indication can be part of a new UE capability. Secondly, in order efficiently utilize PUCCH configurations with no dependency on the SR status, such as in solution 1 of [7] or when SR-PUCCH is configured non-overlapping with other PUCCHs, the gNB should be able to configure (enable or disable) the early SR status indication through an explicit RRC parameter. When UL skipping is configured, it is also conceivable to split the support of intra-UE prioritization for data/data and SR/data.

We note that for overlapping data/data the detailed handling should be treated as part of the ongoing discussion for LCH-based prioritization and UL skipping, which is also addressed in other contributions [8]. But a key point is that the UE can utilize the UCI indication to support simultaneous operation of UL skipping and LCH-based prioritization. For overlapping SR/Data with UL skipping, a new capability could be defined.

There are two main directions:

- Support of a UCI indication only

- Support of a UCI indication together with an early SR status indictation

Proposal 5: RAN2 to discuss the splitting of SR and DATA capability for UL skipping enhancement.

P5 Alt 1> The combination of LCH-based prioritization and UL skipping is supported through separate capabilities for SR/data and data/data.

- UE capability 1: Support LCH-based prioritization between overlapping data/data when UL skipping is configured. The configuration is enabled through a new RRC parameter.

- UE capability 2: Support LCH-based prioritization between overlapping SR/PUSCH when UL skipping is configured. The configuration is enabled through a new RRC parameter.

P5 Alt 2> The combination of LCH-based prioritization and UL skipping supported through sub-features for UCI indication and SR status indication.

- UE capability 1: Support a new UCI indication to provide timing and UCI contents (SR, HARQ-ACK, CSI) of the final PUCCH from PHY to MAC. The configuration is enabled through a new RRC parameter.

- UE capability 2: Support of an early SR status indictation from MAC to PHY. The configuration is enabled through a new RRC parameter. (And the UCI indication is a prerequisite.)

P5 Alt 3> UL skipping and intra-UE prioritization are not supported to be configured at the same time.

# Conclusions

This paper discusses aspects related with overlapping SR/Data and proposes a potential enhancement. We have the following observations and proposals:

Observation 1: If MAC delivers both SR and PUSCH and they overlap in time, PHY drops either SR or PUSCH. For same L1 priority, as PHY does not have a notion of LCH priority, it can happen that the transmission of higher LCH priority gets dropped. This is not intended by the RAN2 procedures.

Proposal 1: For SR and PUSCH of equal L1 priority overlapping in time, in order to facilitate an ability to drop the transmission of lower LCH priority, MAC delivers to the physical layer only SR or PUSCH but not both.

Observation 2: When MAC is unaware of the final PUCCH resource the UE may end up with unnecessary SR/PUSCH dropping or unexpected SR dropping.

Proposal 2: RAN2 to discuss the understanding of ‘valid PUCCH resource’ in TS 38.321 with respect to its relation to the final PUCCH after UCI multiplexing in PHY.

Proposal 3: The UE utilizes additional information from PHY which assists MAC in the SR vs PUSCH selection process. Whether or not MAC can instruct the physical layer to signal SR is guided by a new UCI indication from PHY to MAC that provides timing and UCI contents (SR, HARQ-ACK, CSI) based on the final PUCCH resource Z selected during UCI multiplexing in PHY.

Proposal 4: To assist UCI multiplexing when UL skipping is configured, MAC provides an early SR status indication to PHY.

Proposal 5: RAN2 to discuss the splitting of SR and DATA capability for UL skipping enhancement.

P5 Alt 1> The combination of LCH-based prioritization and UL skipping is supported through separate capabilities for SR/data and data/data.

- UE capability 1: Support LCH-based prioritization between overlapping data/data when UL skipping is configured. The configuration is enabled through a new RRC parameter.

- UE capability 2: Support LCH-based prioritization between overlapping SR/PUSCH when UL skipping is configured. The configuration is enabled through a new RRC parameter.

P5 Alt 2> The combination of LCH-based prioritization and UL skipping supported through sub-features for UCI indication and SR status indication.

- UE capability 1: Support a new UCI indication to provide timing and UCI contents (SR, HARQ-ACK, CSI) of the final PUCCH from PHY to MAC. The configuration is enabled through a new RRC parameter.

- UE capability 2: Support of an early SR status indictation from MAC to PHY. The configuration is enabled through a new RRC parameter. (And the UCI indication is a prerequisite.)

P5 Alt 3> UL skipping and intra-UE prioritization are not supported to be configured at the same time.

# References

[1] R2-2011124, LS on overlapped data and SR are of equal L1 priority, RAN2 to RAN1

[2] R1-2102244, Reply LS on overlapped data and SR are of equal L1 priority, RAN1 to RAN2

[3] R2-2011058, Summary of [AT112-e][041][IIOT] MAC I, Huawei

[4] Draft_R2-113-e_Meeting_Report_v1

[5] R2-2102458, Report of [AT113-e][019][NR16 IIOT] UL Skipping, vivo

[6] R2-2103846, Overlapped SR and PUSCH of equal L1 priority, Apple

[7] R1-2103083, UCI multiplexing and PUSCH skipping design in URLLC, Apple

[8] R2-2103845, UL skipping and intra-UE prioritization, Apple