3GPP TSG-RAN WG2 Meeting #113bis Electronic	 R2-2104510

Elbonia, 12 – 20 April 2021

Agenda item:	5.2

Source:	Nokia (Rapporteur)

Title:	Offline 002 on Stage 2 Corrections

WID/SID:	NR_newRAT-Core - Release 15

Document for:	Discussion and Decision

# 1	Introduction

This document is the report of the following email discussion:

[AT113bis-e][002][NR15] Stage-2 (Nokia)

Scope: Treat R2-2102901, R2-2102902, R2-2102903, R2-2102941, R2-2102942, R2-2103479, R2-2103485, R2-2103653, R2-2103654, R2-2103983, R2-2103984, R2-2102674, R2-2103337, R2-2103338, R2-2103339, R2-2104010, R2-2104011, R2-2104012, R2-2103651, R2-2103652.

Phase 1, determine agreeable parts, Phase 2, for agreeable parts Work on CRs.

Intended outcome: Report and Agreed-in-principle CRs.

Deadline: Schedule A

# 2	Contact Points

Respondents to the email discussion are kindly asked to fill in the following table.

| Company | Name | Email Address |
| --- | --- | --- |
| Nokia (Rapporteur) | Benoist Sébire | benoist.sebire@nokia.com |
| OPPO | ShiCong | shicong@oppo.com |
| Huawei, HiSilicon | Yang Zhao | zhaoyang@huawei.com |
| Ericsson | Antonino Orsino | antonino.orsino@ericsson.com |
| Qualcomm Incorporated | Masato Kitazoe | mkitazoe@qti.qualcomm.com |
| MediaTek | Felix Tsai | chun-fan.tsai@mediatek.com |
| ZTE | HuangHe | huang.he4@zte.com.cn |
| Samsung | Soenghun kim | Kimsh23@samsung.com |
| LG Electronics | SeungJune Yi | seungjune.yi@lge.com |
| Intel | Sudeep Palat | Sudeep.k.palat@intel.com |
| vivo | Kimba Dit Adamou, Boubacar | kimba@vivo.com |
| Apple | Fangli XU | fangli_xu@apple.com |
| CATT | Jing Liang | liangjing@catt.cn |

# Phase 1

# 3	TS 38.300

## 3.1	Clarification on UL data transmission along with RRCReconfigurationComplete during HO

The suggested CRs are proposed in:

R2-2102901	Clarification on UL data transmission along with RRCReconfigurationComplete during HO	OPPO, Nokia (Rapporteur), CMCC, Xiaomi, Huawei, HiSilicon	CR	Rel-15	38.300	15.12.0	0348	-	F	LTE_NR_DC_CA_enh-Core

R2-2102902	Clarification on UL data transmission along with RRCReconfigurationComplete during HO	OPPO, Nokia (Rapporteur), CMCC, Xiaomi, Huawei, HiSilicon	CR	Rel-16	38.300	16.5.0	0349	-	A	LTE_NR_DC_CA_enh-Core

Question 1: Do companies agree with the issue and if yes, are the suggested changes fine (including target releases) or does the text need to be improved / corrected ?

| Answers to Question 1 |  |  |
| --- | --- | --- |
| Company | Agree with issue Yes/No | If yes, are the suggested changes fine (including target releases) or does the text need to be improved / corrected ? |
| OPPO | Yes | Proponent |
| Huawei, HiSilicon | Yes | We are one of the co-sourcing companies. |
| Ericsson | No with comment | We agree in principle that this is currently supported in stage 3 MAC spec. However, since this is already clear from MAC point of view we do not see the point to enhance the stage 2 description. At the end, stage 2 does not prevent to send data together with the complete message and thus nothing is broken from this point of view. Further, we already took the principle long time ago to not further polish stage 2 unless there is something that is not correct…but this does not look the case. |
| Qualcomm Incorporated | No | We do not see the change is essential. It only covers regular MAC multiplexing behaviour which is already clear from stage-3. |
| MediaTek | Yes |  |
| ZTE | No | Unlike the LTE specs, since we have a NOTE already in section 9.2.3.1 of NR stage-2, we think current specs is clear enough. ------------------------------- From 38.300, 9.2.3.1 ---------------------- 4. The UE moves the RRC connection to the target gNB and replies with the RRCReconfigurationComplete. NOTE 1: User Data can also be sent in step 4 if the grant allows. |
| Nokia, Nokia Shanghai Bell | Yes | Proponent |
| Samsung | No | We agree to the intention of the CR. However as ZTE show above, it is already clear in the current specification |
| LG | No | The texts in 36.300 and 38.300 are not exactly same, and thus the change is not essential. We don’t see any problem with current text in 38.300. |
| Intel | Yes | Though it could be useful clarification (though not that essential). We wonder why this WI code was chosen. |
| vivo | No | Not essential. the stage 3 is clear enough. |
| Apple | Yes | The CR is not essential, but we are fine to merge it in Rapporteur CR. |
| CATT | No | As there is a note to indicate that User Data can also be sent in RRCReconfigurationComplete, the clarification in CR is not needed. |

Summary 1: Out of the 13 companies having expressed an opinion, 6 support the change while 7 do not. It was also pointed out that there is already a Note addressing the issue.

Proposal 1: R2-2102901 and R2-2102901 are not pursued.

## 3.2	Intra and Inter Frequency Scenarios

The suggested CRs are proposed in:

R2-2102941	Intra and Inter Frequency Scenarios	Nokia (Rapporteur)	CR	Rel-15	38.300	15.12.0	0350	-	F	NR_newRAT-Core

R2-2102942	Intra and Inter Frequency Scenarios	Nokia (Rapporteur)	CR	Rel-16	38.300	16.5.0	0351	-	A	NR_newRAT-Core

Question 2: Do companies agree with the issue and if yes, are the suggested changes fine (including target releases) or does the text need to be improved / corrected ?

| Answers to Question 2 |  |  |
| --- | --- | --- |
| Company | Agree with issue Yes/No | If yes, are the suggested changes fine (including target releases) or does the text need to be improved / corrected ? |
| OPPO | Yes | It would be go to clarify |
| Huawei, HiSilicon | Agree with the intention but… | The figures are a bit too complicated. We think the figures are examples, and thus no need to list that much. Probably this can also be moved to Annex. |
| Ericsson | No with comment | Our understanding is that RAN4 specs are clear on this aspect and, once again, we do not need to polish or enhance stage 2 spec. Probably this will bring more uncertainty than clarity. However, if we decide to go along this path, we should be careful about what we introduce. |
| Qualcomm Incorporated | No | We do not understand why the existing text, in the same section, on the definition on intra-frequency “measurement” and “inter-frequency “measurement” need to be almost repeated for intra-frequency “scenario” and intra-frequency “scenario”. We think the current text is sufficiently clear. RAN2 spent good amount of time to come to the existing text. |
| MediaTek | Partial | The original text already define the intra- and inter- frequency measurement for SSB and CSI-RS. So, the part to introduce the scenario and picture seems not needed. Other than that (the editorial part), it looks okay. |
| ZTE | No | Current spec is clear enough, We don’t see any essential issues without the CR. In addition, since we have a NOTE already that “NOTE 3: Extended CP for CSI-RS based measurement is not supported in this release.”, there is no need to mention the cyclic prefix type here. |
| Nokia, Nokia Shanghai Bell | Yes | We would like to clarify that similar figures were in LTE and had been pending (and then forgotten) while awaiting for RAN4 feedback. The existing text does not define what the scenarios are. |
| Samsung | No | It is too late to agree on R15 nice to have CR |
| LG | No | We don’t see the need to update Rel-15 specification. |
| Intel | No strong view but.. | We wonder what these scenarios are used for. During Rel-15 NR discussion, it was difficult to define clearly “intra-frequency” and “inter-frequency” due to complexity of NR PHY structure. That is why only measurement gap related intra-frequency and inter-frequency are described as a minimum. |
| vivo | Yes | This clarification is useful. |
| Apple | Yes | We agree with other companies that the content is overlapped with the existing text. But we see the clarification is easy to read. So we are fine with the CR and remove the overlapped existing text. |
| CATT | No | We think the current text is clear enough. |

Summary 2: Out of the 13 companies having expressed an opinion, 4 support the change, 6 do not and 3 are neutral or would prefer something else.

Proposal 2: R2-2102941 and R2-2102942 are not pursued.

## 3.3	Correction on random access procedure for resume procedure

The suggested CRs are proposed in:

R2-2103479	Correction on random access procedure for resume procedure	Nokia (Rapporteur)	CR	Rel-15	38.300	15.12.0	0355	-	F	NR_newRAT-Core

R2-2103485	Correction on random access procedure for resume procedure	Nokia (Rapporteur)	CR	Rel-16	38.300	16.5.0	0356	-	A	NR_newRAT-Core

Question 3: Do companies agree with the issue and if yes, are the suggested changes fine (including target releases) or does the text need to be improved / corrected ?

| Answers to Question 3 |  |  |
| --- | --- | --- |
| Company | Agree with issue Yes/No | If yes, are the suggested changes fine (including target releases) or does the text need to be improved / corrected ? |
| OPPO | No | Transition from RRC_INACTIVE should already include the concerned cases in the CR, i.e., does not necessarily mean Resume is successful, thus we think the spec is already clear. |
| Huawei, HiSilicon | No | This change seems editorial. We do not see need to have it. |
| Ericsson | No | Agree with Huawei. |
| Qualcomm Incorporated | Yes | It is true that a random access in RRC Connection Resume procedure does not necessarily result in transition from RRC_INACTIVE. |
| MediaTek | No | We think original text is fine. No need to have super details in stage 2 SPEC. Even after this modification. It is still difficult to see the difference between the original text and new text |
| ZTE | Agree | The CR seems fine. |
| Nokia, Nokia Shanghai Bell | Yes | Proponent. |
| Samsung | No | It is not essential. We don’t need to continue aligning stage 2 and stage 3 in such tiny details |
| LG | No | We have same understanding as Oppo. |
| Intel | May be | It seems a nice to have improvement. If agreed, consider merging into a stage 2 small corrections CR. |
| vivo | No | Not very essential. |
| Apple | Yes | It’s not essential but good to have. |
| CATT | No | Agree with Huawei. |

Summary 3: Out of the 13 companies having expressed an opinion, 4 support the change, 8 do not and 1 is neutral or would prefer something else.

Proposal 3: R2-2103479 and R2-2103485 are not pursued.

## 3.4	Clarification to data forwarding at full configuration

The suggested CRs are proposed in:

R2-2103653	Clarification to data forwarding at full configuration	Ericsson	CR	Rel-15	38.300	15.12.0	0360	-	F	NR_newRAT-Core

R2-2103654	Clarification to data forwarding at full configuration	Ericsson	CR	Rel-16	38.300	16.5.0	0361	-	A	NR_newRAT-Core

Question 4: Do companies agree with the issue and if yes, are the suggested changes fine (including target releases) or does the text need to be improved / corrected ?

| Answers to Question 4 |  |  |
| --- | --- | --- |
| Company | Agree with issue Yes/No | If yes, are the suggested changes fine (including target releases) or does the text need to be improved / corrected ? |
| OPPO | No | Current spec is clear to us. |
| Huawei, HiSilicon | No | We do not see this change makes anything different, seems not essential. |
| Ericsson (proponent) | Yes | The reason of this CR is because, in the last meeting, the agreed text uses the terminology “the target NG-RAN node may not send PDCP SDUs” which is not entirely aligned with the typical approach used in stage-2/stage-3 specifications. On top this, “may not” goes against the “Specification drafting rules” in TS 21.801 in which the usage of “may” is indicated for capturing permissible actions (see Annex E). If we use “may not” it means that all the rest that is not included in the “may not” can be done by the network but this is not the case. This is why we need to describe only what the network “may do”. |
| Qualcomm Incorporated | Yes | The current text is indeed unclear. |
| MediaTek | No strong view | Looks like a wording enhancement and it is okay for us. |
| ZTE | Yes, but | We are fine with the CR. However, since there is no real impact on behaviour, we think the CR is an editorial CR and should be merged to the rapporteur CR. |
| Nokia, Nokia Shanghai Bell | No strong view | Seems purely editorial. |
| Samsung | No | It is not an essential correction. The current wording is more aligned with LTE |
| LG | No | The original text contains more useful information, i.e. the target may not send all PDCP SDUs forwarded from the source. The proposed CR removes such useful information. |
| Intel | May be | It seems a nice to have improvement. If agreed, consider merging into a stage 2 small corrections CR. |
| vivo | Yes | Not essential, but it is good to respect specification rules. |
| Apple | Yes | It’s not essential but good to have. |
| CATT | Yes |  |

Summary 4: Out of the 13 companies having expressed an opinion, 5 support the change as such, 4 do not and 4 are neutral or would prefer something else.

Proposal 4: R2-2103479 and R2-2103485 are not pursued as separate CRs, but the changes can be incorporated in the updates of R2-2103983 and R2-2103984 (see proposal 5 below).

## 3.5	SRB PDCP handling upon handover

The suggested CRs are proposed in:

R2-2103983	SRB PDCP handling upon handover	Huawei, HiSilicon, Nokia (rapporteur)	CR	Rel-15	38.300	15.12.0	0363	-	F	NR_newRAT-Core

R2-2103984	SRB PDCP handling upon handover	Huawei, HiSilicon, Nokia (rapporteur)	CR	Rel-16	38.300	16.5.0	0364	-	A	NR_newRAT-Core

Question 5: Do companies agree with the issue and if yes, are the suggested changes fine (including target releases) or does the text need to be improved / corrected ?

| Answers to Question 5 |  |  |
| --- | --- | --- |
| Company | Agree with issue Yes/No | If yes, are the suggested changes fine (including target releases) or does the text need to be improved / corrected ? |
| OPPO | Yes | Seems the CR tries the align the stage 2 description with the behaviour in stage 3 spec, we are open to improve the stage 2 description. |
| Huawei, HiSilicon | Yes | Proponent |
| Ericsson | No | There is nothing wrong with the current text and is up to the network anyway whether to signal to discard the PDCP PDU or not. |
| Qualcomm Incorporated | Yes |  |
| MediaTek | No strong view | We are fine have this change. But we also understand that stage 3 SPEC is clear. So, the CR is not essential. |
| ZTE | Yes | The CR is correct, but maybe not essential. It can be merged to the rapporteur CR. |
| Nokia, Nokia Shanghai Bell | Yes |  |
| Samsung | No | The intention is correct. But we don’t believe stage 2 should be corrected to be perfectly aligned with stage 3 in this late stage |
| LG | No | 38.300 does not have to specify all the events specified in stage-3 specifications. |
| Intel | May be | We agree with the intent but it should be clear already from stage 3. If agreed, consider merging into a stage 2 small corrections CR. |
| vivo | No strong view |  |
| Apple | Yes |  |
| CATT | Yes |  |

Summary 5: Out of the 13 companies having expressed an opinion, 7 support the change, 3 do not and 3 are neutral or would prefer something else.

Some issues with the cover sheets were pointed out offline:

- Tdoc number misspelled on the coversheet

- WI code is misspelled (should be NR_newRAT-Core).

Proposal 5: fix the cover sheet of R2-2103983 and R2-2103984 and agree the corresponding updates.

## 3.6	Handover Terminology

SA5 has sent RAN2 an LS in:

R2-2102674	LS on Handover terminology (S5-211324; contact: Ericsson)	SA5	LS in	Rel-17	E_HOO	To:RAN2, RAN3	Cc:-

The LS states the following:

During the development of specifications for Conditional Handover, companies have experienced terminology difficulties separating "legacy handovers", sometimes called "ordinary handovers" from handovers in general, including "ordinary handovers", Conditional Handover (CHO) and Dual Active Protocol Stack Handover (DAPS Handover.) SA5 notes that the following kinds of handovers need to be explicitly defined. Note that items 2 and 3 are already explicitly defined: - "Ordinary" or "Legacy" Handover. - Conditional Handover, CHO. See TS 38.00 clause 3.2. - Dual Active Protocol Stack Handover, DAPS Handover. See TS 38.300 clause 3.2. - Possible future forms of handover, yet to be specified. Note that in the future, CHO might be considered a "legacy" handover type. - Handovers in general, including items 1 to 4. Currently, SA5 interprets "Legacy HO" as the most basic form of HO where a single HO target is configured and where HO preparation happens right before HO execution while the UE disconnects from source before it re-connects to the target. SA5 asks RAN2 and RAN3 to develop terminologies for separating the different kinds of handover. As CHO and DAPS Handover were specified in Rel-16, this terminology should be available from that release. ACTION: SA5 asks RAN2, RAN3 to develop terminology for separating "legacy" handover from handovers in general.

Two sets of CRs are proposed:

First Set: suggests to replace normal handover by handover without DAPS configured.

R2-2103337	38.300 CR: removing ambiguous HO naming	Nokia, Nokia Shanghai Bell	CR	Rel-17	38.300	16.5.0	0354	-	F	E_HOO

R2-2103338	36.300 CR: removing ambiguous HO naming	Nokia, Nokia Shanghai Bell	CR	Rel-17	36.300	16.5.0	1336	-	F	E_HOO

Second Set: suggests to replace legacy handover by Rel-15 handover.

R2-2104011	Correction on handover terminology	Huawei, HiSilicon	CR	Rel-17	36.300	16.5.0	1337	-	F	TEI17

R2-2104012	Correction on handover terminology	Huawei, HiSilicon	CR	Rel-17	38.300	16.5.0	0365	-	F	TEI17

Question 6: Do companies agree with the issue and if yes, which set of CRs should be pursued further?

| Answers to Question 6 |  |  |  |
| --- | --- | --- | --- |
| Company | Agree with issue Yes/No | If yes, which set of CRs to pursue further, 1st or 2nd ? | Are the suggested changes fine or the text needs to be improved / corrected ? |
| OPPO | Yes | 1st as baseline | For the first change, we are not sure whether normal handover = handover without DAPS configured? i.e., whether CHO should also be precluded? |
| Huawei, HiSilicon | Yes | 2nd We understand we also support other handover like CHO, and we may have other enhanced handover procedures in the future. So using “handover without DAPS configured” may not be accurate and may also include other enhanced HO mechanisms. | The two sets of CRs are changing different sections and we understand all affected sections need to be changed in the end. Another question is the starting Release. Although both sets of CRs are from Rel-17 as the LS was from Rel-17, it needs to be discussed whether we should start change from Rel-16 because DAPS were introduced since Rel-16. |
| Qualcomm Incorporated | Yes | 1st Defining a term after the Release as in 2nd set, is not desirable. |  |
| MediaTek | Yes | 1st as baseline We think the term “Rel-15 handover” is still confusing and should be avoided. | We think the CR should be started from Rel-16. |
| Ericsson | Yes | First. | Better not to change the name of an old term. Rel-15 handover is confusing, whether that is anything different than (normal) handover. |
| ZTE | Yes | 1st | The first set CR seems more clear. |
| Nokia, Nokia Shanghai Bell | Yes | 1st |  |
| Samsung | Yes | 1st | Being more specific would be better |
| LG | Yes |  | Need further discussion with careful check of RRC specification. |
| Intel | Yes | We prefer 1st set. |  |
| vivo | Yes | 1st |  |
| Apple | Yes | 1st | Further clarification is needed about whether “handover without DAPS configured” includes CHO. |
| CATT | Yes | 1st | Replace legacy handover by Rel-15 handover is confusing. |

Summary 6: Out of the 13 companies having expressed an opinion, all but one suggest using the 1st set as basis.

Proposal 6: For handover terminology, R2-2104011 and R2-2104012 are not pursued. R2-2103337 and R2-21033378 are used as baseline.

# 4	TS 37.340

The suggested CRs are proposed in:

R2-2103651	Clarification to data forwarding upon SN change with full configuration	Ericsson	CR	Rel-15	37.340	15.12.0	0259	-	F	NR_newRAT-Core

R2-2103652	Clarification to data forwarding upon SN change with full configuration	Ericsson	CR	Rel-16	37.340	16.5.0	0260	-	A	NR_newRAT-Core

Question 7: Do companies agree with the issue and if yes, are the suggested changes fine (including target releases) or does the text need to be improved / corrected ?

| Answers to Question 7 |  |  |
| --- | --- | --- |
| Company | Agree with issue Yes/No | If yes, are the suggested changes fine (including target releases) or does the text need to be improved / corrected ? |
| OPPO | No | Current spec is clear to us. |
| Huawei, HiSilicon | No | Same as commented in 3.4. |
| Ericsson (proponent) | Yes | The reason of this CR is because, in the last meeting, the agreed text uses the terminology “the target NG-RAN node may not send PDCP SDUs” which is not entirely aligned with the typical approach used in stage-2/stage-3 specifications. On top this, “may not” goes against the “Specification drafting rules” in TS 21.801 in which the usage of “may” is indicated for capturing permissible actions (see Annex E). If we use “may not” it means that all the rest that is not included in the “may not” can be done by the network but this is not the case. This is why we need to describe only what the network “may do”. |
| Qualcomm Incorporated | Yes |  |
| MediaTek | No strong view | Same as commented in 3.4. |
| ZTE | Yes, but | We are fine with the CR. However, since there is no real impact on behaviour, we think the CR is an editorial CR and should be merged to the rapporteur CR. |
| Nokia, Nokia Shanghai Bell | No | Seems purely editorial. |
| Samsung | No | Same as commented in 3.4 |
| LG | No | See our comments to Q4. |
| Intel | May be | It seems a nice to have improvement. If agreed, consider merging into a stage 2 small corrections CR. |
| vivo | Yes | Not essential, but it is good to respect specification rules. |
| Apple | Yes |  |
| CATT | Yes | We are fine with the CR. |

Summary 7: Out of the 13 companies having expressed an opinion, 5 support the change as such, 5 do not and 3 are neutral or would prefer something else.

Proposal 7: R2-2103651 and R2-2103652 are agreed in principle, with the understanding that merging with another 37.340 CR should take place at the next meeting if any is agreed.

# 4	Phase 1 Conclusions

The proposals for Phase 1 are:

Proposal 1: R2-2102901 and R2-2102901 on “Clarification on UL data transmission along with RRCReconfigurationComplete during HO” are not pursued.

Proposal 2: R2-2102941 and R2-2102942 on “Intra and Inter Frequency Scenarios” are not pursued.

Proposal 3: R2-2103479 and R2-2103485 on “Correction on random access procedure for resume procedure” are not pursued.

Proposal 4: R2-2103479 and R2-2103485 are not pursued as separate CRs, but the changes can be incorporated in the updates of R2-2103983 and R2-2103984 (see proposal 5 below).

Proposal 5: fix the cover sheet of R2-2103983 and R2-2103984 on “SRB PDCP handling upon handover” and agree the corresponding updates.

Proposal 6: For handover terminology, R2-2104011 and R2-2104012 are not pursued, and R2-2103337 and R2-21033378 are used as baseline.

Proposal 7: R2-2103651 and R2-2103652 are agreed in principle, with the understanding that merging with another 37.340 CR should take place at the next meeting if any is agreed.

# Phase 2 - Handover Terminology

In Phase 1, it was agreed to use the following CRs as basis:

R2-2103337	38.300 CR: removing ambiguous HO naming	Nokia, Nokia Shanghai Bell	CR	Rel-17	38.300	16.5.0	0354	-	F	E_HOO

R2-2103338	36.300 CR: removing ambiguous HO naming	Nokia, Nokia Shanghai Bell	CR	Rel-17	36.300	16.5.0	1336	-	F	E_HOO

Question 8: Can the CRs be agreed as such or how should they be updated?

| Answers to Question 8 |  |  |
| --- | --- | --- |
| Company | Agree with CRs Yes/No | If not, how does the text need to be improved / corrected? |
| Huawei, HiSilicon |  | Still need to check whether the change “handover without DAPS” means including CHO or not? We think the change is not that urgent, and so could allow companies to check further and comeback to next RAN2 meeting. |
| Nokia | CRs to be agreed as such | Regarding Huawei’s comment: for CHO nothing has changed in the way RLC/PDCP and DRBs are handled, so in our opinion it is still correct to simply say ‘’handover without DAPS’’. Furthermore, the NOTE is clearly about DAPS only, so no point in introducing the notion of CHO there. It is true the issue is not urgent, but at the same time it is also simple enough to be concluded immediately and let’s not spend additional time at next meeting or preparing to next meeting for the topic. |

Summary 8: only two companies expressed a view.

Proposal 8: R2-2103337 and R2-2103337 are agreed in principle. If companies find something to fix revisions can be provided at the next meeting.