3GPP TSG-RAN2 Meeting 113-bis-e	R2-2104579

# on-line, 12th -20th April 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 2527 | rev | 1 | Current version: | 16.4.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on SCellIndex and servCellIndex |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTTDOCOMO, INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2021-3-22 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - Reason for first change: Acoording to the following ASN.1 of cellGroupConfig IE, the PSCell is indicated by servCellIndex and the SCell is indicated by SCellIndex, SCellIndex cannot identify an PSCell. Thus it is incorrect in SCellIndex that the IE concerns a short identity, used to identify an SCell or PSCell CellGroupConfig information element SpCellConfig ::= SEQUENCE { servCellIndex ServCellIndex OPTIONAL, -- Cond SCG reconfigurationWithSync ReconfigurationWithSync OPTIONAL, -- Cond ReconfWithSync rlf-TimersAndConstants SetupRelease { RLF-TimersAndConstants } OPTIONAL, -- Need M rlmInSyncOutOfSyncThreshold ENUMERATED {n1} OPTIONAL, -- Need S spCellConfigDedicated ServingCellConfig OPTIONAL, -- Need M ... } SCellConfig ::= SEQUENCE { sCellIndex SCellIndex, sCellConfigCommon ServingCellConfigCommon OPTIONAL, -- Cond SCellAdd sCellConfigDedicated ServingCellConfig OPTIONAL, -- Cond SCellAddMod ..., [[ smtc SSB-MTC OPTIONAL -- Need S ]], [[ sCellState-r16 ENUMERATED {activated} OPTIONAL, -- Cond SCellAddSync secondaryDRX-GroupConfig-r16 ENUMERATED {true} OPTIONAL -- Cond DRX-Config2 ]]} - Reason for second change: According to the current description of servCellIndex, for assigning servCellIndex to SCell, the SCellIndex that has previously been assinged applies for SCells, meanwhile the servCellIndex can also identify a PSCell. Since it is necessary to keep uniqureness of assigning servCellIndex to PSCell or SCell across the cell groups i.e. the servCellIndex used for PSCell should not be duplicated with the servCellIndex used for SCellIndex (as shown in the following example). - Examaple : ServCellIndex for PCell: 0 ServCellIndex for PSCell: 1 ServCellIndex for SCell: 1,2,3 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Delete the PSCell in the description of SCellIndex as following: SCellIndex The IE SCellIndex concerns a short identity, used to identify an SCell. The value range is shared across the Cell Groups. - Add clarificaiton in the description of ServCellIndex shall be uniquely assigned across the cell groups. ServCellIndex The IE ServCellIndex concerns a short identity, used to uniquely identify a serving cell (i.e. the PCell, the PSCell or an SCell) across the cell groups. Value 0 applies for the PCell, while the SCellIndex that has previously been assigned applies for SCells. Impact Analysis Impacted 5G architecture option: NR Standalone, (NG)EN-DC, NE-DC, NR-DC Impacted functionality - SCellIndex - ServCellIndex Inter-Operability: - If network is implemented according to the CR and UE is not, there is no inter-opterability problem, since the network does not duplicately assign the same servCellIndex to PSCell and SCell for the UE. - If UE is implemented according to the CR and network is not, there is inter-operability problem, since it is possible for the netowork to duplicately assign the same servCellIndex to PSCell and SCell for the UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is possible for network to duplicately assgin the same servCellIndex to PSCell and SCell for the UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision1: The second change is revised as below: The IE ServCellIndex concerns a short identity, used to uniquely identify a serving cell (i.e. the PCell, the PSCell or an SCell) across the cell groups. |  |  |  |  |  |  |  |  |

Start Of Change

Unchanged part are omitted

6.3.2	Radio resource control information elements

Unchanged part are omitted

SCellIndex

The IE SCellIndex concerns a short identity, used to identify an SCell. The value range is shared across the Cell Groups.

Unchanged part are omitted

–	ServCellIndex

The IE ServCellIndex concerns a short identity, used to uniquely identify a serving cell (i.e. the PCell, the PSCell or an SCell) across the cell groups. Value 0 applies for the PCell, while the SCellIndex that has previously been assigned applies for SCells.

Unchanged part are omitted

End Of Change