3GPP TSG RAN WG2 #114-e	R2-2105220

Electronic, 19th – 27th May, 2021

Source: 	Huawei, HiSilicon

Title: 	Discussion on positioning enhancement

Agenda Item:	8.11.6

Document for:	Discussion and decision

Introduction

The SI phase has studied the positioning enhancement solutions in terms of accuracy and latency, such as the enhancement of angle based positioning, information reporting, and etc. In RAN#91e, the new WI on positioning enhancement was approved, which has the following objectives [1]:

- Specify methods, measurements, signalling, and procedures for improving positioning accuracy of the Rel-16 NR positioning methods by mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays, including [RAN1, RAN2, RAN3, RAN4] - DL, UL and DL+UL positioning methods - UE-based and UE-assisted positioning solutions - Specify the procedure, measurements, reporting, and signalling for improving the accuracy of [RAN1, RAN2, RAN3] - UL AoA for network-based positioning solutions. - DL-AoD for UE-based and network-based (including UE-assisted) positioning solutions.

A LS has been received from RAN1 regarding the enhancement of DL-AOD positioning. The LS requires RAN2 and RAN3 to have the following action [2]:

To RAN2, RAN3 ACTION: RAN1 respectfully requests RAN2/RAN3 to consider the option of angle report from gNB to LMF for UE-A DL-AoD in Rel-17.

In this paper, the option of angle reporting from gNB to LMF and the other enhancement solutions are discussed.

Discussion

- 2.1 Enhancement for angle based methods

In Rel-16, the AoD information of multiple gNBs is determined at the LMF in DL-AoD positioning and the accuracy of the AoD information as well as the positioning result is limited. That is because the LMF calculates the AoD based on the beam information reported by gNBs and RSRP measurements reported by the UE and the quantized beam information does not support the high accuracy AoD calculations.

In order to achieve high accuracy positioning, a better way is to let the gNB to calculate the AoD information based on its full knowledge of the DL configurations and the RSRP measurements from the UE. This can be realized by reusing the existing NRPPa message of measurement request sent from LMF to gNB to acquire AoD information from the gNBs. The procedure is illustrated in Figure 1:

LMF

NG-RAN

Measurement Request

(RSRP)

Measurement Response

(Angle)

UE

LPP procedure

UE position calc.

Figure 1, AoD calculation procedure

The angle information calculated at NG-RAN nodes can then be reported to LMF and the calculation of the UE positioning. Therefore, we propose to support AoD calculation at NG-RAN nodes and angle reporting from gNB to LMF in DL-AoD positioning for accuracy enhancement.

Proposal 1: Support AoD calculation at NG-RAN nodes and angle reporting from gNB to LMF in DL-AoD positioning.

Proposal 2: Send LS to RAN1 and RAN3 to confirm the support of angle reporting from gNB to LMF.

- 2.2 Positioning assistance data

Currently the LMF select neighbouring cells for positioning, for which the assistance data is sent to UE based on the serving cell information of the UE. That is to say, LMF knows the location of the TRPs apriori and select those TRPs that close to the serving TRP. From 29.572, according to the current spec, the AMF is above to provision the NCGI/ECGI of the PCell and PSCell of the UE.

However, the UE only includes the serving cell information when send the Request Assistance Data message to the LMF, who then selects the assistance data according to the serving cell and sends back to the UE.

Since LMF does not have exact neighbouring information of the UE, it may send PRS information at as many TRPs as possible when send the assistance data to the UE. This can lead to the high signalling overhead and waste of resources. In order to enable the LMF to select the assistance data at appropriate neighbouring cells, it is proposed to let the UE to include neighbouring cell information when send the Request Assistance Data message to the LMF.  UE can include the SCell or PSCell identityin the message sent to LMF for requesting assistance data.

Proposal 3: Support UE to include cell information (e.g., SCell and PSCell information) when requesting assistance data from LMF.

- 2.3	NR E-CID enhancement

NR E-CID does not support RTT measurements for the serving cell in Rel-16. This makes NR E-CID similar to a DL-based positioning technique solely based on RSRP finger-printing. Therefore, NR E-CID in Rel-16 may have a worse accuracy than LTE E-CID. In order to increase the accuracy of NR E-CID, it is proposed to incorporate the RTT measurements based on the serving gNBs for NR E-CID.

Proposal 4: Incorporate RTT measurement for NR E-CID.

Conclusion

Based on the discussion above, we have the following proposals:

Proposal 1: Support AoD calculation at NG-RAN nodes and angle reporting from gNB to LMF in DL-AoD positioning.

Proposal 2: Send LS to RAN1 and RAN3 to confirm the support of angle reporting from gNB to LMF.

Proposal 3: Support UE to include cell information (e.g., SCell and PSCell information) when requesting assistance data from LMF.

Proposal 4: Incorporate RTT measurement for NR E-CID.

References

- RP-210903, Revised WID on NR Positioning Enhancements.

- R1-2104089 LS on DL-AoD angle calculation enhancement