3GPP TSG-RAN WG2 Meeting #114 electronic	R2-2105280

Online, May 19 – May 27, 2021

Source:	CATT

Title:	Consideration on UP common aspects of SDT

Agenda Item:	8.6.2

Document for:	Discussion and Decision

Introduction

In RAN2#113bis-e meeting, various agreements were achieved on SDT procedure and user plane [1].

Agreements: - RSRP threshold is used to select between SDT and non-SDT procedure, if configured (RSRP refers to the same RSRP measured for carrier selection). - RSRP threshold to select between SDT and non-SDT procedure is used for both CG-SDT and RA-SDT - RSRP threshold to select between SDT and non-SDT procedure is same for both CG-SDT and RA-SDT - RSRP threshold for carrier selection is specific to SDT (i.e. separately configured for SDT). This is optional for the network. - Confirm that cell selection mechanism is not modified - RSRP threshold for RA type selection is specific to SDT (i.e. separately configured for SDT) - Data volume threshold is the same for CG-SDT and RA-SDT (can be checked further in stage 3 if we obtain majority support) 8 FFS on the order and missing pieces (e.g. failure, fallback) of the high level procedure. The details of the procedures are left for stage 3. FFS on the procedure below, but copied for information. A. Upon arrival of data only for DRB/SRB(s) for which SDT is enabled, the high level procedure for selection between SDT and non SDT procedure is as follows: If CG-SDT criteria is met: UE selects CG-SDT. UE initiate SDT procedure Else if RA-SDT criteria is met: UE selects RA-SDT. UE initiate SDT procedure Else: UE initiate non SDT procedure. B. CG-SDT criteria is considered met, if all of the following conditions are met, 1) available data volume <= data volume threshold 2) RSRP is greater than or equal to a configured threshold FFS 3) CG-SDT resources are configured on the selected UL carrier and are valid C. RA-SDT criteria is considered met, if all of the following conditions are met, 1) available data volume <= data volume threshold 2) RSRP is greater than or equal to a configured threshold 3) 4 step RA-SDT resources are configured on the selected UL carrier and criteria to select 4 step RA SDT is met; or 2 step RA-SDT resources are configured on the selected UL carrier and criteria to select 2 step RA SDT is met - Switching from SDT to non-SDT is supported. - FFS Switching from CG-SDT to RA-SDT is not allowed 11 UE switches from SDT to non-SDT in following cases: - Case 1 (27/0): UE receive indication from network to switch to non-SDT procedure. - Network can send RRCResume. FFS whether network can send indication in RAR/fallbackRAR/DCI to switch to non-SDT procedure. - FFS Case 2 (18/9): Initial UL transmission (in msgA/Msg3/CG resources) fails configured number of times

Agreements

1	 The UE performs PDCP re-establishment implicitly, i.e. without explicit indication for PDCP re-establishment, when the UE initiates SDT procedure.

2	As in legacy, whether to support ROHC continuity is explicitly configured by the network.

3	PDCP duplication is not supported for SDT

4	connected mode DRX is not supported for SDT

5	PHR functionality is supported for SDT.   FFS on PHR procedure

6	SR resource is not configured for SDT. When the BSR is triggered by SDT data, the UE will trigger RA because SR resource is not available, same as legacy

In the following, we discuss remaining issues and FFSs of last meeting, our analysis as well as preference are also provided.

Discussion

- SDT procedure

- 2.1.1	UE behavior when initial UL transmission fails

It has been agreed that SDT can fallback to non-SDT procedure, but there is one FFS on whether CG-SDT can fallback to RA-SDT. Actually, when the UE fails in CG-SDT, the solutions can be:

- the UE switches to RRC re-establishment;

- the UE goes to RRC IDLE;

- the UE switches CG-SDT to RA-SDT;

- the UE switches CG-SDT to non-SDT procedure;

- leave this to UE implementation;

In the following, we analyze the advantages as well as disadvantages of each solution.

Switching to RRC re-establishment

If the UE switches to RRC re-establishment procedure, UE has to perform cell selection and then reestablish PDCP, RLC and etc. It is a bit of complex. In addition, the RSRP of serving is beyond the configured threshold, in light of that UE has selected CG-SDT with all the selection criteria are satisfied. Therefore, it is more likely that UE selects to the same serving cell. Besides, in RAN2#113bis meeting, it was agreed that RRC re-establishment procedure is not supported when cell reselection happens in SDT. Therefore, we think one unified solution should be adapted, i.e. RRC re-establishment is not pursued when CG-SDT fails.

Switching to RRC IDLE

If the UE goes to RRC IDLE mode directly, this will greatly impact the service performance and should not be supported.

Switching to RA-SDT

Based on the email discussion in RAN2#113bis e-meeting [2], it can be seen that majority of companies does not support CG-SDT fallback to RA-SDT. This is because the UE has high probability to fail for RA-SDT since the CG-SDT is not successful. Therefore, we think this option is not preferable.

Switching to non-SDT procedure

If UE switches CG-SDT to non-SDT procedure, for example, legacy RRC resume procedure, network may indicate UE to go to CONNECTED mode. Then, dedicated resources and configurations can be assigned by the network. This is more accurate. Besides, it has been agreed that switching from SDT to non-SDT is supported. Hence, this solution aligns with the agreement.

UE implementation

Leaving this to UE implementation is LTE solution. We can support this if no consensus can be reached.

But it is still FFS failure cases of CG-SDT. If autonomous failure detection of CG-SDT by UE is supported, we prefer to support to switch CG-SDT to non-SDT procedure.

Proposal 1: Support to switch CG-SDT to non-SDT procedure if autonomous failure detection of CG-SDT by UE is supported.

Another FFS left in RAN2#113bis meeting was whether to support UE switches from SDT to non-SDT when initial UL transmission (in msgA/Msg3/CG resources) fails configured number of times.

This issue is similar to the above one. Per proposal 1, we agree UE switched to non-SDT when CG-SDT fails. To make the procedure simple, we tend to agree one unified solution, i.e. UE can switch from SDT to non-SDT when initial UL transmission (in MSGA/MSG3/CG resources) fails configured number of times.

Proposal 2: Support to switch from SDT to non-SDT when initial UL transmission (in msgA/Msg3/CG resources) fails.

- 2.1.2	Indication used to switch UE to non-SDT procedure

In email[2], there is one FFS addressing whether network can send indication in RAR/fallbackRAR to switch to non-SDT procedure, which is highlighted in yellow.

Proposal 10: UE switches from SDT to non-SDT in following cases: - Case 1: UE receive indication from network to switch to non-SDT procedure. - Network can send RRCResume. FFS whether network can send indication in RAR/fallbackRAR to switch to non-SDT procedure. - Case 2: Initial UL transmission (in msgA/Msg3/CG resources) fails configured number of times

In EDT, when the UL grant provided in RAR is not for EDT, UE will indicate to the RRC layer the EDT is cancelled and normal RRC setup/resume procedure is conducted [4]-[5]. Therefore, we think the same principle can be reused.

36.321 - if the Random Access Preamble associated with EDT was transmitted and UL grant provided in the Random Access Response message is not for EDT: - indicate to upper layers that EDT is cancelled due to UL grant not being for EDT; - for CP-EDT, flush the Msg3 buffer. - for UP-EDT, update the MAC PDU in the Msg3 buffer in accordance with the uplink grant received in the Random Access Response.s

36.331 5.3.3.3c UE actions upon receiving EDT fallback indication from lower layers Upon indication from lower layers that EDT is cancelled, the UE shall: 1> start or restart timer T300; 1> if the fallback is indicated by lower layers in response to the RRCEarlyDataRequest: 2> initiate transmission of RRCConnectionRequest message in accordance with 5.3.3.3; 1> else if the fallback is indicated by lower layers in response to the RRCConnectionResumeRequest for EDT when connected to EPC and the fallback is not due to the UL grant provided in Random Access Response not being for EDT: 2> perform the actions as specified in 5.3.3.9a; 2> initiate transmission of the RRCConnectionResumeRequest message in accordance with 5.3.3.3a; NOTE: It is up to UE implementation to avoid data loss due to EDT fallback.

Therefore, we propose that

Proposal 3: Network can send indication in RAR/fallback RAR to switch to non-SDT procedure.

- PDCP status report

In RAN2#113bis meeting, whether UE implicitly disables PDCP status report was discussed in Summary of UP open issues [3].

If the UE implicitly disables PDCP status report, there are specification impacts on PDCP. If PDCP status report is triggered explicitly by the network, the network may need to send RRCReconfig to the UE in advance which disables PDCP status report and then RRCRelease is sent to the UE. When the UE goes into RRC inactive state, the UE can maintain default configuration and apply this default configuration to SDT. There is no specification impact. But, extra signanling may be introduced. Therefore, we slightly prefer UE implicitly disables PDCP status report to avoid signaling overhead.

Proposal 4: UE implicitly disable PDCP status report when SDT is initiated.

- Failures handling

- 2.3.1	RLC failure

There is no consensus on whether to support RLC failure handling in the last meeting. From our perspective, RLC failure is helpful to notify RRC that radio link is not reliable and RRC may perform RRC re-establishment. But considering RLC failure handling may take a long time, the benefits of RLC failure handling in SDT are marginal. Besides, there are some other mechanisms to enable UE to detect transmission failures, e.g. SDT failure detection timer which is agreed in RAN2#113 meeting. In our understanding, duplicate function should be avoided in SDT. Therefore, we support that:

Proposal 5: RLC failure handling is not supported in SDT.

- 2.3.2	BFR

Regarding whether BFR is supported, it was suggested consulting RAN1 on this issue. BFR procedure is used for indicating to the serving cell of a new SSB or CSI-RS when beam failure is detected on the serving SSB(s)/CSI-RS(s) [4]. Beam failure is declared when the number of beam failure instance exceeds beamFailureInstanceMaxCount during beamFailureDetectionTimer. Beam failure detection procedure is defined in 38.213 [6], which is closely related to RAN1 work. Thus RAN1 is in better place for deciding on whether to support BFR or not in SDT.

Proposal 6: Send LS to RAN1 and ask RAN1 whether BFR is supported.

- PHR procedure

- 2.4.1	PHR type

In RAN2#113bis meeting, it was agreed that PHR is supported in SDT. PHR is used to report the power header room to support for power-aware packet scheduling. It implies how much transmission power left for the UE to be used in addition to the power being used by current transmission.

In NR connected mode, the following information is reported in reported in PHR [4]:

- Type 1 power headroom: the difference between the nominal UE maximum transmit power and the estimated power for UL-SCH transmission per activated Serving Cell; - Type 2 power headroom: the difference between the nominal UE maximum transmit power and the estimated power for UL-SCH and PUCCH transmission on SpCell of the other MAC entity (i.e. E-UTRA MAC entity in EN-DC, NE-DC, and NGEN-DC cases); - Type 3 power headroom: the difference between the nominal UE maximum transmit power and the estimated power for SRS transmission per activated Serving Cell; - MPE P-MPR: the power backoff to meet the MPE FR2 requirements for a Serving Cell operating on FR2.

We will study the PHR type case by case:

Type 2 power headroom

Type 2 power headroom is not introduced in NR and only be used in EN-DC, NE-DC and NGEN-DC cases. In RAN2#113bis meeting, it was agreed that:

1	gNB can only configure MN terminated MCG bearer type for SDT

This implies that SDT can only be configured for the MN terminated MCG bearer, but it does not mean that DC can not be supported in SDT. In our understanding, DC is beneficial to increase the user perspective throughput. Meanwhile, if the UE which is going through poor channel condition with the MN can be connected to SN to achieve better cell coverage. However, in SDT, the data amount is not so large. Besides, if DC is supported, UE has to send first UL message through MN and then MN sends SN configuration to the UE. Therefore, the benefit of using DC in SDT is not so obvious. Hence, it is suggested DC is not supported in SDT. Besides, default MAC configuration is used in SDT. Hence, CA is not supported in SDT, either. Therefore, multiplePHR will be set to false in SDT and Multiple Entry PHR MAC CE will not be generated.

Proposal 7: Single Entry PHR MAC CE is applied in SDT.

Type 3 power headroom

There is no conclusion on whether SRS is supported in SDT. Considering SRS expends a large bandwidth, supporting SRS in SDT will introduce power consumption. But we can send LS to RAN1 to ask whether SRS is supported in SDT.

MPR P-MPR

MPE P-MPR reported in PHR is to mitigate RLF due to sudden and unpredictable large back off caused UE’s actions to ensure compliance with MPE regulation [7]. When mpe-Reporting-FR2 is configured, UE can report the PHR which is referred below to as 'MPE P-MPR report'. In SDT, default PHR configuration is used [8] where mpe-Reporting-FR2 is not configured as default.

Proposal 8: Type 2 and MPE P-MPR report are not supported in SDT.

Proposal 9: Send LS to RAN1 to check whether SRS is supported in SDT and then decide whether Type 3 power headroom is supported in SDT.

- 2.4.2	PHR trigger

PHR is triggered when the following conditions are satisfied [4]:

- Timer expiration: phr-ProhibitTimer or phr-PeriodicTimer;

- Path loss has changed more than phr-Tx-PowerFactorChange dB;

- Configuration or reconfiguration of the PHR which does not disable the function;

- Activation of an SCell of any MAC entity with CG of which is not dormant BWP;

- Upon switching of activated BWP from dormant BWP to non-dormant DL BWP;

- MPE P-MPR report;

There is no conclusion on when MAC configuration will be applied during SDT. Since default MAC configuration is used for SDT, apparently, it should take effect when SDT is initiated. In consequence, PHR which is part of default MAC configuration is configured to the UE during SDT initiation.

CA, dormant BWP and MPE P-MPR report are not configured in MAC with default configuration. In consequence, condition d, e and f are not to be satisfied in SDT.

Proposal 10: PHR can be triggered when:

- phr-ProhibitTimer and phr-PeriodicTimer expires;

- path loss has changed more phr-Tx-PowerFactorChange dB;

- default PHR configuration is applied to the UE during SDT initiation.

Proposal 11 Current PHR triggering procedure can be reused in SDT.

- 2.4.3	PHR Prioritization

PHR MAC CE needs to be sent to the network together with SDT data. According to the logical channel prioritization procedure, PHR MAC CE has higher priority than DRB data. Someone may argue that this may occupy large UL grant. However, taking Proposal 7 into account, i.e. Single Entry PHR MAC CE is reported to the network, the overhead is tolerable. Therefore, the current logical channel prioritization procedure can be reused.

Proposal 12: Current logical channel prioritization procedure can be reused in SDT.

- LCH restriction

LCH can be configured with some logical channel restrictions, for example, SCS, PUSCH duration and CG configurations to make sure the suitable UL grant is used for the logical channel. In SDT, it has been assumed that multiple CG configurations can be supported. Under this assumption, associate CG configuration to logical channel is helpful to resource assignment. For example, traffic of LCH1 requires large periodicity but small TBS, while traffic of LCH2 requires small periodicity but large TBS. And there are two CG configurations. CG configuration 1 is configured with large periodicity and small TBS corresponding to LCH1. And CG configuration 2 is configured with small periodicity and large TBS corresponding to LCH2. In this way, data can be transmitted on the appropriate CG resource with low latency and resource waste. While other parameters of LCH restriction are almost the same in SDT procedure, they have no impacts on SDT transmission. Therefore, we support that:

Proposal 13: Support LCH restriction in SDT.

- MSG3 content

In RA-SDT, T-C-RNTI in RAR will turn to C-RNTI and be used in the subsequent scheduling procedure. Meanwhile, C-RNTI which is assigned in RRCRelease message is also valid during the CG-SDT procedure. It can be seen that network can recognize the UE by the unique C-RNTI.

In RAN2#113bis meeting, it has been agreed that:

6	SR resource is not configured for SDT. When the BSR is triggered by SDT data, the UE will trigger RA because SR resource is not available, same as legacy

It implies that in subsequent SDT, data may arrive and trigger RA. In this case, C-RNTI can be multiplexed together with BSR to the network in MSG3/MSGA. When the network receives MSG3/MSGA, it can find which UE initiates the RA procedure and the data volume in the buffer of the UE. Then, network can decide whether to indicate the UE to go the RRC connected mode or just continue SDT transmission.

In accordance to the current specification, UE can multiplex C-RNTI when there is no data from CCCH. So we think this can be supported without much complexity.

Proposal 14: Include C-RNTI as well as BSR in the content of MSG3/MSGA during the RA which is triggered by the BSR when new SDT data arrives during subsequent SDT.

Conclusion

This contribution analyzed the UP common issues for both RA-based and CG-based SDT. We conclude the following proposals:

Proposal 1: Support to switch CG-SDT to non-SDT procedure if autonomous failure detection of CG-SDT by UE is supported.

Proposal 2: Support to switch from SDT to non-SDT when initial UL transmission (in msgA/Msg3/CG resources) fails.

Proposal 3: Network can send indication in RAR/fallback RAR to switch to non-SDT procedure.

Proposal 4: UE implicitly disable PDCP status report when SDT is initiated.

Proposal 5: RLC failure handling is not supported in SDT.

Proposal 6: Send LS to RAN1 and ask RAN1 whether BFR is supported.

Proposal 7: Single Entry PHR MAC CE is applied in SDT.

Proposal 8: Type 2 and MPE P-MPR report are not supported in SDT.

Proposal 9: Send LS to RAN1 to check whether SRS is supported in SDT and then decide whether Type 3 power headroom is supported in SDT.

Proposal 10: PHR can be triggered when:

- phr-ProhibitTimer and phr-PeriodicTimer expires;

- path loss has changed more phr-Tx-PowerFactorChange dB;

- default PHR configuration is applied to the UE during SDT initiation.

Proposal 11 Current PHR triggering procedure can be reused in SDT.

Proposal 12: Current logical channel prioritization procedure can be reused in SDT.

Proposal 13: Support LCH restriction in SDT.

Proposal 14: Include C-RNTI as well as BSR in the content of MSG3/MSGA during the RA which is triggered by the BSR when new SDT data arrives during subsequent SDT.

Reference

- RAN2#113bis meeting report;

- R2-2102707, Report from email discussion [POST113-e][501][SDT] Selection criteria and overall Procedure, Samsung Electronics Co., Ltd, RAN2#113bis;

- R2-2104395, Summary of UP SDT open issues, LG, RAN2#113bis;

- 36.321, Evolved Universal Terrestrial Radio Access (E-UTRA); Medium Access Control (MAC) protocol specification;

- 36.331, Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC);

- 38.213, NR; Physical layer procedures for control;

- R4-2005670 LS on MPE enhancements, Nokia, Nokia Shanghai Bell;

- 38.331, Radio Resource Control (RRC) protocol specification;