3GPP TSG-RAN WG2 Meeting #114 electronic	               _R2-2106522

Online, May 19 – May 27, 2021

Agenda Item:	8.12.2.2

Source: 	Huawei, HiSilicon

Title:	 Summary of [AT114-e][106][RedCap] Identification and access restrictions

Document for:	Discussion and Decision

# Introduction

This paper aims at capturing the summary of offline discussion.

- [AT114-e][106][RedCap] Identification and access restrictions (Huawei)

Initial scope: Discuss the proposals from R2-2106487

Initial intended outcome: Summary of the offline discussion with e.g.:

    - List of proposals for agreement (if any)

    - List of proposals that require online discussions

    - List of proposals that should not be pursued (if any)

Initial deadline (for companies' feedback): Thursday 2021-05-20 07:00 UTC

Initial deadline (for rapporteur's summary in R2-2106522): Thursday 2021-05-20 09:00 UTC

# Open issue to discuss

- 2.1: Early identification

- 2.1.1 Msg1 early identification

Issue A: whether there is need to support Msg1 early identification

The proposal in R2-2106487 is

Draft_Proposal 1: [To discuss][12/17] Msg1 early identification is needed from RAN2 perspective.

Option 1: Yes, it is needed from R2 perspective (with detailed solution as FFS)

Option 2: No, it is not needed from R2 perspective

Option 3: Just let RAN1 to discuss this

Question 1: Which option do you prefer on the need of Msg1 early identification from RAN2 perspective?

| Companies | Preferred option? | Comments |
| --- | --- | --- |
| OPPO | Option 3 |  |
| Ericsson | Option 2/3 | There is no need from RAN2 perspective so RAN1 should indicate whether such is needed. In our understanding there are some deployments where such can be beneficial. We are OK to have this as an optional, configurable, indication. |
| Apple | Option 3/1 | If RAN2 wants to identiy RedCap UE at RACH, then we see this as option -1. Also differentiation based on Rx ports can be useful (but this is also RAN1 related). |
| Qualcomm | Option 1 | As discussed during the SI phase, there are multiple use cases for “early identification during Msg1”. Some of them, e.g. coverage enhancements for Msg2/3 transmission, maybe should be discussed by RAN1. But there are also issues such as initial BWP configuration and PRACH configuration that clearly should be discussed by RAN2. |
| MediaTek | 3 (and not needed from R2 perspective) | The reasons for an early indication are R1 centric (BWP operation, coverage recovery and so on). We prefer to let R1 discuss this and conclude on the need. If a need is identified by R1, R2 can introduce this indication in msg1 or msg3 as required. |

- 2.1.2 Msg3 early identification

Issue A: whether there is need to support Msg3 early identification

The proposal in R2-2106487 is

Draft_Proposal 2: [To discuss][9/14] RAN2 to discuss the need of Msg3 early identification from RAN2 perspective (in case Msg1 early identification is optionally configured or not supported).

Option 1: Yes, it is needed from R2 perspective, in case Msg1 early identification is optionally configured or not supported (with detailed solution as FFS)

Option 2: No, it is not needed from R2 perspective

Option 3: Just let RAN1 to discuss this

Question 2: Which option do you prefer on the need of Msg3 early identification from RAN2 perspective?

| Companies | Preferred option? | Comments |
| --- | --- | --- |
| OPPO | Option 2 | Since SA1 don’t agree on the need to differentiate access control for RedCap UEs and non-RedCap UEs, we don’t think Msg3 identification is needed. |
| Ericsson | Option 1 | Prefer to have Msg3 indication, e.g. when Msg1 indication is not configured. |
| Apple | Option 2 | No need to differentiatiate, similar view as Oppo. |
| Qualcomm | Option 1 | In our view, if Msg1 indication is not configured, early identification in Msg3 is needed for subscription validation. |
| MediaTek | 3 (and not needed from R2 perspective) | Similar to Q1, we prefer to let R1 determine the need for such an indication. If the need is identified by R1, R2 can introduce necessary signaling for this. |

- 2.1.2 Rx braches specific early identification

Issue A: whether there is a need for Rx branches specific early identification

The proposal in R2-2106487 is

Draft_Proposal 3: There is no need to support Rx branches specific early identification from RAN2 perceptive.

Option 1: Yes, early identification of RedCap UE’s Rx capabilities

Option 2: No, only early identification of RedCap UE (not to differentiate 1Rx and 2Rx)

Option 3: Just let RAN1 to discuss this.

Question 3: Which option do you prefer for Rx branches specific early identification?

| Companies | Preferred option? | Comments |
| --- | --- | --- |
| OPPO | Option 3 |  |
| Ericsson | Option 2 |  |
| Apple | Option 3 |  |
| Qualcomm | Option 2 | Binary indication of RedCap is sufficient for initial access. We are also fine with leaving its discussion to RAN1. |
| MediaTek | 2 | While this is again a R1 centric discussion, we do not see a need for an early indication of UE’s RX capabilities at this point. This information will anyways be provided to the NW at UE capability exchange. |

- 2.2: Access and Camping Restriction

- 2.2.1 Cell barring

Issue A: Cell barring specific to the number of Rx branches of the UE

The proposal in R2-2106487 is

Draft_Proposal 4: [Easy][17/18] SIB1 indicates cell barring for 1 Rx branch and 2 Rx branches separately for RedCap UEs.

The proposal above is aligned with WID objective and clear majority view. Rapporteur would like to ask if this proposal is agreeable. Silence means agreement in this case.

Question 4: Do you agree above draft proposal 4? If not, please indicate whether you have any strong concern on the proposal.

| Companies | Agree or not? | Comments |
| --- | --- | --- |
| OPPO | Agree |  |
| Ericsson | Agree |  |
| Apple | agree |  |
| Qualcomm | Agree |  |
| MediaTek | Agree | This is aligned with the WID objective |

Issue B: whether cell barring for RedCap UE is per PLMN

The issue is discussed in R2-2105161.

Companies propose that the cell barring for RedCap UE can be per PLMN [ZTE].

Rapporteur understands the baseline is cell specifci cell barrring. Therefore, below quesiton is asked to check if companies want to enhance this for RedCap as per PLMN cell barring.

Question 5: Do you prefer the cell barring is per cell or per PLMN for RedCap?

| Companies | Per cell, or per PLMN? | Comments |
| --- | --- | --- |
| OPPO | Per cell | Stick to the baseline where current cell barring is cell specific. Don’ t see the need for optimization. |
| Ericsson | Per cell | Also prefer baseline as OPPO explains. |
| Apple | Per cell |  |
| Qualcomm | Per cell | Cell barring for RedCap UEs could depend on cell loading. So per-cell barring makes more sense. |
| MediaTek | Per cell | Agree with Oppo |

Issue C: whether to ignore the cellbarred in MIB

The proposal in R2-2106487 is

Draft_Proposal 6: [To discuss] RAN2 to discuss whether RedCap UE should ignore or apply the legacy cellbarred in MIB.

Option 1: RedCap UE ingores the cellBarred in MIB (i.e. supporting the case of barring non-RedCap UE but allowing RedCap UE to camp)

Option 2: RedCap UE applies the cellBarred in MIB (i.e. not supporting the case of barring non-RedCap UE but allowing RedCap UE to camp)

Rapporteur understands this depends on whether RAN2 consider the case of barring non-RedCap UE but allowing RedCap UE to camp as valid.

Question 6: Do you prefer whether RedCap UEs ignore or apply the legacy cellbarred in MIB (in addition to its RedCap specific indication)?

| Companies | Ignore, or apply? | Comments |
| --- | --- | --- |
| OPPO | Ignore | This is most flexible from network control’s perspective. |
| Ericsson | Apply | The question is whether we want to support “RedCap-only” cells. We don’t see this necessary at the moment but can discuss further. |
| Apple | Apply | Same view as Ericsson. |
| Qualcomm | Ignore | Option 1 supports all four cases (only non-RedCap is barred, only RedCap is barred, both are barred, no barring for both). Option 2 does not support the case only non-RedCap is barred. |
| MediaTek | Ignore | This option is more future proof, as it allows the case of barring non-RedCap UEs while allowing RedCap UEs to camp onto the cell. |

- 2.2.2 IFRI

Issue A: whether to introduce RedCap specific IFRI

The proposal in R2-2106487 is

Draft_Proposal 7a: [Easy] RedCap UE supports the Intra Frequency Reselection Indicator.

Draft_Proposal 7b: [To discuss] RAN2 to discuss whether RedCap UEs reuse the legacy IFRI in MIB or use new RedCap specific IFRI in SIB1.

Rapporteur understands there is no clear majority view on the solution to support IFRI for RedCap UE in the contributions. But, it seems companies indeed support the application of IFRI to RedCap UE. Therefore, rapporteur would like to ask if companies are fine to agree on the high level P7a.

Question 7: do you agree above draft proposal 7a (i.e. RedCap UE supports the Intra Frequency Reselection Indicator)? If not, please indicate whether you have any strong concern on the proposal.

| Companies | Agree or not? | Comments |
| --- | --- | --- |
| OPPO | Agree |  |
| Ericsson | Agree |  |
| Apple | Agree to 7a |  |
| Qualcomm | Agree |  |
| MediaTek | Agree |  |

As for the support of Intra Freq Reselection indicator for RedCap UE, companies propose the following options:

Option 1: Reuse the legacy IFRI in MIB (i.e. no RedCap specific IFRI)

Option 2: Introduce RedCap specific IFRI in SIB1

- Option 2a: not to differentiate 1Rx and 2Rx

- Option 2b: specific to the number of Rx branches

Question 8: Which option do you prefer on Intra Freq Reselection Indicator for RedCap UE?

| Companies | Preferred option | Comments |
| --- | --- | --- |
| OPPO | Option 2 | Whether 2a or 2b can be further discussed. |
| Ericsson | Option 1 | We don’t currently see use case to differentiate this between RedCap and non-RedCap. |
| Apple | Option 1 |  |
| Qualcomm | Option 2 | If Rx branch specific barring is supported, the same reasoning applies to IFRI (i.e. Option 2b). |
| MediaTek | 2a | In case all cells in the frequency do not support RedCap operation, it is useful to indicate this information to the UE to avoid unnecessarily searching for cells that do not support RedCap on the frequency. |

- 2.2.3 Neighbor cell RedCap support indication

Issue A: For cell reselection

The issue is discussed in R2-2104775, R2-2105137, R2-2105161, R2-2105399.

Knowing whether a neighbour cell accepts access by Redcap can avoid RedCap UEs’ unnecessary RRM measurements and save power. Companies propose to include an indication in system information on whether a neighbour cell accepts (support and/or allow) access by RedCap UEs. [QC, Apple, ZTE, Fujitsu]

The proposal in R2-2106487 is

Draft_Proposal 8: [2nd batch] [To discuss] RAN2 to discuss the need for an indication in system information on whether a neighbour cell accepts access by RedCap UEs.

Question 9: Do you support to indicate in system information on whether a neighbour cell accepts the access by RedCap UEs?

| Companies | Yes or No | Comments |
| --- | --- | --- |
| OPPO | No | We don’t think such optimization is needed. |
| Ericsson | No |  |
| Apple | Yes | It would be a waste of UE power to perform re-selection actions only to realize that the NCell does not support RedCap or bars certain RedCap UE. If NW broadcasts the support/no-support of RedCap for cell selection, it helps for the NW to also broadcasts this for re-selection. |
| Qualcomm | Yes | We do not consider it an optimization, because cell barring for RedCap can be more dynamic than in legacy (e.g. barring can depend on cell loading, which can be dynamic). So a RedCap UE can’t check its neighbor cells for only once and then know whether it can skip RRM measurements on a neighbor cell forever. It still needs to check periodically on all neighbor cells to see if they have changed their barring status. Having this indication in the system information of its serving cell can help a RedCap UE avoid those periodic check on its neighbor cells. |
| MediaTek | Yes | It is likely that not all basestations will be upgraded to Rel-17 and support RedCap operation. In this scenario, it is beneficial to provide information the UE as to which neighbour cells allow RedCap UE access. |

Issue B: For handover

The issue is discussed in R2-2104775.

the companies proposes than knowing whether a neighbour cell accepts access by Redcap or not can help gNB ensures not to handover a RedCap UE to a target cell that it can’t access. [QC]

The proposal in R2-2106487 is

Draft_Proposal 9: [2nd batch] [To discuss] RAN2 to discuss the need for coordination between gNBs on whether a neighbour/target gNB supports RedCap UEs. (Send LS to RAN3, if agreed).

Question 10: Is it needed of the coordination between gNBs on whether a neighbour/target gNB supports RedCap UEs, to avoid handover RedCap to a target cell that it can’t access?

| Companies | Yes or No | Comments |
| --- | --- | --- |
| OPPO | No | Existing handover preparation procedure already allows such, e.g. if target gNB does not support RedCap UEs, it can reject the handover request. |
| Ericsson | Yes | But not clear whether anything is needed in RAN2 – there could be RAN3 impact though. |
| Apple | Co-ordination is needed across gNBs. | The signaling details can be discussed if RAN2 or RAN3 needs to handle and if existing signaling is enough. |
| Qualcomm | Yes | Without this enhancement, a source cell has to reply on “trial and error” to select a target cell that supports RedCap, e.g. it picks a target cell based on measurement report, sends handover request to the selected target cell. If the request is denied, the source cell has to try another target cell. This increases likelihood of handover failure and handover latency. On the other hand, if the source cell is able to know whether a neighbor cell supports RedCap, it can select a target cell with much better chance of success. |
| MediaTek | Yes | The NW should ensure that a RedCap UE should only do a handover to a cell that supports RedCap operation. To enable this, inter-gNB coordination is needed. We can send an LS to RAN3 to trigger this discussion. |

- 2.2.4 RedCap specific cell (re)selection configurations

Issue A: whether to support the RedCap specific cell reselection priorities

The issue is discussed in R2-2105161, R2-2105472, R2-2105399.

Companies propose that the network may provide different cell reselection priority (or other parameters) for non-RedCap UE and RedCap UEs or for RedCap UEs with 1Rx branch. [ZTE, Samsung, Fujitsu]

Issue B: whether to support the RedCap specific cell selection parameter

The issue is discussed in R2-2105472, R2-2104790, R2-2105399, R2-2105443, R2-2106243.

Companies propose that the network may provide separate S-criteria parameters for a RedCap UE with 1 Rx branch. [Samsung, Thales, Fujitsu, DENSO, CMCC]

The proposal in R2-2106487 is

Draft_Proposal 10: [2nd batch] [To discuss] RAN2 to discuss whether to support RedCap specific Cell (re)selection parameters. (FFS only for 1 RX branches RedCap UE or all RedCap UEs; FFS on which parameters e.g. cell reselection priorities, cell reselection parameters and cell selection parameters)

Question 11: Do you prefer to support RedCap specific cell (re)selection parameters?

Please also comment on the details, if you support this: FFS only for 1 RX branches RedCap UE or all RedCap UEs; FFS on which parameters e.g. cell reselection priorities, cell reselection parameters and cell selection parameters.

| Companies | Yes or No | Comments |
| --- | --- | --- |
| OPPO | No | For the need of separate S-criteria parameters for a RedCap UE with 1 Rx branch, this should be discussed and decided by RAN1. Regarding Redcap specific reselection parameters, we think this is a general issue for every new feature, and we don’t think they are needed. |
| Ericsson | No |  |
| Apple | No strong view. |  |
| Qualcomm | Yes | For RedCap UEs which have reduced Rx capabilities, their Qrxlevmin and Qqualmin should be different from those for non-RedCap UEs to compensate the loss in their received power. |
| MediaTek | No | Given that the cell layout doesn’t change with the introduction of RedCap, we do not see a strong need to enhance cell reselection procedures. |

# Conclusion and proposals

Based on the above summary, following proposals are given.

- 3.1 Proposals in number order

TBD

- 3.2 For chair notes (proposals in priority order)

TBD

# Reference

- R2-2104775	Access and camping restrictions for RedCap UEs	Qualcomm Incorporated

- R2-2104777	Discussion on early identification and SI indication	CAICT

- R2-2104790	NR-REDCAP identification and SI indication	THALES

- R2-2104809	Discussion on RedCap UE’s access control	OPPO

- R2-2104911	Identification and access restrictions for RedCap UEs	vivo, Guangdong Genius

- R2-2104928	Early identification and camping restrictions  for RedCap UE	Intel Corporation

- R2-2105014	Methods for barring and for capability reporting	Sierra Wireless, S.A.

- R2-2105071	Discussion on UAC for Redcap devices	Xiaomi Communications

- R2-2105072	Discussion on Identification and UE access restrictions for Redcap devices	Xiaomi Communications

- R2-2105137	Power-saving aspects from cell access and camping of RedCap UEs	Apple

- R2-2105161	Identification and Access Restriction for RedCap	ZTE Corporation, Sanechips

- R2-2105235	Early indication & access restriction for RedCap UEs	Ericsson

- R2-2105320	Early Identification and camping restrictions for Redcap UEs	CATT

- R2-2105399	Camping restrictions of RedCap UE	Fujitsu

- R2-2105443	Camping restriction and cell selection criterion	DENSO CORPORATION

- R2-2105472	Access control for RedCap UEs	Samsung

- R2-2105540	Discussion on early indication design for Redcap UE	Spreadtrum Communications

- R2-2105635	Identification and access restriction of RedCap UE	Huawei, HiSilicon

- R2-2105793	Early identification and SI indication	NEC

- R2-2105814	Cell barring for REDCAP UEs	Lenovo, Motorola Mobility

- R2-2105879	Access for REDCAP UE	Nokia, Nokia Shanghai Bell

- R2-2105883	Identification and access restrictions of RedCap Ues	LG Electronics UK

- R2-2105957	Discussion on access and camping restrictions for RedCap UEs	Futurewei Technologies

- R2-2106052	Identification and restriction of RedCap UE	InterDigital

- R2-2106243	Access control for RedCap UEs	cmcc

- R2-2106244	Discussion on early identification	cmcc

- R2-2106274	Early identification and camping restrictions of RedCap UE	China Telecommunications

- R2-2106487	[Pre114-e][106][RedCap] Summary 8.12.2.2 - Identification and access restrictions (Huawei)