3GPP TSG-RAN WG2 Meeting #114-e                                                  R2-210xxxx

E-Meeting: May 19-27, 2021

Agenda item:	8.10.3.1

Source:	Qualcomm Incorporated

Title:	Report on [Offline 107] TAC update

Document for:	Discussion and Decision

# Introduction

This document provides summary report of the following email discussion.

- [AT114-e][107][NTN] TAC update (Qualcomm)

Initial scope: Discuss mechanism for TAC update

Initial intended outcome: Summary of the offline discussion with e.g.:

    - List of proposals for agreement (if any)

    - List of proposals that require online discussions

    - List of proposals that should not be pursued (if any)

Initial deadline (for companies' feedback): Friday 2021-05-21 10:00 UTC

Initial deadline (for rapporteur's summary in R2-2106525): Friday 2021-05-21 18:00

Proposals marked "for agreement" in R2-2106525 not challenged until Monday 2021-05-24 10:00 UTC will be declared as agreed via email by the session chair.

For the rest the discussion will continue online in the Monday CB session.

# Discussion

In RAN2#113e and RAN2#113bis-e meeting, following agreements were made.

Agreements:

1.	In NTN, the network may broadcast more than one TACs per PLMN in a cell, which is to up to network implementation.

2.	When the network stops broadcasting a TAC, the UE needs to know it (FFS on further details)

3.	RAN2 assume UE does not do TAU if one of the currently broadcasted TAC belongs to UE’s registration area.

4.	RAN2 confirm that in NTN when TAC change in SI happens is up to network implementation, i.e. it may not exactly sync up with real-time illumination on ground.

5.	Send a LS to CT1 and SA2, with Cc RAN3. The content is: currently RAN2 has two options on table, and the preference is “AS indicates all received TACs to NAS layer when more than one TAC per PLMN is broadcasted in NTN cell”, compared to “AS still reports only one TAC to NAS layer”, and ask for CT1’s feedback. Also include justification for RAN2 preference

One outstanding issue for RAN2 is how UE is notified when there is a change in TAC. There are currently three solutions brought up.

Option#1: Network can trigger SI update procedure (i.e., sending short paging message) [1]

Option#2: Time information is provided to  UE informing when to check SIB for update [2]

Option#3: Virtual Tracking Area (mapping of TA and time) is configured to UE [3]

Let’s begin with option#1. But before that it is important to understand the previous agreements.

RAN2#113bis agreement:

RAN2 confirm that in NTN when TAC change in SI happens is up to network implementation, i.e. it may not exactly sync up with real-time illumination on ground.

RAN2#113 agreement:

In NTN, the UE determines the TA based on the broadcast information (the use of other information is not excluded). In any case RAN2 will not go in a different direction than other groups

Figure 1 TAC update scenarios

Therefore, the UE would not know if it is physically in TAC1 or TAC2 geographical area. What the UE knows is it is in the geographical area covered by the broadcast TAC.

Q1. Is  it your understanding that if the UE does not map it’s geographical location to TAC, the UE registered with TAC1 might be physically present in (or moved into) the earth fixed tracking area logically belonging to TAC2 without triggering registration (where the UE may not be registered with TAC2)?

| Company | Hard TAC update (Yes/No) | Soft TAC update (Yes/No) | Comments |
| --- | --- | --- | --- |
| Samsung | The network may erroneously register the UE in TAC1 if it is physically in TAC2. |  | Sorry- the question is a little unclear. If the TAC change occurs on the ground while SIB is broadcasting the TAC(s) being illuminated when SIB was created, the UE physically present in TAC2 (just to the right of the TAC1-TAC2 border) would miss a page message from the network if the network has correctly registered the UE in TAC2. Furthermore, such UE would need to send a registration request to the network, because it would not find its correct TAC2 in the SIB (which is broadcasting TAC1 because TAC1 was being illuminated when SIB was created). To avoid such missed paging and to avoid registration signaling, the gNB should broadcast TACs that its beam/cell expects to illuminated uring the entire SIB window (=currently illuminated TACs plus any future TACs up to the end of the SIB period). In general, soft TAC update approach requires multiple TACs to be broadcast. Hence, to reduce SIB overhead, a more compact version of TAIs/TACs should be broadcast by including the common part just once and non-common parts as needed for TACs. |
| Qualcomm | Yes | Yes | Since UE is unaware of TAC boundary, it can freely move without registration as long as cell broadcasts UE’s registered TAC. So when the cell stops broadcasting UE’s registered TAC, there might be many UEs which are camping on the cell based on the same TAC that was just stopped. |

RAN2#113bis agreement

When the network stops broadcasting a TAC, the UE needs to know it (FFS on further details)

For option#1, network will send short paging message in all possible POs of all UEs at least once during SI modification period. Network would not know if the UE missed the paging or didn’t receive correctly, therefore, network implementation might repeat the paging to UEs.

This would require consuming precious paging resource. For UEs, any paging for MT call will be delayed. In addition, UE would not know if the paging message received is due to solely TAC update or really due to the change in important SI parameters.

Q2. Do you agree that paging for SI update notification triggered by the stop of TAC1 (but keeps broadcasting TAC2) is unnecessary to those UEs that are registered to TAC2? (i.e., paging does not indicate whether the cell stopped broadcasting TAC1 or TAC2).

| Company | Yes/No | Comments |
| --- | --- | --- |
| Samsung | Pl. see comment | If a gNB constructs the TAC List as we described in Q1 Response, this situation would not arise. Additionally, if one gNB cell stops broadcasting a given TAC, another cell should be broadcasting it and the UE would have done cell reselection to such better cell. Hence, it is possible to avoid such situation through proper gNB implementation. Please note that the soft update approach is very complex to implement at the gNB for Earth-moving beams, because the gNB needs to keep the time mapping between the Earth-fixed TACs and the beam coverage of its cells to determine exactly what TACs to broadcast in a given SIB at a given instant. To avoid missed paging or unnecessary paging to inform UEs, the gNB should properly create the TAC List as we specified in Q1 Response. |
| Qualcomm | Yes | If cell is broadcasting multiple TACs, say TAC1 and TAC2, then when cell stops broadcasting TAC1, the UEs which are registered to TAC2 are not required to receive the paging (not required to know the update). However, network does not know how many UEs or which UEs in the cell are registered to TAC1 but not TAC2 so it cannot avoid sending paging notification. |

Q3. Do you agree that paging for SI update notification triggered by the stop of a TAC is inefficient in terms of paging resource consumption?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Samsung | Yes | If the gNB does not create SIBs properly and if the UE procedure is not modified, excessive paging can occur. We need to broadcast a predictive TAC list (as we explained in Q1 Repsonse) and then enhance the UE procedure such that the UE processes the SIB only once per cell reselection. |
| Qualcomm | Yes | TS 38.331 says “Repetitions of SI change indication may occur within preceding modification period.” It will be very inefficient for such repetitions. In addition, main concern is that paging notification has to be sent every time a cell crosses a tracking area boundary, and this may become frequent in a cell. For example, with LEO and fixed/non-steerable antenna, cell velocity over the ground will be around 7.5 km/s. With tracking areas of 500 km in size, a new tracking area would be entered about every 67s and, separately, an old tracking area would be exited every 67s. |

Q4. Do you agree that paging for SI update notification triggered by the stop of a TAC will impact (or delay) delivery of paging for MT call?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Samsung | Yes | This can happen but it is possible to avoid this situation as explained earlier. |
| Qualcomm | Yes | If every possible PO in a paging cycle is used, the paging for MT call will be delayed. |

For Option#2, network does not need to trigger SI update notification procedure. Instead, a time information can be provided to let UE know when it can come back and check SIB to get updated on TAC.

According to TS 38.331 SIB9, the timeInfo field can provide, among other things, timeInfoUTC (39 bits) which indicates the UTC time counting the number of UTC seconds in 10 ms units since 00:00:00 on Gregorian calendar date 1 January, 1900 (midnight between Sunday, December 31, 1899 and Monday, January 1, 1900). However, according to [2], reference time can be SFN = 0 significantly reducing the signalling overhead. The additional overhead with such time information can be as low as 8 bits.

Also, according to [2] the time information signalling does not need to be per TAC (its not overhead per TAC). Rather it can be per PLMN or per cell.

Q5. Do you agree that UTC time does not need to be broadcast to provide (hard or soft) TAC update time information to UE to reduce signalling overhead (i.e., simply count since SFN = 0 can be broadcast)?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Samsung | Yes | If time-based option is used, the signaling load would reduce. However, the problem with both option 1 and option 2 is that SIB CANNOT be changed in the middle of the SIB window (e.g., SIB1 must have the same info in the SIB1 period of 160 ms) and TAC change on the Earth’s surface areas can occur anytime including within the SIB window. A realistic, reliable, and efficient soft TAC update approach in our view is creation of a predictive list by the gNB to cover the entire SIB period and requiring the UE to process TACs just once per cell reselection. For reliability, we should not implement option 1 or option 2. To avoid paging costs, we should not implement option 1. A suitable TAC list with compact TAI representation is adequate. Of course, the gNB and the UE complexity would be high whenever the soft update approach is used for Earth-moving beams. |
| Qualcomm | Yes | But we certainly can discuss the other options. |

Q6. Do you agree that time information on when a TAC broadcasting is stopped can be signalled to UE via NTN specific SIB that carries ephemeris (not necessarily in SIB1)?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Samsung | Yes | However, both option 1 and option 2 have reliability issue. |
| Qualcomm | Yes but | If a single time for all TACs per cell is signalled, then SIB1 can be used as the extra overhead would be small. |

Q7. Do you agree that providing cell specific time information for the next TAC update can be considered as an option for network to notify UE in addition to legacy SI update procedure? Please indicate for both hard and soft TAC mechanisms.

| Company | Hard TAC update (Yes/No) | Soft TAC update (Yes/No) | Comments |
| --- | --- | --- | --- |
| Samsung | No | No | The achievable time resolution would not be adequate to address intra-SIB period TAC change that can occur anytime and is essentially beyond the network control. |
| Qualcomm | Yes | Yes | Regardless of whether this is supported or not, network can always have option to send paging notification. However, main point is network should not have only this inefficient option of paging to notify TAC update. If such time information is absent, we can define that UE depends on paging notification. There is no additional complexity involved in broadcasting time information. |

The third option is virtual tracking area proposed in [3]. In this proposal, the UE and network need to have same mapping (for VTA#x) to be synchronized at what time UE belongs to what TA broadcast by a cell.

Table 1. Mapping between VTAs and TAs [2]

| Time Window | VTA # | TA Set | VTA # | TA Set | VTA # | TA Set | And so on… |
| --- | --- | --- | --- | --- | --- | --- | --- |
| t1 to t2 | 1 | X, Y | 2 | Y, Z | 3 | Z, A | . |
| t2 to t3 | 1 | Y, Z | 2 | Z, A | 3 | A, B | . |
| t3 to t4 | 1 | Z, A | 2 | A, B |  | B, C | . |
| And, so on… | . | . | . | . | . | . | . |

In this case, SA2 may need to be involved in the VTA configuration and mapping of TA.

Note that this concept is similar (though configuration detail can be different) to solution 1 proposed during SI phase, see TR 38.821 subclause “8.2.2.1  Solution 1: Timing window based Registration update and paging” and this was not agreed.

Q8. Do you agree RAN2 can consider the concept of a Virtual Tracking Area (VTA) as a candidate option for the Tracking Area management in addition to (and not as a replacement of) the soft TAC update approach? If yes, please indicate whether LS needs to be sent to SA2.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Samsung | Yes | Dear NTN Colleague: We understand that we are in a minority at this time about the VTA approach! However, for Earth-moving beams, implementation of the soft TAC update approach is quite complex for the gNB because the gNB needs to keep updating its TACs in the NTN cells it controls. Different NTN cells would cross different Earth-fixed TAs at different times and such changes can easily occur within SIB windows. Furthermore, the UE needs to compare its TAI List with multiple TAIs. The VTA approach simply uses the mapping between the Earth-fixed TAs and the TAC broadcast by the gNB. The UE and the AMF has such mapping. The gNB has zero impact and does not need to know the VTA-TAC mapping. SIB has zero impact. The gNB simply transmits a single constant TAC like R16 regardless of the geographic area it is illuminating. The UE compares this single broadcast TAC with its TAI List (=list of VTAs) only once per cell reselection. We request that RAN2 give the gNB vendors an option of the VTA-based TA management in addition to the soft TAC update approach. At the very least, the VTA approach should be compared with the soft TAC update approach for Earth-moving cells before deciding not to pursue it. The VTA approach can be used for all types of beams. We would be happy to answer any questions about the VTA approach. Feel free to review R2-2106069 (co-authored by Samsung, Apple, and Rakuten Mobile) for VTA details. Thank you. |
| Qualcomm | No | CN and UE need to have same mapping for sending paging, so this solution requires discussion in SA2. During the SI in SA2, there was a proposal in TR 23.737 (as part of Solution #12 in clause 6.12) to support virtual cells and virtual TAs but it was not agreed due to expected extra high impacts. We think that SA2 agreement on VTAs at this stage would be extremely unlikely (e.g. SA2 stage 2 finishes in June). |

# Conclusion

To be update

No table of contents entries found.

# References

[1] R2-2105611, “Discussion on remaining issues on soft TAU”, Huawei, HiSilicon.

[2] R2-2105432, “Hard and soft TAC update timing”, Qualcomm Incorporated.

[3] R2-2106069, “Tracking Area Management using Virtual Tracking Areas in an NTN”, Samsung Research America, Apple, Rakuten Mobile.