3GPP TSG-RAN WG2 #115 electronic	R2-20xxxxx

e-Meeting, Aug16th– 27th  2021

Agenda Item:	5.4.3

Source:	ZTE, Sanechips

Title:	Summary of offline [AT115-e][017][NR15] UE Capabilties III (ZTE)

Document for:	Discussion, Decision

# 1	Introduction

This contribution summarizes the following discussion:

[AT115-e][017][NR15] UE Capabilties III (ZTE)

Scope: Determine agreeable parts in a first phase, for agreeable parts agree on CRs. Treat R2-2107600, R2-2107601, R2-2106908, R2-2108346, R2-2106956, R2-2108038, R2-2108039, R2-2108718, R2-2108719, R2-2108749, R2-2108751,

Intended outcome: Report, agreed CRs if applicable

Deadline: Schedule 1

Deadline: Email discussions with Deadline Schedule 1: A first round with Deadline for comments Thursday Aug 19 1200 UTC to settle scope what is agreeable etc A Final round with Final deadline Thursday Aug 26 1200 UTC. to settle details / agree CRs etc.

Contact form

| Company | Email |
| --- | --- |
| Nokia | amaanat.ali@nokia.com |

# Discussion

## 2.1	Part 1: Intended to determine agreeable parts

Part 1 discussion is focusing on reaching conclusion whether the proposals/CRs can be agreed in principle, and Part 2 discussion would then focus on detailed changes for those agreeable contributions.

### MIMO

R2-2107600	Correction to the description of additionalActiveTCI-StatePDCCH	Apple	CR	Rel-15	38.306	15.14.0	0612	-	F	NR_newRAT-Core

R2-2107601	Correction to the description of additionalActiveTCI-StatePDCCH	Apple	CR	Rel-16	38.306	16.5.0	0613	-	A	NR_newRAT-Core

Q1: Do companies agree with the intention/modification of the CRs above?

| Company | Agree Intention (Yes or No) | Agree Modifications (Yes or No) | Comments |
| --- | --- | --- | --- |
| Nokia | No |  | As we read it, if UE indicates n1, then it must support also this bit in R16. This CR would change that and mean UE only supports single TCI state in total. |

### RI bit in EN-DC

R2-2106908	Reply LS on RI bit width for Cat5 UE in EN-DC mode (R1-2106108; contact: Nokia)	RAN1	LS in	Rel-15	NR_newRAT-Core	To:RAN2

RAN1 would like to thank RAN2 of the LS on RI bit width for Cat5 UE in EN-DC mode [R1-2104161/R2-2104583]. RAN1 would like to confirm the RAN2 interpretation that: “the RI bit width for a Cat5 UE is NOT affected by the number of MIMO layers it supports in EN-DC mode but only by the network configuration parameter maxLayersMIMO-r10, PBCH antenna ports and the UE category (without suffix), as in the legacy LTE.”

R2-2108346	Clarification to RI bit width for Cat5 in EN-DC	Nokia, Nokia Shanghai Bell	discussion	Rel-15	NR_newRAT-Core

Observation 1: RAN1 LS reply indicates the RAN2 understanding was correct and a Cat5 UE shall use 2-bit RI bit width in EN-DC by default even if it's not capable of 4-layer spatial multiplexing in EN-DC. Proposal 1: Capture a NOTE in the field description of fourLayerTM3-TM4-r15 in (Rel-16) 36.306 about the RI bit width for Cat5 UEs as per below.. NOTE 1: Cat5 UE supporting only 2-layer spatial multiplexing for EN-DC will still determine the RI bit width according TS36.212 [22], which means it may still use 2-bit RI bit width despite not supporting more than 2-layer spatial multiplexing.

Q2: Do companies agree to capture the above NOTE 1 in the field description of fourLayerTM3-TM4-r15 in (Rel-16) 36.306 about the RI bit width for Cat5 UEs?

| Company | Yes or No | Comments |
| --- | --- | --- |
| Nokia | Yes | Proponent, we would like to clarify this to avoid the IODT issue. |

### Intra-band and Inter-band UE capability

R2-2106956	Reply LS on the Intra-band and Inter-band (NG)EN-DC/NE-DC Capabilities (R4-2107907; contact: ZTE)	RAN4	LS in	Rel-15	NR_newRAT-Core	To:RAN2	Cc:RAN1

Moved from 5.1

Question 1: For which (NG)EN-DC/NE-DC BC types the above capabilities (ul-TimingAlignmentEUTRA-NR/ pa-PhaseDiscontinuityImpacts /dualPA-Architecture/ /simultaneousRxTxInterBandENDC /asyncIntraBandENDC ) are applicable? Answer: From RAN4 perspective, - dualPA-Architecture is applicable to (NG)EN-DC/NE-DC BC Type 1, Type 2 and Type 5, - simultaneousRxTxInterBandENDC is applicable to (NG)EN-DC/NE-DC BC Type 2, Type 3 and Type 4, - asyncIntraBandENDC is applicable to (NG)EN-DC/NE-DC BC Type 1, Type 2, Type 3 and Type 5. Question 2: If the capability ul-TimingAlignmentEUTRA-NR/ pa-PhaseDiscontinuityImpacts /dualPA-Architecture/ asyncIntraBandENDC are applicable to the (NG)EN-DC/NE-DC BC Type 1/2/3, whether they are used to indicate the restriction to the intra-band (NG)EN-DC/NE-DC BC part? Answer: Yes, dualPA-Architecture/ asyncIntraBandENDC are used to indicate the restriction to the intra-band (NG)EN-DC/NE-DC BC part.

R2-2108038	CR on the Intra-band and Inter-band EN-DC Capabilities - R15	ZTE Corporation, Sanechips	CR	Rel-15	38.306	15.14.0	0517	3	F	NR_newRAT-Core	R2-2105182

R2-2108039	CR on the Intra-band and Inter-band EN-DC Capabilities - R16	ZTE Corporation, Sanechips	CR	Rel-16	38.306	16.5.0	0518	3	A	NR_newRAT-Core	R2-2105183

About this topic, in the previous meeting, RAN2 sent an LS (R2-2104550) to RAN1/4 to ask 2 questions as below:

Question 1: For which (NG)EN-DC/NE-DC BC types the above capabilities (ul-TimingAlignmentEUTRA-NR/ pa-PhaseDiscontinuityImpacts /dualPA-Architecture/ /simultaneousRxTxInterBandENDC /asyncIntraBandENDC ) are applicable? Question 2: If the capability ul-TimingAlignmentEUTRA-NR/ pa-PhaseDiscontinuityImpacts /ul-dualPA-Architecture/ asyncIntraBandENDC are applicable to the (NG)EN-DC/NE-DC BC Type 1/2/3, whether they are used to indicate the restriction to the intra-band (NG)EN-DC/NE-DC BC part?

RAN4 has sent the reply LS as above, however, we haven’t got the reply LS from RAN1 even RAN1 has made the below agreement in the last meeting.

Agreement: Regarding questions mentioned in RAN2 LS R1-2104162/R2-2104550, - UE feature 6-24 (ul-TimingAlignmentEUTRA-NR) is applicable to Type 1 and Type 2 (NG)EN-DC BC types. - UE feature 6-23 (pa-PhaseDiscontinuityImpacts) is applicable to Type 1 and Type 2 (NG)EN-DC/NE-DC BC types. - Both 6-24 (ul-TimingAlignmentEUTRA-NR) and 6-23 (pa-PhaseDiscontinuityImpacts) are used to indicate the restriction to the intra-band (NG)EN-DC/NE-DC BC part. - RAN1 further discusses whether 6-24 (ul-TimingAlignmentEUTRA-NR) and 6-23 (pa-PhaseDiscontinuityImpacts) can be applicable to Type 5 (NG)EN-DC/NE-DC BC type by taking RAN4 discussion outcome into account.

To accelerate the discussion, we’d like to collect the companies views on the modifications to the RAN4 related features first, including dualPA-Architecture/ /simultaneousRxTxInterBandENDC /asyncIntraBandENDC. For the RAN1 feature (ul-TimingAlignmentEUTRA-NR/pa-PhaseDiscontinuityImpacts), we can wait for the RAN1’s reply LS.

Q3: Do companies agree that the modifications to the dualPA-Architecture/ /simultaneousRxTxInterBandENDC /asyncIntraBandENDC in the R2-2108038/R2-2108039 are aligned with RAN4’s Reply LS R2-2106956?

| Company | Yes or No | Comments |
| --- | --- | --- |
| Nokia | Yes | We did not check this in detail but the changes seem aligned to the response in the LS. |

### IMS Capability

R2-2108718	Clarification on IMS video over split bearer in (NG)EN-DC	Google Inc.	CR	Rel-15	36.306	15.10.0	1811	1	F	NR_newRAT-Core	R2-2105188

R2-2108719	Clarification on IMS video over split bearer in (NG)EN-DC	Google Inc.	CR	Rel-16	36.306	16.5.0	1812	1	A	NR_newRAT-Core	R2-2105189

Q4: Do companies agree with the intention/modification of the CRs above?

| Company | Agree Intention (Yes or No) | Agree Modifications (Yes or No) | Comments |
| --- | --- | --- | --- |
| Nokia | No |  | The discussion is not in RAN2. |

R2-2108749	Clarification on IMS video over split bearer in NR-DC and NE-DC	Google Inc.	CR	Rel-15	38.306	15.14.0	0581	1	F	NR_newRAT-Core	R2-2105190

R2-2108751	Clarification on IMS video over split bearer in NR-DC and NE-DC	Google Inc.	CR	Rel-16	38.306	16.5.0	0582	1	A	NR_newRAT-Core	R2-2105191

Q5: Do companies agree with the intention/modification of the CRs above?

| Company | Agree Intention (Yes or No) | Agree Modifications (Yes or No) | Comments |
| --- | --- | --- | --- |
| Nokia | No |  | The discussion is not in RAN2. |

## 2.2	Part 2: Intended to progress discussion on agreeable parts

- To be updated after discussion on part 1 -

# 3	Conclusion

# 4	References

- R2-2107600	Correction to the description of additionalActiveTCI-StatePDCCH	Apple	CR	Rel-15	38.306	15.14.0	0612	-	F	NR_newRAT-Core

- R2-2107601	Correction to the description of additionalActiveTCI-StatePDCCH	Apple	CR	Rel-16	38.306	16.5.0	0613	-	A	NR_newRAT-Core

- R2-2106908	Reply LS on RI bit width for Cat5 UE in EN-DC mode (R1-2106108; contact: Nokia)	RAN1	LS in	Rel-15	NR_newRAT-Core	To:RAN2

- R2-2108346	Clarification to RI bit width for Cat5 in EN-DC	Nokia, Nokia Shanghai Bell	discussion	Rel-15	NR_newRAT-Core

- R2-2106956	Reply LS on the Intra-band and Inter-band (NG)EN-DC/NE-DC Capabilities (R4-2107907; contact: ZTE)	RAN4	LS in	Rel-15	NR_newRAT-Core	To:RAN2	Cc:RAN1

- R2-2108038	CR on the Intra-band and Inter-band EN-DC Capabilities - R15	ZTE Corporation, Sanechips	CR	Rel-15	38.306	15.14.0	0517	3	F	NR_newRAT-Core	R2-2105182

- R2-2108039	CR on the Intra-band and Inter-band EN-DC Capabilities - R16	ZTE Corporation, Sanechips	CR	Rel-16	38.306	16.5.0	0518	3	A	NR_newRAT-Core	R2-2105183

- R2-2108718	Clarification on IMS video over split bearer in (NG)EN-DC	Google Inc.	CR	Rel-15	36.306	15.10.0	1811	1	F	NR_newRAT-Core	R2-2105188

- R2-2108719	Clarification on IMS video over split bearer in (NG)EN-DC	Google Inc.	CR	Rel-16	36.306	16.5.0	1812	1	A	NR_newRAT-Core	R2-2105189

- R2-2108749	Clarification on IMS video over split bearer in NR-DC and NE-DC	Google Inc.	CR	Rel-15	38.306	15.14.0	0581	1	F	NR_newRAT-Core	R2-2105190

- R2-2108751	Clarification on IMS video over split bearer in NR-DC and NE-DC	Google Inc.	CR	Rel-16	38.306	16.5.0	0582	1	A	NR_newRAT-Core	R2-2105191

- R2-2104550  LS on the Intra-band and Inter-band (NG)EN-DC/NE-DC Capabilities  To: RAN4/RAN1 Rel15 NR_newRAT-Core