3GPP TSG-RAN WG2 #115 electronic	R2-20xxxxx

e-Meeting, Aug16th– 27th  2021

Agenda Item:	5.4.3

Source:	ZTE, Sanechips

Title:	Summary of offline [AT115-e][017][NR15] UE Capabilties III (ZTE)

Document for:	Discussion, Decision

# 1	Introduction

This contribution summarizes the following discussion:

[AT115-e][017][NR15] UE Capabilties III (ZTE)

Scope: Determine agreeable parts in a first phase, for agreeable parts agree on CRs. Treat R2-2107600, R2-2107601, R2-2106908, R2-2108346, R2-2106956, R2-2108038, R2-2108039, R2-2108718, R2-2108719, R2-2108749, R2-2108751,

Intended outcome: Report, agreed CRs if applicable

Deadline: Schedule 1

Deadline: Email discussions with Deadline Schedule 1: A first round with Deadline for comments Thursday Aug 19 1200 UTC to settle scope what is agreeable etc A Final round with Final deadline Thursday Aug 26 1200 UTC. to settle details / agree CRs etc.

Contact form

| Company | Email |
| --- | --- |
| Nokia | amaanat.ali@nokia.com |
| Qualcomm Incorporated | mkitazoe@qti.qualcomm.com |
| Lenovo | hchoi5@lenovo.com |

# Discussion

## 2.1	Part 1: Intended to determine agreeable parts

Part 1 discussion is focusing on reaching conclusion whether the proposals/CRs can be agreed in principle, and Part 2 discussion would then focus on detailed changes for those agreeable contributions.

### MIMO

R2-2107600	Correction to the description of additionalActiveTCI-StatePDCCH	Apple	CR	Rel-15	38.306	15.14.0	0612	-	F	NR_newRAT-Core

R2-2107601	Correction to the description of additionalActiveTCI-StatePDCCH	Apple	CR	Rel-16	38.306	16.5.0	0613	-	A	NR_newRAT-Core

Q1: Do companies agree with the intention/modification of the CRs above?

| Company | Agree Intention (Yes or No) | Agree Modifications (Yes or No) | Comments |
| --- | --- | --- | --- |
| Nokia | No |  | As we read it, if UE indicates n1, then it must support also this bit in R16. This CR would change that and mean UE only supports single TCI state in total. |
| Qualcomm Incorporated | Yes |  | The current text just says the UE capability parameter is applicable (as opposed to mandatory) when the UE indicates ‘n1’ in maxNumberActiveTCI-PerBWP. |

### RI bit in EN-DC

R2-2106908	Reply LS on RI bit width for Cat5 UE in EN-DC mode (R1-2106108; contact: Nokia)	RAN1	LS in	Rel-15	NR_newRAT-Core	To:RAN2

RAN1 would like to thank RAN2 of the LS on RI bit width for Cat5 UE in EN-DC mode [R1-2104161/R2-2104583]. RAN1 would like to confirm the RAN2 interpretation that: “the RI bit width for a Cat5 UE is NOT affected by the number of MIMO layers it supports in EN-DC mode but only by the network configuration parameter maxLayersMIMO-r10, PBCH antenna ports and the UE category (without suffix), as in the legacy LTE.”

R2-2108346	Clarification to RI bit width for Cat5 in EN-DC	Nokia, Nokia Shanghai Bell	discussion	Rel-15	NR_newRAT-Core

Observation 1: RAN1 LS reply indicates the RAN2 understanding was correct and a Cat5 UE shall use 2-bit RI bit width in EN-DC by default even if it's not capable of 4-layer spatial multiplexing in EN-DC. Proposal 1: Capture a NOTE in the field description of fourLayerTM3-TM4-r15 in (Rel-16) 36.306 about the RI bit width for Cat5 UEs as per below.. NOTE 1: Cat5 UE supporting only 2-layer spatial multiplexing for EN-DC will still determine the RI bit width according TS36.212 [22], which means it may still use 2-bit RI bit width despite not supporting more than 2-layer spatial multiplexing.

Q2: Do companies agree to capture the above NOTE 1 in the field description of fourLayerTM3-TM4-r15 in (Rel-16) 36.306 about the RI bit width for Cat5 UEs?

| Company | Yes or No | Comments |
| --- | --- | --- |
| Nokia | Yes | Proponent, we would like to clarify this to avoid the IODT issue. |
| Qualcomm Incorporated | Yes |  |

### Intra-band and Inter-band UE capability

R2-2106956	Reply LS on the Intra-band and Inter-band (NG)EN-DC/NE-DC Capabilities (R4-2107907; contact: ZTE)	RAN4	LS in	Rel-15	NR_newRAT-Core	To:RAN2	Cc:RAN1

Moved from 5.1

Question 1: For which (NG)EN-DC/NE-DC BC types the above capabilities (ul-TimingAlignmentEUTRA-NR/ pa-PhaseDiscontinuityImpacts /dualPA-Architecture/ /simultaneousRxTxInterBandENDC /asyncIntraBandENDC ) are applicable? Answer: From RAN4 perspective, - dualPA-Architecture is applicable to (NG)EN-DC/NE-DC BC Type 1, Type 2 and Type 5, - simultaneousRxTxInterBandENDC is applicable to (NG)EN-DC/NE-DC BC Type 2, Type 3 and Type 4, - asyncIntraBandENDC is applicable to (NG)EN-DC/NE-DC BC Type 1, Type 2, Type 3 and Type 5. Question 2: If the capability ul-TimingAlignmentEUTRA-NR/ pa-PhaseDiscontinuityImpacts /dualPA-Architecture/ asyncIntraBandENDC are applicable to the (NG)EN-DC/NE-DC BC Type 1/2/3, whether they are used to indicate the restriction to the intra-band (NG)EN-DC/NE-DC BC part? Answer: Yes, dualPA-Architecture/ asyncIntraBandENDC are used to indicate the restriction to the intra-band (NG)EN-DC/NE-DC BC part.

R2-2108038	CR on the Intra-band and Inter-band EN-DC Capabilities - R15	ZTE Corporation, Sanechips	CR	Rel-15	38.306	15.14.0	0517	3	F	NR_newRAT-Core	R2-2105182

R2-2108039	CR on the Intra-band and Inter-band EN-DC Capabilities - R16	ZTE Corporation, Sanechips	CR	Rel-16	38.306	16.5.0	0518	3	A	NR_newRAT-Core	R2-2105183

About this topic, in the previous meeting, RAN2 sent an LS (R2-2104550) to RAN1/4 to ask 2 questions as below:

Question 1: For which (NG)EN-DC/NE-DC BC types the above capabilities (ul-TimingAlignmentEUTRA-NR/ pa-PhaseDiscontinuityImpacts /dualPA-Architecture/ /simultaneousRxTxInterBandENDC /asyncIntraBandENDC ) are applicable? Question 2: If the capability ul-TimingAlignmentEUTRA-NR/ pa-PhaseDiscontinuityImpacts /ul-dualPA-Architecture/ asyncIntraBandENDC are applicable to the (NG)EN-DC/NE-DC BC Type 1/2/3, whether they are used to indicate the restriction to the intra-band (NG)EN-DC/NE-DC BC part?

RAN4 has sent the reply LS as above, however, we haven’t got the reply LS from RAN1 even RAN1 has made the below agreement in the last meeting.

Agreement: Regarding questions mentioned in RAN2 LS R1-2104162/R2-2104550, - UE feature 6-24 (ul-TimingAlignmentEUTRA-NR) is applicable to Type 1 and Type 2 (NG)EN-DC BC types. - UE feature 6-23 (pa-PhaseDiscontinuityImpacts) is applicable to Type 1 and Type 2 (NG)EN-DC/NE-DC BC types. - Both 6-24 (ul-TimingAlignmentEUTRA-NR) and 6-23 (pa-PhaseDiscontinuityImpacts) are used to indicate the restriction to the intra-band (NG)EN-DC/NE-DC BC part. - RAN1 further discusses whether 6-24 (ul-TimingAlignmentEUTRA-NR) and 6-23 (pa-PhaseDiscontinuityImpacts) can be applicable to Type 5 (NG)EN-DC/NE-DC BC type by taking RAN4 discussion outcome into account.

To accelerate the discussion, we’d like to collect the companies views on the modifications to the RAN4 related features first, including dualPA-Architecture/ /simultaneousRxTxInterBandENDC /asyncIntraBandENDC. For the RAN1 feature (ul-TimingAlignmentEUTRA-NR/pa-PhaseDiscontinuityImpacts), we can wait for the RAN1’s reply LS.

Q3: Do companies agree that the modifications to the dualPA-Architecture/ /simultaneousRxTxInterBandENDC /asyncIntraBandENDC in the R2-2108038/R2-2108039 are aligned with RAN4’s Reply LS R2-2106956?

| Company | Yes or No | Comments |
| --- | --- | --- |
| Nokia | Yes | We did not check this in detail but the changes seem aligned to the response in the LS. |
| Qualcomm Incorporated | Yes, but | We are still waiting for RAN1 input for ul-TimingAlignmentEUTRA-NR and pa-PhaseDiscontinuityImpacts, but we have the same understanding as proposed in the CRs. |

### IMS Capability

R2-2108718	Clarification on IMS video over split bearer in (NG)EN-DC	Google Inc.	CR	Rel-15	36.306	15.10.0	1811	1	F	NR_newRAT-Core	R2-2105188

R2-2108719	Clarification on IMS video over split bearer in (NG)EN-DC	Google Inc.	CR	Rel-16	36.306	16.5.0	1812	1	A	NR_newRAT-Core	R2-2105189

Q4: Do companies agree with the intention/modification of the CRs above?

| Company | Agree Intention (Yes or No) | Agree Modifications (Yes or No) | Comments |
| --- | --- | --- | --- |
| Nokia | No |  | The discussion is not in RAN2. |
| Qualcomm Incorporated | Yes |  | Makes sense to apply the same handling as IMS voice. |
| Lenovo | No |  | The specified note is about IMS voice, so it looks odd to mention IMS video. In general, we wonder about the reason for change as it seems a pure UE implementation problem: As far we understood the scenario is that the UE indicates to NW that it supports split bearer (by indicating drb-TypeSplit-r12) and (NG)EN-DC (by indicating en-DC-r15 and ng-EN-DC-r15). But only for IMS video the UE doesn’t want to support split bearer. So question to proponent is what kind of extra implementation burden do they have for IMS video in case of split bearer? |

R2-2108749	Clarification on IMS video over split bearer in NR-DC and NE-DC	Google Inc.	CR	Rel-15	38.306	15.14.0	0581	1	F	NR_newRAT-Core	R2-2105190

R2-2108751	Clarification on IMS video over split bearer in NR-DC and NE-DC	Google Inc.	CR	Rel-16	38.306	16.5.0	0582	1	A	NR_newRAT-Core	R2-2105191

Q5: Do companies agree with the intention/modification of the CRs above?

| Company | Agree Intention (Yes or No) | Agree Modifications (Yes or No) | Comments |
| --- | --- | --- | --- |
| Nokia | No |  | The discussion is not in RAN2. |
| Qualcomm Incorporated | Yes |  | Makes sense to apply the same handling as IMS voice. |
| Lenovo | No |  | Same comments as for Q4. Proponent to clarify the scenario (the UE supports split bearer and NR-DC and NE-DC?) and the extra implementation burden for IMS video in case of split bearer. |

## 2.2	Part 2: Intended to progress discussion on agreeable parts

- To be updated after discussion on part 1 -

# 3	Conclusion

# 4	References

- R2-2107600	Correction to the description of additionalActiveTCI-StatePDCCH	Apple	CR	Rel-15	38.306	15.14.0	0612	-	F	NR_newRAT-Core

- R2-2107601	Correction to the description of additionalActiveTCI-StatePDCCH	Apple	CR	Rel-16	38.306	16.5.0	0613	-	A	NR_newRAT-Core

- R2-2106908	Reply LS on RI bit width for Cat5 UE in EN-DC mode (R1-2106108; contact: Nokia)	RAN1	LS in	Rel-15	NR_newRAT-Core	To:RAN2

- R2-2108346	Clarification to RI bit width for Cat5 in EN-DC	Nokia, Nokia Shanghai Bell	discussion	Rel-15	NR_newRAT-Core

- R2-2106956	Reply LS on the Intra-band and Inter-band (NG)EN-DC/NE-DC Capabilities (R4-2107907; contact: ZTE)	RAN4	LS in	Rel-15	NR_newRAT-Core	To:RAN2	Cc:RAN1

- R2-2108038	CR on the Intra-band and Inter-band EN-DC Capabilities - R15	ZTE Corporation, Sanechips	CR	Rel-15	38.306	15.14.0	0517	3	F	NR_newRAT-Core	R2-2105182

- R2-2108039	CR on the Intra-band and Inter-band EN-DC Capabilities - R16	ZTE Corporation, Sanechips	CR	Rel-16	38.306	16.5.0	0518	3	A	NR_newRAT-Core	R2-2105183

- R2-2108718	Clarification on IMS video over split bearer in (NG)EN-DC	Google Inc.	CR	Rel-15	36.306	15.10.0	1811	1	F	NR_newRAT-Core	R2-2105188

- R2-2108719	Clarification on IMS video over split bearer in (NG)EN-DC	Google Inc.	CR	Rel-16	36.306	16.5.0	1812	1	A	NR_newRAT-Core	R2-2105189

- R2-2108749	Clarification on IMS video over split bearer in NR-DC and NE-DC	Google Inc.	CR	Rel-15	38.306	15.14.0	0581	1	F	NR_newRAT-Core	R2-2105190

- R2-2108751	Clarification on IMS video over split bearer in NR-DC and NE-DC	Google Inc.	CR	Rel-16	38.306	16.5.0	0582	1	A	NR_newRAT-Core	R2-2105191

- R2-2104550  LS on the Intra-band and Inter-band (NG)EN-DC/NE-DC Capabilities  To: RAN4/RAN1 Rel15 NR_newRAT-Core