3GPP TSG-RAN WG2 Meeting #115-e	R2-2109053

Elbonia, Online, 16 – 27th of August 2021

Agenda item:	6.1.4.1.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Report from [AT115-e][024][NR16] DAPS & CHO (Nokia)

WID/SID:	NR_Mob_enh-Core - Release 16

Document for:	Discussion and Decision

# 1	Brief scope of the paper

This document aims at collecting companies’ views regarding the following CHO and DAPS related CRs:

[AT115-e][024][NR16] DAPS & CHO (Nokia)

Scope: Await on-line, take into account online outcomes. Determine agreeable parts and agree CRs, Treat R2-2108090, R2-2107775, R2-2107085, R2-2107086, R2-2107087, R2-2107776, R2-2108817, R2-2106933, R2-2108164, R2-2107526, R2-2107527, R2-2108102, R2-2108103, R2-2108776, R2-2108777

Intended outcome: Report, Agreed CRs.

Deadline: on-line first, Schedule 1

The following sections elaborate on the topics listed in the scope above.

# 2	DAPS

## 2.1 	On bearer release handling for DAPS

The authors of [1] discuss the issue of bearer releasing in DAPS HO. It is mentioned that the specification does not describe how the bearers which are released at DAPS HO command are handled when the DAPS fallback occurs. [1] proposes two options how to address the problem: either to postpone the release of RLC bearers until random access success (Option 1) or to define how RLC bearers are restored on DAPS fallback (Option 2). Companies are asked to provide their views regarding the problem and which option shall be used for addressing it.

| Question 1: Do you agree with the problem stated in [1]? Which option should be adopted to address it? |  |  |
| --- | --- | --- |
| Company | Yes/No | Comment |
| ZTE | No | We have discussed the similar issue on bearer release (R2-2102821/2822) at RAN2#113bis-e meeting. And it’s agreed that the intent is correct (UE only does fallback to non-DAPS bearers configured by source) but CR is not needed. |
| MediaTek | Clarification is needed | It is not so clear to us whether it is still a DAPS bearer if the source RLC bearer is released and target RLC bearer are added. This bearer could not receive DL data from source during DAPS handover. Anyway, in our understanding, for the RLC bearers which the NW releases in RRCReconfiguration which commands DAPS handover, the correct UE behavior should be - When DAPS handover is initiated, the RLC bearer is suspended (No transmit or receive). The RLC bearer configuration is kept. - When DAPS handover succeeds, the RLC bearer is released. - When DAPS handover fails, the original RLC bearer configuration is restored/resumed We can discuss how to clarify the SPEC once we have common understanding. |
| Ericsson | No | We would support the intent in principle, but this type of change has already been concluded as not essential. This issue was discussed at RAN2#1113bis, based on E/// CRs to both LTE and NR: Addition/release of bearers during DAPS: R2-2102821 Addition and release of DRBs in DAPS HO Command Ericsson CR Rel-16 36.331 16.4.0 4607 - F LTE_feMob-Core - Rapporteur thinks we can either go with the CRs or have clarification on non-DAPS bearer. - Ericsson clarifies this is to handle what happens if UE falls back to source while target added non-DAPS bearers. QC thinks Ericsson is correct but likely UEs will not implement it this way. Could just add a NOTE. Huawei thinks the CR is not needed and we would have 3 types of non-DAPS DRBs with this. There's no real risk of errors. - Ericsson wonders why we wouldn't capture this in specification. - The intent is correct (UE only does fallback to non-DAPS bearers configured by source) but CR is not needed. R2-2102822 Addition and release of DRBs in DAPS HO Command Ericsson CR Rel-16 38.331 16.4.0 2478 - F NR_Mob_enh-Core - The intent is correct (UE only does fallback to non-DAPS bearers configured by source) but CR is not needed. |
| QCOM | Yes Option-1 | Agree with the intention, clarification is needed. Option-1 where UE delays the release of the bearer(s) till DAPS is successful seems the simplest approach to resolve this issue. |
| Huawei, HiSilicon | No | No need to update the spec. Current wording “revert back to the UE configuration used for the DRB in the source PCell” has covered all aspects. |
| NEC | No | We agree with ZTE and Ericsson that we have discussed similar issue before, and concluded that CR is not needed. |
| Sharp | No | Agree that current wording “revert back to the UE configuration used for the DRB in the source PCell” has covered this. |
| Samsung | No | We already discussed this issue several meetings ago and concluded no change. |
| Apple | No | We agree with the intention, and as Ericsson and ZTE mentioned, RAN2 has discussed this issue and concluded that no CR was needed. |
| OPPO | No | Agree with the motivation of the CR, while the current spec already covers the issue for non-daps bearer handling as following, the CR is not needed: 3> for each non DAPS bearer: 4> revert back to the UE configuration used for the DRB in the source PCell, includes PDCP, RLC states variables, the security configuration and the data stored in transmission and reception buffers in PDCP and RLC entities ; |
| LG | No | We agree with the intention of the CR. But we are not sure if the consequence without this CR is really serious. On the other hand, the comments from ZTE seems also valid in that R2-2102821/2822 treats the similar issue, i.e., the handling of RBs that are released by DAPS HO command at DAPS fallback |
| CATT | No | As has been pointed out by some companies this had been discussed. |
| Lenovo | No | Agree with ZTE and Ericsson that we have discussed the similar issue. |
| vivo | No | Agree ZTE and Ericsson. Following the agreement in RAN2#113bis-e meeting, we agree with the intention for this CR (UE only does fallback to non-DAPS bearers configured by source), but think the CR is not needed. |
| Intel | No | Agree with Ericsson and ZTE, the issue has been discussed and concluded before. In addition, as mentioned by Huawei, current sentence has covered the issue. 3> for each non DAPS bearer: 4> revert back to the UE configuration used for the DRB in the source PCell, includes PDCP, RLC states variables, the security configuration and the data stored in transmission and reception buffers in PDCP and RLC entities ; |
| ITRI | No | Agree with ZTE and Ericsson that the issue has been discussed. |
| Nokia | Yes | Proponent. The current wording only says about reverting back to the configuration of DRB in source cell. But the problem is that as a part of processing the received configuration the DRB itself is released. The bearer does not exist anymore. If that is a common understanding the UE keeps the ‘’released bearer” and can later restore it at fallback, then OK. But still we think that shall be expressed somehow more clearly. |
| Google | No | There is no need for the CR as the sentence “revert back to the UE configuration used for the DRB in the source PCell” covers the intention. In addition, similar changes were discussed and concluded not needed, as indicated by Ericsson and ZTE. |

Summary for Q1:

- 18 companies responded to this question.

- 3 companies think clarification is needed, the remainder claims this was discussed and concluded no specification changes are required

- The rapporteur still thinks the behavior is still unclear in the specification, but companies claim it is clear how to implement this case.

Proposal 1: The changes in R2-2108090 are not pursued.

## 2.2 	Source SDAP configuration fallback

In [2] it is suggested to remove the part on reverting back to the SDAP configuration used in the source PCell in section 5.3.5.8.3 of NR RRC. It is claimed that there is nothing to revert, as the target SDAP configuration is not applied until UL switching. Do you agree with the changes proposed in [2]?

| Question 2: Do you agree with the change proposed in [2]? |  |  |
| --- | --- | --- |
| Company | Yes/No | Comment |
| ZTE | Yes | The target SDAP configuration is not applied until indication of successful completion of random access towards target cell is received (i.e. only source SDAP configuration is applied), so no need to revert back to the source SDAP configuration. |
| MediaTek | No strong view | Agree that the SDAP configuration is not applied so not necessary to revert upon handover failure. We however also think that keeping this clause is also fine. |
| Ericsson | No | Not an essential change. The line removed in the CR does not have any effect and is not necessary. So with that logic, removing it does not have effect either. |
| QCOM | No strong view | The source configuration is still there if DAPS fails, so the clause is useless … leaning toward removing it, but will go with majority. |
| Huawei, HiSilicon | No | The reasoning given in cover sheet is only for DAPS bears, but the UE behaviour “revert back to the SDAP configuration used in the source PCell;” is common for both DAPS bears and non-DAPS bears. For non-DAPS bears, it is still necessary to make it clear. |
| NEC | Yes | The sentence will mislead the readers that the UE has already applied target SDAP configuration. And keeping this useless and maybe even misleading sentence is no good to the specification. |
| Sharp | No | Agree that current wording “revert back to the UE configuration used for the DRB in the source PCell” has covered this. |
| Samsung | Yes | We are fine with this. |
| Apple | No strong view | We agree with the analysis but think current spec can work. We can follow majority view. |
| OPPO | Yes | Agree with the CR. |
| LG | Yes | The SDAP configuration for target is applied only when the DAPS handover is successfully performed. Thus, there is no reason to revert back the SDAP configuration for source when fallback procedure is performed. |
| Lenovo | Yes | Agree with CR. |
| Vivo | No strong view | Since the target SDAP configuration is not applied until UL switch, the behavior of SDAP configuration revert is not necessary. However, the change is not essential. |
| Intel | No | We agree the sentence is useless. But current sentence also will not cause any problem. Therefore we do not see the need for the change. |
| ITRI | Yes | Agree with the CR. |
| Nokia | No strong view | Agree with some of the preceding comments. The sentence is not correct, but even if it stays in the standard, there will be no issue in UE’s behavior. |
| Google | Yes | Although nothing is broken, we prefer to remove the incorrect sentence. |

Summary for Q2:

- 17 companies responded to this question.

- 8 companies are in favour of supporting this CR. 4 companies are against, others think it is not essential, but also OK to have

- As the currently available procedure will not cause an issue (even if there is some inconsistency), the proposal is not to pursue the CR, as there is no clear majority for having it.

Proposal 2: Changes in R2-2107775 are not pursued.

## 2.3 	T301 for DAPS

The authors of [3][4][5] discuss the issue concerning T301 handling in DAPS HO. It has been noticed that UE in DAPS is configured with various timers for the target cell, provided in RLF-TimersAndConstants. However, there is no value for T301 which may negatively impact the potential re-establishment performance. Two options how to address the problem are provided in [4] and [5].

| Question 3: Do you agree with the problem found in [3]? Which alternative to resolve it do you prefer? |  |  |
| --- | --- | --- |
| Company | Yes/No | Answer (Alt-1, Alt-2, other) |
| ZTE | No | Since the UE will acquire SIB1 of the target cell after completion of RA to the target cell, the UE can use the T301 value included in ue-TimersAndConstants received in SIB1, as the legacy HO. So the change is not needed. - if reconfigurationWithSync was included in spCellConfig of an MCG or SCG, and when MAC of an NR cell group successfully completes a Random Access procedure triggered above: */omit unrelated part/* 3> if the active downlink BWP, which is indicated by the firstActiveDownlinkBWP-Id for the target SpCell of the MCG, has a common search space configured by searchSpaceSIB1: 4> acquire the SIB1, which is scheduled as specified in TS 38.213 [13], of the target SpCell of the MCG; 4> upon acquiring SIB1, perform the actions specified in clause 5.2.2.4.2; |
| MediaTek | No | There is no need to have T301 value in RLF-TimersAndConstants IE, because T301 is started only during RRC connection re-establishment when SIB1 from that cell (in which RRCRestablishmentRequest is to be sent) has always been received, i.e. the UE has ue-TimersAndConstants. |
| Ericsson | Maybe | We are not sure there is an issue here - it depends on how the spec is interpreted. The DAPS and non-DAPS case is written in a similar way: 2> if any DAPS bearer is configured: 3> configure the value of timers and constants for the target cell group in accordance with received rlf-TimersAndConstants; 2> else: 3> (re-)configure the value of timers and constants in accordance with received rlf-TimersAndConstants; So, for the non-DAPS case, since rlf-TimersAndConstants does not include T301 there is an implicit assumption that the value in UE-TimersAndConstants in SIB1 is used for T301. It is unclear if anything is needed. If there is consensus around doing anything here, we prefer alternative 1 as it has a lot less impact. |
| QCOM | Yes | We support Alt-2, i.e. extending the IE RLF-TimersAndConstants to include T301, as we don’t see the need to change the UE behaviour when obtaining this info, i.e. some info from the dedicated message and other info from broadcast messages. |
| Huawei, HiSilicon | No | No need to make any change. If T301 is not configured in HO command, it can still maintain the length value used in source cell. |
| NEC | Maybe | This T301 handling is the same for DAPS handover and legacy handover. The legacy handover case can be discussed first, and DAPS applies the same way. |
| Sharp | Maybe | In LTE, T301 is included in RLF-TimersAndConstants, so maybe it’s better to align also in NR(i.e. Alt-2 ). |
| Samsung | No | Same view with ZTE. |
| Apple | No | We share ZTE’s view. |
| OPPO | Yes | When network sets the rlf-TimersAndConstants to setup in DAPS HO command, UE has all values for RLF timers and constants other than T301 to be used for the target cell group. And we prefer Alt1 to add the text that UE will use the value for timer T301 for the target cell group, as included in ue-TimersAndConstants received in SIB1. |
| LG | No | We think that if re-establishment happens in the target cell, the UE will apply T301 in the SIB1 in accordance with the re-establishment procedure. There is no real problem in the current procedure. |
| CATT | No | Agree with ZTE. |
| Lenovo | Yes | Prefer Alt2. |
| Vivo | No | As we know, T301 is started upon transmission of RRCReestabilshmentRequest. At that time, UE has performed cell selection and get essential system information, including ue-TimersAndConstants received in SIB1, which contains T301 value. Hence, it seems no problem. |
| Intel | No | It is not DAPS specific issue. Current DAPS just followed legacy HO behaviour. We agree with ZTE that the UE shall use the T301 from SIB1. |
| ITRI | No | Agree with ZTE that UE shall the T301 value from SIB1. |
| Nokia | No | Agree with ZTE and Ericsson. DAPS behavior does not differ from legacy HO, where it is assumed SIB1 value will be used. |
| Google | No | We share the same view as ZTE. |

Summary for Q3:

- 18 companies responded to this question

- 3 companies are OK to have those changes pursued. 3 companies said ‘maybe’. The remainder does not want this CRs to be agreed.

- It has been pointed out that DAPS case is not different from the legacy HO, where it is assumed SIB1 values are used.

- Thus, we propose not to pursue changes in [4] and [5].

Proposal 3: Changes in R2-2107086 and R2-2107087 are not pursued.

## 2.4 	Other DAPS corrections

[6] and [7] propose various corrections to DAPS HO. In [6] some modifications to the procedural text concerning how to handle PDCP configuration and SRBs when security key is (not) changed during DAPS. [7] clarifies that the UE shall not resume SRBs for source cell when applying the target cell configuration during DAPS HO. Companies are kindly asked to express their views if changes in [6] and [7] are needed.

| Question 4: Do you support the changes proposed in [6]? Please comment especially if you think not all changes are relevant. |  |  |
| --- | --- | --- |
| Company | Yes/No | Comment |
| ZTE | Yes | Since all changes are editorial changes, we prefer to merge them into the rapporteur CR |
| MediaTek | Yes | Also suggest to put this in rapporteur CR |
| Ericsson | No | 1st change (adding "for target cell group"): Not essential. The "target cell group" clarification has been discussed several times before (at RAN2#112 and RAN2#113bis) and RAN2 agreed that configuration during DAPS handover applies to target cell group and no further RRC clarifications are needed. 2nd and 3rd change: Not essential. It is proposed to remove text which is not needed as it is claimed it does not have any effect. So the change would not have any effect either and there is no risk of misunderstanding. Potentially this can go into a rapporteur CR. Editorials can be sent to the spec rapporteur. |
| QCOM | No strong view | Editorial changes, good to have. |
| Huawei, HiSilicon | No | Only the first change, i.e. “1) Add “for the target cell group” for the case of DAPS with security key change in 5.3.5.6.3;”, is acceptable and can be merged to Rapp RRC CR. The other changes are editorial and unnecessary. |
| NEC | Yes | We are also fine to merge the corrections into rapporteur CR. |
| Sharp | No strong view. |  |
| Samsung | See comments | The first change is not needed since it is already clear: 1> If any DAPS bearer is configured, for each SRB: 2> establish a PDCP entity for the target cell group as specified in TS 38.323 [5], with the same configuration as the PDCP entity for the source cell group; 2> if the masterKeyUpdate is received: 3> configure the PDCP entity with the security algorithms according to securityConfig and apply the keys (KRRCenc and KRRCint) associated with the master key (KgNB); It seems OK with other editorial changes but not essential. |
| Apple | No strong view | The changes are correct but not essential. |
| OPPO | Agree | Agree with the editorial changes. |
| LG | Yes | All changes can be merged into the rapporteur CR. |
| CATT | No strong view |  |
| Lenovo | No strong view | Better to merge to rapporteur CR. |
| vivo | Partial Yes | Agree with 1st change, it’s a kind of clarification, although there should be no problem in current text. But 2nd & 3rd changes are not suitable. “The status variables” should not be removed. |
| Intel | No | As mentioned by Ericsson, first change has been discussed several times, and RAN2 concluded no change is needed. Therefore we do not see the need to discuss this again. 2nd/3rd changes are editorial, not essential. Can be included in Rapp CR. |
| ITRI | No strong view |  |
| Nokia | No | Agree with Ericsson regarding the first change. In addition, we understand there is no rapporteur’s CR at this stage already and we shall agree individual CRs only if they propose truly needed changes. |
| Google | No strong view | The changes are correct but not essential. |

Summary for Q4:

- 18 companies provided their feedback.

- 6 companies agree (at least partially) to capture some of those changes in the standard. 4 companies are clearly against, the remainder has no strong view, claims this are editorial changes and could be added to the rapporteur’s CR.

- As there is no rapporteur’s CR run at this (closed) stage of the WI and only essential corrections shall be pursued in individual CRs, based on the views expressed above, we think it is fair to conclude the CR is not pursued.

Proposal 4: Changes in R2-2107776 are not pursued.

And a separate question for CR in [7].

| Question 5: Do you support the changes proposed in [7]? |  |  |
| --- | --- | --- |
| Company | Yes/No | Comment |
| ZTE | Yes |  |
| MediaTek | No strong view | Seems okay but not critical |
| Ericsson | Yes | This correction seems correct and essential. |
| QCOM | Yes |  |
| Huawei, HiSilicon | Yes |  |
| NEC | Yes |  |
| Samsung | Yes | It would be better to include this in Rapp CR. |
| Apple | Yes |  |
| OPPO | No strong view |  |
| LG | Yes |  |
| CATT | Yes |  |
| Lenovo | No strong view | Better to merge to rapporteur CR. |
| vivo | Yes | Upon the DAPS handover, the UE suspends SRBs for the source cell group until receiving daps-SourceRelease |
| Intel | Yes | Ok to have this change. “suspended” seems redundant, can be removed. 2> resume all suspended radio bearers except the suspended SRBs for the source cell group |
| ITRI | Yes |  |
| Nokia | OK to have it |  |
| Google | Yes | Proponent. The change fixes the issue that the UE resumes suspended radio bearers for the source cell group during DAP handover. Therefore, the change is not just an editorial correction. We will update the CR to remove “suspended” as suggested by Intel. |

Summary for Q5:

- 17 views have been provided

- 14 companies are supportive, 3 companies have no strong opinion.

- Intel suggests to remove the word ‘suspended’, which seems to be redundant

- The CR can be pursued, with the change suggested above.

Proposal 5: CR in R2-2108817 is pursued with the removal of ‘’suspended’’ in change proposed for section 5.3.5.5.1.

# 3 	Conditional Reconfiguration

## 3.1 	SCG with CHO configuration

Papers [8][9][10][11] have been discussed during the online session at RAN2#115. It has been decided RAN2 will not ask RAN3 to specify the support of CHO with SCG configuration for Rel-16. However, several open issues remain:

- How to capture in RAN2 specifications that CHO with SCG configuration is not supported as per Rel-16

- Whether to ask RAN3 to define a related support in Rel-17

- What exactly to respond to RAN3

Regarding a) there are multiple ways how to capture this restriction in RAN2 specification. E.g. a Stage-2 modification (37.340 or 38.300) can be pursued. Alternatively, NR RRC may be updated with a restriction that RRC Reconfiguration comprising conditionalReconfiguration IE cannot contain a target node SCG configuration. Companies are kindly asked to express their preference:

| Question 6: How to capture in RAN2 specification the restriction CHO with SCG configuration is not supported in Rel-16? |  |  |
| --- | --- | --- |
| Company | TS number | Comment |
| ZTE | TS 38.331 and TS36.331 for stage-3 description; TS 37.340 and TS 36.300 for stage-2 description | We think it’s better to clearly capture the restriction for CHO with SCG configuration in both stage-2 and stage-3 specs, similar to the restriction for other non-coexistence features, e.g. CHO and DAPS. And we also provide the corresponding TPs in our discussion paper (R2-2108164), which can be considered as one solution (the change below is highlighted by yellow). TS 38.331 condRRCReconfig The RRCReconfiguration message to be applied when the condition(s) are fulfilled. The RRCReconfiguration message contained in condRRCReconfig cannot contain the field conditionalReconfiguration, the field daps-Config or the SCG configuration. TS 36.331 condReconfigurationToApply The RRCConnectionReconfiguration message to be applied when the condition(s) are fulfilled. The RRCConnectionReconfiguration message contained in condReconfigurationToApply can not contain the SCG configuration. TS 37.340 In case MR-DC is configured, CHO is only supported in Master Node to eNB/gNB Change procedure in this release. CHO is not supported in eNB/gNB to Master Node Change procedure in this release. NOTE 3: CHO is only supported from E-UTRA with EPC/EN-DC to E-UTRA with EPC and from NR/NE-DC/NR-DC to NR. TS 36.300 NOTE 2: In case LTE-DC is configured, CHO is only supported in MeNB to eNB change procedure in this release of the specification. NOTE 3: CHO is not supported in eNB to MeNB change procedure in this release of the specification. |
| MediaTek | No strong view | The proposal from ZTE is in general okay to us. We prefer to have this clarification in stage 3 but also fine to clarify both stage 2 and stage 3. |
| Ericsson | - | We think we can do nothing for now. RAN2 has more important things to focus on than removing this from the rel-16 specs. We don’t think there is a problem if this is introduced on the UE side in one release and on the network side in another release. |
| QCOM | Stage-2 is enough | The CHO feature in DC becomes almost useless, as network needs to ensure before configure the CHO that the SCG cell can still be the same after CHO is executed, otherwise, the network can’t trigger the CHO. No need to do many changes in the spec, at the end of the day, this is controlled by the network, and no interoperability issue will rise if network stick to the agreement. In the future release (e.g. Rel-17), the restriction can be simply removed from the stage-2 spec only with no further changes to other specs. |
| Huawei, HiSilicon | 38.331 and 36.331 | Suggest to add a clarification in the field description of the conditionalReconfiguration IE. |
| NEC | Stage 2 (37.340) at least, fine with Stage 3 as well |  |
| Sharp | 38.331, 36.331 | Ok to add stage3 clarification. |
| Samsung | 38.331, 36.331 | It seems to be effective and also easy to find that capturing the restriction in NR RRC, such as field description of (mrdc-)SecondaryCellGroup in RRCReconfiguration message. |
| Apple | But stage-2 and stage-3 spec | We are fine with ZTE’s proposal. The restriction of DAPS configuration is captured both stage-2 and stage-3, so we should follow the same way. |
| OPPO | 38.331, 36.331 | We are generally fine with ZTE’s CR. |
| LG | Stage-2 and possibly in stage-3. | We need to state the configuration restriction in stage-2 spec (36.331 and 37.340). Stage-3 changes are not essential (but no strong view) |
| CATT | at least in stage 3 |  |
| Lenovo | 38.331, 36.331 | More information suggested from ZTE to be added in field description is fine. |
| vivo | At least 38.331 | NR RRC may be updated with a restriction that RRC Reconfiguration comprising conditionalReconfiguration IE cannot contain a target node SCG configuration. Suggest to consider similar change for LTE. |
| Intel | Stage 3 | For stage 2, should not current sentence (as below) is clear enough? TS 37.340 In case MR-DC is configured, CHO is only supported in Master Node to eNB/gNB Change procedure in this release. TS 36.300 NOTE 2: In case LTE-DC is configured, CHO is only supported in MeNB to eNB change procedure in this release of the specification. |
| ITRI | 38.331, 36.331 | The stage 3 CR is needed and we also fine with ZTE’CR. |
| Nokia | Perhaps just Stage-2 changes | We think it is not essential to add such clarification to multiple specifications, especially as in Rel-17 it is expected this restriction will be removed. |
| Google | Both stage-2 and stage-3 | ZTE’s proposal is ok to us. |

Summary for Q6:

- 18 companies took part in this discussion

- Stage-3 changes are mentioned in 14 responses. Stage-2 changes are less popular.

- Even though we think Stage-2 simple clarification would be sufficient (expecting the support for CHO with SCG configuration in Rel-17), we are OK to pursue what the majority prefers

Proposal 6: Capture in Stage-3 specification (TS 38.331, TS 36.331) that CHO with SCG configuration is not supported in Rel-16.

Regarding b), we understand RAN2 is fine to support CHO with SCG configuration from Rel-17 onwards. This would align the CHO with HO behaviour. If there is no objection to support it in Rel-17, RAN2 shall indicate that in our response LS.

| Question 7: Do you support CHO with SCG configuration in Rel-17? If the answer is yes, should we include this request in the response LS to RAN3? |  |  |
| --- | --- | --- |
| Company | Yes/No | Comment |
| ZTE | Yes | We are fine to include the willing in the response LS to RAN3, e.g. R2 assumes this will be supported in Rel-17.. |
| MediaTek | Yes | We could inform RAN3 our agreements. |
| Ericsson | Yes | Yes |
| QCOM | Yes |  |
| Huawei, HiSilicon | Yes |  |
| NEC | Yes | fine to add this in the LS |
| Sharp | Yes |  |
| Samsung | Yes | It needs to inform RAN3 on the support in R17, and good to include this in the LS to R3. |
| Apple | Yes |  |
| OPPO | Yes |  |
| LG | Yes | We are fine to support CHO with SCG configuration in R17 if time allows. RAN2 can ask this in the LS to RAN3. |
| CATT | Yes |  |
| Lenovo | Yes |  |
| vivo | Yes | We are fine to support CHO with SCG configuration from Rel-17, and include this request in the response LS to RAN3 |
| Intel | FFS | Just to check, do companies assume that RAN3 will do this in TEI 17 or R17 DCCA (so far not in the scope)? Or a new WI? |
| ITRI | Yes |  |
| Nokia | Yes | Regarding Intel’s comment: it should not bother RAN2 how to address it formally in RAN3 (indeed, maybe RAN changes to the scope would be needed). But in the LS, RAN3 has clearly indicated this can be done – either in Rel-16 or Rel-17. |
| Google | Yes |  |

Summary for Q7:

- All companies but one support having CHO with SCG configuration in Rel-17

- Intel wants to keep it FFS as it is not clear where it shall be formally addressed in Rel-17 (i.e. TEI17, Rel-17 DCCA or other)

Proposal 7: CHO with SCG configuration is supported in Rel-17. This is indicated to RAN3 in our response LS.

Regarding c), what else should be included in the response LS, besides the decision not to support CHO with SCG configuration in Rel-16 and (potential) willingness to specify it for Rel-17?

| Question 8: What should be included in the response LS, besides the decision not to support CHO with SCG configuration in Rel-16 and (potential) willingness to specify it for Rel-17? |  |
| --- | --- |
| Company | Comment |
| ZTE | No. Including RAN2 agreement is enough. |
| MediaTek | R2 agreement seems enough |
| Ericsson | RAN2 agreements. |
| Huawei, HiSilicon | RAN2 agreements. |
| NEC | RAN2 agreements |
| Samsung | Nothing special for now. |
| OPPO | RAN2 agreements |
| LG | RAN2 agreements |
| CATT | RAN2 agreements |
| Lenovo | RAN2 agreements |
| vivo | RAN2 agreements. |
| Intel | RAN2 agreements |
| ITRI | RAN2 agreements |
| Nokia | No support in Rel-16, support in Rel-17. |
| Google | RAN2 agreements |

Summary for Q8:

- All companies are OK to insert RAN2 agreements into the LS and let RAN3 know we do not support CHO with SCG in Rel-16, but support having it in Rel-17

- No proposal is made.

The draft response LS will be prepared based on the view expressed above.

## 3.2 	RRC connection re-establishment with CPC configuration

The authors of [12][13] notice that a UE that is configured with conditional reconfiguration and initiates the connection re-establishment procedure will skip a substantial part of the procedure in 5.3.7.2 (Initiation of RRC Connection Re-establishment). It is proposed to make certain parts of the procedure conditional to whether the attemptCondReconfig is available, not to whether conditionalReconfiguration is provided. Do you think the problem is valid and the solution proposed is agreeable?

| Question 9: Do you find the problem found in [12][13] valid and agree to correct as proposed in those CRs? |  |  |
| --- | --- | --- |
| Company | Yes/No | Comment |
| ZTE | Yes | Based on the current spec, it is possible that the UE may trigger CPC execution during cell re-selection in RRC re-establishment procedure. So we think the change is needed. |
| MediaTek | Yes |  |
| Ericsson (proponent) | Yes | This is an error that needs to be corrected. |
| QCOM | Yes |  |
| Huawei, HiSilicon | Yes |  |
| NEC | Yes |  |
| Sharp | No | We agree with the intention,it is possible that CPC execution is triggered during cell re-selection, but anyway the UE will do MR-DC release action when the selected cell is not for CHO recovery, so no problem is found. |
| Samsung | Yes |  |
| Apple | Yes |  |
| OPPO | No | Agree with Sharp. And the attemptCondReconfig is conditional configured if the UE is configured with at least a candidate SpCell for CHO. We see no issue for the current spec. |
| LG | Neutral | The observation in the CRs seems correct that the UE may trigger CPC during cell selection within re-establishment, which we think is not what RAN2 intended. The changes in the CR prevents such case from happening by releasing MR-DC in the concerned case. However, RAN2 may need to check if the triggered/executed CPC during re-establishment really causes any serious problem, given that upon selecting a suitable cell during cell selection within re-establishment, the UE eventually releases MR-DC and continues legacy re-establishment steps. |
| CATT | Yes |  |
| Lenovo | No | In T38.331(5.7.3b), after UE initiates MCG link recovery, UE stops conditional reconfiguration evaluation for CPC, if configured. That means UE has stop evaluating CPC condition already when UE initiates re-establishment. If MCG link recovery is not configured, to evaluate CPC condition during re-establishment is not forbidden according to the specification. However, UE will not perform CPC evaluation since it is useless because CPC cannot be performed after UE initiates re-establishment procedure. Therefore, CPC will not be triggered at all during re-establishment procedure. |
| vivo | Yes | It’s correct to use consistent condition “attemptCondReconfig ” |
| Intel | No | The scenario mentioned by CR should be covered by “else” part, i.e. if conditional HO is configured, the UE will not perform MAC reset, but once select suitable cell, the UE will check if attemptCondReconfig is configured or not. If not, the UE will perform MAC reset and remove MR-DC. 1> if attemptCondReconfig is configured; and 1> if the selected cell is one of the candidate cells for which the reconfigurationWithSync is included in the masterCellGroup in VarConditionalReconfig: 2> apply the stored condRRCReconfig associated to the selected cell and perform actions as specified in 5.3.5.3; NOTE 1: It is left to network implementation to how to avoid keystream reuse in case of CHO based recovery after a failed handover without key change. 1> else: So the question is, whether the UE can perform CPC after reestablishment? I think no since the MR-DC configuration will be removed. |
| ITRI | No | We share the same view as Intel. |
| Nokia | No | We agree with Intel that the ‘’1> else’’ part of the procedure in 5.3.7.3 (NR RRC) will serve the purpose (MR-DC config will be released, etc.) when the UE is configured with (any) conditionalReconfiguration, while attemptCondReconfig is not present. |
| Google | Yes |  |

Summary for Q9:

- 17 companies responded

- 10 companies want to pursue such changes, 6 companies are against, one company is neutral

- The rapporteur thinks the specification correctly addresses also the case of CPC, as was explained by Intel and others. Thus, due to the lack of massive support and 6 opposing views, the proposal is to not pursue these CRs.

Proposal 8: The CRs in R2-2108102 and R2-2108103 are not pursued.

## 3.3 	On HO Request Acknowledge in CHO

Finally, the authors of [14][15] underline the HO Request ACK needs to always convey an RRC container with the target cell configuration, even if the procedure did not change the previously provided configuration (for CHO which was prepared earlier and then source node triggered a reconfiguration not impacting the target cell config). We see some value in what is discussed in [14][15], but perhaps this should be discussed in RAN3 directly. Do you think the problem is valid and should be highlighted to RAN3?

| Question 10: Do you agree with what is proposed in [14][15] and see the need to inform RAN3? |  |  |
| --- | --- | --- |
| Company | Yes/No | Comment |
| ZTE | No | This issue has been discussed at RAN2#111e (i.e. R2-2007229). And it’s agreed no support to do this in Rel-16. So we think no need to discuss this again for R16 and no need to inform RAN3. |
| MediaTek | Maybe | We tend to agree with this concern and it seems reasonable for RAN3 to look at it. However, maybe companies could trigger this discussion directly in RAN3. |
| Ericsson | Maybe | We think this should be raised directly in RAN3 as it only impacts RAN3. No need for RAN2 to spend time on it. |
| Huawei, HiSilicon | No | We share similar views as ZTE. Inter-node signalling was discussed in RAN2#111-e and RAN2 agreed to not do it in Rel-16. |
| NEC | No | same view as ZTE |
| Sharp | No | Agree with ZTE. RAN agreed not to support this in Rel-16. |
| Samsung (proponent) | Yes | Even though the problem happens at UE side, the solution can be discussed in RAN3, so it is necessary to inform them on the request for the discussion and solution. @ZTE, Huawei: We are not proposing the RAN2 solution as in R2-2007229, but just send the LS to RAN3 to identify the problem and request for the solution from them. One more reason for this LS: problem happens at RRC in UE, and RAN3 usually doesn’t care about the RRC contents, they just encapsulating this in their X2/Xn message. Therefore, it is important to let them know there is a problem in their HO preparation procedure with respect to RRC content included. |
| Apple | See comment | It can be discussed in RAN3 directly. |
| OPPO | No | Agree with ZTE. |
| LG | No | Agree with ZTE. Also, this would be better to discuss in RAN3 directly. |
| CATT | Maybe | We agree the intention, maybe it can be discussed in RAN3 directly |
| Lenovo | No | Agree with ZTE. |
| vivo | No | Agree with ZTE that this has been discussed in RAN2 before. |
| Intel | No | Agree with ZTE. It has been discussed before. To understanding, it is optimization since nothing is broken, and could be resolved by network implementation, e.g. source to determine whether target configuration will be impacted. |
| ITRI | No | Agree with ZTE. |
| Nokia | Agree with the intention | We think the issue may be valid, but it shall be taken directly to RAN3, if necessary. |
| Google | No | The issue can be raised to RAN directly so we don’t see a need to send a LS. Besides, we think the issue can be solved by the network implementation at least for intra-vendor cases. The source can decide whether the updated source configuration would cause the CHO configuration to be updated. Based on the decision, the source can decide whether to trigger the conditional HO preparation procedure with the target to update the CHO configuration. |

Summary for Q10:

- 17 companies responded to this question.

- 5 companies possibly see a need to discuss this issue. The remainder thinks this is not needed or shall be considered directly in RAN3.

- Thus, we suggest not to discuss this topic in RAN2.

Proposal 9: The proposals in R2-2108776 are not pursued.

# 4	Conclusion

Based on the views expressed in the previous sections, we propose the following:

Proposal 1: The changes in R2-2108090 are not pursued.

Proposal 2: Changes in R2-2107775 are not pursued.

Proposal 3: Changes in R2-2107086 and R2-2107087 are not pursued.

Proposal 4: Changes in R2-2107776 are not pursued.

Proposal 5: CR in R2-2108817 is pursued with the removal of ‘’suspended’’ in change proposed for section 5.3.5.5.1.

Proposal 6: Capture in Stage-3 specification (TS 38.331, TS 36.331) that CHO with SCG configuration is not supported in Rel-16.

Proposal 7: CHO with SCG configuration is supported in Rel-17. This is indicated to RAN3 in our response LS

Proposal 8: The CRs in R2-2108102 and R2-2108103 are not pursued.

Proposal 9: The proposals in R2-2108776 are not pursued.

# 5	List of referenced documents

- R2-2108090	On bearer release handling for DAPS HO	Nokia, Nokia Shanghai Bell

- R2-2107775	Correction on fallback to source SDAP configuration in case of DAPS failure	NEC

- R2-2107085	Discussion on T301 issue for DAPS HO	OPPO

- R2-2107086	Correction on T301 for DAPS HO (alternative 1)	OPPO

- R2-2107087	Correction on T301 for DAPS HO (alternative 2)	OPPO

- R2-2107776	Correction on SRB handling for DAPS	NEC

- R2-2108817	Correction to DAPS handover	Google Inc.

- R2-2106933	Response LS on Conditional Handover with SCG configuration scenarios (R3-212848; contact: Nokia)

- R2-2108164	Discussion on CHO with SCG configuration	ZTE Corporation, Sanechips

- R2-2107526	On supporting CHO with SCG configuration	Nokia, Nokia Shanghai Bell

- R2-2107527	Response LS on CHO with SCG configuration	Nokia, Nokia Shanghai Bell

- R2-2108102	RRC connection re-establishment with CPC configuration	Ericsson

- R2-2108103	RRC connection re-establishment with CPC configuration	Ericsson

- R2-2108776	Signalling of HOReqACK msg upon serving cell configuration update

- R2-2108777	[Draft] LS on reflecting source cell configuration update in Conditional Handover

# Contact information

| Company | Delegate contact |
| --- | --- |
| ZTE | zhang.mengjie@zte.com.cn |
| MediaTek | chun-fan.tsai@mediatek.com |
| Ericsson | cecilia.eklof@ericsson.com |
| Qualcomm | Mouaffac (mambriss@qti.qualcomm.com) |
| Huawei, HiSilicon | jun.chen@huawei.com |
| NEC | Wang_da@nec.cn |
| Samsung | June77.hwang@samsung.com |
| OPPO | youxin@oppo.com |
| Lenovo | Wulh5@lenovo.com |
| vivo | Chenli5g@vivo.com |
| ITRI | moumou3@itri.org.tw |
| Google | frankwu@google.com |