3GPP TSG RAN WG2 Meeting #115-e                                                      draft R2-2109129

# Electronic meeting, 16th-27th August 2021

Agenda item:			8.12.2.1

Source:			Intel Corporation

Title:		Summary of [AT115-e][109][RedCap] Capabilities (Intel)-2nd round

Document for:	 	Discussion and decision

- Introduction

This document is the summary of following offline discussion:

[AT115-e][109][RedCap] Capabilites (Intel)

Initial scope: Continue the discussion on p5, p6, p8-p13, p16-p18 (p19-p20 can be discussed during the running CR drafting). In general discuss whether, for (some of) these proposals, we need to ask anything to RAN1. Also discuss p1 and p2 from R2-2107677, i.e. need to send an LS to SA2/CT1

Intended outcome: Summary of the offline discussion with e.g.:

    - List of proposals for agreement (if any)

    - List of proposals that require online discussions

    - List of proposals that should not be pursued (if any)

Initial deadline (for companies' feedback): Monday 2021-08-23 10:00 UTC

Initial deadline (for rapporteur's summary in R2-2108891): Monday 2021-08-23 16:00 UTC

Proposals marked "for agreement" in R2-2108891 not challenged until Tuesday 2021-08-24 0800 UTC will be declared as agreed via email by the session chair (for the rest the discussion will further continue online).

Final scope: Continue the discussion on p3, p13 and p14 from R2-2108891 and draft the LS to RAN1 on L2 buffer size reduction

Intended outcome: LS(s) and summary of the offline discussion with e.g.:

    - List of proposals for agreement (if any)

    - List of proposals that require online discussions

Final deadline (for companies' feedback): Thursday 2021-08-26 1000 UTC

Final deadline (for rapporteur's summary in R2-2109129 and LS in R2-2109130): Thursday 2021-08-26 1500 UTC

Proposals marked "for agreement" in R2-2109129 not challenged until Friday 2021-08-27 0300 UTC will be declared as agreed via email by the session chair (for the rest the discussion might continue online during the CB session).

Discussion

Based on [12], RAN2 discussed the supported capabilities for RedCap UE and how to capture them into specification. Following agreements have been made:

Agreements:

- The number of DRBs supported by RedCap UEs is less than legacy value (which is 16). There will be a single mandatory value (FFS if 4 or 8). FFS if it will be possible to have an optional capability

- “RRC processing delay” is not relaxed for RedCap UE

- PDCP/RLC AM 12 bits SN is mandatory for RedCap UE, and PDCP/RLC AM 18bits SN is optional supported by RedCap UE; FFS on how to capture this in specification

- NE-DC, and (NG)EN-DC are not supported by RedCap UE; FFS on how to capture it in the specification

- DAPS and CAPC related capabilities are not applicable for RedCap UE; [8/20] FFS on CHO. FFS on how to capture this in the specification;

Based on [15], following agreements have been made:

Agreements via email - from offline 109:

- Maximum 8 DRBs is mandatory supported by RedCap UEs.

- From RAN2 perspective, inter RAT mobility related capabilities are applicable for RedCap UE;

- From RAN2 perspective, measurement related capabilities are applicable for RedCap UE;

- From RAN2 perspective, URLLC related capabilities are applicable for RedCap UE except those affected by CA/DC;

- From RAN2 perspective, IAB related capabilities are not applicable for RedCap UE, i.e. the RedCap UE is not expected to act as IAB node;

- Do not introduce capability signalling on the supported Rx number for RedCap UE since the number of Rx branches for RedCap is implicitly indicated by the corresponding capability parameter maxNumberMIMO-LayersPDSCH in the existing UE capability framework;

Following proposals will be treated in this offline discussion based on the guidance from Chair:

Proposal 3. [To discuss] [11/22] on whether to introduce an optional capability to indicate the number of DRBs that the RedCap can additionally support. - Continue in offline 109 Proposal 13. [To discuss] [6/17] whether RAN2 needs to send LS to RAN1/4 asking them to check features, URLLC, measurement, V2X, IAB, positioning - Continue in offline 109 Proposal 14. [To discuss] [9/19] Send LS to SA2/CT1 to check subscription solution, and whether core network should know that the UE is a RedCap UE. - Continue in offline 109 - Send an LS to RAN1 (contact Intel & Spreadtrum) asking to discuss L2 buffer size reduction and provide feedback to RAN2

Max DRB number

The discussion situation is

RAN2 has agreed “1. Maximum 8 DRBs is mandatory supported by RedCap UEs.” In phase 1 discussion: Summary on Discussion point 2.1.2-2 on max DRB number. 22 companies provided inputs to this discussion point: Introduce the optional capability to indicate the number of DRBs that the RedCap can additionally support - Yes: 11 companies (ZTE, Qualcomm, Apple, BT, vivo, Sequans, Turkcell, MediaTek, LGE, Deutsche Telekom, OPPO ) Companies who support to introduce optional capability would like to allow high-end UE and offer more flexibility to implementation; - No: 9 companies (Intel, Huawei, Spreadtrum, CMCC, Ericsson, CATT, NEC, Telecom Italia, Nokia ) Futurewei commented that if 8 is selected as the mandatory value, no optional capability is needed. Rapporteur: There is no clear majority on whether optional capability should be introduced for RedCap UE. Rapporteur would suggest to postpone the discussion. Proposal 3. [To discuss] [11/22] on whether to introduce an optional capability to indicate the number of DRBs that the RedCap can additionally support. - Continue in offline 109

Considering RAN2 has agreed 8 is mandatory value, do companies still see the value to introduce optional capability to indicate the support of 16 DRBs? .Rapporteur would like to check companies’ view again,

Discussion point 2.1.2 on max DRB number: Companies are invited to provide view on whether to introduce the optional capability to indicate the support of 16 DRBs for RedCap UEs?

| Company’s name | Yes/No | Comments, if any |
| --- | --- | --- |
| Intel | No | 8 is sufficient for RedCap UE. |
| Apple | Yes | We should not shortchange all RedCap UEs. Wearable can have similar applications as legacy NR UEs and adding a blanket to all RedCap UEs is ineffective. |
| BT | Yes | We consider 8 DRBs are enough for RedCap UEs but we don’t want to reduce flexibility. At the end, it is up to how the network is engineered to support more than 8 DRBs in case this is accepted. |
| OPPO | Yes | For most RedCap use cases, 8 DRBs are sufficient. Introducing optional capability for support of 16 DRBs could provide more flexibility for UE implementation. |
| Huawei, HiSilicon | No | 16 will be not really useful to RedCap UE. In addition, single value will make the gNB implementation simple, just like legacy. |

Summary on Discussion point 2.1.2 on max DRB number.

LS to RAN1, RAN4

The discussion situation is:

Discussion point 2.3-1 on LS to RAN1 and RAN4: Companies are invited to provide view on whether RAN2 needs to send LS to RAN1/4, ask them to check features, URLLC, measurement, V2X, IAB, positioning ? If yes, please indicate which feature should be included in the LS. Summary on the Discussion point 2.3-1 on LS to RAN1 and RAN4 17 companies provided inputs to this discussion point. - Need of LS to RAN1/4: Yes: 6 companies (Spreadtrum, ZTE, Apple, Squans, NEC, OPPO ) No: 2 companies (Intel, vivo , ) Postpone: 6 companies (Qualcomm, Ericsson, Turkcell, MediaTek, LGE, Nokia) Qualcomm mentioned RAN1 is discussing these issues, and we can wait for their agreements. Rapporteur: There is no consensus on the need of LS. As Qualcomm commented “RAN1 is discussing these issues”, Rapporteur tend to think no LS is needed at least for now. Proposal 13. [To discuss] [6/17] whether RAN2 needs to send LS to RAN1/4 asking them to check features, URLLC, measurement, V2X, IAB, positioning - Continue in offline 109

As commented by Qualcomm RAN1 is discussing these issues, then should we still need to send LS to trigger the discussion in RAN1 and RAN4? Or we just postpone the discussion, and may send LS based on the situation. Rapporteur would like to check companies’ view again.

Discussion point 2.2 on LS to RAN1 and RAN4: Companies are invited to provide view on whether RAN2 needs to send LS to RAN1/4, ask them to check features, URLLC, measurement, V2X, IAB, positioning ?

- Option 1: send the LS in this meeting;

- Option 2: postpone the LS, and only send LS when needed, e.g. check the situation in RAN1 and RAN4 in next meeting;

| Company’s name | Option 1 or option 2 | Comments, if any |
| --- | --- | --- |
| Intel | Option 2 | Let’s see the discussion in RAN1 and RAN4 first. Can come back in next meeting. |
| Apple | No strong view. Ok with majority. |  |
| BT | Option 2 | RAN2 can wait to see RAN1 evolution |
| OPPO | Option 1 | We notice that RAN1 is not discussing all these features, so maybe an LS is needed and all features (URLLC, measurement, V2X, IAB, positioning) can be included. |
| Huawei, HiSilicon | Option 2, or? | The LS seems not essential. But we are fine if the LS is just copy RAN2 agreements: - From RAN2 perspective, inter RAT mobility related capabilities are applicable for RedCap UE; - From RAN2 perspective, measurement related capabilities are applicable for RedCap UE; - From RAN2 perspective, URLLC related capabilities are applicable for RedCap UE except those affected by CA/DC; - From RAN2 perspective, IAB related capabilities are not applicable for RedCap UE, i.e. the RedCap UE is not expected to act as IAB node; To be honest, RAN1 will check the feature by their own anyway. Maybe it is also another option that we just copy all the R2 agreements this meeting to RAN1, not limited to this AI, but cover all the RedCap items. |

Summary on the Discussion point 2.2 on LS to RAN1 and RAN4

xx companies provided inputs to this discussion point.

- Option 1: send the LS in this meeting;

- Option 2: postpone the LS, and only send LS when needed, e.g. check  the situation in RAN1 and RAN4 in next meeting;

Constrain the use of RedCap

The discussion situation is

Summary on the Discussion point 2.3-1 on LS to SA2, CT1 19 companies provided inputs to this discussion point. - Need of LS to SA2/CT1: Yes: 9 companies (Intel, Spreadtrum, ZTE, Qualcomm, CMCC, vivo, NEC, Telecom Italia, Deutsche Telekom ) No: 9 companies (Huawei, Apple, Sequans, Ericsson, Turkcell, MediaTek, LGE, Nokia, OPPO) Companies who do not want to send LS believe SA2/CT1 can initiate the discussion directly. Rapporteur: The different between LS to RAN1/4 and LS to SA2/CT1 is, RAN1/4 is responsible for RedCap WI and RAN1 is discussing the same issues. But SA2/CT1 are out of RedCap WI. Any work in SA2/CT1 should be triggered based on LS. Proposal 14. [To discuss] [9/19] Send LS to SA2/CT1 to check subscription solution, and whether core network should know that the UE is a RedCap UE. - Continue in offline 109

As mentioned in phase 1 summary, the difference between LS to RAN1/4 and LS to SA2/CT1 is, RAN1/4 is responsible for RAN WI RedCap and RAN1 is discussing the same issues. But SA2/CT1 are out of RedCap WI. They are still discussing whether a WI is needed to handle eDRX issue. RedCap related discussion in SA2/CT1 is triggered by RAN LS.

Discussion point 2.3 on LS to SA2, CT1: Companies are invited to provide view on whether Send LS to SA2/CT1 to ask question “whether core network should know that the UE is a RedCap UE”?

| Company’s name | Yes/No | Comments, if any |
| --- | --- | --- |
| Intel | Yes | We have to trigger the discussion in SA2/CT1 since they are not in WI scope. |
| Apple | No | CT1/SA2 is already discussing the signaling between RAN and CN, and anyway eDRX and UAC LS already triggered the discussion on NAS signaling. We think there is no need to remind them about the CN knowing whether the UE is a redcap UE. |
| BT | Yes | It is required to inform SA2/CT1 that RedCap has 1Rx and 2Rx. As mentioned by Intel, 1Rx and 2Rx is not in the WI description therefore SA2/CT1 need to be notified. |
| OPPO | No | Agree with Apple. |
| Huawei, HiSilicon | No | As to the rapporteur’s statement “They are still discussing whether a WI is needed to handle eDRX issue. RedCap related discussion in SA2/CT1 is triggered by RAN LS.” Please note SA2 is going to approve their own WID this week, see S2-2106148, where the asked question will be included. It seems no need to trigger their discussion by LS. |

Summary on the Discussion point 2.3 on LS to SA2, CT1

- Summary report and proposals

Aiming to help with the meeting discussion/progress, the proposals are categorized starting with:

- [To agree] when there is large support and hence proposed for easy agreement.

- [To discuss] when there is substantial level of support and agreement may be possible.

- [FFS] when there is low support or companies propose new solutions or options to possibly consider further e.g. if there is sufficient support (understanding that these topic have not been discussed by all companies when providing their views in the different discussion points).

The proposals also start with a number: for the format [x], ‘x’ represents the number of supportive companies (i.e. these solutions are marked as FFS as the proposed solutions were not discussed by all companies) and, for the format [x/y], ‘x’ represents the number of supportive companies, and (y-x) the number of companies with different view.

The observations captured are the following:

Observation 1.	xxxx.

The proposals captured are the following:

Proposal 1.

The following list shows the proposals above organized based on the suggested priority aiming to help during its meeting discussion:

Proposals for easy agreement

Proposals for discussion (1st priority) or to be captured as FFS

Proposals for discussion (2nd priority) or to be captured as FFS

Annex: companies’ point of contact

| Company | Point of contact | Email address |
| --- | --- | --- |
| Intel Corporation | Yi Guo | Yi.guo@intel.com |
| Apple | Naveen Palle | naveen.palle@apple.com |
| BT | Salva Diaz | salva.diazsendra@bt.com |
| OPPO | Haitao Li | lihaitao@oppo.com |
| Huawei, HiSilicon | Yulong Shi | shiyulong5@huawei.com |

- Reference

- R2-2106462 Summary 8.12.2.1 - Definition of RedCap UE and reduced capabilities (Intel)

- R2-2106521	[offline 105] Definition of RedCap UE and reduced capabilities (Intel)	Intel

- R2-2106528	[offline 105] Definition of RedCap UE and reduced capabilities - second round	Intel

- TR 38.875

- RP-210918, “Revised WID on support of reduced capability NR devices”

- R2-2102017	Summary of offline 107 - [REDCAP] L2 capabilties and UE types	Huawei

- R2-2105234	Definition of RedCap UE and first look on capability signaling		Ericsson

- R2-2105634	Definition of RedCap UE type and reduced capabilities	Huawei, HiSilicon

- R2-2104927	RedCap UE capability and constraining of reduced capabilities	Intel Corporation

- TS 38.306 g40

- TS 38.331 g41

- R2-2107676	Email discussion report on [105][RedCap] Capabilities (Intel)	Intel Corporation

- R1-2108316 FL summary #1 on other aspects of UE complexity reduction for RedCap Moderator (Intel Corporation)

- R2-2107677	Constraining network access for UE with reduced capabilities	Intel Corporation

- R2-2108891	[offline 109] RedCap capabilities	Intel

- R2-2109103	WF on Rel-17 RedCap L2 soft buffer Reduction	Spreadtrum, Apple, CAICT, CEPRI, CMCC, CTC, CUC, GDCNI, Guangdong Genius, OPPO, Sequans, Xiaomi, u-blox AG, vivo, ZTE, Sanechips