3GPP TSG-RAN2 Meeting #115e	Draft of R2-2108851

Online, 9th – 27th, August 2021

Agenda Item:	4.5, 7.1.1, 7.4

Source: 	Samsung

Title:  	[AT115-e][201][LTE] Miscellaneous LTE CRs

Document for:	Discussion and Decision

# Introduction

This report gives a summary of this offline email discussion.

[AT115-e][201][LTE] Miscellaneous LTE CRs (Samsung)

Scope:

    - Discuss LTE CRs marked for this discussion (if needed)

Intended outcome:

    - Discussion report in R2-2108851

    - Agreeable CRs (if any)

Deadline for providing comments, for rapporteur inputs, conclusions and CR finalization:

    - Initial deadline (for company feedback):  1st week Thu, UTC 0900

    - Initial deadline (for rapporteur summary):  1st week Thu, UTC 1700

    - Deadline for CR finalization: 2nd week Wed, UTC 0900

The deadline for the first round is Thu, UTC 0900.

# Contact information

| Company | Email |
| --- | --- |
| Samsung | seungri.jin@samsung.com |
| Lenovo | hchoi5@lenovo.com |
| Huawei, HiSilicon | jun.chen@huawei.com |
| Qualcomm | uphuyal@qti.qualcomm.com |

# Discussion

## On T330 resetting

R2-2108312	On T330 resetting	Ericsson, ZTE Corporation, Sanechips	CR	Rel-15	36.331	15.14.0	4712	-	F	LTE_5GCN_connect-Core

T330 (logged MDT DurationTimer) is used to indicate the effectiveness period of a logged MDT configuration which shall keep running irrespective of state transition or RAT type.

However, according current specs T330 will be stopped when UE enters RRC_INACTIVE or when UE HO to another RAT, as a consequence, UE cannot continue logging MDT results when transiting to idle or connected (if configured to log MBSFN measurements) or coming back to LTE since T330 is stopped and not restarted.

Question 1)
Do companies agree on this CR?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Lenovo | No | In stage 2 TS 37.320 the support of logged measurements for inactive state and NR was specified in R16. So, it’s correct in R15 to stop T330 when UE enters RRC_INACTIVE or when UE HO to NR. |
| Huawei, HiSilicon | No | In Rel-15 LTE-5GC, the support of MDT was discussed, and RAN2 agreed on the LS R2-1714203 (at RAN2#100 meeting). In the LS, RAN2 agreed that: - From pure radio interfacce perspective, MDT could be supported with the same functionality as that of LTE connected to EPC but MDT is not part of the Rel-15 NR WID scope For Rel-15 TS 36.331, currenlty the UE stops T330 upon entering RRC_Inactive and HO to another RAT. We think that both cases are related to LTE-5GC, so the UE behaviours are reasonable as MDT for LTE-5GC was not supported in Rel-15. |
| Samsung | No | We have same view with Lenovo and Huawei. |
| Qualcomm | ? | The CR has 2 changes for Rel-15. Both already exist in Rel-16 spec, which seems to be introduced by two separate Rel-16 Cat-F CRs. It is unclear why now there is a need for these to be ported to Rel-15. If these were essential changes, why were they not done from Rel-15 when Rel-16 was corrected? Therefore, no strong view on the need. |

## LTE RRC Rapporteur CRs

R2-2108634	Minor changes collected by Rapporteur for Rel-15	Samsung	CR	Rel-15	36.331	15.14.0	4718	-	F	LTE_eMTC4-Core, LTE_sTTIandPT, LTE-L23

R2-2108635	Minor changes collected by Rapporteur for Rel-16	Samsung	CR	Rel-16	36.331	16.5.0	4719	-	A	LTE_eMTC4-Core, LTE_sTTIandPT, LTE-L23

This CRs include the minor changes, especially removing the redundant editorial Notes which are reamined in the specifications.

- In PDSCH-ConfigDedicated IE, below Editor’s note for eMTC is removed.

-- eNote (ToDo): Clarify that eMTC fields (i.e. fields starting with ce-) do not apply

-- for SCell (merging issue)

- In SPS-ConfigUL IE, below Editor’s note for STTI is removed.

-- eNote (TBC) that no separate STTI field is required (alike in merged CR)

- Editor’s note in Annex A is removed.

Editor's note	No agreements have been reached concerning the extension of RRC PDUs so far. Any statements in this clause about the protocol extension mechanism should be considered as FFS.

- In A.4.3.1, some below two TBD parts are removed.

<TBD: ref to seperate example>

Question 2)
Do companies agree on this CR?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Lenovo | Yes but | - For the eNote in PDSCH-ConfigDedicated IE: it can be expected that an eMTC does not support CA, so further clarification might not be needed. - For the eNote in SPS-ConfigUL IE: it is not fully clear whether it applies only for the field harq-ProcID-Offset-r15 or for all fields below the eNote. Anyway, it seemed that there was no further discussion on this eNote in the past, so it looks ok to remove it. - Further issues can be fixed as well: For the R15 CR: - In 6.3.5: in the description of IE MTC-SSB-NR the highlighted changes can be made. The IE MTC-SSB-NR specifies the SS/PBCH measurement timing configuration (SMTC) applicable for SSB based NR measurements i.e. the time occasions for performing these measurements, see 5.5.2.13. - In 5.5.2.9: the reference to TS 38.133 should be corrected to "[84]". 2> if mgta is set to TRUE, apply a timing advance value of 0.5ms to the gap occurrences calculated above according to TS 38.133 [16]; - In 6.3.2, AUL-Config field descriptions: to be aligned with ASN.1 the letter “M” should be added in the field name “aul-StartingPartialBW-OutsideCOT”. aul-StartingPartialBW-OutsideMCOT - In 6.3.5, MeasConfig field descriptions: in the description of mgta the reference to TS 38.133 should be corrected to "[84]". “Indicates whether a timing advance value of 0.5 ms is applicable to the measurement gap configuration provided by E-UTRAN according to TS 38.133 [16].” For the R16 CR: - Same corrections as for R15 above. - In 5.6.1.3: in the sentence below a redundant word “the” can be removed. " Upon receiving DLInformationTransfer message, the the IAB-MT shall: - In 5.5.2.13: in the paragraph below, the name “duration” should be corrected to “ssb-Duration”. If smtc2-LP is present, for cells indicated in the pci-List parameter in smtc2-LP for inter-RAT cell reselection, the UE shall setup an additional SS/PBCH block measurement timing configuration (SMTC) in accordance with the received periodicity parameter in the smtc2-LP configuration and use the Offset (derived from parameter periodicityAndOffset) and duration parameter from the measTimingConfig configuration for that frequency. |
| Huawei, HiSilicon | Yes |  |
| Samsung | Yes | Further minor corrections (e.g. commented from Lenovo above) could be added on top of these CRs. |
| Qualcomm | Yes | Regarding Lenovo’s suggestions: regarding suggestion in 6.3.5: shouldn’t it be SS/PBCH block measurement timing configuration (SMTC)…? Also, adding abbreviation in 3.2 for SMTC could be helpful. In 5.2.2.9: gapOffset can be made italics. 2> if the gapOffset in measGapConfig indicates a non-uniform gap pattern: Fine with other suggestions from Lenovo. And thanks for thorough check. |

## Early security reactivation upon reception of RRCConnectionReject

R2-2107774	Correction on early security reactivation upon reception of RRCConnectionReject	NEC	CR	Rel-16	36.331	16.5.0	4696	-	F	TEI16, LTE_eMTC5-Core

According to 5.3.3.18, early security reactivation includes UP transmission using PUR and resuming a suspended RRC connection in 5GC. However, in 5.3.3.8, early security reactivation, transmission using PUR and RRC connection resume in 5GC are listed as separate cases.

This misalignment would be caused by two overlapping approved CRs in RAN#87, where one was introducing 5.3.3.18 for TEI16 (CR4167r2/ RP-200357) and the other was adding PUR and RRC connection resume in 5GC for eMTC (CR4191r1/ RP-200360).

This CR proposed to delete “or for transmission using PUR or for resuming a suspended RRC connection in 5GC”, and add a reference to 5.3.3.18.

Question 3)
Do companies agree on this CR?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes | This also applies to NB-IoT and ‘NB_IOTenh3-Core’ should be added to the list of WI codes. We think this could be merged in the rapporteur CR. |
| Samsung | Yes | We also think this change can be merged in the Rap CR because the change is just an alignment of procedures. |
| Qualcomm | Yes | It was merging issue of two CRs. Agree with above comments about WI code and that this can be merged with Rapp CR. |

## ReportConfigEUTRA for CHO/CPAC

R2-2108701	36.331 Correction on ReportConfigEUTRA for CHO/CPAC	CATT	CR	Rel-16	36.331	16.5.0	4720	-	F	LTE_feMob-Core

In the current specification, the field of triggerType is mandatory present in IE ReportConfigEUTRA. However, if condReconfigurationTriggerEUTRA-r16 is configured for conditional reconfiguration (CHO/CPC), both the fields of triggerType and condReconfigurationTriggerEUTRA-r16 will be present in the IE of ReportConfigEUTRA. It is not clear which trigger type should be applied for the associated reprot configuration. Considering that for CHO/CPC, the field of triggerType is not applicable, and the UE should ignore the field of triggerType when condReconfigurationTriggerEUTRA-r16 is configured.

Moreover, the fields of triggerQuantity/ reportQuantity/ maxReportCells/ reportInterval/ reportAmount are all mandatory present fields. Except the field of triggerQuantity, all the other fields are related with measurement report configuration which is not applicable for CHO/CPC. Hence if the condReconfigurationTriggerEUTRA-r16 is configured, the UE should ignore them.

Question 4)
Do companies agree on this CR?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes | In TS 36.331, the procedural text (as below) mentions that the UE will not perform legayc meaurement reporting (related to legacy measurement configurations), and the UE only goes to CHO execution section. Based on the text, we think the UE will anyway ignore the mandatory fields included in the same ReportConfigEUTRA IE. 2> perform the evaluation of reporting criteria as specified in 5.5.4, except if reportConfig is condReconfigurationTriggerEUTRA; NOTE 2c: The evaluation of conditional reconfiguration execution criteria is specified in 5.3.5.9.4. For the CR, the changes are technically correct, and they are aligned with our above analysis. |
| Samsung | Yes | We share the intention of this CR. |
| Qualcomm | Yes |  |

# Summary of email discussion

TBD

# Conclusions

TBD

# References