3GPP TSG-RAN WG2 #115-e	R2-210xxxx

E-meeting, August 2021

Agenda Item:	8.x.x

Source:	OPPO

Title:	Summary of 	[AT115-e][703][V2X/SL] SL DRX configuration for GC/BC (OPPO)

Document for:	Discussion, Decision

Introduction

This is for the following email discussion

[AT115-e][703][V2X/SL] SL DRX configuration for GC/BC (OPPO)

Scope: Discuss following FFS/TBD/open issues:

Q1: Whether the dedicated RRC is also used to configure SL DRX configuration for GC/BC?

Q2: How to configure SL DRX on-duration and inactivity timers for GC/BC?

Q3: How to configure SL DRX RTT and retransmission timers for GC/BC?

Q4: Need of down-select other DRX configurations for a specific L2 DST ID if the UE has multiple QoS profiles for same DST L2 ID? If needed, how to do down-selection?

Q5: Need to define default DRX configuration for GC/BC?

Q6: Need for SL DRX MAC CE for GC/BC?

Intended outcome: Discussion summary in R2-2108983

Discussion

Q1: Whether the dedicated RRC is also used to configure SL DRX configuration for GC/BC?

According to RAN2#113e agreement,

3:	For broadcast/groupcast, for in-coverage case, for RRC_CONNECTED TX-UE/RX-UE can obtain DRX configuration from SIB. FFS on whether dedicated-RRC is also used.

the only FFS point for BC/GC DRX configuration is whether dedicated-RRC can be used for RRC_CONNECTED TX-UE/RX-UE.

Firstly, rapp understand this issue is independent of the case of SIB delivery using dedicated-RRC, which is still based on the cell-specific SIB information, i.e., not UE-specific configuration.

RRCReconfiguration-v1530-IEs ::=            SEQUENCE {

masterCellGroup                         OCTET STRING (CONTAINING CellGroupConfig)                              OPTIONAL, -- Need M

fullConfig                              ENUMERATED {true}                                                      OPTIONAL, -- Cond FullConfig

dedicatedNAS-MessageList                SEQUENCE (SIZE(1..maxDRB)) OF DedicatedNAS-Message                     OPTIONAL, -- Cond nonHO

masterKeyUpdate                         MasterKeyUpdate                                                        OPTIONAL, -- Cond MasterKeyChange

dedicatedSIB1-Delivery                  OCTET STRING (CONTAINING SIB1)                                         OPTIONAL, -- Need N

dedicatedSystemInformationDelivery      OCTET STRING (CONTAINING SystemInformation)                            OPTIONAL, -- Need N

otherConfig                             OtherConfig                                                            OPTIONAL, -- Need M

nonCriticalExtension                    RRCReconfiguration-v1540-IEs                                           OPTIONAL

}

Q2.1-1: For SL BC and GC, for in-coverage case, do you agree RRC_CONNECTED TX-UE/RX-UE can obtain DRX configuration from SIB which is delivered via dedicated RRC signalling?

| Company | Agree/Disagree | Comment |
| --- | --- | --- |
| Xiaomi | Agree |  |
| InterDigital | Agree |  |
| Ericsson | Agree |  |
| Apple | Agree |  |
| OPPO | Agree | As in legacy |
| vivo | Disagree | We don’t think it is necessary, although the UEs who are in IC can receive the RRC (re)configuration. We believe, the situation in a groupcast or broadcast is more complicated than in unicast; some UEs are in IC with (re)configuration and some are in OOC with pre-configuration, this could result in somewhat mismatching of DRX configuration in between. |

Secondly, as we did for Rx pool, which is normally delivered via SIB instead of dedicated-RRC, one exceptional case is handover

SL-BWP-PoolConfig-r16 ::=        SEQUENCE {

sl-RxPool-r16                    SEQUENCE (SIZE (1..maxNrofRXPool-r16)) OF SL-ResourcePool-r16        OPTIONAL,    -- Cond HO

sl-TxPoolSelectedNormal-r16      SL-TxPoolDedicated-r16                                               OPTIONAL,    -- Need M

sl-TxPoolScheduling-r16          SL-TxPoolDedicated-r16                                               OPTIONAL,    -- Need N

sl-TxPoolExceptional-r16         SL-ResourcePoolConfig-r16                                            OPTIONAL     -- Need M

}

| Conditional Presence | Explanation |
| --- | --- |
| HO | This field is optionally present, need M, in an RRCReconfiguration message including reconfigurationWithSync; otherwise it is absent, Need M. |

I.e., it can be delivered to UE in a dedicated RRC during handover procedure, i.e., in an RRCReconfiguration message including reconfigurationWithSync

Q2.1-2: For SL BC and GC, for in-coverage case, do you agree RRC_CONNECTED TX-UE/RX-UE can obtain DRX configuration from dedicated RRC signalling during handover, i.e., in an RRCReconfiguration message including reconfigurationWithSyn?

| Company | Agree/Disagree | Comment |
| --- | --- | --- |
| Xiaomi | Agree |  |
| InterDigital | Agree |  |
| Ericsson | Agree |  |
| Apple | Agree |  |
| OPPO | Agree | As in legacy |
| vivo | Disagree | Same as in Q2.1-1, to avoid the mismatching between the UEs. |

Other than the two cases above, one may ask whether there is any other left use case for dedicated RRC signalling.

Q2.1-3: For SL BC and GC, for in-coverage case, other than the two cases in Q2.1-1/2, is there any other use case for RRC_CONNECTED TX-UE/RX-UE to obtain DRX configuration from dedicated RRC signalling?

| Company | Yes/No | Comment |
| --- | --- | --- |
| Xiaomi | Yes | We understand it’s optional for NW to include SL BC/GC DRX in SIB, even NW is capable of SL DRX. If NW choose not to include these in SIB, dedicated signalling shall be used to carry SL BC and GC DRX configuration to CONNECTED UE. Otherwise, SL DRX capable gNB is mandated to include BC/GC DRX configuration in SIB. |
| InterDigital | No |  |
| Ericsson | No |  |
| Apple | No | We need ensure the common DRX configuration is consistent among UEs in broadcast, even for RRC_CONNECTED UEs. The legacy SL-configDedicated IE defined in R16 does not guarantee that as the RRC spec is very ambiguous about this aspect (e,g. it is unclear whether a resource pool configured in the dedicated signaling is the same common pool used in SIB), so we prefer to not reuse dedicated RRC IE for this DRX configuration. |
| OPPO | No | As clarified in 8830, whether UE-specific RRC is feasible: - it is obviously not feasible for Rx-UE, since data delivery of group-cast and broadcast is naturally not per-UE; - it could be feasible for Tx-UE, i.e., it can be used to configure per-UE TX resources, as a subset of RX time occasions allowed by the DRX configuration for RX-UE; But then if one for the latter point goes for the UE-specific DRX configuration, it should be achievable already by configuring UE-specific TX pool (mode-2) or via UE-specific SL grant provisioning (mode-1). |
| vivo | No |  |

Q2: How to configure SL DRX on-duration and inactivity timers for GC/BC?

Based on the RAN2#114 agreement,

4:	For GC/BC, DRX cycle is configured per QoS profile.

5a:	For GC/BC, RAN2 understands that sl-drx-startoffset does not take QoS requirement into consideration.

5b:	For GC/BC, For GC/BC, sl-drx-startoffset is set based on DST L2 ID.

So there are left issues on the configuration granularity for on-duration timer length and inactivity timer length.

Q2.2-1: For BC/GC, what is the granularity for configuration of on-duration timer length?

Option-1: Per QoS profile

Option-2: Per DST L2 ID

Option-3: Others (if this option is selected, please indicate the preferred granularity)

| Company | Option(s) | Comment |
| --- | --- | --- |
| Xiaomi | Option-1 | We prefer common solution. |
| InterDigital | Option-1 | We think the QoS has an effect on the on-duration (similar to the DRX cycle) |
| Ericsson | Option-1 | Agree with Interdigital |
| Apple | Option-1 |  |
| OPPO | Option-1 |  |
| vivo | Option-1 | The granularity of DRX cycle is already agreed relying on the QoS profile. However, regardless of single or multiple QoS profiles, however, the SL DRX on-duration timers should be finally condensed to a single configuration, mapped to L2 destination ID. |

Considering the following agreement, inactivity time is not applicable to BC

15:	SL Inactivity timer is not supported for broadcast transmissions.

Q2.2-2: For GC, what is the granularity for configuration of inactivity timer length?

Option-1: Per QoS profile

Option-2: Per DST L2 ID

Option-3: Others (if this option is selected, please indicate the preferred granularity)

| Company | Option(s) | Comment |
| --- | --- | --- |
| Xiaomi | Option-1 |  |
| InterDigital | Option-1 | We think the QoS has an effect on the inactivity timer (similar to the DRX cycle) |
| Ericsson | Option-1 |  |
| Apple | Option-1 |  |
| OPPO | Option-1 |  |
| vivo | Option-1 | The granularity of DRX cycle is already agreed relying on the QoS profile. However, regardless of single or multiple QoS profiles, the SL DRX inactivity timers should be finally condensed to a single configuration, mapped to L2 destination ID. |

Q3: How to configure SL DRX RTT and retransmission timers for GC/BC?

The issue here is similar to Q2 above. Besides, considering that although the need of RTT/Re-tx timer for BC is still FFS, pending on the conclusion in [POST114-e][706], but it is of clear majority support, rapp understand the questions for Q3 can be limited to GC for now.

Proposal 13 –SL HARQ RTT timer and SL Retransmission timer are not used for broadcast transmissions [13/15].

Q2.3-1: For GC, what is the granularity for configuration of RTT timer length?

Option-1: Per QoS profile

Option-2: Per DST L2 ID

Option-3: Per retransmission mode

Option-4: Others (if this option is selected, please indicate the preferred granularity)

| Company | Option(s) | Comment |
| --- | --- | --- |
| Xiaomi | Option-3 | We understand the RTT timer is decided in following way. RTT is configured per resource pool considering the PSFCH allocation in feedback based retransmission. RTT timer should be a fixed value in blind retransmission. RTT timer is selected per retransmission mode. We don’t see the relation between RTT and QoS profile, since the RTT is to describe the round trip time. |
| InterDigital | Option-4 (others) | RTT timer length can does not depend on QoS profile, but can have granularity that goes beyond L2 ID. Namely, HARQ RTT should depend at least on whether SCI contains the a retransmission resource, whether HARQ is enabled/disabled. Note: these aspects have already been discussed in [706] and pending RAN1 LS, so suggest to not discuss HARQ RTT in this email discussion. |
| Ericsson | comments | Uncertain whether a configuration granularity is needed. It is obvious that neither QoS profile nor L2 ID is suitable. |
| Apple | Neither 1 or 2 | We do not think Option1/2 is right. But it is too early to decide how to determine “granularity” for GC or even whether there is a need to introduce multiple granularity. We prefer to discuss this after SL unicast solutions is fully resolved. |
| OPPO | Option-4 | We do not think either per-QoS or per-L2-ID configuration is needed, a QoS/L2-ID agnostic configuration is sufficient, i.e., a single value, which is especially helpful for mode-1 scheduling where it is hard for the network to know the associated QoS to derive the length of RTT/Re-tx timer correctly. |
| vivo | Option-2 | The RTT timer is not related to the QoS profile. For simplicity, the granularity for configuration of RTT timer length is based on DST L2 ID. As working assumption (waiting for RAN1 to feedback), meanwhile, SL HARQ RTT timer can be derived from the retransmission resource timing when the SCI indicates a retransmission resource, as option-3. |

Q2.3-2: For GC, what is the granularity for configuration of re-transmission timer length?

Option-1: Per QoS profile

Option-2: Per DST L2 ID

Option-3: Others (if this option is selected, please indicate the preferred granularity)

| Company | Option(s) | Comment |
| --- | --- | --- |
| Xiaomi | Option-1 |  |
| InterDigital | Option-3 (Others) | Similar response to previous question. |
| Ericsson | comments | Uncertain whether a configuration granularity is needed. It is obvious that neither QoS profile nor L2 ID is suitable. |
| Apple | Neither 1 or 2 | Same concern as Q2.3-1 |
| OPPO | Option-3 | We do not think either per-QoS or per-L2-ID configuration is needed, a QoS/L2-ID agnostic configuration is sufficient, i.e., a single value, which is especially helpful for mode-1 scheduling where it is hard for the network to know the associated QoS to derive the length of RTT/Re-tx timer correctly. |
| vivo | Option-2 | The re-transmission timer is not tightly related to the QoS profile. For simplicity, the granularity for configuration of re-transmission timer length is based on DST L2 ID. |

Q4: Need of down-select other DRX configurations for a specific L2 DST ID if the UE has multiple QoS profiles for same DST L2 ID? If needed, how to do down-selection?

So far, R2 has already conclude on the need of per-QoS configuration for DRX cycle, so this question is at least valid for DRX cycle.

Q2.4-1a: If the UE has multiple QoS profiles, and thus they associate with different DRX cycle length value(s), for same DST L2 ID, do you think TX/RX UE has to down-select to a single associated DRX cycle length value?

| Company | Yes/No | Comment |
| --- | --- | --- |
| Xiaomi | Yes | Without down selection, the UE has to wake in multiple DRX cycle, so the power saving gain is marginal. |
| InterDigital | Yes | The MAC layer cannot see QoS profile, and so down selection to a single DRX cycle length per L2 ID is needed for the MAC layer to manage this timer. |
| Ericsson | No | In our views the down-selection is not needed, as it will increase design complexity without any clear benefit. If a UE has multiple QoS profiles for the same DST L2 ID, the UE could just have multiple DRX cycles configured at the same time, the UE will be in active time if any of the on-duration timers associated to the DRX cycles is running. |
| Apple | No with comments | In regards of saving power, it is reasonable to achieve a single on-duration ( aligned in time domain) period for the L2 address in this case even when multiple different DRX cycles are used. Therefore, the DRX cycle per QoS profile needs to be configured in some form like T, 2T, 4T, 8T so that wake-up time can overlaps automatically. Then, there is no need for down-selection. |
| OPPO | No | As clarified in 8830, SL QoS have multiple dimensions, and the ordering of one dimension is not necessarily the same as the other dimension (one has to consider of non-standardized QoS and has to consider there might be new PQI added into the standardized PQI table). So down-selection based on a single dimension of the QoS is not feasible. Another way-out is to select - With the shortest DRX cycle within the ones corresponding to the QoS associated with the service; - With the longest on-duration timer within the ones corresponding to the QoS associated with the service; Although this is a feasible way-out to derive a same / single DRX pattern for both Tx and Rx UE, it does not necessarily achieve better performance than following multiple DRX patterns, e.g., considering two associated QoS - DRX1 for QoS1, shorter on-duration + shorter DRX cycle; - DRX2 for QoS2, longer on-duration + longer DRX cycle; So, the final decided DRX of longer on-duration + shorter DRX cycle would have higher power consumption for both DRX1 and DRX2 – in the extreme case, if the longer on-duration timer length > shorter DRX cycle, UE may end up with no power saving gain at all. |
| vivo | Yes | We prefer to have single DRX configuration condensed per L2 destination ID. |

If one answer Yes to Q2.4-1a, how to do the down-selection? Rapp observed some proposals in companies submitted tdocs

- Select the DRX configuration associated with the QoS profile whose priority level is the highest

- Select the DRX configuration associated with the QoS profile whose PDB is the smallest

- Select the DRX configuration whose DRX cycle is the smallest

Q2.4-1b: If one answer Yes to Q2.4-1a, how to do the down-selection

Option-1: Select the DRX configuration associated with the QoS profile whose priority level is the highest

Option-2: Select the DRX configuration associated with the QoS profile whose PDB is the smallest

Option-3: Select the DRX configuration whose DRX cycle is the smallest

Option-4: Others (if this option is selected, please indicate the preferred granularity)

| Company | Option(s) | Comment |
| --- | --- | --- |
| Xiaomi | Option-3 | We understand option-2 and option-3 should be the same in practice. Small PDB requires small DRX cycle. Regarding option-1, high priority level doesn’t necessarily requrie small DRX cycle. If a long DRX cycle is selected, the delay requirement may not be fulfilled for the low priority QoS profile which requires low latency. |
| InterDigital | Option-3 | We think this approach is the simplest to specify in the MAC, since the MAC layer is not aware of the QoS profile. |
| Apple | Option 4 | Given that there may multiple service types mapped to the same L2 address, using Option 3 does not guarantee the UEs are still wake-up at the same time because the “smallest” DRX cycle may be different among the UEs. It is still reasonable to ensure DRX cycle is configured in a exponential sequence so that the onDurations are always overlapping. |
| vivo | Option-2 | It should rely on the minimum PDB among the QoS profiles. |

The same issue is applicable to the other DRX settings.

For on-duration timer length:

Q2.4-2a: If one selected option-1 for Q2.2-1, and if the UE has multiple QoS profiles, and thus they associate with different on-duration timer length value(s), for same DST L2 ID, do you think TX/RX UE has to down-select to a single associated on-duration timer length value?

| Company | Yes/No | Comment |
| --- | --- | --- |
| Xiaomi | Yes | Since we prefer to down select to one DRX cycle, on-duration timer should also down-select to one. |
| InterDigital | Yes | Same reasoning as our response for Q2.4-1a |
| Ericsson | No | Same reasoning as our response for Q2.4-1a |
| Apple | No | As explained in Q2.4-1, the key is to make sure the on-Duration overlaps (with the same offset), no need of the down-selection. |
| OPPO | No | As clarified in 8830, SL QoS have multiple dimensions, and the ordering of one dimension is not necessarily the same as the other dimension (one has to consider of non-standardized QoS and has to consider there might be new PQI added into the standardized PQI table). So down-selection based on a single dimension of the QoS is not feasible. Another way-out is to select - With the shortest DRX cycle within the ones corresponding to the QoS associated with the service; - With the longest on-duration timer within the ones corresponding to the QoS associated with the service; Although this is a feasible way-out to derive a same / single DRX pattern for both Tx and Rx UE, it does not necessarily achieve better performance than following multiple DRX patterns, e.g., considering two associated QoS - DRX1 for QoS1, shorter on-duration + shorter DRX cycle; - DRX2 for QoS2, longer on-duration + longer DRX cycle; So, the final decided DRX of longer on-duration + shorter DRX cycle would have higher power consumption for both DRX1 and DRX2 – in the extreme case, if the longer on-duration timer length > shorter DRX cycle, UE may end up with no power saving gain at all. |
| vivo | Yes | Regardless of single or multiple QoS profiles, the SL DRX on-duration timers are finally condensed to a single configuration, mapped to L2 destination ID. |

Q2.4-2b: If one answer Yes to Q2.4-2a, how to do the down-selection

Option-1: Select the DRX configuration associated with the QoS profile whose priority level is the highest

Option-2: Select the DRX configuration associated with the QoS profile whose PDB is the smallest

Option-3: Select the DRX configuration whose on-duration timer length is the largest

Option-4: Select the on-duration timer associated with the QoS profile, which is associated with the selected DRX cycle.

Option-5: Others (if this option is selected, please indicate the preferred granularity)

| Company | Option(s) | Comment |
| --- | --- | --- |
| Xiaomi | Option-4 | As DRX cycle has been selected as in Q2.4-1b, the on-duration timer, which is associated with the same QoS profile, should also be selected. Otherwise, there may be the case that on-duration timer is longer than DRX cycle and UE can’t go to sleep. Initial transmission is restricted within the on-duration timer running. Inactivity timer could provide extended wakeup time for subsequent transmission. |
| InterDigital | Option 3 | Same reasoning as our response for Q2.4-1b |
| vivo | Option-3 |  |

For inactivity timer length:

Q2.4-3a: If one selected option-1 for Q2.2-2, and if the UE has multiple QoS profiles, and thus they associate with different inactivity timer length value(s), for same DST L2 ID, do you think TX/RX UE has to down-select to a single associated inactivity timer length value?

| Company | Yes/No | Comment |
| --- | --- | --- |
| Xiaomi | Yes | MAC entity is not aware of the QoS profile of the received MAC PDU. It’s not feasible for MAC to start different inactivity timer based on QoS profile. |
| InterDigital | Yes | Same reasoning as our response for Q2.4-1a |
| Ericsson | No | Same reasoning as our response for Q2.4-1a. in addition, the SCI carries the priority information, which may be used to identify the associated QoS profile. |
| Apple | No | If multiple inactivity timers are configured, the RX UE only need to (re)start the corresponding timer based on SCI priority. We do not see a need for further restriction of UE behaviour. |
| vivo | Yes | Regardless of single or multiple QoS profiles, the SL DRX inactivity timers are finally condensed to a single configuration, mapped to L2 destination ID. |

Q2.4-3b: If one answer Yes to Q2.4-3a, how to do the down-selection

Option-1: Select the DRX configuration associated with the QoS profile whose priority level is the highest

Option-2: Select the DRX configuration associated with the QoS profile whose PDB is the smallest

Option-3: Select the DRX configuration whose inactivity timer length is the largest

Option-4: Others (if this option is selected, please indicate the preferred granularity)

| Company | Option(s) | Comment |
| --- | --- | --- |
| Xiaomi | Option-3 | The largest length should be selected to cover the longet interval between initial and subsquent transmission. |
| InterDigital | Option-3 | Same reasoning as our response for Q2.4-1b |
| vivo | Option-3 |  |

For RTT timer length:

Q2.4-4a: If one selected option-1 for Q2.3-1, and if the UE has multiple QoS profiles, and thus they associate with different RTT timer length value(s), for same DST L2 ID, do you think TX/RX UE has to down-select to a single associated RTT timer length value?

| Company | Yes/No | Comment |
| --- | --- | --- |
| InterDigital | No | The HARQ RTT timer and retransmission timers are determined per HARQ process, based on the factors we commented in Q2.3-1, so there is no discussion of down-selection needed for this question. |
| Ericsson | No | See our comments for Q2.3-1 |
| Apple | No | We are not sure multiple granularity is needed in the first place. |
| OPPO | No | See our comments for Q2.3-1 |

Q2.4-4b: If one answer Yes to Q2.4-4a, how to do the down-selection

Option-1: Select the DRX configuration associated with the QoS profile whose priority level is the highest

Option-2: Select the DRX configuration associated with the QoS profile whose PDB is the smallest

Option-3: Select the DRX configuration whose RTT timer length is the smallest

Option-4: Others (if this option is selected, please indicate the preferred granularity)

| Company | Option(s) | Comment |
| --- | --- | --- |

For Retransmission timer length:

Q2.4-5a: If one selected option-1 for Q2.3-2, and if the UE has multiple QoS profiles, and thus they associate with different Retransmission timer length value(s), for same DST L2 ID, do you think TX/RX UE has to down-select to a single associated Retransmission timer length value?

| Company | Yes/No | Comment |
| --- | --- | --- |
| Xiaomi | Yes |  |
| InterDigital | No | The HARQ RTT timer and retransmission timers are determined per HARQ process, based on the factors we commented in Q2.3-1, so there is no discussion of down-selection needed for this question. |
| Ericsson | No | See our comments for Q2.3-1 |
| Apple | No | We are not sure multiple granularity is needed in the first place. |
| OPPO | No | See our comment for Q2.3-2 |

Q2.4-5b: If one answer Yes to Q2.4-5a, how to do the down-selection

Option-1: Select the DRX configuration associated with the QoS profile whose priority level is the highest

Option-2: Select the DRX configuration associated with the QoS profile whose PDB is the smallest

Option-3: Select the DRX configuration whose Retransmission timer length is the largest

Option-4: Others (if this option is selected, please indicate the preferred granularity)

| Company | Option(s) | Comment |
| --- | --- | --- |
| Xiaomi | Option-3 | The largest length should be selected to cover the longest transmission duration. |

Q5: Need to define default DRX configuration for GC/BC?

As raised in [7], a default DRX configuration is helpful so that in case a QoS profile cannot be mapped to the configured QoS-to-DRX mapping (considering it is difficult for network configuration to exhaust all possible QoS profiles), it can be mapped to the default DRX configuration.

Q2.5-1: Do you agree to introduce a default DRX configuration for GC/BC, so that in case a QoS profile cannot be mapped to the DRX parameters configured for the dedicated QoS profiles, it can be mapped to the default DRX configuration.

| Company | Agree/Disagree | Comment |
| --- | --- | --- |
| Xiaomi | Agree |  |
| InterDigital | Agree |  |
| Ericsson | agree | We think RTT timer and retransmission timer can be mapped to the default DRX configuration. |
| Apple | Agree |  |
| OPPO | Agree |  |
| vivo | Agree | There are two purposes to introduce a default DRX configuration for GC/BC. One is, as interpreted in Q2.5-1, it can be used in case that a QoS profile cannot be mapped to the DRX parameters. The other is, more efficiently, we can define a default DRX configuration for GC/BC, whereby the Rx UEs can operate the DRX cycle with the minimum power consumption, independently of the service types and the destination Layer 2 IDs. This is because either considering the down-selected DRX configuration or DRX configuration corresponding to multiple QoS profiles leads the Rx UEs to be awake on most occasions, and results in an inefficient power saving. It is noted that, as long as the Rx UE(s) receives the first packet in On-duration of the default DRX cycle, the Rx UE(s) can start to run the DRX pattern configured with the QoS profile(s) and L2 destination ID, parallelly in the default DRX pattern. |

Q6: Need for SL DRX MAC CE for GC/BC

DRX MAC CE was adopted for Uu in unicast case, it is questionable whether it should be adopted for GC/BC case for SL.

Q2.6-1: Do you agree NOT to adopt DRX MAC CE for sidelink GC/BC case in Rel-17?

| Company | Agree/Disagree | Comment |
| --- | --- | --- |
| Xiaomi | Agree |  |
| InterDigital | Agree |  |
| Ericsson | Agree |  |
| Apple | No MAC CE for GC/BC |  |
| OPPO | Agree, i.e., no MAC CE for GC/BC |  |
| vivo | Disagree |  |

- xxx.

Conclusion

We have the following proposals

Proposal 1	xxx.

Reference

- R2-2106985	Leftover Issues for Sidelink Unicast DRX	CATT	discussion	Rel-17	NR_SL_enh-Core

- R2-2106986	Leftover Issues for Sidelink Groupcast and Broadcast DRX	CATT	discussion	Rel-17	NR_SL_enh-Core

- R2-2106987	Further Issues Regarding to the Tx Profile	CATT	discussion	Rel-17	NR_SL_enh-Core

- R2-2106988	Impacts of SL DRX on Other Procedures	CATT	discussion	Rel-17	NR_SL_enh-Core

- R2-2107041	Discussion on left issue from [704][705][706]	OPPO	discussion	Rel-17	NR_SL_enh-Core

- R2-2107151	NR SL DRX	Fraunhofer IIS, Fraunhofer HHI	discussion	Rel-17

- R2-2107155	Consideration on sidelink DRX for groupcast and broadcast	Huawei, HiSilicon	discussion	Rel-17	NR_SL_enh-Core

- R2-2107156	Remaining issues on the sidelink DRX for unicast	Huawei, HiSilicon	discussion	Rel-17	NR_SL_enh-Core

- R2-2107157	Discussion on SL communication impact on Uu DRX	Huawei, HiSilicon	discussion	Rel-17	NR_SL_enh-Core

- R2-2107190	Left issues on SL-DRX	OPPO	discussion	Rel-17	NR_SL_enh-Core

  - Revised in R2-2108830

- R2-2108830	Left issues on SL-DRX	OPPO	discussion	Rel-17	NR_SL_enh-Core

- R2-2107191	Discussion on SL-DRX impact to mode-1 scheduling	OPPO	discussion	Rel-17	NR_SL_enh-Core

- R2-2107238	Leftover issues on overall flow of unicast TX-UE centric mechanism	NEC Corporation	discussion

- R2-2107239	Discussion on DRX suspend/resume mechanism	NEC Corporation	discussion

- R2-2107242	Further discussion on Uu/SL DRX timer	LG Electronics France	discussion	Rel-17	NR_SL_enh-Core

- R2-2107269	Resource Allocation Considering DRX	InterDigital	discussion	Rel-17	NR_SL_enh-Core

- R2-2107270	Open Issues on SL DRX Timers	InterDigital	discussion	Rel-17	NR_SL_enh-Core

- R2-2107271	DRX Configuration Determination in Unicast	InterDigital	discussion	Rel-17	NR_SL_enh-Core

- R2-2107310	On SL DRX Configuration aspects	Intel Corporation	discussion	Rel-17	NR_SL_relay-Core

- R2-2107311	Discussion on SL DRX Timers	Intel Corporation	discussion	Rel-17	NR_SL_enh-Core

- R2-2107312	On DRX wake-up time alignment	Intel Corporation	discussion	Rel-17	NR_SL_enh-Core

- R2-2107355	Remaining issues on DRX Timers for SL Unicast	Spreadtrum Communications	discussion	Rel-17

- R2-2107432	Consideration on Backward compatibility for SL DRX	ZTE Corporation, Sanechips	discussion	Rel-17	NR_SL_enh-Core

- R2-2107433	Further consideration on DRX configuration	ZTE Corporation, Sanechips	discussion	Rel-17	NR_SL_enh-Core

- R2-2107434	Discussion on  SL DRX  timer	ZTE Corporation, Sanechips	discussion	Rel-17	NR_SL_enh-Core

- R2-2107472	Remaining aspects of SL DRX	Ericsson	discussion	Rel-17	NR_SL_enh-Core

- R2-2107474	Handling coexistence between UEs supporting different releases	Ericsson	discussion	Rel-17	NR_SL_enh-Core

- R2-2107626	Discussion on remaining issues of SL DRX configurations	Apple	discussion	Rel-17	NR_SL_enh-Core

- R2-2107627	Discussion on remaining issues of SL impact of Uu-DRX	Apple	discussion	Rel-17	NR_SL_enh-Core

- R2-2107653	Remaining details on HARQ RTT and Retransmission Timer for SL DRX	Fujitsu	discussion	Rel-17	NR_SL_enh-Core	R2-2105400

- R2-2107654	SL DRX impact on LCP	Fujitsu	discussion	Rel-17	NR_SL_enh-Core	R2-2105401

- R2-2107968	DRX impact on Uu	Xiaomi communications	discussion

- R2-2107969	Discussion on Sidelink DRX for unicast	Xiaomi communications	discussion

- R2-2107970	Discussion on Sidelink DRX for broadcast and groupcast	Xiaomi communications	discussion

- R2-2108014	DRX Configuration for UC BC GC and its interaction with Sensing	Lenovo Mobile Com. Technology	discussion	NR_SL_enh-Core

- R2-2108016	DRX coordination between Uu and SL	Lenovo Mobile Com. Technology	discussion	NR_SL_enh-Core

- R2-2108072	Proposals for Sidelink DRX	Sony	discussion	Rel-17	NR_SL_enh-Core

- R2-2108151	Consideration on TX centric SL DRX configuration and alignment	LG Electronics Inc.	discussion	Rel-17	NR_SL_enh-Core

- R2-2108214	Discussion on Compatible Issues with Rel 16 UEs 	Qualcomm Finland RFFE Oy	discussion

- R2-2108215	Discussion on RLF and PC5 RRC Connection with SL DRX 	Qualcomm Finland RFFE Oy	discussion

- R2-2108217	Discussion on Remaining Issues 	Qualcomm Finland RFFE Oy	discussion

- R2-2108222	A Default PC5 DRX Configuration for Broadcast/Groupcast/Unicast	vivo	discussion

- R2-2108223	DRX duration calculation	vivo, Xiaomi, ZTE corporation	discussion

- R2-2108224	Remaining issues on SL DRX for unicast/groupcast/broadcast	vivo	discussion

- R2-2108426	Discussion on TBD/FFS	Samsung Research America	discussion

- R2-2108427	Further consideration for SL DRX operation in groupcast	Samsung Research America	discussion

- R2-2108428	Further consideration for SL DRX and Uu DRX alignments	Samsung Research America	discussion

- R2-2108469	Discussion on alignment of mode 1 RA of Tx UE and SL DRX of Rx UE	Nokia, Nokia Shanghai Bell	discussion	NR_SL_enh-Core

- R2-2108470	Further Issues on Sidelink Traffic Pattern for SL DRX Configuration	Nokia, Nokia Shanghai Bell	discussion	Rel-17	NR_SL_enh-Core	R2-2105958

- R2-2108471	SL DRX for SL groupcast	Nokia, Nokia Shanghai Bell	discussion	NR_SL_enh-Core

- R2-2108765	SL DRX enabled UE Mode 2 operation 	ITL	discussion	Rel-17

- R2-2108822	Remaining issues of SL DRX	MediaTek Inc.	discussion	Rel-17	NR_SL_enh-Core