3GPP TSG RAN2 #115-e								R2-21xxxxx

Online, 16 – 27 August 2021

Agenda Item:		8.13.2.1	Handover related SON aspects

Source:			Huawei (email rapporteur)

Title:			Report of [AT115e][852][SONMDT] Procedures and Modeling of successful HO (Huawei)

Document for:		Discussion and Decision

1	Introduction

This is the email report of [AT115e][852] SONMDT:

[AT115e][852][SON/MDT] Procedures and Modeling of successful HO (Huawei)

Scope: Focus on the agreeable proposals in R2-2108564

Intended outcome: Report with Agreements

Deadline: 11:00 UTC, Wednesday August 25th

The company contact list is the same as the list in R2-2108564 [1].

| Company | Name | Email Address |
| --- | --- | --- |
| Qualcomm | Rajeev Kumar | rkum@qti.qualcomm.com |
| OPPO | Liu yang | liuyangbj@oppo.com |
| Lenovo | Lianhai | Wulh5@lenovo.com |
| Samsung | Sangbum Kim | Sb07.kim@samsung.com |
| Huawei, HiSilicon | Jun Chen | jun.chen@huawei.com |
| NEC | Wangda | wang_da@nec.cn |
| Sharp | Ningjuan Chang | Ningjuan.chang@cn.sharp-world.com |
| CATT | Erlin Zeng | erlin.zeng@catt.cn |
| vivo | Wen-Ming | ming.wen@vivo.com |
| Ericsson | Marco Belleschi | Marco.belleschi@ericsson.com |
| Nokia, Nokia Shanghai Bell | Malgorzata Tomala | malgorzata.tomala@nokia.com |
| LGE | SangWon Kim | sangwon7.kim@lge.com |
| ZTE | Zhihong-QIU | qiu.zhihong@zte.com.cn |

2	Discussion

The agreeable proposals in R2-2108564 are listed as below:

Proposal 2: For the thresholds of T310/T312 in the source cell, the source cell configures the values.

Proposal 3: Introduce a UE capability indication for SHR.

Proposal 4: The UE may discard the SHR, i.e. release the UE variable VarSuccHO-Report, 48 hours after the SHR is stored.

Q1: which of the above proposals are acceptable?

| Company | All or some | Comments |
| --- | --- | --- |
| vivo | all |  |
| Qualcomm | All |  |
| Ericsson | All |  |
| Sharp | All |  |
| Huawei, HiSilicon | All |  |

P1 is also an agreeable proposal in R2-2108564. In addition, some alternatives on ASN.1 definition are also provided (see section 5), so it is proposed to collect companies’ opinions on the alternatives.

Proposal 1: Defines new values or mix of existing values and new values for the threshold, and explicit values or percentages are candidate solutions.

Q2: Do you agree with P1? In addition, which of the alternatives (shown in section 5) are preferred?

| Company | Yes/No for P1 | Alt1, Alt2, Alt2a, or others | Comments |
| --- | --- | --- | --- |
| vivo | Yes | Either Alt2 or Alt2a is fine to us | We prefer to adopt the options where percentage is used. This offers a cleaner and minimal signalling overheaded compared to the use of explicit values. |
| Qualcomm | Yes | Any alternative seems okay. | However, the threshold should be set properly. 20% seems quite low. In alternatives 2, 2a, we should remove 20%. similarly, in alternative 1, we should avoid setting a quite low value as a threshold. |
| Ericsson | Yes | Any alternative seems ok |  |
| Sharp | Yes | Any alternative seems ok | Agree with Qualcomm that some quite low values may not be useful. |
| Huawei, HiSilicon | Yes | Either Alt2 or Alt2a is ok | Alt1 may introduce lots of values, and thus the signalling overhead is significant. In addition, if T310/T312/T304 is to be extended in the future, more threshold values may also be needed. |

3	Conclusion

[To be updated]

4	References

[1] R2-2108564	Report of [Post114-e][851][SONMDT] Procedures and Modeling of successful HO report (Huawei)	Huawei	discussion

5	Draft changes for the thresholds of T310/T312/T304

The ASN.1 definition is as below:

Alt1: explicit values

Here is an example:

ReportConfigSuccHO-Report-r17 ::=        SEQUENCE {

t310_threshold-r17			ENUMERATED {value1, value2}		OPTIONAL,

t312_threshold-r17			ENUMERATED {value1, value2}		OPTIONAL,

t304_threshold-r17			ENUMERATED {value1, value2}		OPTIONAL

}

t310_threshold The T310_threshold is to indicate the threshold value.

the above field descriptions can be applied for other fields To be added

Alt2: percentages

Here is an example:

ReportConfigSuccHO-Report-r17 ::=        SEQUENCE {

t310_threshold_ratio-r17			ENUMERATED {20%, 40%, 60%, 80%}		OPTIONAL,

t312_threshold_ratio-r17			ENUMERATED {20%, 40%, 60%, 80%}		OPTIONAL,

t304_threshold_ratio-r17			ENUMERATED {20%, 40%, 60%, 80%}		OPTIONAL

}

t310_threshold_ratio The T310_threshold_ratio is to indicate the ratio of the threshold value over the siganlled T310 value. For example, if the signalled T310 value is ms100, and T310_threshold_ratio is 40%, the T310 theshold for the successful handover report is 40ms.

the above field descriptions can be applied for other fields To be added

Alt2a: a common set of percentages + a flag indicating the corresponding event

Here is an example:

ReportConfigSuccHO-Report-r17 ::=        SEQUENCE {

shr_threshold_ratio-r17			ENUMERATED {20%, 40%, 60%, 80%}         	   OPTIONAL,

shr_t310_flag-r17				ENUMERATED {true}          					   OPTIONAL,

shr_t312_flag-r17				ENUMERATED {true}          					   OPTIONAL,

shr_t304_flag-r17				ENUMERATED {true}          					   OPTIONAL

}

shr_threshold_ratio The shr_threshold_ratio is to indicate the ratio of the threshold value over the siganlled timer value. For example, if the signalled T310 value is ms100, and T310_threshold_ratio is 40%, the T310 theshold for the successful handover report is 40ms.

shr_t310_flag The presence of the field indicates that the threshold ratio for triggering the successful HO report applies to T310.

the above field descriptions can be applied for other fields To be added