3GPP TSG-RAN WG2 #116-e	Tdoc R2-2109393

E-meeting, 1st – 12th Month 2021

Agenda Item:	8.24.1

Source:	OPPO, Ericsson, ZTE Corporation, Sanechips

Title:	Discussion on MIMO-dependent BW class and frequency separation

Document for:	Discussion, Decision

Introduction

This contribution is to discuss issue from R4-2114754.

Discussion

Q1

In the LS from R4 in R4-2114754,

Whether current signalling can support UE’s different CA bandwidth class capabilities or different frequency separation class capabilities for UL CA with and without UL MIMO for the same band?

For example:

–	when maxNumberMIMO-LayersCB-PUSCH = 1, the UE can support CA_ n77C;

–	when maxNumberMIMO-LayersCB-PUSCH = 2, the UE can support CA_ n77B;

where maxNumberMIMO-LayersCB-PUSCH is defined as per-CC feature set capability, while BW class is defined per-BC-per-band-entry capability. So

- In a single BC entry, it cannot indicate per-MIMO-layer BW class;

- In two BC entries, it can implement: 1) in one BC-entry, it can indicate the BW class for UL CA (e.g. CA_ n77B) w/ UL-MIMO and 2) in the other BC-entry, it can indicate the BW class for UL CA (e.g. CA_ n77C) w/o UL-MIMO.

RAN2 reply Q1 as Yes, i.e., current signalling can support UE’s different CA bandwidth class capabilities or different frequency separation class capabilities for UL CA with and without UL MIMO for the same band.

Q2

In the LS from R4 in R4-2114754,

Is UE allowed to indicate the capabilities separately in two different band combination entries?

- If yes, whether gNB take account on the different capabilities across different Band combination entries?

If the answer to the above questions is no, RAN4 respectfully ask RAN2 to consider designing new signalling to differentiate such different UE capabilities in UL CA with and without UL MIMO.

To “take into account”, in the context above, meant whether the gNB would be able to reconfigure the UE based on the different reported BCs. As an example (considering the BC 1) and BC 2) defined in the section above), the gNB would “take into account” both band combinations if it could first configure the UE according to the BC 2) and later reconfigure the UE according to the BC 1). Not take into account would imply that e.g. once the gNB configured the UE according to BC 2), it could never consider to configure the UE according to BC 1), and the UE would thus lose the possibility to be configured with UL MIMO.

Therefore, the request above highlight that the “gNB take account on the different capabilities across different Band combination entries”.

However, this does not mean RAN4 tends to put a restriction on the gNB implementation, i.e., force the gNB implementation to “combine” the two entries to define the configurable parameter for a specific BC.

I.e., the intention does not collide with the behavior that expressed in R4-2107621,

In addition, RAN2 would like to point out that UE capability signalling is considered per BC when deciding RRC configuration. Network is not required to derive UE configuration for a BC based on multiple band combination capabilities.

It should be noted, however, that it is up to the network on how to configure the UE. Hence, while replying yes to the first question in the RAN4 LS (i.e. that the UE can indicate support of different CA bandwidth class capabilities or different frequency separation class capabilities for UL CA with and without UL MIMO for the same band), it already implies that the network may take such capabilities into account when configuring the UE. But to address the concern raised, it is suggested to clarify using the following wording

RAN2 reply Q2 as “The network may configure the UE according to a band combination entry, and it may later reconfigure the UE according to another band combination entry” to avoid concern by RAN4.

We can also include in the LS response a further example to RAN4, as below:

“For instance, the network may configure the UE according to a band combination where the UE indicates support of CA_ n77C and maxNumberMIMO-LayersCB-PUSCH = 1; and it may reconfigure the UE according to a band combination where the UE indicates support of CA_ n77B and maxNumberMIMO-LayersCB-PUSCH = 2”.

Conclusion

Based on the discussion in section 2 we propose the following:

Proposal 1	RAN2 reply Q1 as Yes, i.e., current signalling can support UE’s different CA bandwidth class capabilities or different frequency separation class capabilities for UL CA with and without UL MIMO for the same band.

Proposal 2	RAN2 reply Q2 as “The network may configure the UE according to a band combination entry, and it may later reconfigure the UE according to another band combination entry” to avoid concern by RAN4.

References

R4-2114754, LS on signalling for intra-band CA with UL-MIMO