3GPP TSG-RAN WG2 #116-e	R2-2109417

E-meeting, Novemer 2021

Agenda Item:	6.2.3

Source:	OPPO, Apple, Ericsson, Lenovo, Motorola Mobility

Title:	Left issue on MaxTransNum

Document for:	Discussion, Decision

Introduction

This is to discuss the left issues on SL-CG-MaxTransNum.

Discussion

In RAN2#113bis, the following issue has been discussed

Proposal 1	When FB is disabled and if sl-CG-MaxTransNumList is NOT configured, UE judges “next retransmission(s) of the MAC PDU is not required” based on its implementation.

- Agreed.

Proposal 2	When FB is disabled, for CG, if sl-CG-MaxTransNumList is configured with a value not larger than the number of CG resources, when sl-CG-MaxTransNum is reached, UE assumes that next retransmission(s) of the MAC PDU is not required

- Agreed.

- Working assumption: “UE assumes that next retransmission(s) of the MAC PDU is required when FB is disabled, for CG, if sl-CG-MaxTransNumList is configured with a value not larger than the number of CG resources, when sl-CG-MaxTransNum is not reached”

This is for UE to understand how to behave w.r.t the following procedure, i.e.,

2>	else if HARQ feedback has been disabled for the MAC PDU and next retransmission(s) of the MAC PDU is not required:

3>	instruct the physical layer to signal a positive acknowledgement corresponding to the transmission on the PUCCH according to clause 16.5 of TS 38.213 [6].

2>	else if HARQ feedback has been disabled for the MAC PDU and no sidelink grant is available for next retransmission(s) of the MAC PDU, if any:

3>	instruct the physical layer to signal a negative acknowledgement corresponding to the transmission on the PUCCH according to clause 16.5 of TS 38.213 [6].

According to the discussion in RAN1#105, RAN1 concluded as no LS back, which means no concern from RAN1.

Considering UE may not start transmission from the first CG resource, in order to make the WA clearer, the WA can be confirmed by RAN2 for a more general case.

RAN2 confirm the revised WA that “UE assumes that next retransmission(s) of the MAC PDU is required when FB is disabled, for CG, if sl-CG-MaxTransNumList is configured with a value not larger than the number of CG resources available, when sl-CG-MaxTransNum is not reached”.

Then, since RAN2 not agreed to restrict the value for SL-CG-MaxTransNum to be equal to or less than the number of CG resources, there is a missing area, i.e., how for the UE to report A/N when the value of SL-CG-MaxTransNum is configured with larger than CG resources.

RAN2 has not clarified how for the UE to behave in case SL-CG-MaxTransNum is configured as larger than CG resource.

Considering even if “sl-CG-MaxTransNumList is configured with a value NOT larger than the number of CG resources”, UE may not start transmission from the first CG resource, i.e., it may end up with the same result as for the case where “sl-CG-MaxTransNumList is configured with a value larger than the number of CG resources”.

Figure 1 A/N reporting in case of different values of SL-CG-maxTransNum

As shown in the above figure, for CG resources

- In Case1, if the UE is configured with a value of SL-CG-maxTransNum = CG resources = 3, in case the UE start transmission since the last CG resource, the gap between the number of CG resource that can be used for the TB transmission and SL-CG-maxTransNum is 2. According to the WA, UE assumes the next retransmission is required at the last CG resource (report NACK on PUCCH);

- In Case2, if the UE is configured with a value of SL-CG-maxTransNum =5, while CG resources = 3, in case the UE start transmission since the first CG resource, the gap between the number of CG resource that can be used for the TB transmission and SL-CG-maxTransNum is also 2, i.e. for CG resources, there is no difference between Case1 and Case2.

When FB is disabled, if sl-CG-MaxTransNumList is configured with a value larger than the number of CG resources available, when CG resource is exhausted and sl-CG-MaxTransNum is not reached, UE assumes that next retransmission(s) of the MAC PDU is required and thus reporting NACK.

For the following DG resources, the legacy mechanism of DG should be applied for the UE to determine whether the next retransmission(s) of the MAC PDU is required.

When FB is disabled, if sl-CG-MaxTransNumList is configured with a value larger than the number of CG resources, for DG resource allocated when CG resource is exhausted, it is up to UE implementation to report ACK or NACK.

Conclusion

We have the following proposal:

Observation 1	RAN2 has not clarified how for the UE to behave in case SL-CG-MaxTransNum is configured as larger than CG resource.

We have the following proposal:

Proposal 1	RAN2 confirm the revised WA that “UE assumes that next retransmission(s) of the MAC PDU is required when FB is disabled, for CG, if sl-CG-MaxTransNumList is configured with a value not larger than the number of CG resources available, when sl-CG-MaxTransNum is not reached”.

Proposal 2	When FB is disabled, if sl-CG-MaxTransNumList is configured with a value larger than the number of CG resources available, when CG resource is exhausted and sl-CG-MaxTransNum is not reached, UE assumes that next retransmission(s) of the MAC PDU is required and thus reporting NACK.

Proposal 3	When FB is disabled, if sl-CG-MaxTransNumList is configured with a value larger than the number of CG resources, for DG resource allocated when CG resource is exhausted, it is up to UE implementation to report ACK or NACK.

Reference

- R2-2008782	Correction on CG maximum retransmission number	OPPO	CR	Rel-16	38.321	16.2.1	0892	-	F	5G_V2X_NRSL-Core