3GPP TSG-RAN2 Meeting #116 Electronic                  R2- 2109527

# 1st November – 12th November 2021

Agenda item:	8.6.4

Source:	Samsung

Title:	RACH configuration for Small Data Transmission

Document for:	Discussion & Decision

# Introduction

In this contribution we discuss the following aspects of RACH-based SDT

- RACH configuration for SDT

# Discussion

## RACH configuration for SDT

In RAN2#112e, RACH configuration for RACH based SDT was discussed and following was agreed.

RAN2#112e Agreements [1]:

As a baseline, the RACH resource i.e. (RO+preamble combination) is different between SDT and non-SDT

-	If ROs for SDT and non SDT are different, preamble partitioning between SDT and non SDT is not needed.

-	If ROs for SDT and non SDT are same, preamble partitioning is needed

### RA Preambles for SDT

The preambles to be used for SDT depends on whether ROs used for SDT are same as ROs for non SDT or ROs used for SDT are different from ROs for non SDT. The understanding is that both these options for RO configuration will be supported and it's up to network configuration to configure ROs for SDT according to one of these option.

Preambles for SDT using 4 step RA when ROs used for SDT using 4 step RA are same as ROs used for non SDT 4 step RA

Case 1: 2 step RA is not configured; or 2 step RA is configured but ROs for 2 step RA are not shared with 4 step RA

In this case preambles for SDT using 4 step RA can start from end of preambles used for non SDT 4 step RA as described below.

  - ssb-perRACH-Occasion (N1) configured for non SDT 4 step RA

  - CB-PreamblesPerSSB (R1) configured for non SDT 4 step RA

  - CB-PreamblesPerSSB-SDT (X) is configured for SDT using 4 step RA

  - if N1<1, one SS/PBCH block is mapped to 1N1 consecutive valid PRACH occasions and X contention based preambles with consecutive indexes associated with the SS/PBCH block per valid PRACH occasion start from preamble index R1. An example is illustrated in Figure 1(A).

  - If N1≥1, X contention based preambles with consecutive indexes associated with SS/PBCH block n, 0≤n≤N1-1, per valid PRACH occasion start from preamble index n⋅NpreambletotalN1+R1, where Npreambletotal is provided by totalNumberOfRA-Preambles for 4 step random access procedure. An example is illustrated in Figure 1(B).

Figure 1

Case 2: 2 step RA is configured and ROs for 2 step RA are shared with 4 step RA

In this case preambles for SDT using 4 step RA can start from end of preambles used for non SDT 4 step RA and non SDT 2 step RA as described below. Note that non SDT 4 step RA preambles always follow non SDT 2 step RA preambles when ROs for

  - ssb-perRACH-Occasion (N1) configured for non SDT 4 step RA

  - CB-PreamblesPerSSB (R1) configured for non SDT 4 step RA

  - CB-PreamblesPerSSB (R2) configured for non SDT 2 step RA

  - CB-PreamblesPerSSB-SDT (X) is configured for SDT using 4 step RA

  - if N1<1, one SS/PBCH block is mapped to 1N1 consecutive valid PRACH occasions and X contention based preambles with consecutive indexes associated with the SS/PBCH block per valid PRACH occasion start from preamble index R1+R2. An example is illustrated in Figure 2(A).

  - If N1≥1, X contention based preambles with consecutive indexes associated with SS/PBCH block n, 0≤n≤N1-1, per valid PRACH occasion start from preamble index n⋅NpreambletotalN1+R1+R2, where Npreambletotal is provided by totalNumberOfRA-Preambles for 4 step random access procedure. An example is illustrated in Figure 2(B).

Figure 2

Preambles for SDT using 4 step RA when ROs used for SDT are different from ROs for non SDT.

- CB-PreamblesPerSSB-SDT (X) is configured for SDT using 4 step RA

- ssb-perRACH-Occasion-SDT (Y) is configured for SDT using 4 step RA

- if Y<1, one SS/PBCH block is mapped to 1Y consecutive valid PRACH occasions and X contention based preambles with consecutive indexes associated with the SS/PBCH block per valid PRACH occasion start from preamble index 0.

- If Y ≥1, X contention based preambles with consecutive indexes associated with SS/PBCH block n, 0≤n≤Y-1, per valid PRACH occasion start from preamble index n⋅NpreambletotalY, where Npreambletotal is provided by totalNumberOfRA-Preambles for SDT 4 step random access procedure. If totalNumberOfRA-Preambles for SDT 4 step random access procedure is not configured, UE assumes the value is 64.

Instead of defining different rules for different cases/options as described above, simplest approach would be to configure a starting preamble index (S) for SDT using 4 step RA. Preambles can then be determined as follows:

- CB-PreamblesPerSSB-SDT (X) is configured for SDT using 4 step RA

- ssb-perRACH-Occasion-SDT (Y) is configured for SDT using 4 step RA

- if Y<1, one SS/PBCH block is mapped to 1Y consecutive valid PRACH occasions and X contention based preambles with consecutive indexes associated with the SS/PBCH block per valid PRACH occasion start from preamble index S.

- If Y ≥1, X contention based preambles with consecutive indexes associated with SS/PBCH block n, 0≤n≤Y-1, per valid PRACH occasion start from preamble index n⋅NpreambletotalY+S, where Npreambletotal is provided by totalNumberOfRA-Preambles for SDT 4 step random access procedure. If totalNumberOfRA-Preambles for SDT 4 step random access procedure is not configured, UE assumes the value is 64.

Proposal 1: Starting preamble index is configured in 4 step RA configuration for SDT.

Proposal 2: Preambles per SSB per RACH occasion for SDT using 4 step RA is determined as follows:

- X is the number of contention based preambles per SSB for SDT using 4 step RA

- Y is the number of SSBs per RACH occasion for SDT using 4 step RA

- S is the starting preamble index for SDT using 4 step RA

- if Y<1, one SS/PBCH block is mapped to 1Y consecutive valid PRACH occasions and X contention based preambles with consecutive indexes associated with the SS/PBCH block per valid PRACH occasion start from preamble index S. If Y ≥1, X contention based preambles with consecutive indexes associated with SS/PBCH block n, 0≤n≤Y-1, per valid PRACH occasion start from preamble index n⋅NpreambletotalY+S, where Npreambletotal is provided by totalNumberOfRA-Preambles for SDT 4 step random access procedure. If totalNumberOfRA-Preambles for SDT 4 step random access procedure is not configured, UE assumes the value is 64.

Similarly a starting preamble index (S) for SDT using 2 step RA can be configured. Preambles for SDT using 2 step RA can then be determined as follows:

- CB-PreamblesPerSSB-SDT (X) is configured for SDT using 2 step RA

- ssb-perRACH-Occasion-SDT (Y) is configured for SDT using 2 step RA

- if Y<1, one SS/PBCH block is mapped to 1Y consecutive valid PRACH occasions and X contention based preambles with consecutive indexes associated with the SS/PBCH block per valid PRACH occasion start from preamble index S. If Y ≥1, X contention based preambles with consecutive indexes associated with SS/PBCH block n, 0≤n≤Y-1, per valid PRACH occasion start from preamble index n⋅NpreambletotalY+S, where Npreambletotal is provided by totalNumberOfRA-Preambles for SDT 2 step random access procedure. If totalNumberOfRA-Preambles for SDT 2 step random access procedure is not configured, UE assumes the value is 64.

Proposal 3: Starting preamble index is configured in 2 step RA configuration for SDT.

Proposal 4: Preambles per SSB per RACH occasion for SDT using 2 step RA is determined as follows:

- X is the number of contention based preambles per SSB for SDT using 2 step RA

- Y is the number of SSBs per RACH occasion for SDT using 2 step RA

- S is the starting preamble index for SDT using 2 step RA

- if Y<1, one SS/PBCH block is mapped to 1Y consecutive valid PRACH occasions and X contention based preambles with consecutive indexes associated with the SS/PBCH block per valid PRACH occasion start from preamble index S.

- If Y ≥1, X contention based preambles with consecutive indexes associated with SS/PBCH block n, 0≤n≤Y-1, per valid PRACH occasion start from preamble index n⋅NpreambletotalY+S, where Npreambletotal is provided by totalNumberOfRA-Preambles for SDT 2 step random access procedure. If totalNumberOfRA-Preambles for SDT 2 step random access procedure is not configured, UE assumes the value is 64.

### ROs for SDT using 4 step RA

4 step ROs for SDT can be shared with 4 step ROs for non SDT or can be separately configured. Following parameters can be signaled by gNB for configuring ROs for small data transmission using 4 step RA.

prach-ConfigurationIndex-SDT: PRACH configuration index for SDT using 4 step RA. The gNB has the option to not signal prach-ConfigurationIndex-SDT. If prach-ConfigurationIndex-SDT is not signaled by gNB, UE determine PRACH occasions for SDT according to prach-ConfigurationIndex configured/signaled by gNB for non SDT (i.e. in RACH-ConfigGeneric for 4 step RA). This field may only be present in the case of separate ROs for 4 step RA based SDT.

msg1-FDM-SDT: The number of msg1 PRACH transmission occasions Frequency-Division Multiplexed in one-time instance for small data transmission. If the field is absent, UE shall use value of msg1-FDM configured/signaled by gNB for non SDT (i.e. in RACH-ConfigGeneric for 4 step RA). This field may only be present in the case of separate ROs for 4 step RA based SDT.

msg1-FrequencyStart-SDT: Offset of lowest PRACH transmissions occasion in frequency domain with respect to PRB 0. If the field is absent, UE shall use value of msg1-FrequencyStart configured/signaled by gNB for non SDT (i.e. in RACH-ConfigGeneric for 4 step RA). This field may only be present in the case of separate ROs for 4 step RA based SDT.

For flexible signaling of ROs for SDT, following parameters can be configured in RACH configuration for 4 step RA based SDT. Note that these parameters are currently supported only for IAB RACH configuration.

prach-ConfigurationPeriodScaling-SDT Scaling factor to extend the periodicity of the baseline configuration indicated by prach-ConfigurationIndex-SDT. Value scf1 corresponds to scaling factor of 1 and so on.

prach-ConfigurationFrameOffset-SDT Scaling factor for ROs defined in the baseline configuration indicated by prach-ConfigurationIndex-SDT.

prach-ConfigurationSOffset-SDT Subframe/Slot offset for ROs defined in the baseline configuration indicated by prach-ConfigurationIndex-SDT.

Proposal 5: ROs for 4 step RA based SDT are signaled using following parameters: prach-ConfigurationIndex-SDT, msg1-FDM-SDT, msg1-FrequencyStart-SDT, prach-ConfigurationPeriodScaling-SDT, prach-ConfigurationFrameOffset-SDT, prach-ConfigurationSOffset-SDT.

### ROs for SDT using 2 step RA

2 step ROs for SDT can be shared with 2 step ROs for non SDT or can be separately configured. Following parameters are signaled by gNB for configuring ROs for small data transmission using 2 step RA.

msgA-PRACH-ConfigurationIndex-SDT: PRACH configuration index for SDT using 2 step RA. The gNB has the option to not signal msgA-PRACH-ConfigurationIndex-SDT. If msgA-PRACH-ConfigurationIndex-SDT is not signaled by gNB, UE determine PRACH occasions for 2 step RA based SDT according to msgA-PRACH-ConfigurationIndex configured/signaled by gNB for non SDT (i.e. in RACH-ConfigGenericTwoStepRA). This field may only be present in the case of separate ROs for 2 step RA based SDT.

msgA-RO-FDM –SDT: The number of msgA PRACH transmission occasions Frequency-Division Multiplexed in one-time instance for SDT using 2 step RA. If the field is absent, UE shall use value of msgA-RO-FDM configured/signaled by gNB for non SDT in RACH-ConfigGenericTwoStepRA. This field may only be present in the case of separate ROs for 2 step RA based SDT.

msgA-RO-FrequencyStart –SDT: Offset of lowest PRACH transmissions occasion in frequency domain with respect to PRB 0. If the field is absent, UE shall use value of msgA-RO-FrequencyStart configured/signaled by gNB for non SDT in RACH-ConfigGenericTwoStepRA. This field may only be present in the case of separate ROs for 2 step RA based SDT.

For flexible signaling of ROs for SDT, following parameters can be configured in RACH configuration for SDT using 2 step RA. Note that these parameters are currently supported only for IAB RACH configuration.

prach-ConfigurationPeriodScaling-SDT Scaling factor to extend the periodicity of the baseline configuration indicated by prach-ConfigurationIndex. Value scf1 corresponds to scaling factor of 1 and so on.

prach-ConfigurationFrameOffset-SDT Scaling factor for ROs defined in the baseline configuration indicated by prach-ConfigurationIndex.

prach-ConfigurationSOffset-SDT Subframe/Slot offset for ROs defined in the baseline configuration indicated by prach-ConfigurationIndex.

Proposal 6: ROs for 2 step RA based SDT are signaled using following parameters: msgA-PRACH-ConfigurationIndex-SDT, msgA-RO-FDM –SDT, msgA-RO-FrequencyStart –SDT, msgA-prach-ConfigurationPeriodScaling-SDT, msgA-prach-ConfigurationFrameOffset-SDT, msgA-prach-ConfigurationSOffset-SDT

## Handling msgATransMax for 2-step RA-SDT

In RAN2 #115e it was agreed that, as in legacy, UE can be configured to switch from 2-step RA-SDT to 4-step RA-SDT after N times of MsgA transmission. Network may not configure 4-step RA-SDT resources but configure 4-step RA resources. In this case, the issue is whether UE switches to 4 step RA or not after N times of MsgA transmission. Switching to 4 step RA (non SDT) would require UE to rebuild MAC PDU. So we propose that

After N times of MsgA transmission, UE switches to 4 step RA only if 4 step RA resources for SDT are configured. Otherwise UE continue 2 step RA SDT procedure.

Proposal 7: After N times of MsgA transmission, UE switches to 4 step RA only if 4 step RA resources for SDT are configured. Otherwise UE continue 2 step RA SDT procedure.

# Conclusion

Based on the above, RAN2 is requested to discuss and agree on the following proposals:

Proposal 1: Starting preamble index is configured in 4 step RA configuration for SDT.

Proposal 2: Preambles per SSB per RACH occasion for SDT using 4 step RA is determined as follows:

- X is the number of contention based preambles per SSB for SDT using 4 step RA

- Y is the number of SSBs per RACH occasion for SDT using 4 step RA

- S is the starting preamble index for SDT using 4 step RA

- if Y<1, one SS/PBCH block is mapped to 1Y consecutive valid PRACH occasions and X contention based preambles with consecutive indexes associated with the SS/PBCH block per valid PRACH occasion start from preamble index S.

- If Y ≥1, X contention based preambles with consecutive indexes associated with SS/PBCH block n, 0≤n≤Y-1, per valid PRACH occasion start from preamble index n⋅NpreambletotalY+S, where Npreambletotal is provided by totalNumberOfRA-Preambles for SDT 4 step random access procedure. If totalNumberOfRA-Preambles for SDT 4 step random access procedure is not configured, UE assumes the value is 64.

Proposal 3: Starting preamble index is configured in 2 step RA configuration for SDT.

Proposal 4: Preambles per SSB per RACH occasion for SDT using 2 step RA is determined as follows:

- X is the number of contention based preambles per SSB for SDT using 2 step RA

- Y is the number of SSBs per RACH occasion for SDT using 2 step RA

- S is the starting preamble index for SDT using 2 step RA

- if Y<1, one SS/PBCH block is mapped to 1Y consecutive valid PRACH occasions and X contention based preambles with consecutive indexes associated with the SS/PBCH block per valid PRACH occasion start from preamble index S. If Y ≥1, X contention based preambles with consecutive indexes associated with SS/PBCH block n, 0≤n≤Y-1, per valid PRACH occasion start from preamble index n⋅NpreambletotalY+S, where Npreambletotal is provided by totalNumberOfRA-Preambles for SDT 2 step random access procedure. If totalNumberOfRA-Preambles for SDT 2 step random access procedure is not configured, UE assumes the value is 64.

Proposal 5: ROs for 4 step RA based SDT are signaled using following parameters: prach-ConfigurationIndex-SDT, msg1-FDM-SDT, msg1-FrequencyStart-SDT, prach-ConfigurationPeriodScaling-SDT, prach-ConfigurationFrameOffset-SDT, prach-ConfigurationSOffset-SDT.

Proposal 6: ROs for 2 step RA based SDT are signaled using following parameters: msgA-PRACH-ConfigurationIndex-SDT, msgA-RO-FDM –SDT, msgA-RO-FrequencyStart –SDT, msgA-prach-ConfigurationPeriodScaling-SDT, msgA-prach-ConfigurationFrameOffset-SDT, msgA-prach-ConfigurationSOffset-SDT.

Proposal 7: After N times of MsgA transmission, UE switches to 4 step RA only if 4 step RA resources for SDT are configured. Otherwise UE continue 2 step RA SDT procedure.

# References

[1] RAN2-112-e_NR-U_PowSav_2sRA -Rel-17 Sdata_IIoT Notes (Diana)_eom