3GPP TSG-RAN WG2 Meeting #116-e                  	R2-2109744

e-Meeting, 1st-12th Nov, 2021

Source:	vivo, Guangdong Genius

Title:	RRM Relaxation for Neighboring Cell for RedCap UEs

Agenda Item:	8.12.3.2

Document for:	Discussion and Decision

- Introduction

In this paper we discuss some remaining open issues in RRM relaxation for RedCap UEs.

- Discussion

  - RRM Relaxation Criteria for RRC_CONNECTED

    - Procedure of RRM relaxation

During the RAN2#114 meeting, the following was agreed on the framework of the Rel-17 RRM relaxation procedure for RRC_CONNECTED:

Agreements:

- An RSRP/RSRQ based stationarity criterion (Working Assumption: the same as in idle/inactive) can be configured for UEs in RRC Connected. If the criterion is met, this is reported to the network (FFS how/when). It is FFS whether, based on this, besides possibly reconfiguring RRM measurements (up to network implementation), the network can enable RRM measurement relaxation (FFS whether same method as in Idle/Inactive)

In the following RAN2#115 meeting, the following was further agreed.

- The network provides the configuration of stationarity criterion to the UE via dedicated signalling (e.g. RRCReconfiguration message) in RRC_CONNECTED.

Based on the above agreements, one possible procedure for Rel-17 RRM relaxation solution can be following:

- UE is configured with RSRP/RSRQ based stationarity criterion for Rel-17 RRM relaxation via dedicated signalling .

- UE reports the RRM measurement results when the configured criterion is fulfilled.

- NW configures UE to relax the RRM measurements based on the UE reporting.

- UE performs relaxed RRM measurements according to the configuration.

- NW could configure UE to stop performing RRM relaxation when necessary(e.g. based on UE measurement report).

This procedure could be considered as the starting point for RRM relaxation mechanism in RRC_CONNECTED.

Proposal 1：RAN2 to discuss the following Rel-17 RRM relaxation procedure for RRC_CONNECTED:

- UE is configured with RSRP/RSRQ based stationarity criterion for Rel-17 RRM relaxation via dedicated signalling .

- UE reports the RRM measurement results when the configured criteria is fulfilled.

- NW configures UE to relax the RRM measurements based on the UE reporting.

- UE performs relaxed RRM measurements according to to the configuration.

- NW could configure UE to stop performing RRM relaxation when necessary(e.g. based on following UE measurement report).

    - Report Fulfillment of RRM Relaxation Criteria

For RRC_CONNECTED UE to report whether the stationarity criterion is met or not, there are two options on the table[1]:

- Option 1 (12/25): Reuse UEAssistanceInformation message for the report.

- Option 2 (16/25): Reuse RRM measurement reporting mechanism

We prefer option2 with the following 2 reasons:

- In the last meeting, it was agreed that Do not introduce nor reuse not-at-cell-edge threshold for R17 RRC_CONNECTED UEs. The agreement was reached based on the assumption that network can estimate UE’s position(i.e. whether not-at-cell-edge criterion is met or not) based on A1/A2 events. Hence, option2 allows UE to report the fulfillment of not-at-cell-edge and stationarity criterion with the same mechanism, i.e. RRM measurement reporting mechanism.

- If the UE's signal strength is floating around close to the threshold of stationary criterion, it may trigger the UE to send frequent reports indicating that the criterion is fulfilled or not fulfilled. To avoid such frequent reports, the hysteresis and time-to-trigger mechanisms, which has been used in legacy RRM measurement reporting was proposed[2]. Obviously, compared with option1 which requires to introduce hysteresis and time-to-trigger mechanisms for UAI, option2 is a more straightforward selection.

Proposal2: Reuse RRM measurement reporting mechanism to indicates the fulfillment of the Rel-17 RRM relaxation criterion.

    - RRM Relaxation Methods

In Rel-16, when the RRM relaxation criteria are fulfilled, UE in RRC_IDLE/RRC_INACTIVE applies the corresponding relaxation methods defined by RAN4, e.g. relaxed measurements with longer intervals (scaling factor) or stop measurement for up to 1hour.

However, for RRC_CONNECTED UE, there is no Rel-16 like RRM relaxation methods has be defined. Hence, only network implementation-based RRM relaxation methods can be applied. For example, network can reconfigure measurement configuration for RRC_CONNECTED UE via removing some MOs or providing white/black cell list. In our understanding, network implementation-based RRM measurement relaxation methods are not enough and have some drawbacks, e.g. frequently reconfigure measurement configuration will cause a lot of RRC signalling overhead, the effect of power saving is not as good as relaxed measurements with longer intervals, etc. Hence, it makes sense to define RRM measurement relaxation methods for RRC_CONNECTED UE. But anyway, the details of the RRM measurement relaxation methods should be discussed and decided in RAN4.

Proposal 3：To relax RRM measurement in RRC_CONNECTED, in addition to reconfigure measurement with the existing mechanism (e.g. reduce MO number, include white/black cell list), other RRM measurement relaxation methods (e.g. relaxed measurements with longer intervals, i.e. scaling factor, or stop measurement for some time) should be supported. Details of the relaxation methods is up to RAN4 discussion.

    - Configure the Rel-17 RRM Relaxation Criteria via System Information

In the RAN2#115 meeting, it was agreed that the network provides the configuration of stationarity criterion to the UE via dedicated signalling (e.g. RRCReconfiguration message) in RRC_CONNECTED. But it is still FFS whether to support providing the configuration of Rel-17 criterion to RRC_CONNECTED UE via system information[1].

In our understanding, the issue is related to the previous discussion on how to report the fulfillment of criteria in Section 2.1.2. If the proposal2(reuse RRM measurement reporting mechanism) is agreed, the configuration of Rel-17 RRM relaxation criteria can only be sent to UE via dedicated signalling, as the measId, which needs to be included in the measurement report, needs to be configured via dedicated signalling.

Proposal 4: If P2 is agreed, the Rel-17 RRM relaxation criteria cannot be configured via system information for RRC_CONNECTED UE.

    - Transition from RRC_CONNECTED to RRC_IDLE/INACTIVE

Based on the current procedure, UE determines whether the criteria for RRM measurement relaxation are fulfilled in RRC_CONNECTED and RRC_IDLE/RRC_INACTIVE independently. However, as the criteria are defined based on the UE measurement on serving cell, in this way, whether the criteria for RRM measurement relaxation are fulfilled is not changed during UE’s RRC state transition (since the measurement will not change).

For example, a UE in RRC_CONNECTED is stationarity and/or at cell center, it is not likely it starts to move or enters cell edge in a short time after leaving RRC_CONNECTED. Hence, better power saving performance on RRM relaxation can be expected if UE can continue to perform RRM relaxation after RRC state transition as soon as possible. Since the network knows the RRM relaxation criteria for all RRC states, we think it is easy for the network to decide whether a RRC_CONNECTED UE can continue RRM measurement relaxation after RRC state transition. Hence, we propose:

Proposal 5：To allow UE to continue to relax RRM measurement after RRC state transition, NW can indicate the UE via dedicated RRC signaling whether and which criteria for RRM relaxation is considered as satisfied after leaving RRC_CONNECTED state.

  - RRM Relaxation Criteria for RRC_IDLE/INACTIVE

    - Coexistence with R16 configuration

In RAN2#114 meeting, the following scenario was discussed: NW configures R17 RRM relaxation configuration with R16 RRM relaxation configuration together, and a UE fulfills both R16 relaxation criterion and R17 relaxation criterion. For a such case, there were two options on how UE performs RRM relaxation:

- Option 1) UE performs Rel-17 RRM relaxation method

- Option 2) It is up to UE implementation to select either Rel-16 or Rel-17 relaxation operation

There is no agreement on which option should be adopted.

In our understanding, when both Rel-16 and Rel-17 relaxation criterion are fulfilled, UE’s mobility performance can be ensured with either Rel-16 or Rel-17 RRM relaxation methods. Then, it makes sense to select RRM relaxation method which can provide more power saving gain. In the light of this idea, we provide the preference on RRM relaxation method for all the possible coexistence cases of R16 and R17 relaxation criteria as following:

|  | R16 criterion fulfilled | R17 criterion fulfilled | RRM relaxation method which can provide more power saving gain |
| --- | --- | --- | --- |
| Case1 | low mobility criterion | stationarity criterion | Rel-17 RRM relaxation method corresponding to Rel-17 stationarity criterion It is common understanding that the Rel-17 RRM relaxation method for stationarity criterion should provide more RRM relaxation gain than Rel-16 RRM relaxation method for low mobility criterion to target UEs(e.g stationary UE). Otherwise, there is no motivation to specify Rel-17 stationarity criterion on top of Rel-16 low mobility criterion. Hence, UE should of course apply the Rel 17 RRM relaxation method. |
| Case2 | not-at-cell-edge criterion | stationarity criterion | FFS, not sure which relaxation method can provide more power saving gain This case may occur when Rel-16 low mobility criterion is not configured. As the fulfillment of R16 not-at-cell-edge criterion and R17 stationarity criterion indicates the attributes of UE from two different aspects, it is hard to say the RRM relaxation method corresponding to which criterion(i.e. R16 or R17) can provide more power saving gain. Hence, it is difficult to say which relaxation method should be performed. |
| Case3 | low mobility and not-at-cell-edge criteria | stationarity criterion | FFS, not sure which relaxation method can provide more power saving gain It is common understanding that the Rel-16 RRM relaxation method corresponding to both Rel-16 low mobility and not-at-cell-edge criteria can provide more power saving gain than the Rel-16 RRM relaxation method corresponding to Rel-16 low mobility criterion. It is also common understanding that the Rel-17 RRM relaxation method corresponding to R17 stationarity criterion can provide more power saving gain than the RRM relaxation method corresponding to Rel-16 low mobility criterion. But is it hard to tell which of the following RRM relaxation method can provide more power saving gain: - Rel-17 RRM relaxation method corresponding to R17 stationarity criterion - Rel-16 RRM relaxation method corresponding to both Rel-16 low mobility and not-at-cell-edge criteria |
| Case4 | low mobility criterion | stationarity and not-at-cell-edge criteria | Rel-17 RRM relaxation method corresponding to both Rel-17 stationarity and not-at-cell-edge criteria It is common understanding that the Rel-17 RRM relaxation method corresponding to both Rel-17 stationarity and not-at-cell-edge criteria can provide more power saving gain than the Rel-17 RRM relaxation method corresponding to Rel-17 stationarity criterion. It is also common understanding that the Rel-17 RRM relaxation method corresponding to Rel-17stationarity criterion can provide more power saving gain than the Rel-16 RRM relaxation method corresponding to Rel-16 low mobility criterion. Hence, R17 Relaxation method should be selected |
| Case5 | not-at-cell-edge criterion | stationarity and not-at-cell-edge criteria | FFS, not sure which relaxation method can provide more power saving gain Rel-17 stationary criterion is expected to be stricter than Rel-16 low mobility criterion. As Rel-17 not-at-cell-edge criterion can only be configured along with Rel-17 stationarity criterion, it is reasonable to allow looser Rel-17 not-at-cell-edge criterion than Rel-16 to allow more stationary UEs (which do not fulfill the R16 not-at-cell-edge criterion) to perform RRM relaxation. Hence, it is hard to tell which method can will provide more power saving gain. |
| Case6 | low mobility and not-at-cell-edge criteria | stationarity and not-at-cell-edge criteria | FFS, not sure which relaxation method can provide more power saving gain Similar as case5, it is hard to tell which method can will provide more power saving gain. |

According to the above table, Rel-17 RRM relaxation method should be selected for case 1 and 4, while it is hard to tell which relaxation method should be selected for case2,3,5,6. Hence, we propose:

Proposal6：For a UE fulfills both R16 relaxation criterion and R17 relaxation criterion, it is up to UE implementation to select either Rel-16 or Rel-17 relaxation operation.

  - LS to RAN4

Based on the above discussion, we think an LS should be sent to RAN4 to inform them our conclusions on RRM relaxation, e.g. criteria and mechanism, and request RAN4 to discuss the RRM relaxation method on neighboring cells for RedCap UEs in RRC_CONNECTED. A draft LS is provided in Annex.

Proposal 7：Send an LS to inform RAN4 of our conclusions on RRM Relaxation for RRC_CONNECTED and request them to discuss the RRM relaxation methods on neighboring cells for RedCap UEs in RRC_CONNECTED.

- Conclusion

In this contribution, we discussed some remaining open issues in RRM relaxation for RedCap UEs. The proposals are following:

- For RRC_CONNECTED

Proposal 1：RAN2 to discuss the following Rel-17 RRM relaxation procedure for RRC_CONNECTED:

- UE is configured with RSRP/RSRQ based stationarity criterion for Rel-17 RRM relaxation via dedicated signalling .

- UE reports the RRM measurement results when the configured criteria is fulfilled.

- NW configures UE to relax the RRM measurements based on the UE reporting.

- UE performs relaxed RRM measurements according to to the configuration.

- NW could configure UE to stop performing RRM relaxation when necessary(e.g. based on following UE measurement report).

Proposal2: Reuse RRM measurement reporting mechanism to indicates the fulfillment of the Rel-17 RRM relaxation criterion.

Proposal 3：To relax RRM measurement in RRC_CONNECTED, in addition to reconfigure measurement with the existing mechanism (e.g. reduce MO number, include white/black cell list), other RRM measurement relaxation methods (e.g. relaxed measurements with longer intervals, i.e. scaling factor, or stop measurement for some time) should be supported. Details of the relaxation methods is up to RAN4 discussion.

Proposal 4: If P2 is agreed, the Rel-17 RRM relaxation criteria cannot be configured via system information for RRC_CONNECTED UE.

Proposal 5：To allow UE to continue to relax RRM measurement after RRC state transition, NW can indicate the UE via dedicated RRC signaling whether and which criteria for RRM relaxation is considered as satisfied after leaving RRC_CONNECTED state.

- For RRC_IDLE/INACTIVE

Proposal6：For a UE fulfills both R16 relaxation criterion and R17 relaxation criterion, it is up to UE implementation to select either Rel-16 or Rel-17 relaxation operation.

- LS to RAN4

Proposal 7：Send an LS to inform RAN4 of our conclusions on RRM Relaxation for RRC_CONNECTED and request them to discuss the RRM relaxation methods on neighboring cells for RedCap UEs in RRC_CONNECTED.

- References

- R2-2109133 [offline 110] RRM relaxation - second round	Huawei

- R2-2108275 Details on RRM relaxation	Ericsson

Annex - Draft LS to RAN4 on RRM relaxation for RedCap UEs

# 1. Overall Description:

RAN2 have discussed the RRM relaxation on neighboring cells for RedCap UEs in RRC_CONNECTED states, and the following agreements [to be updated after discussion] have been achieved:

The Agreement reached in RAN2#116-e are following [to be updated after discussion]：

- RAN2 to discuss the following Rel-17 RRM relaxation procedure for RRC_CONNECTED: - UE is configured with RSRP/RSRQ based stationarity criterion for Rel-17 RRM relaxation via dedicated signalling . - UE reports the RRM measurement results when the configured criteria is fulfilled. - NW configures UE to relax the RRM measurements based on the UE reporting. - UE performs relaxed RRM measurements according to to the configuration. - NW could configure UE to stop performing RRM relaxation when necessary(e.g. based on following UE measurement report). - Reuse RRM measurement reporting mechanism to indicates the fulfillment of the Rel-17 RRM relaxation criterion. - To relax RRM measurement in RRC_CONNECTED, in addition to reconfigure measurement with the existing mechanism (e.g. reduce MO number, include white/black cell list), other RRM measurement relaxation methods (e.g. relaxed measurements with longer intervals, i.e. scaling factor, or stop measurement for some time) should be supported. Details of the relaxation methods is up to RAN4 discussion. - the Rel-17 RRM relaxation criteria cannot be configured via system information for RRC_CONNECTED UE. - To allow UE to continue to relax RRM measurement after RRC state transition, NW can indicate the UE via dedicated RRC signaling whether and which criteria for RRM relaxation is considered as satisfied after leaving RRC_CONNECTED state. - For a UE fulfills both R16 relaxation criterion and R17 relaxation criterion, it is up to UE implementation to select either Rel-16 or Rel-17 relaxation operation. - Send an LS to inform RAN4 of our conclusions on RRM Relaxation for RRC_CONNECTED and request them to discuss the RRM relaxation methods on neighboring cells for RedCap UEs in RRC_CONNECTED.

# 2. Actions:

# To RAN WG4

# RAN2 kindly request RAN4 to:

- take the above information into account during the following work for RRM relaxation for RedCap UEs.

- Provide feedback, if any.