3GPP TSG-RAN WG2 #116-e	R2-2109914

Electronic meeting, November 1st – 12th, 2021

Agenda Item:	9.1.4

Source: 	Ericsson

Title:	Support of 16-QAM for unicast in UL and DL in NB-IoT

Document for: 	Discussion, Decision

# 1	Introduction

In the Work Item (WI) on “Rel-17 enhancements for NB-IoT and LTE-MTC” [1], one of the objectives is to specify the following enhancement for NB-IoT:

- Specify 16-QAM for unicast in UL and DL, including necessary changes to DL power allocation for NPDSCH and DL TBS. This is to be specified without a new NB-IoT UE category. For DL, increase in maximum TBS of e.g. 2x the Rel-16 maximum, and soft buffer size will be specified by modifying at least existing Category NB2. For UL, the maximum TBS is not increased. [NB-IoT] [RAN1, RAN4] - Extend the NB-IoT channel quality reporting based on the framework of Rel-14—16, to support 16-QAM in DL. [NB-IoT] [RAN2, RAN1, RAN4]

In this contribution, we will discuss 16-QAM support for NB-IoT from a RAN2 perspective, including suggested signaling design.

# Discussion

## Signaling

In RAN2 #115-e, the following agreements were reached.

Agreements: For 16QAM - Confirm the working assumption: The support of 16-QAM uses separate UE capabilities for DL and UL. - 16QAM is configured via dedicated signaling separately for UL and DL.

Based on RAN2#115-e agreement, 16-QAM UE capability and dedicated RRC signaling can be defined.

Text proposals for UE capabilities are provided in Annex A.1 and Annex A.2.

A text proposal for dedicated RRC signaling to configure 16-QAM is provided in Annex A.3.

## Downlink power allocation and uplink power control

Regarding the downlink power configuration to support 16-QAM, RAN1 has reached the following agreements:

Confirm working assumption: Working Assumption - For downlink power allocation to support 16QAM: - For standalone and guard-band deployments: - One power ratio is signalled optionally - NPDSCH EPRE to NRS EPRE in symbols without NRS - The same transmit power is assumed across different symbols. - If the signalling is not indicated, the legacy power allocation is used. - i.e., the ratio of NPDSCH EPRE to NRS EPRE is 0dB for one NRS antenna port, and -3dB for two NRS antenna ports - UE specific signalling is used Working Assumption For downlink power allocation to support 16QAM: - For inband deployments, a power ratio is signaled in addition to the signalling for standalone and guard-band deployments which in this case applies to “symbols with NRS” and “symbols without NRS nor CRS”. - the power ratio between NPDSCH EPRE and NRS EPRE in symbols with CRS is signalled - the signalling is UE specific Note: “symbols with NRS” and “symbols without NRS nor CRS” have the same power.

Based on RAN1 working assumption, UE specific power ratio is optionally configured for standalone and guard-band deployment, and for in-band deployment, UE specific power ratio in symbols with CRS should be mandatory configured.

Optional UE specific power ratio for symbols without NRS nor CRS and mandatory UE specific power radio for IB in symbols with CRS should be added in NPDSCH-Config-NB.

Regarding the uplink power control to support 16-QAM, RAN1 has not concluded with the final solution,

Working Assumption For the new term introduced for power control of NPUSCH, - Reuse the LTE definition simplified for NB-IoT: for and for , where is given by higher layer parameter deltaMCS-Enabled, and where K is the code block size. - FFS: whether the new term applies to QPSK when configured with 16QAM, if it does not, whether an additional term is introduced to avoid jump between QPSK and 16QAM

Based on RAN1 working assumption, the new term delta_TF should be enabled and disabled by RRC configuration.

UE dedicated switching on uplink power control term should be added in UplinkPowerControlDedicated-NB.

## PUR

Regarding the PUR support of 16-QAM, RAN1 and RAN2 separately has reached the following agreements:

Agreement: Confirm the following working assumption: - Working Assumption - Support 16-QAM for NPUSCH in PUR procedure.

Agreement - A NPUSCH 16QAM activation indication is needed in PUR configuration.

RAN1 and RAN2 had already reached an agreement to support 16QAM for NPUSH in PUR procedure, however it is still FFS in RAN1 if 16QAM for NPDSCH in PUR should be supported or not. If 16QAM for NPDSCH in PUR is to be supported, given the different applicability of 16QAM in DL and UL for PUR, the configuration for DL and UL should be decoupled.

For the support of NPUSCH 16QAM in PUR, to save RRC signaling load, it is suggested to use one flag to activate 16QAM and repurpose the legacy multitone for indication of 16QAM MCS.

A text proposal to configure NPUSCH 16QAM in PUR is provided in Annex A.4.

To save RRC signaling load, it is suggested to use a flag to activate 16QAM and repurpose legacy multitone for indication of 16QAM MCS.

## Quality report

Regarding the downlink power configuration to support 16-QAM, RAN1 has reached the following agreements:

Conclusion The channel quality report is not supported in Msg3 in connected mode in Rel-17. Possible Agreement For CQI table for downlink 16-QAM, down-select between following options: - Option 1: More than three candidate values for 16-QAM are added in the legacy table. - FFS: Which of the legacy entries are removed - Option 3: More than three candidate values are added in the legacy table, e.g., based on the entries in the eMTC table (CQI Tables in 36.213)

For quality report, RAN1 has no final agreement on the CQI table design. For RRC_CONNECTED mode, DCQR and AS RAI MAC control element should be updated. There are 4 bits for Quality Report and 2 reserved bits in DCQR and AS RAI MAC CE, Thus, the reserved bit can be used to indicate the rel-17 DCQR and legacy DCQR.

Figure 6.1.3.19-1: DCQR and AS RAI MAC control element

The reserved bit in DCQR and AS RAI MAC CE can be used to indicate the rel-17 DCQR and legacy DCQR.

It was concluded in RAN1 that channel quality report is not supported in Msg3 in connected mode, however there is no discussion if channel quality report should be supported in Msg3 in idle mode. For the RAN1 CQI table design in 16QAM, both option 1 and option 3 will require more than three candidate values for 16QAM, while there are only three remaining values in CQI-NPDCCH-NB and no new value can be added in CQI-NPDCCH-Short-NB. If to support 16-QAM quality report in Msg3 for idle mode, there will be much specification impact. Thus we propose not to support idle mode 16QAM quality report.

The channel quality report is not supported in Msg3 in idle mode in Rel-17.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Optional UE specific power ratio for symbols without NRS nor CRS and mandatory UE specific power radio for IB in symbols with CRS should be added in NPDSCH-Config-NB.

Proposal 2	UE dedicated switching on uplink power control term should be added in UplinkPowerControlDedicated-NB.

Proposal 3	To save RRC signaling load, it is suggested to use a flag to activate 16QAM and repurpose legacy multitone for indication of 16QAM MCS.

Proposal 4	The reserved bit in DCQR and AS RAI MAC CE can be used to indicate the rel-17 DCQR and legacy DCQR.

Proposal 5	The channel quality report is not supported in Msg3 in idle mode in Rel-17.

# References

RP-201306, "WID revision: Additional enhancements for NB-IoT and LTE-MTC”, RAN #88e, Electronic Meeting, June 29th – July 3rd, 2020.

# Annex A

## Annex A.1	TP of UE capabilities for TS 36.331

#### 6.7.3.6	NB-IoT Other information elements

<Omitted>

#### –	UE-Capability-NB

The IE UE-Capability-NB is used to convey the NB-IoT UE Radio Access Capability Parameters, see TS 36.306 [5]. The IE UE-Capability-NB is transferred in NB-IoT only.

UE-Capability-NB information element

-- ASN1START

UE-Capability-NB-r13 ::=		SEQUENCE {

accessStratumRelease-r13		AccessStratumRelease-NB-r13,

ue-Category-NB-r13				ENUMERATED {nb1}					OPTIONAL,

multipleDRB-r13					ENUMERATED {supported}				OPTIONAL,

pdcp-Parameters-r13				PDCP-Parameters-NB-r13				OPTIONAL,

phyLayerParameters-r13			PhyLayerParameters-NB-r13,

rf-Parameters-r13				RF-Parameters-NB-r13,

dummy							SEQUENCE {}							OPTIONAL

}

<Omitted>

UE-Capability-NB-v1610-IEs ::=		SEQUENCE {

earlySecurityReactivation-r16		ENUMERATED {supported}			OPTIONAL,

earlyData-UP-5GC-r16				ENUMERATED {supported}			OPTIONAL,

pur-Parameters-r16					PUR-Parameters-NB-r16			OPTIONAL,

mac-Parameters-v1610				MAC-Parameters-NB-v1610,

phyLayerParameters-v1610			PhyLayerParameters-NB-v1610		OPTIONAL,

son-Parameters-r16					SON-Parameters-NB-r16		OPTIONAL,

meas-Parameters-r16					Meas-Parameters-NB-r16,

tdd-UE-Capability-v1610				TDD-UE-Capability-NB-v1610		OPTIONAL,

nonCriticalExtension				UE-Capability-NB-v17xy-IEs		OPTIONAL

}

UE-Capability-NB-v17xy-IEs ::=		SEQUENCE {

phyLayerParameters-v17xy			PhyLayerParameters-NB-v17xy		OPTIONAL,

nonCriticalExtension				SEQUENCE {}						OPTIONAL

}

<Omitted>

PhyLayerParameters-NB-v1610	::=		SEQUENCE {

npdsch-MultiTB-r16					ENUMERATED {supported}			OPTIONAL,

npdsch-MultiTB-Interleaving-r16		ENUMERATED {supported}			OPTIONAL,

npusch-MultiTB-r16					ENUMERATED {supported}			OPTIONAL,

npusch-MultiTB-Interleaving-r16		ENUMERATED {supported}			OPTIONAL,

multiTB-HARQ-AckBundling-r16		ENUMERATED {supported}			OPTIONAL,

slotSymbolResourceResvDL-r16			ENUMERATED {supported}			OPTIONAL,

slotSymbolResourceResvUL-r16			ENUMERATED {supported}			OPTIONAL,

subframeResourceResvDL-r16				ENUMERATED {supported}			OPTIONAL,

subframeResourceResvUL-r16			ENUMERATED {supported}			OPTIONAL

}

PhyLayerParameters-NB-v17xy	::=		SEQUENCE {

npdsch-16QAM-r17						ENUMERATED {supported}			OPTIONAL,

npusch-16QAM-r17						ENUMERATED {supported}			OPTIONAL

}

<Omitted>

-- ASN1STOP

| UE-Capability-NB field descriptions | FDD/TDD appl | FDD/TDD diff |
| --- | --- | --- |
| accessStratumRelease Set to rel16 in this version of the specification. | FDD/TDD | No |

<Omitted>

| npusch-MultiTB-Interleaving Indicates whether the UE supports interleaved transmission when multiple TBs is scheduled in RRC_CONNECTED for UL. | FDD | - |
| --- | --- | --- |
| npdsch-16QAM Indicates whether the UE supports 16QAM in DL. | FDD/TDD | - |
| npusch-16QAM Indicates whether the UE supports 16QAM in UL. | FDD/TDD | - |
| powerClassNB-14dBm Defines whether the UE supports power class 14dBm in all the bands supported by the UE as specified in TS 36.101 [42]. If powerClassNB-20dBm is included, the UE shall not include the field powerClassNB-14dBm. | FDD/TDD | No |

<Omitted>

## Annex A.2	TP of UE Capability for TS 36.306

### 4.3.4	Physical layer parameters

<Omitted>

#### 4.3.4.xx	npdsch-16QAM-r17

This field indicates whether the UE supports 16QAM for unicast NPDSCH in RRC_CONNECTED. This feature is only applicable if the UE supports category NB2.

#### 4.3.4.xy	npusch-16QAM-r17

This field indicates whether the UE supports 16QAM for unicast NPUSCH in RRC_CONNECTED. This feature is only applicable if the UE supports category NB2.

## Annex A.3	TP of eNB Configuration for TS 36.331

#### 6.7.3.2	NB-IoT Radio resource control information elements

<Omitted>

#### –	NPDSCH-Config-NB

The IE NPDSCH-ConfigCommon-NB is used to specify the common NPDSCH configuration. The IE NPDSCH-ConfigDedicated-NB is used to specify the UE specific NPDSCH configuration.

NPDSCH-Config-NB information element

-- ASN1START

NPDSCH-ConfigCommon-NB-r13 ::=	SEQUENCE {

nrs-Power-r13					INTEGER (-60..50)

}

NPDSCH-ConfigDedicated-NB-r16 ::=	SEQUENCE {

npdsch-MultiTB-Config-r16			NPDSCH-MultiTB-Config-NB-r16	 OPTIONAL	-- Cond twoHARQ

}

NPDSCH-ConfigDedicated-NB-r17 ::=	SEQUENCE {

npdsch-16QAM-Config-r17			ENUMERATED {true}	 OPTIONAL	-- Need OR

}

NPDSCH-MultiTB-Config-NB-r16 ::=	SEQUENCE {

multiTB-Config-r16					ENUMERATED {interleaved, nonInterleaved},

harq-AckBundling-r16				ENUMERATED {true}		OPTIONAL	-- Cond interleaved

}

-- ASN1STOP

NPDSCH-Config-NB field descriptions

multiTB-Config For FDD: Activation of multiple TBs scheduling in DL, see TS 36.213 [23]. Value interleaved indicates that multiple TBs scheduling with interleaved transmission is enabled, value nonInterleaved indicates that multiple TBs scheduling without interleaved transmission is enabled.

harq-AckBundling For FDD: Activation of HARQ ACK bundling for DL multiple TBs scheduling with interleaved transmission, see TS 36.213 [23].

npdsch-16QAM-Config Activation of 16 QAM for unicast NPDSCH.

nrs-Power Provides the downlink narrowband reference-signal EPRE, see TS 36.213 [23], clause 16.2. The actual value in dBm.

| Conditional presence | Explanation |
| --- | --- |
| interleaved | The field is optionally present, Need OR, if multiTB-Config is set to interleaved; otherwise the field is not present and the UE shall delete any existing value for this field. |
| twoHARQ | The field is optionally present, Need OR, if twoHARQ-ProcessesConfig is configured; otherwise the field is not present and the UE shall delete any existing value for this field. |

<Omitted>

#### –	NPUSCH-Config-NB

The IE NPUSCH-ConfigCommon-NB is used to specify the common NPUSCH configuration. The IE NPUSCH-ConfigDedicated-NB is used to specify the UE specific NPUSCH configuration.

NPUSCH-Config-NB information element

-- ASN1START

NPUSCH-ConfigCommon-NB-r13 ::=		SEQUENCE {

ack-NACK-NumRepetitions-Msg4-r13	SEQUENCE (SIZE(1.. maxNPRACH-Resources-NB-r13)) OF

ACK-NACK-NumRepetitions-NB-r13,

srs-SubframeConfig-r13				ENUMERATED {

sc0, sc1, sc2, sc3, sc4, sc5, sc6, sc7,

sc8, sc9, sc10, sc11, sc12, sc13, sc14, sc15

}							OPTIONAL,	-- Need OR

dmrs-Config-r13						SEQUENCE {

threeTone-BaseSequence-r13			INTEGER (0..12)			OPTIONAL,	-- Need OP

threeTone-CyclicShift-r13			INTEGER (0..2),

sixTone-BaseSequence-r13			INTEGER (0..14)			OPTIONAL,	-- Need OP

sixTone-CyclicShift-r13				INTEGER (0..3),

twelveTone-BaseSequence-r13			INTEGER (0..30)			OPTIONAL	-- Need OP

}		OPTIONAL,	-- Need OR

ul-ReferenceSignalsNPUSCH-r13		UL-ReferenceSignalsNPUSCH-NB-r13

}

UL-ReferenceSignalsNPUSCH-NB-r13 ::=	SEQUENCE {

groupHoppingEnabled-r13					BOOLEAN,

groupAssignmentNPUSCH-r13				INTEGER (0..29)

}

NPUSCH-ConfigDedicated-NB-r13 ::=	SEQUENCE {

ack-NACK-NumRepetitions-r13			ACK-NACK-NumRepetitions-NB-r13	OPTIONAL,	-- Need ON

npusch-AllSymbols-r13				BOOLEAN							OPTIONAL,	-- Cond SRS

groupHoppingDisabled-r13			ENUMERATED {true}				OPTIONAL	-- Need OR

}

NPUSCH-ConfigDedicated-NB-v1610 ::=	SEQUENCE {

npusch-MultiTB-Config-r16			ENUMERATED {interleaved, nonInterleaved}

}

NPUSCH-ConfigDedicated-NB-r17 ::=	SEQUENCE {

npusch-16QAM-Config-r17				ENUMERATED {true}				OPTIONAL	-- Need OR

}

ACK-NACK-NumRepetitions-NB-r13	::=	ENUMERATED {r1, r2, r4, r8, r16, r32, r64, r128}

-- ASN1STOP

NPUSCH-Config-NB field descriptions

ack-NACK-NumRepetitions Number of repetitions for the ACK NACK resource unit carrying HARQ response to NPDSCH, see TS 36.213 [23], clause 16.4.2. If this field is absent and no value was configured via dedicated signalling, the value used for reception of Msg4 is used.

ack-NACK-NumRepetitions-Msg4 Number of repetitions for ACK/NACK HARQ response to NPDSCH containing Msg4 per NPRACH resource, see TS 36.213 [23], clause 16.4.2.

groupAssignmentNPUSCH See TS 36.211 [21], clause 10.1.4.1.3.

groupHoppingDisabled See TS 36.211 [21], clause 10.1.4.1.3.

groupHoppingEnabled See TS 36.211 [21], clause 10.1.4.1.3.

npusch-AllSymbols If set to TRUE, the UE shall use all NB-IoT symbols for NPUSCH transmission. If set to FALSE, the UE punctures the NPUSCH transmissions in the symbols that collides with SRS. If the field is not present, the UE uses all NB-IoT symbols for NPUSCH transmission. See TS 36.211 [21], clause 10.1.3.6.

npusch-16QAM-Config Activation of 16 QAM for unicast NPUSCH.

npusch-MultiTB-Config For FDD: Activation of multiple TBs scheduling in UL, see TS 36.213 [23]. Value interleaved indicates that multiple TBs scheduling with interleaved transmission is enabled, value nonInterleaved indicates that multiple TBs scheduling without interleaved transmission is enabled.

sixTone-BaseSequence The base sequence of DMRS sequence in a cell for 6 tones transmission; see TS 36.211 [21], clause 10.1.4.1.2. If absent, it is given by NB-IoT CellID mod 14. Value 14 is not used.

sixTone-CyclicShift Define 4 cyclic shifts for the 6-tone case, see TS 36.211 [21], clause 10.1.4.1.2.

srs-SubframeConfig SRS SubframeConfiguration. See TS 36.211 [21], table 5.5.3.3-1. Value sc0 corresponds to value 0, sc1 to value 1 and so on.

threeTone-BaseSequence The base sequence of DMRS sequence in a cell for 3 tones transmission; see TS 36.211 [21], clause 10.1.4.1.2. If absent, it is given by NB-IoT CellID mod 12. Value 12 is not used.

threeTone-CyclicShift Define 3 cyclic shifts for the 3-tone case, see TS 36.211 [21], clause 10.1.4.1.2.

twelveTone-BaseSequence The base sequence of DMRS sequence in a cell for 12 tones transmission; see TS 36.211 [21], clause 10.1.4.1.2. If absent, it is given by NB-IoT CellID mod 30. Value 30 is not used.

ul-ReferenceSignalsNPUSCH Used to specify parameters needed for the transmission on NPUSCH.

| Conditional presence | Explanation |
| --- | --- |
| SRS | This field is optionally present, need OP, if srs-SubframeConfig is broadcasted. Otherwise, the IE is not present. |

<Omitted>

#### –	PhysicalConfigDedicated-NB

The IE PhysicalConfigDedicated-NB is used to specify the UE specific physical channel configuration.

PhysicalConfigDedicated-NB information element

-- ASN1START

PhysicalConfigDedicated-NB-r13 ::=	SEQUENCE {

carrierConfigDedicated-r13			CarrierConfigDedicated-NB-r13		OPTIONAL,	-- Need ON

npdcch-ConfigDedicated-r13			NPDCCH-ConfigDedicated-NB-r13		OPTIONAL,	-- Need ON

npusch-ConfigDedicated-r13			NPUSCH-ConfigDedicated-NB-r13		OPTIONAL,	-- Need ON

uplinkPowerControlDedicated-r13		UplinkPowerControlDedicated-NB-r13	OPTIONAL,	-- Need ON

...,

[[	twoHARQ-ProcessesConfig-r14		ENUMERATED {true}	OPTIONAL	-- Need OR

]],

[[	interferenceRandomisationConfig-r14	ENUMERATED {true}	OPTIONAL	-- Need OR

]],

[[	npdcch-ConfigDedicated-v1530	NPDCCH-ConfigDedicated-NB-v1530		OPTIONAL	-- Cond TDD

]],

[[	additionalTxSIB1-Config-v1540	ENUMERATED {true}	OPTIONAL	-- Cond additionalSIB1

]],

[[	npusch-ConfigDedicated-v1610		NPUSCH-ConfigDedicated-NB-v1610

OPTIONAL,	-- Cond twoHARQ

npdsch-ConfigDedicated-r16			NPDSCH-ConfigDedicated-NB-r16

OPTIONAL,

resourceReservationConfigDL-r16		SetupRelease {ResourceReservationConfig-NB-r16}

OPTIONAL,	-- Cond dl-NonAnchor

resourceReservationConfigUL-r16		SetupRelease {ResourceReservationConfig-NB-r16}

OPTIONAL	-- Cond ul-NonAnchor

]],

[[	npdsch-ConfigDedicated-r17		NPDSCH-ConfigDedicated-NB-r17		OPTIONAL,	-- Need ON

npusch-ConfigDedicated-r17		NPUSCH-ConfigDedicated-NB-r17		OPTIONAL	-- Need ON

]]

}

-- ASN1STOP

<Omitted>

## Annex A.4	TP of eNB Configuration for TS 36.331

#### PUR-Config-NB

The IE PUR-Config-NB is used to specify PUR configuration.

PUR-Config-NB information element

-- ASN1START

PUR-Config-NB-r16	::=				SEQUENCE {

pur-ConfigID-r16					PUR-ConfigID-NB-r16				OPTIONAL,	--Need OR

pur-TimeAlignmentTimer-r16			INTEGER (1..8)				OPTIONAL,	--Need OR

pur-NRSRP-ChangeThreshold-r16		SetupRelease {PUR-NRSRP-ChangeThreshold-r16}

OPTIONAL,	--Need ON

pur-ImplicitReleaseAfter-r16		ENUMERATED {n2, n4, n8, spare}	OPTIONAL,	--Need OR

pur-RNTI-r16						C-RNTI							OPTIONAL,	--Need ON

pur-ResponseWindowTimer-r16			ENUMERATED {pp1, pp2, pp3, pp4, pp8, pp16, pp32, pp64}

OPTIONAL,	--Need ON

pur-StartTimeParameters-r16			SEQUENCE {

periodicityAndOffset-r16			PUR-PeriodicityAndOffset-NB-r16,

startSFN-r16						INTEGER (0..1023),

startSubframe-r16					INTEGER (0..9),

hsfn-LSB-Info-r16					BIT STRING (SIZE(1))

}																	OPTIONAL,	--Need ON

pur-NumOccasions-r16				ENUMERATED {one, infinite},

pur-PhysicalConfig-r16				SEQUENCE {

carrierConfig-r16					CarrierConfigDedicated-NB-r13,

npusch-NumRUsIndex-r16				INTEGER (0..7),

npusch-NumRepetitionsIndex-r16		INTEGER (0..7),

npusch-SubCarrierSetIndex-r16		CHOICE {

khz15								INTEGER (0..18),

khz3dot75							INTEGER (0..47)

},

npusch-MCS-r16						CHOICE {

singleTone							INTEGER (0..10),

multiTone							INTEGER (0..13)

},

p0-UE-NPUSCH-r16					INTEGER (-8..7),

alpha-r16							ENUMERATED {al0, al04, al05, al06,

al07, al08, al09, al1},

npusch-CyclicShift-r16				ENUMERATED {n0, n6},

npdcch-Config-r16					NPDCCH-ConfigDedicated-NB-r13

}	OPTIONAL,	-- Need ON

pur-PhysicalConfig-r17				SEQUENCE {

npusch-16qam-r17				ENUMERATED {true} OPTIONAL,	-- Need ON

}

...

}

PUR-NRSRP-ChangeThreshold-r16 ::=	SEQUENCE {

increaseThresh-r16						NRSRP-ChangeThresh-NB-r16,

decreaseThresh-r16						NRSRP-ChangeThresh-NB-r16	OPTIONAL	--Need OP

}

NRSRP-ChangeThresh-NB-r16 ::= ENUMERATED {dB4, dB6, dB8, dB10, dB14, dB18, dB22, dB26, dB30, dB34, spare6, spare5, spare4, spare3, spare2, spare1}

-- ASN1STOP

PUR-Config-NB field descriptions

alpha Parameter: αc(3). See TS 36.213 [23], clause 16.2.1.1.1.

carrierConfig Carrier used for PUR.

hsfn-LSB-Info LSB of the H-SFN corresponding to the last subframe of the first transmission of RRCConnectionRelease message containing pur-Config.

npusch-16qam Activation of 16 QAM for PUR NPUSCH.

npdcch-Config NPDCCH configuration for PUR.

npusch-CyclicShift Parameter: ncs. See TS 36.211 [21], clause 10.1.4.1.2. Value n0 corresponds to value 0 and value n6 corresponds to value 6.

npusch-MCS Index to tables specified in TS 36.213 [23], Table 16.5.1.2-1 and Table 16.5.1.2-2 for single tone and multi tone respectively, that defines modulation and TBS index for NPUSCH for PUR. When npush-16qam is enabled, then multiTone will be interpreted as 16QAM mcs, i.e value 0 indicates mcs index 14 and value 1 indicates mcs index 15.

npusch-NumRepetitionsIndex Index to a table specified in TS 36.213 [23], Table 16.5.1.1-3, that defines number of repetitions for NPUSCH for PUR.

npusch-NumRUsIndex Index to a table specified in TS 36.213 [23], Table 16.5.1.1-2, that defines number of resource units for NPUSCH for PUR.

npusch-SubCarrierSetIndex For NPUSCH transmission with subcarrier spacing 3.75 kHz, indicates the subcarrier used for PUR specified in TS 36.213 [23]. For NPUSCH transmission with subcarrier spacing 15 kHz, index to a table specified in TS 36.213 [23], Table 16.5.1.1-1, that defines the set of subcarriers for NPUSCH for PUR.

p0-UE-NPUSCH Parameter: . See TS 36.213 [23], clause 16.2.1.1.1, unit dB.

pur-ImplicitReleaseAfter Number of consecutive PUR occasions that can be skipped before implicit release of PUR configuration. Value n2 corresponds to 2 PUR occasions, value n4 corresponds to 4 PUR occasions, and so on.

pur-NRSRP-ChangeThreshold Threshold(s) of change in serving cell NRSRP in dB for TA validation. Value dB4 corresponds to 4 dB, value dB6 corresponds to 6 dB, and so on. When pur-NRSRP-ChangeThreshold is set to setup, if decreaseThrsh is absent the value of increaseThresh is also used for decreaseThresh.

pur-NumOccasions Number of PUR occasions. Value one corresponds to 1 PUR occasion, and value infinite corresponds to an infinite number of PUR occasions.

pur-PeriodicityAndOffset Indicates the periodicity for the PUR occasions and time offset until the first PUR occasion.

pur-ResponseWindowTimer Duration of the PUR response window in TS 36.321 [6]. Value in PDCCH periods. Value pp2 corresponds to 2 PDDCH periods, pp3 corresponds to 3 PDCCH periods, and so on. The value considered by the UE is: pur-ResponseWindowSize = Min (signaled value x PDCCH period, 10.24s).

pur-TimeAlignmentTimer Value of the time alignment timer for PUR. Value in number of periodicity of PUR.