3GPP TSG-RAN WG2 Meeting #116e  	               			   R2-2110296

Online, November 1st – November 12th 2021                        Revision of R2-2108091

Agenda item:	8.2.2.1

Source:	LG Electronics Inc.

Title:	Deactivation of SCG

Document for:	Discussion and Decision

# 1. Introduction

In this contribution, we continue the discussion on deactivation of SCG, including UE-requested SCG deactivation and PSCell change scenarios in the SCG deactivated state.

# 2. Discussion

## 2.1 UE-requested SCG deactivation

The following is the agreement reached in the RAN2 # 113bis-e meeting.

RAN2 #113bis-e meeting - 2 The UE can indicate to the MN that the UE would like the SCG to be deactivated. FFS on the details (e.g., reusing UAI or existing messages, information included, etc.). Network can configure whether UE is allowed to do the indication.

As per the agreement, the UE can indicate preference for SCG deactivation and other detailed procedures were left FFS. In our view, it would be good to reuse the existing method for indicating its preference so that having a minimum specification change. For example, UE Assistance Information message can be used for UE-requested SCG deactivation. In the current specification, the UE may transmit the UE's preferred configurations according to various purposes to the network through the UE Assistance Information message, and the preference for SCG deactivation may be added as one of the preferred configurations.

Proposal 1. The UE indicates a UE preference for SCG deactivation in UE Assistance Information message.

In addition to the case where there is no UL data, the UE may perceive whether SCG deactivation is required for power saving or overheating. In the previous discussion [1], a company mentioned that the network needs to know why the UE wants to deactivate SCG. However, since the UE may send the preference for power saving or overheating support information along with SCG deactivation preference in the UE Assistance Information message, the network can implicitly know why the UE wants to deactivate SCG. Therefore, additional information such as the reason for deactivation does not need to be included explicitly.

And, in the e-mail discussion [Post114-e][231], there is further discussion of whether it is necessary for the UE to indicate that it wants to release the SCG. The network can decide whether to deactivate or release SCG by considering other situations as well as the UE Assistance information message, and the reason for the SCG deactivation or SCG release will not be much different. So, it is sufficient to transmit a preference for SCG deactivation.

That is, the network can recognize the UE’s situation and decide desired SCG state by considering all other information such as the preferences of the UE assistance information message, the radio quality, and the state of the network. Therefore, it is sufficient that the UE indicates only the preference for SCG deactivation without additional information on the reason for SCG deactivation or other SCG state preference.

Observation 1. The network can recognize the UE’s situation and decide desired SCG state by considering all other information such as the preferences of the UE assistance information message, the radio quality, and the state of the network.

Proposal 2. The UE indicates only the preference for SCG deactivation without additional information on the reason for SCG deactivation or other SCG state preference.

Even if the current preference for SCG deactivation is same as that indicated in the last transmission, the it can be transmitted to the network as needed. In this operation, a prohibit timer is needed to prevent frequent UE’s transmission. If the prohibit timer is not running and the UE determines that it prefers to transition to SCG deactivation, the UE can start timer with the prohibit timer and transmit the UE Assistance Information message to provide the SCG deactivation preference. So, it is guaranteed not to send the same preference while the timer is running.

That is, in order to avoid frequent UE’s transmission for SCG deactivation, the network should configure the prohibit timer for the SCG deactivation preference.

Proposal 3. The network should configure a prohibit timer for UE Assistance Information message concerning SCG deactivation preference.

## 2. 2 PSCell Change in deactivated state

The agreement has been made as follow in the RAN2 #112e meeting. As per the agreement, the network can select the SCG state (activated/deactivated) at PSCell change.

RAN2 #112e meeting - SCG RRC reconfiguration can select the SCG activation state (activated/deactivated) at PSCell addition/change, RRC resume or HO

If the network configures the SCG state to deactivate at PSCell change, the UE should not perform RACH toward the target PSCell to maintain the deactivated state. And the UE should not start timer T304 to avoid SCG failure triggered by expiration of timer T304, even if the configuration includes reconfigurationWithSCG.

Proposal 4. If the network configures the SCG state to deactivate with the legacy PSCell change command, the UE with deactivated SCG should not perform RACH toward the target PSCell.

Proposal 5. If the network configures the SCG state to deactivate with the legacy PSCell change command, the UE with deactivated SCG should not start T304 timer in the target PSCell.

In case of absent of SCG sate in the network configuration at PSCell change, the UE may assume that the target PSCell does not support SCG deactivation or that the target PSCell wants to activate the SCG. Thus, the UE needs to activate the SCG. However, if PSCell is changed due to Conditional PSCell change (CPC), the UE may maintain the SCG state of the source PSCell, which will be dealt with in the next paragraph.

Proposal 6. If the network does not configure the SCG state with the legacy PSCell change command, the UE with deactivated SCG should transition SCG state to the activated state.

It is not appropriate for the network to configure the SCG state in CPC configuration because the network does not know when the condition will be met in the UE. For example, in the following situation, SCG may be unnecessarily activated due to the SCG state in the CPC configuration.

- If the UE receives CPC configuration which includes activating the SCG state, and if CPC is executed in SCG deactivated state.

In case of that the target PSCell have a desired SCG state as shown below, the network can (de)activate SCG right after CPC is executed. The network (PCell) can know whether the target PSCell supports SCG deactivation and whether CPC has been executed by the UE notifying the network (PCell) of CPC completion. It is also applicable to the case that the target PSCell does not support SCG deactivated state.

- SCG is (de)activated in source PSCell but needs to be activated in target PSCell, or

- SCG is (de)activated in source PSCell but needs be deactivated in target PSCell.

Therefore, the network should not configure the SCG state at CPC configuration. And in the case of a PSCell change due to a CPC, the UE can maintain the previous SCG state of source PSCell in the target PSCell.

Proposal 7. The network should not configure SCG activation(activated/deactivated) state in the CPC configuration.

Proposal 8. In the case of a PSCell change due to a CPC, the UE maintains the previous SCG state of source PSCell when executing CPC.

# 3. Conclusion

Follows are our proposal:

Proposal 1. The UE indicates a UE preference for SCG deactivation in UE Assistance Information message.

Observation 1. The network can recognize the UE’s situation and decide desired SCG state by considering all other information such as the preferences of the UE assistance information message, the signal condition, and the state of the network.

Proposal 2. The UE indicates only the preference for SCG deactivation without additional information on the reason for SCG deactivation or other SCG state preference.

Proposal 3. The network should configure a prohibit timer for UE Assistance Information message concerning SCG deactivation preference.

Proposal 4. If the network configures the SCG state to deactivate with the legacy PSCell change command, the UE with deactivated SCG should not perform RACH toward the target PSCell.

Proposal 5. If the network configures the SCG state to deactivate with the legacy PSCell change command, the UE with deactivated SCG should not start T304 timer in the target PSCell.

Proposal 6. If the network does not configure the SCG state with the legacy PSCell change command, the UE with deactivated SCG should transition SCG state to the activated state.

Proposal 7. The network should not configure SCG activation(activated/deactivated) state in the CPC configuration.

Proposal 8. In the case of a PSCell change due to a CPC, the UE maintains the previous SCG state of source PSCell when executing CPC.

# 4. References

[1] R2-2104301	 RAN2-113bis-e LTE DCCA Mobility RAN slicing and Multi-SIM (Tero)_2021-04-20_final.docx

[2] R2-2101969	Summary of [AT113-e][230][eDCCA] Solution alternatives for SCG activation and deactivation (Huawei)	Huawei	discussion