3GPP TSG RAN WG2 Meeting #116-e                                                           R2-2110496

e-Meeting, 1st – 12th November, 2021

Agenda item:	8.5.2

Source:	Samsung

Title:	Issues on Propagation Delay Compensation

Document for:	Discussion and Decision

1	Introduction

In RAN2#115-e, RAN2 made the following agreements:

Agreements

- RAN2 assumes that gNB can perform pre-compensation.  RAN2 agrees to introduce signalling to enable/disable UE-side PDC.

- The gNB can enable/disable UE-side PDC via unicast-RRC signalling for Rel-17

- RAN2 shall wait for RAN1 to decide the measurement framework for RTT based PDC method and does not preclude UE-side PDC or gNB based pre-compensation at this point.  RAN2 is expecting guidance from RAN1 on what is needed.

- UE Assistance information from the UE which could for example be used by gNB to activate PDC is not supported

- Implicit activation of UE-side PDC when a pre-configured threshold is met is not supported

- UE-based trigger for TA update or RACH procedure for PDC are deprioritized for Release 17

This document discusses open RAN2 issues on propagation delay compensation.

2	Discussion

In Rel-16 IIOT, PDC mechanism was not defined. However, RAN2’s assumption is to leave it up to UE implementation which is the same as LTE HRLLC. Although the specification does not mandate anything, it does not mean UE does nothing for PDC. A sensible implementation should perform the PDC anyway. A possible way or even a common understanding is to use the existing Timing Advance (TA) value, where TA/2 is theoretically the same as the propagation delay. Hence, we can interpret that Rel-16 IIOT UE has to be equipped with proprietary PDC mechanism.

In addition to the legacy UE implementation-based PDC, RAN2 agreed to introduce gNB-side pre-compensation where UE shall not perform any compensation. Since the propagation delay highly depends on the distance between gNB and UE, it is true that different UEs have different value of the propagation delay. It means that the pre-compensation is not feasible for broadcast delivery method of ReferenceTimeInfo by system information. Therefore it is reasonable to limit the usage of this pre-compensation to only unicast delivery.

Proposal 1.	Enabling/disabling UE-side PDC is supported only for ReferenceTimeInfo by unicast delivery.

Proposal 2.	Enabling/disabling UE-side PDC is not supported for ReferenceTimeInfo by broadcast delivery (SIB).

Rel-17 IIOT WI aims to develop a PDC mechanism(s) including PD estimation, signalling, and compensation. In compensation part, the difference of Rel-17 would be whether newly defined PD signalling is used by a UE. Therefore, an RRC signalling should indicate whether Rel-17 PDC is applied by a UE or not and which entity should perform the compensation i.e. (1) Rel-17 standardized PDC, (2) No UE side compensation (i.e. gNB-side compensation), (3) Rel-16 PDC based on UE implementation. Rel-17 PDC implies that NW periodically delivers the accurate PD value to UE. The indication can be included in ReferenceTimeInfo.

Proposal 3. RRC signalling indicates whether gNB provides PD value for UE-side compensation, i.e. (1) PDC based on gNB’s signalling on PD value (2) No UE side compensation (3) UE implementation based Rel-16 PDC.

Proposal 4. This indication is included in ReferenceTimeInfo.

3	Conclusion

Based on the discussion above, RAN2 is requested to discuss and agree the following proposals:

Proposal 1.	Enabling/disabling UE-side PDC is supported only for ReferenceTimeInfo by unicast delivery.

Proposal 2.	Enabling/disabling UE-side PDC is not supported for ReferenceTimeInfo by broadcast delivery (SIB).

Proposal 3. RRC signalling indicates whether gNB provides PD value for UE-side compensation, i.e. (1) PDC based on gNB’s signalling on PD value (2) No UE side compensation (3) UE implementation based Rel-16 PDC.

Proposal 4. This indication is included in ReferenceTimeInfo.