3GPP TSG-RAN WG2 Meeting #116 electronic             	R2-2110652

Online, 1st – 12th November 2021

Source:	vivo

Title:	Remaining issues on sl-MaxTransNum configuration and UE behaviour

Agenda Item:	6.2.3

Document for:	Discussion and Decision

# Introduction

In RAN2 #113bis-e meeting, there is an online discussion on how to handle DG for retransmissions in case of different configurations of sl-CG-MaxTransNumList based on post-meeting email discussion [1]. However, there are still some remaining issues that need to be discussed:

- UE’s behaviour when sl-MaxTransNum is configured with a value larger than the number of CG resources;

In RAN2 #115e meeting, this issue is agreed to be postponed to RAN2 #116e.

R2-2108221 Remaining issues on sl-MaxTransNum configuration and UE behaviour vivo discussion Proposal 1: Besides the WA, RAN2 to clarify that the UE will decide whether the next retransmission(s) of the MAC PDU is required based on implementation when SL HARQ FB is disabled and when sl-MaxTransNum is not reached, in case that sl-CG-MaxTransNumList is configured with a value larger than the number of CG resources. - Postponed to the next meeting.

# Discussion

The contribution is to discuss the skipped proposal 4 in offline discussion [1] in RAN2 #113bis-e meeting:

Proposal 4 RAN2 discuss to further clarify in the field description that UE does not expect a configuration of sl-MaxTransNum larger than the number of CG resources. - Skipped to the decision in proposal 5.

In proposal 3 of the same offline discussion we agreed the WA which is also informed to RAN1 in [2]that:

Working assumption: “UE assumes that next retransmission(s) of the MAC PDU is required when FB is disabled, for CG, if sl-CG-MaxTransNumList is configured with a value not larger than the number of CG resources, when sl-CG-MaxTransNum is not reached”

However, with agreeing on this, it is still unclear that when sl-CG-MaxTransNumList is configured with a value larger than the number of CG resources, what is the UE’s behaviour? E.g.:

  - UE may take this configuration as a worng configuration;

  - UE follows the behaviour in WA in this case, i.e. same as when sl-CG-MaxTransNumList is configured with a value NOT larger than the number of CG resources;

  - UE handles it based on implementation

According to the agreement in RAN2 #113e [3]:

- RAN2 confirms sl-CG-MaxTransNumList covers {only CG resources}.

Therefore, if the UE follows the behaviour in WA in this case, then it will always assume that next retransmission(s) of the MAC PDU is required, because the sl-MaxTransNum is always NOT reached, which is obviously not reasonable. The simplest way is to leave it to UE implementation.

Observation 1: UE’s behaviour is still unclear when FB is disabled, for CG, if sl-CG-MaxTransNumList is configured with a value larger than the number of CG resources, when sl-MaxTransNum is not reached.

Proposal 1: Besides the WA, RAN2 to clarify that the UE will decide whether the next retransmission(s) of the MAC PDU is required based on implementation when SL HARQ FB is disabled and when sl-MaxTransNum is not reached, in case that sl-CG-MaxTransNumList is configured with a value larger than the number of CG resources.

# Conclusion

We have the following observation and proposals:

Observation 1: UE’s behaviour is still unclear when FB is disabled, for CG, if sl-CG-MaxTransNumList is configured with a value larger than the number of CG resources, when sl-MaxTransNum is not reached.

Proposal 1: Besides the WA, RAN2 to clarify that the UE will decide whether the next retransmission(s) of the MAC PDU is required based on implementation when SL HARQ FB is disabled and when sl-MaxTransNum is not reached, in case that sl-CG-MaxTransNumList is configured with a value larger than the number of CG resources.

# Reference

- R2-2102885, Summary of [POST113-e][708][V2X/SL] How to handle DG for retransmissions (OPPO), 3GPP TSG-RAN WG2 #113bis-e, E-meeting, April 2021

- R2-2104475, LS on R16 V2X for PUCCH reporting and for minimum time gap, RAN2, 3GPP TSG-RAN WG2 Meeting #113bis-e, E-meeting, 12 – 20 April 2021

- RAN2 #113-e meeting chairman notes