3GPP TSG RAN WG2#116-e	R2-2110770

e-Meeting, 1st - 12th November, 2021

Agenda item:		9.2.4     Control Plane Impact

Source:		NEC

Title:		Analysis on Mobility Aspects for IoT NTN

Document for:		Discussion and Decision

# 1. Introduction

At RAN2 #115e meeting, IoT NTN topic has made following agreements regarding idle mode and connected mode mobility:

- Idle mode mobility relevant agreements: Cell selection / reselection procedures for NB-IoT and LTE-M in TN is the baseline in NB-IoT/LTE-M NTN. RAN2 assumes that Satellite assistance information, e.g. for cell selection reselection, for serving cell is provided to UE. FFS if Satellite assistance information for neighbour cell(s) is provided to UE for cell selection/reselection (justification would be needed). Wait for the progress in RAN1 before discussion on whether satellite assistance information is broadcast in a separate information block. The timing information on when a cell is going to stop serving the area is broadcast at least for the quasi-earth fixed case. FFS details. Connected mode mobility relevant agreement: Rel-16 LTE CHO mechanism is supported for LTE-M devices in IoT NTN. FFS which CE Mode(s) to apply Send an LS to RAN4 to inform that RRM impacts for supporting CHO should be taken into consideration. No procedural update is required to support connected mode mobility for LTE-M. The value range for parameter t304 is not extended with larger values. Rel-16 RLF / connection re-establishment mechanisms are supported in IoT NTN assuming that minor adjustments to UE specific timers and constants would be sufficient. Postpone the discussion on whether specific timers and constants for RLF and RRC connection re-establishment procedures require extended value range and/or new behaviour till next meeting.

In this contribution, we further analysis idle mode and connected mode mobility for IoT NTN

# 2. Discussion

2.1 Idle mode mobility

For cell selection and reselection, we made following agreement for IoT NTN at last meeting:

- The timing information on when a cell is going to stop serving the area is broadcast at least for the quasi-earth fixed case. FFS details

For NR NTN, above has been agreed in past and further detail of using this information was agreed at last meeting:

- For quasi-earth fixed cell, specify that UE should start measurements on neighbour cells before the broadcast stop time of the serving cell, i.e. the time when the serving cell stops covering the current area, and the exact time to start measurements is up to UE implementation.

We think above agreement regarding how to use the timing information of serving cell can be adopted for IoT NTN:

Proposal 1: Same as NR NTN, for IoT NTN, to specify that UE should start measurements on neighbour cells before the broadcast stop time of the serving cell, i.e., the time when the serving cell stops covering the current area, and the exact time to start measurements is up to UE implementation

Another point is neighbour cell’s timing information, whether to broadcast it and how to use the information. This has been discussed in NR NTN e.g., some companies propose to only consider the neighbour cells which have longer serving time then a threshold or add an offset to neighbour cells which have longer serving time. On the other hand, other companies pointed out the gain is negligible, this would only avoid number of cell reselection to a neighbour cell with short serving time. considering the limit benefit, we could either delay this discussion to next release or wait for progress in NR NTN.

Proposal 2: Regarding using neighbour cell’s timing information, wait for progress in NR NTN or delay the discussion to future release.

2.2 Conditional HO

In has been agree that Rel-16 LTE CHO mechanism is supported for LTE-M devices in IoT NTN. In NR-NTN, we agreed to have location-based event, time-based event and A4 event as additional conditions for CHO, even though some details are still under discussion. For IoT NTN, the challenge of slow changing of radio signalling from cell centre to cell edge would remain the same, when UE is moving away from a serving cell to a neighbouring cell or a serving cell is moving from a UE, UE may not observe that the signal level of the serving cell is getting worse obviously, hence in addition to A3 and A5, A4 based HO condition would be useful especially for earth moving cell.

Proposal 3: Support CondEvent A4 for IoT NTN CHO

In WID, GNSS capability is assumed but simultaneous GNSS and NTN NB-IoT/eMTC operation is not assumed. Moreover, only intermittent delay-tolerant small packet transmissions should be considered. It would be challenging for UE to track its location during a short connection session, it seems less urgent to introduce location-based condition for IoT NTN. And it could be considered in later release when the specification on NR NTN became stable.

Proposal 4: not to support location-based trigger for IoT NTN CHO in Rel-17

On one hand, we see the use case of timer-based condition e.g., in service/feeder link switch scenario, on the other hand, considering short data transmission session, there would be other network implementation to handover out or make sure no UE connection around the service/feeder link switch time.

Proposal 5: not to support timer-based trigger for IoT NTN CHO in Rel-17

2.2 RLF and RRC re-establishment specific timers and constants

At RAN115e meeting, it was postponed the discussion on whether specific timers and constants for RLF and RRC connection re-establishment procedures require extended value range and/or new behaviour till this meeting

In current specification, PCell RLF could be triggered by different reasons:

- Physical layer problem: PCell RLF detection starts with N310 number of out of sync indications, then UE expects to receive N311 number of in sync within T310, if not, RLF at MCG will be declared and RRC re-establishement will be triggered

- Random access problem: PreambleTransMax

- Maximum time of RLC retransmission: maxRetxThreshold

After RLF is detected, there are two stages for RLF recovery:

- Cell selection: find a suitable cell for RRC connection re-establishment within T311, (if not suitable cell is found, UE will go to RRC-IDLE mode.

- RRC connect re-establishment procedure: send out RRC connection reestablishment message, and expect to receive response within T301

All the timers or constants are highlighted as above. And the corresponding IEs and value set are also copied as following:

RLC-Config maxRetxThreshold ENUMERATED {t1, t2, t3, t4, t6, t8, t16, t32} PreambleTransMax ::= ENUMERATED { n3, n4, n5, n6, n7, n8, n10, n20, n50,n100, n200} RLC-Config-NB maxRetxThreshold-r13 ENUMERATED {t1, t2, t3, t4, t6, t8, t16, t32} RLF-TimersAndConstants RLF-TimersAndConstants-r9 ::= CHOICE { release NULL, setup SEQUENCE { t301-r9 ENUMERATED {ms100, ms200, ms300, ms400, ms600, ms1000, ms1500, ms2000}, t310-r9 ENUMERATED {ms0, ms50, ms100, ms200, ms500, ms1000, ms2000}, n310-r9 ENUMERATED {n1, n2, n3, n4, n6, n8, n10, n20}, t311-r9 ENUMERATED {ms1000, ms3000, ms5000, ms10000, ms15000, ms20000, ms30000}, n311-r9 ENUMERATED {n1, n2, n3, n4, n5, n6, n8, n10}, ... } } RLF-TimersAndConstants-r13 ::= CHOICE { release NULL, setup SEQUENCE { t301-v1310 ENUMERATED {ms2500, ms3000, ms3500, ms4000, ms5000, ms6000, ms8000, ms10000}, ..., [[t310-v1330 ENUMERATED {ms4000, ms6000} OPTIONAL -- Need ON ]] } } RLF-TimersAndConstants-NB-r13 RLF-TimersAndConstants-NB-r13 ::= CHOICE { release NULL, setup SEQUENCE { t301-r13 ENUMERATED { ms2500, ms4000, ms6000, ms10000, ms15000, ms25000, ms40000, ms60000}, t310-r13 ENUMERATED {ms0, ms200, ms500, ms1000, ms2000, ms4000, ms8000}, n310-r13 ENUMERATED {n1, n2, n3, n4, n6, n8, n10, n20}, t311-r13 ENUMERATED {ms1000, ms3000, ms5000, ms10000, ms15000, ms20000, ms30000}, n311-r13 ENUMERATED {n1, n2, n3, n4, n5, n6, n8, n10}, ..., [[ t311-v1350 ENUMERATED {ms40000, ms60000, ms90000, ms120000} OPTIONAL -- Need OR]], [[t301-v1530 ENUMERATED {ms80000, ms100000, ms120000} OPTIONAL, -- Cond TDD t311-v1530 ENUMERATED {ms160000, ms200000} OPTIONAL -- Cond TDD ]] } } -- ASN1STOP

Apart from timer T301, all above timers and constants are not impacted by long RTT of NTN network in our understanding. Currently maximum value of T301 is 10s for LTE-M and 60s for NB-IOT. It looks like big enough.

Proposal 6: Timers and constants for RLF and RRC connection re-establishment procedures does not require extended value range.

2.3 Reduce RLF/RRC-reestablishment events

It has been agreed that only intermittent delay-tolerant small packet transmissions should be considered in IoT-NTN communication. This means the transmission session usually is short, and the packets can tolerate certain delay. Considering the short transmission session of small packets, any RLF/HO during it would increase not only the service delay but also increase system overhead/complexity a lot, it would be better to avoid (or at least reduce) RLF/HO during small data transmission session. There are two possible solutions:

when uplink data arrival, and by prediction the serving/camping cell will stop service very soon, and the UE cannot complete the short data transmission session before

- Option1: allow UE to delay a data transmission session initiation. This option mainly relies on UE implementation possibly with no or minor network control. We think this will largely reduce the amount of RLF/RRC-reestablishment events happening in the system. On the other hand, it will increase the packet delay.

- Option2: allow UE to advance the upcoming cell reselection before initiating the data transmission session. This option may need some network control. We think this will also largely reduce the amount of RLF/RRC-reestablishment events happening in the system. And it will increase the packet delay but potentially not as much as option 1.

Both options mentioned above require minor changes but with the cost of extra data transmission delay.  Moreover, we have agreed to broadcast the timing information of a cell stop serving the area.

Proposal 7: RAN2 discuss to have one of following solutions to avoid RLF/Handover during a short data transmission session:

- Option1: allow UE to delay a data transmission session initiation until finishing upcoming cell reselection

- Option2: allow UE to advance the upcoming cell reselection if there is data arrival for transmission

# 3. Conclusion

In this contribution, in terms of mobility aspects of IoT NTN, we made following proposals for RAN2 discussion.

For idle mode mobility:

Proposal 1: Same as NR NTN, for IoT NTN, to specify that UE should start measurements on neighbour cells before the broadcast stop time of the serving cell, i.e., the time when the serving cell stops covering the current area, and the exact time to start measurements is up to UE implementation

Proposal 2: Regarding using neighbour cell’s timing information, wait for progress in NR NTN or delay the discussion to future release.

For CHO:

Proposal 3: Support CondEvent A4 for IoT NTN CHO

Proposal 4: not to support location-based trigger for IoT NTN CHO in Rel-17

Proposal 5: not to support timer-based trigger for IoT NTN CHO in Rel-17

Regarding RLF and RRC connection re-establishment specific timers and constants:

Proposal 6: Timers and constants for RLF and RRC connection re-establishment procedures does not require extended value range.

Last regarding minor enhancement:

Proposal 7: RAN2 discuss to have one of following solutions to avoid RLF/Handover during a short data transmission session:

- Option1: allow UE to delay a data transmission session initiation until finishing upcoming cell reselection

- Option2: allow UE to advance the upcoming cell reselection if there is data arrival for transmission

# References

[1] Chairman’s Notes RAN2# 115e