3GPP TSG-RAN WG2 #116e	Tdoc R2-2110776

Electronic meeting, Nov 1st – 12th, 2021 	Revision of R2-2107976

Agenda Item:	8.3.4

Source:	Ericsson

Title:	Introduction of a Paging cause indication

Document for:	Discussion, Decision

# 1	Introduction

Multi-USIM devices enable use of multiple subscriptions in one device, e.g. more than one USIM and concurrent registration in more than one network. The USIMs can be from the same or different operator(s) (PLMN).

A Multi-USIM device needs to handle “simultaneous state” situations like:

- RRC_IDLE/INACTIVE + RRC_IDLE/INACTIVE, e.g.

  - check paging in multiple systems

  - receive system information for multiple systems

  - perform measurements on multiple systems

- RRC_CONNECTED+ RRC_INACTIVE/IDLE, e.g.

  - voice + paging/SI/measurements

RAN2 WID RP-210316 [1] defines three objectives for the support for Multi-USIM devices in Rel-17.  In this contribution the focus is on the objective 3, reported here:

- Unless SA2 find an alternative solution or decides otherwise, specify mechanism for an incoming page to indicate to the UE whether the service is voLTE/VoNR.

  - RAT Concurrency: Network A is either LTE or NR. Network B is either LTE or NR.

  - Applicable UE architecture: Single-Rx/Dual-Rx/Single-Tx

This contribution was revised to account the discussions taken in SA2 group.

# 2	Discussion

A Multi-USIM UE with a single Rx processing unit/chain receives transmissions from one PLMN at a time.

Once the Paging message is received from PLMN2, the Multi-USIM UE shall be able to decide whether to interrupt the ongoing service on the PLMN1 to respond to the paging.

This is possible if a paging cause is included in the Paging message. Depending on the cause, the UE can avoid responding to the Paging if it is for a service with lower priority than the ongoing service in PLMN1, thus minimizing the impact on the ongoing service in the current network.

In TR 23.761 [2], SA2 group agreed to the following solution:

-	For both EPS and 5GS, only the voice is supported to be indicated as the paging cause to the UE.

NOTE 1:	During normative phase, it will be determined whether the Paging Cause is applied 1) only for UEs with the request, or 2) to all UEs indiscriminately.

The inclusion of a paging cause in the paging message was also confirmed by RAN2 in RAN2#113bis-e:

1: RAN2 works to support the MUSIM paging cause feature that SA2 is working on and also addresses the paging cause issue raised by SA2 LS.

2: RAN2 attempts to reply LS to SA2 once we progress on solution and agree on CR(s) that support/address the above feature/issue.

5: If RAN2 agrees to add a paging cause value (or any other information that could lead to a specific paging cause) in Uu paging message, RAN2 specifies the relevant UE behavior (i.e. inform or passing to the upper layer) upon its reception in both LTE and NR specifications.

For the UE in RRC_IDLE, it is expected that the CN performs mapping from incoming IP packet (e.g, ToS/DSCP) to a paging cause and sends the cause to the gNB/eNB using the NGAP/S1AP Paging message. Then the gNB/eNB forwards transparently the paging cause to the Multi-USIM UEs via RRC Paging.

For the UE in RRC_IDLE, it is up to the CN to set the cause and send it to the gNB/eNB using the NGAP/S1AP Paging message.

For the UE in RRC_INACTIVE, the gNB formulates the paging cause, e.g. based on QoS parameters (5QI, ARP) and Paging Policy Indicator (PPI) associated with an incoming DL flow, and sends it to the Multi-USIM UEs via RRC Paging.

For the UE in RRC_INACTIVE, the gNB formulates the paging cause based on e.g. QoS parameters associated with an incoming DL flow.

Based on the RAN2 objective 3) and the SA2 agreement, a new IE containing the Paging cause would be needed to indicate if the service is VoLTE/VoNR. It will be included in the Paging Records carried in the Paging message, only for the intended Multi-USIM UEs

The Paging cause will be included in the Paging Records carried in the Paging message, only for the intended Multi-USIM UEs.

Since not all the paging records will include the cause (only the ones for the Multi-USIM UEs), we expect that the additional bits of the cause have neglectable impacts to the RRC Paging message, both regarding the message size and the possible coverage shrinking.

It is expected that the additional bits of the cause will have neglectable impacts to the RRC Paging message, both regarding the Paging message size and the possible coverage shrinking.

From RAN2 point of view, a paging cause indicating if the incoming service is “VoLTE/VoNR” can be added to the Paging message with no considerable impacts on the message.

As mentioned previously, the paging cause is created by the CN and sent to the gNB/eNB via NGAP/S1AP Paging message. Hence, no capability exchange is expected to be performed between the CN and gNB/eNB in relation with the support of the cause. That is, the CN can always include the cause in the Paging message for the Multi-USIM UE and, if the gNB/eNB supports it, it will be forwarded to the UE, otherwise it will be discarded.

No capability exchange is needed between CN and gNB/eNB in relation with the paging cause support.

An aspect to consider is how to distinguish the case when the paging cause is not present because the service is not voice from the case when the legacy RAN does not support sending the cause.

Some solutions have been proposed, based on the UE - CN capability exchange (agreed by SA2), but they do not solve this issue: the capability exchange is between UE and CN, but it does not take into consideration the RAN capability. So, if UE and CN supports the paging cause, but not the RAN, the cause will not be sent to the UE anyway.

UE – CN capability exchange does not include what the RAN supports.

A possible solution to the issue is to include an additional cause (e.g. “Other”) to indicate all other no-voice services and make the sending of the cause (“Voice” or “Other”) mandatory.

However, a simpler solution is to have only the “Voice” cause and have it as optional IE. The solution of the issue above depends on the presence or not of the following IEs:

- If the Paging record for Rel17 is present and Paging cause “Voice” is present, the Paging is for voice service.

- If the Paging record for Rel17 is present and Paging cause “Voice” is not present, the Paging is for other services (i.e. data).

- If the Paging record for Rel17 is not present, the network does not support sending the cause.

So, the addition of the optional cause “Voice” is enough to distinguish the three cases.

Proposal 3	Include only “Voice” as optional cause IE in Paging message.

The proposed ASN.1 text for LTE [3] and NR [4] to include the new causes is shown in Appendix.

# Conclusion

In the previous sections we made the following observations:

- For the UE in RRC_IDLE, it is up to the CN to set the cause and send it to the gNB/eNB using the NGAP/S1AP Paging message.

- For the UE in RRC_INACTIVE, the gNB formulates the paging cause based on e.g. QoS parameters associated with an incoming DL flow.

- It is expected that the additional bits of the cause will have neglectable impacts to the RRC Paging message, both regarding the Paging message size and the possible coverage shrinking.

- No capability exchange is needed between CN and gNB/eNB in relation of the paging cause support.

- UE – CN capability exchange does not include what the RAN supports.

- Based on the discussion in the previous sections we propose the following:

- The Paging cause will be included in the Paging Records carried in the Paging message, only for the intended Multi-USIM UEs.

- From RAN2 point of view, a paging cause indicating if the incoming service is “VoLTE/VoNR” can be added to the Paging message with no considerable impacts on the message.

- Include only “Voice” as optional cause IE in Paging message.

# References

RP-210316, WID: Support for Multi-SIM devices for LTE/NR.

TR 23.761 v 17.0.0, Study on system enablers for devices having multiple Universal Subscriber Identity Modules (USIM)

TS 36.331, v.16.4.0, E-UTRA; Radio Resource Control (RRC); Protocol specification

TS 38.331, v.16.4.1, NR; Radio Resource Control (RRC); Protocol specification

R2-2104331, Summary of [AT113bis-e][230][MUSIM] Reply LS to SA2 on paging cause (Intel)

# Appendix: ASN.1 text for Paging cause introduction

- LTE Paging (TS 36.331 [3]):

Paging message

-- ASN1START

Paging ::=					SEQUENCE {

pagingRecordList				PagingRecordList					OPTIONAL,	-- Need ON

systemInfoModification			ENUMERATED {true}					OPTIONAL,	-- Need ON

etws-Indication					ENUMERATED {true}					OPTIONAL,	-- Need ON

nonCriticalExtension			Paging-v890-IEs						OPTIONAL

}

Paging-v890-IEs ::=			SEQUENCE {

lateNonCriticalExtension		OCTET STRING						OPTIONAL,

nonCriticalExtension			Paging-v920-IEs						OPTIONAL

}

Paging-v920-IEs ::=			SEQUENCE {

cmas-Indication-r9				ENUMERATED {true}					OPTIONAL,	-- Need ON

nonCriticalExtension			Paging-v1130-IEs					OPTIONAL

}

Paging-v1130-IEs ::=			SEQUENCE {

eab-ParamModification-r11		ENUMERATED {true}					OPTIONAL,	-- Need ON

nonCriticalExtension			Paging-v1310-IEs					OPTIONAL

}

Paging-v1310-IEs ::=			SEQUENCE {

redistributionIndication-r13	ENUMERATED {true}					OPTIONAL,	-- Need ON

systemInfoModification-eDRX-r13	ENUMERATED {true}					OPTIONAL,	-- Need ON

nonCriticalExtension			Paging-v1530-IEs					OPTIONAL

}

Paging-v1530-IEs ::=			SEQUENCE {

accessType						ENUMERATED {non3GPP}				OPTIONAL,	-- Need ON

nonCriticalExtension			Paging-v1610-IEs					OPTIONAL

}

Paging-v1610-IEs ::=			SEQUENCE {

pagingRecordList-v1610			PagingRecordList-v1610				OPTIONAL,	-- Need ON

uac-ParamModification-r16		ENUMERATED {true}					OPTIONAL,	-- Need ON

nonCriticalExtension			Paging-v17xy-IEs					OPTIONAL

}

Paging-v17xy-IEs ::=			SEQUENCE {

pagingRecordList-v17xy			PagingRecordList-v17xy				OPTIONAL,	-- Need ON

nonCriticalExtension			SEQUENCE {}							OPTIONAL

}

PagingRecordList ::=				SEQUENCE (SIZE (1..maxPageRec)) OF PagingRecord

PagingRecordList-v1610 ::=			SEQUENCE (SIZE (1..maxPageRec)) OF PagingRecord-v1610

PagingRecordList-v17xy ::=			SEQUENCE (SIZE (1..maxPageRec)) OF PagingRecord-v17xy

PagingRecord ::=					SEQUENCE {

ue-Identity							PagingUE-Identity,

cn-Domain							ENUMERATED	{ps, cs},

...

}

PagingRecord-v1610 ::=				SEQUENCE {

accessType-r16						ENUMERATED {non3GPP}			OPTIONAL,		-- Need ON

mt-EDT-r16							ENUMERATED {true}				OPTIONAL		-- Need ON

}

PagingRecord-v17xy ::=				SEQUENCE {

pagingForVoice-r17					ENUMERATED {true}     		    OPTIONAL  	 	-- Need ON

}

PagingUE-Identity ::=				CHOICE {

s-TMSI								S-TMSI,

imsi								IMSI,

...,

ng-5G-S-TMSI-r15					NG-5G-S-TMSI-r15,

fullI-RNTI-r15						I-RNTI-r15

}

IMSI ::=							SEQUENCE (SIZE (6..21)) OF IMSI-Digit

IMSI-Digit ::=						INTEGER (0..9)

-- ASN1STOP

| Paging field descriptions |
| --- |
| accessType It indicates whether Paging is originated due to the PDU sessions from the non-3GPP access when E-UTRA is connected to 5GC. E-UTRAN does not include both accessType (i.e., without suffix) and accessType-r16 in a single paging message. |
| cmas-Indication If present: indication of a CMAS notification. |
| cn-Domain Indicates the origin of paging. |
| eab-ParamModification If present: indication of an EAB parameters (SIB14) modification. |
| etws-Indication If present: indication of an ETWS primary notification and/ or ETWS secondary notification. |
| imsi The International Mobile Subscriber Identity, a globally unique permanent subscriber identity, see TS 23.003 [27]. The first element contains the first IMSI digit, the second element contains the second IMSI digit and so on. |
| mt-EDT Indication of mobile terminating EDT. |
| pagingRecordList If E-UTRAN includes pagingRecordList-v1610, it includes the same number of entries, and listed in the same order, as in pagingRecordList (i.e. without suffix). |
| redistributionIndication If present: indication to trigger E-UTRAN inter-frequency redistribution procedure as specified in TS 36.304 [4], clause 5.2.4.10. |
| systemInfoModification If present: indication of a BCCH modification other than SIB10, SIB11, SIB12 and SIB14. This indication does not apply to UEs using eDRX cycle longer than the BCCH modification period. |
| systemInfoModification-eDRX If present: indication of a BCCH modification other than SIB10, SIB11, SIB12 and SIB14. This indication applies only to UEs using eDRX cycle longer than the BCCH modification period. |
| uac-ParamModification If present: indication of UAC parameters (SIB25) modification. |  |
| ue-Identity Provides the NAS identity of the UE that is being paged. The IMSI is not applicable for E-UTRA/5GC. |
| pagingForVoice If present: indication that voice service triggered the Paging. This indication is sent only to Multi-USIM UEs. |

- NR Paging (TS 38.331 [4]):

Paging message

-- ASN1START

-- TAG-PAGING-START

Paging ::=                          SEQUENCE {

pagingRecordList                    PagingRecordList                       OPTIONAL, -- Need N

lateNonCriticalExtension            OCTET STRING                           OPTIONAL,

nonCriticalExtension                Paging-v17xy-IEs                       OPTIONAL

}

Paging-v17xy-IEs ::=			     SEQUENCE {

pagingRecordList-v17xy			     PagingRecordList-v17xy				OPTIONAL,	-- Need N

nonCriticalExtension			     SEQUENCE {}						OPTIONAL

}

PagingRecordList ::=              	 SEQUENCE (SIZE(1..maxNrofPageRec)) OF PagingRecord

PagingRecordList-v17xy ::=			 SEQUENCE (SIZE(1..maxNrofPageRec)) OF PagingRecord-v17xy

PagingRecord ::=                    SEQUENCE {

ue-Identity                         PagingUE-Identity,

accessType                          ENUMERATED {non3GPP}    OPTIONAL,   -- Need N

...

}

PagingRecord-v17xy ::=				  SEQUENCE {

pagingForVoice-r17					ENUMERATED {true}     		    OPTIONAL  	 	-- Need ON

}

PagingUE-Identity ::=               CHOICE {

ng-5G-S-TMSI                        NG-5G-S-TMSI,

fullI-RNTI                          I-RNTI-Value,

...

}

-- TAG-PAGING-STOP

-- ASN1STOP

PagingRecord field descriptions

accessType Indicates whether the Paging message is originated due to the PDU sessions from the non-3GPP access.

pagingForVoice If present: indication that voice service triggered the Paging. This indication is sent only to Multi-USIM UEs.