3GPP TSG-RAN WG2 Meeting #116 electronic	R2-2110996

Electronic meeting 1st - 12th November, 2021

Source:	CATT

Title:	Discussion on QoE collection start and stop

Agenda Item:	8.14.2.2

Document for:	Discussion and Decision

- Introduction

In RAN2#114-e meeting, the following agreements regarding to the pause/resume mechanism for QoE reporting were achieved [1]:

- At reception of QoE release, the UE shall discard any unsent QoE reports corresponding to the released QoE configuration. - FFS whether pause resume will affect all configurations or whether pause resume can act selectively per configuration.

In this contribution, we will further discuss the Pause/Resume mechanism for QoE reporting when the RAN is overload.

- Discussion

For the issue of QoE report handling during RAN overload, there are three options provided by RAN2 #114-e meeting agreement:

- Option 1: Application layer is responsible for storing QoE reports when the UE receives QoE pause indication.

- Option 2: AS layer is responsible for storing QoE reports when the UE receives QoE pause indication.

- Option 3: The QoE container received from application layer is discarded during pause.

RAN2 sent LS to SA4/SA5 for the above issue and SA4/SA5 send reply LS in this meeting.

In the liaison [2] from SA5, they state either of option 1 and Option 2 is equivalent.

In the liaison [3] from SA4, they state it is feasible to buffer QoE data during temporary stop/QoE pause in application layer. However, SA4 points out that fully-reliable resumption of QoE reporting by the application layer, upon receiving a restart directive, may not always be possible in the current Rel-17 QoE architecture. When the RAN overload occurs, it may be last for hours. If application layer is responsible for storing QoE reports during QoE pause, the QoE reports may not valid when network expects to resume it.

The pause/resume mechanism for QoE reporting is similar as logged MDT. For the logged MDT, the network sends the logged measurement configuration when the UE is in RRC_CONNECTED state. The UE performs the logged measurement in RRC_INACTIVE/ RRC_IDLE state and store the logged measurement report in AS layer. There are also some reports are stored in the AS layer, for example, RLF Report/RACH report/CEF report. Therefore, for the QoE report, it is possible to store QoE reports in AS layer when the UE receives QoE pause indication.

On the other hand, if the QoE reports can be stored in AS layer, the AS layer can know the situations of the stored QoE report, e.g. the stored duration and the size of QoE reports. For avoiding network overloading when resuming the QoE reports, the AS layer of UE can send the QoE reports related information to network for assisting the network to resume QoE reporting in order.

Proposal 1: AS layer is responsible for storing QoE reports when the UE receives QoE Pause indication.

The ongoing session will be continued and the recorded information will be stored in the UE until restart received. But the memory of UE is limited. In order to save the memory of UE, the continued time and the size of storage QoE reports should be defined when pausing the QoE reporting. The timer and/or size limitation may be sent with the pause command.

Proposal 2: The timer and/or size limitation should be defined for UE AS layer storing QoE report when pausing the QoE reporting.

If the timer expires or the size is over limitation, the handling of storage QoE reports and corresponding QoE measurements should be defined. There are two options of UE’s behaviors for QoE reports and corresponding QoE measurements when the timer expires.

- Option 1: keep the stored QoE reports and inform Application layer to stop or store the QoE measurements for the corresponding QoE configuration.

- Option 2: discard the stored first-in QoE reports and continue the QoE measurements for the corresponding QoE configuration.

Proposal 3: RAN2 discuss the following two options of UE’s behaviors for QoE reports and corresponding QoE measurement when the timer expires or the size is over limitation

Option 1: Keep the stored QoE reports and inform Application layer to stop or store the QoE measurements for the corresponding QoE configuration.

Option 2: Discard the stored first-in QoE reports and continue the QoE measurements for the corresponding QoE configuration.

If no timer and/or size limitation may be sent with the pause command, considerring the buffer size of AS layer, the suitable solution may be that AS layer firstly store the QoE report according the configuration of the buffer.  When the buffer  for storing the QoE report is full, the AS layer may inform  the APP layer to stop send report to AS layer and store the QoE report in APP layer.

Proposal 4: AS layer to store QoE report firstly and then informs the APP layer to store the QoE report when buffer in AS layer is full.

In RAN2 #114-e meeting, whether pause/resume will affect all configurations or whether pause/resume can act selectively per configuration is for further study. For this issue, the granularity of all QoE configurations to pause/resume QoE reporting is coarser than per QoE configuration to pause/resume QoE reporting. For some real-time application, e.g.VR, the granularity of per QoE configuration is needed for that this application type needs to report immediately. Based on different services and requirements, the granularity of per QoE configuration is more suitable.

Proposal 5: The pause/resume mechanism can act selectively per QoE configuration.

In RAN3 #112-e meeting, the open issues associated pause was listed as following:

Whether a prioritization mechanism of different service types or slices is needed for the RAN to pause or release ongoing QoE measurements in case of RAN overload.

Whether a prioritization mechanism is needed for UE to send to RAN pending QoE reports when RAN overload is solved.

If the granularity of per QoE configuration is selected, what follow are the evaluation criteria in this way, i.e. which QoE configurations need to be pause/resume reporting and which configurations do not need to be pause/resume reporting. The aim of NR QoE measurement is to collect experience parameters of different service types, therefore when the network overload, it is flexible for network to pause/resume the QoE reporting based on the service type priority.

Proposal 6: Service type priority of QoE configuration can be used to evaluate which QoE configurations should be paused or resumed.

If the RAN confirms the QoE configurations that need to pause/resume reporting, RAN should include the corresponding the RRC level ID or the service type in pause/resume command.

Proposal 7:  Include the RRC level ID or the service type of QoE configuration that needs to pause/resume reporting in pause/resume command.

When the UE receive the pause/resume command, the AS layer should use AT command to inform APP layer which QoE configuration needs to pause/resume reporting.

Proposal 8: When the UE receive the pause/resume command, the AS layer should use AT command to inform APP layer which QoE configuration needs to pause/resume reporting.

- Conclusion

According to the analysis in section 2, it is proposed:

Proposal 1: AS layer is responsible for storing QoE reports when the UE receives QoE Pause indication.

Proposal 2: The timer and/or size limitation should be defined for UE AS layer storing QoE report when pausing the QoE reporting.

Proposal 3: RAN2 discuss the following two options of UE’s behaviors for QoE reports and corresponding QoE measurement when the timer expires or the size is over limitation

Option 1: Keep the stored QoE reports and inform Application layer to stop or store the QoE measurements for the corresponding QoE configuration.

Option 2: Discard the stored first-in QoE reports and continue the QoE measurements for the corresponding QoE configuration.

Proposal 4: AS layer to store QoE report firstly and then informs the APP layer to store the QoE report when buffer in AS layer is full.

Proposal 5: The pause/resume mechanism can act selectively per QoE configuration.

Proposal 6: Service type priority of QoE configuration can be used to evaluate which QoE configurations should be paused or resumed.

Proposal 7:  Include the RRC level ID or the service type of QoE configuration that needs to pause/resume reporting in pause/resume command.

Proposal 8: When the UE receive the pause/resume command, the AS layer should use AT command to inform APP layer which QoE configuration needs to pause/resume reporting.

- Reference

- RAN2 #114-e Meeting Report.

- S5-214519 Reply LS on QoE report handling at QoE pause, Huawei

- S4-211290 LS Reply on QoE report handling at QoE pause, Huawei