3GPP TSG-RAN WG2 #116-e	R2-211xxxx

Electronic meeting, 1st November – 12th November 2021

Agenda Item:	5.4.1.2

Source:	Ericsson

Title:	Summary of [AT116-e][002][NR15] RRC Inter Node Other and LTE

Document for:	Discussion, Decision

# 1	Introduction

This document is to handle the following email discussion:

[AT116-e][002][NR15] RRC Inter Node Other and LTE (Ericsson)

Scope: Determine agreeable parts in a first phase, for agreeable parts agree on CRs. Treat R2-2110460, R2-2110461, R2-2110462, R2-2110463, R2-2110696, R2-2109370, R2-2111182, R2-2110022, R2-2110796, R2-2110939, R2-2110942

Intended outcome: Report, agreed CRs if applicable

Deadline: Schedule 1

Regarding the deadlines, I would like to set the following 2 deadlines:

1) First deadline on Thursday Nov 4 1200 UTC to settle scope what is agreeable.

2) Second deadline on Thursday Nov 11 1200 UTC to agree the CRs (where applicable) and final check.

# 2	Contact information

| Company (Name) | Email |
| --- | --- |
| Nokia | amaanat.ali@nokia.com |

# 3	Discussion

Companies are encouraged to provide comments for each CR/document under this email discussion:

## 3.1	Inter-Node RRC messages

R2-2110460	Correction on reestablishmentInfo	ZTE Corporation, Sanechips	CR	Rel-15	38.331	15.15.0	2834	-	F	NR_newRAT-Core

R2-2110461	Correction on reestablishmentInfo(R16)	ZTE Corporation, Sanechips	CR	Rel-16	38.331	16.6.0	2835	-	A	NR_newRAT-Core

R2-2110462	Correction on reestablishmentInfo	ZTE Corporation, Sanechips	CR	Rel-15	36.331	15.15.0	4732	-	F	LTE_5GCN_connect-Core

R2-2110463	Correction on reestablishmentInfo(R16)	ZTE Corporation, Sanechips	CR	Rel-16	36.331	16.6.0	4733	-	A	LTE_5GCN_connect-Core

Question 1: Do company agree with the changes proposed in the CRs in R2-2110460, R2-2110461, R2-2110462, and R2-2110463?

| Company | Agree (y/n) | Comments |
| --- | --- | --- |
| Nokia | Agree | Yes the alignment seems to be required |

## 3.2	RRC Rapporteur CR

This CR usually continues in a short email discussion after the meeting, but companies are encouraged to provide preliminary comments, if there is something to be highlighted.

R2-2110696	Miscellaneous non-controversial corrections Set XII	Ericsson	CR	Rel-15	38.331	15.15.0	2843	-	F	NR_newRAT-Core

Question 2: Do company agree with the changes proposed in the CRs in R2-2110696?

| Company | Agree (y/n) | Comments |
| --- | --- | --- |
| Nokia | Agree |  |

## 3.3	Measurements

### 3.3.1	Association between serving cell and measurements object

R2-2109370	Association between serving cell and measurement object (R5-215762; contact: HiSilicon)	RAN5	LS in	Rel-15	5GS_NR_LTE-UEConTest	To:RAN2

Moved from 3

R2-2111182	Discussion on association between serving cell and measurement object	MediaTek Inc.	discussion	Rel-15

Regarding this issue, the contribution in R2-2111182 formulates the following proposals:

Proposal 1: Reply RAN5 that the servingCellMO indication is used to determine the association between serving cell and measurement object in TS 38.331.

Proposal 2: Reply RAN5 that, for event A3/A5 triggering reporting configured on SCC, it is compulsory to configure servingCellMO for SCell in order to enable UE considering SCell to be a neighbouring cell.

Question 3: Do company agree to reply RAN5 that the servingCellMO indication is used to determine the association between serving cell and measurement object in TS 38.331?

| Company | Agree (y/n) | Comments |
| --- | --- | --- |
| Nokia | Agree |  |

Question 4: Do company agree to reply RAN5 that, for event A3/A5 triggering reporting configured on SCC, it is compulsory to configure servingCellMO for SCell in order to enable UE considering SCell to be a neighbouring cell?

| Company | Agree (y/n) | Comments |
| --- | --- | --- |
| Nokia | Agree |  |

### 3.3.2	L3 filtering clarification

R2-2110022	L3 Filtering (filterCoefficient) Clarification	Apple, Ericsson	discussion	Rel-16	NR_newRAT-Core, TEI16

Moved from 6.1.4.1.2

R2-2110796	Draft LS to RAN4 on L3 filter configuration	Apple, Ericsson	LS out	Rel-16	NR_newRAT-Core, TEI16	To:RAN4

Moved from 6.1.4.1.2

Regarding this issue, the contribution in R2-2110022 fomulates the following proposals:

Proposal 1: Confirm that the UE operation on the adaptation of the filter coefficient configuration is independent from the L1/L2 mechanism.

Proposal 2: It is expected that NW and UE have the same understanding on the sample rate X.

Proposal 3: Send an LS to RAN4 on RAN2’s understanding of the impact of L1/L2 mechanism on the L3 sampling rate X used for filter co-efficient configuration.

Question 5: Do company agree to confirm that the UE operation on the adaptation of the filter coefficient configuration is independent from the L1/L2 mechanism?

| Company | Agree (y/n) | Comments |
| --- | --- | --- |
| Nokia | P1 is very confusingly worded. Here is our understanding. | Already during WCDMA and LTE specification work it was agreed that L3 filter coefficients are provided based on the RAN4 (L1) measurement period and UE implementations need to scale the practical L3 filter coefficients based on this information to match with the actual sampling rate(s) used in the implementation. The L3 filtering should not change when UE implementation changes its internal sampling rate also all UEs should use the same reference period, which is UE measurement period, when defining the actual L3 filter coefficients in the UE implementation. Each UE vendor may decide its own actual sampling rate in the implementation, also UE vendor can vary sampling rate if it likes but this should not impact the outcome of L3 filtering output or effective length of L3 filter. |

Question 6: Do company agree that it is expected that NW and UE have the same understanding on the sample rate X?

| Company | Agree (y/n) | Comments |
| --- | --- | --- |
| Nokia | No | L3 filter coefficients are provided based on the RAN4 (L1) measurement period and UE implementations need to scale the practical L3 filter coefficients based on this information to match with the actual sampling rate(s) used in the implementation. So we are not sure why network needs to be in sync to the UE implementation choice as the black box just expects L3 filter coefficients are provided based on the RAN4 (L1) measurement period. Note the L3 filtering process is agnostic to UE implementation of sample rate etc. It is the UEs responsibility to ensure that UE vendor can vary sampling rate if it likes but this should not impact the outcome of L3 filtering output or effective length of L3 filter. |

Question 7: Do company agree to send an LS to RAN4 (be R2-2110796 the baseline) on RAN2’s understanding of the impact of L1/L2 mechanism on the L3 sampling rate X used for filter co-efficient configuration?

| Company | Agree (y/n) | Comments |
| --- | --- | --- |
| Nokia | Yes, but | We can ask a question to confirm an understanding but we are really unsure what changes in this and what is the exact question as the filtering is there for several RAT generations now and hasn’t changed at all. In short, L3 filtering coefficient k is associated with sampling rate X (L1 value), but that cannot be avoided. The question in the LS is also ambiguous. |

## 3.4	LTE changes – Correction to nas-Container

R2-2110939	Correction to nas-Container	Sequans Communications	CR	Rel-15	36.331	15.15.0	4741	-	F	NR_newRAT-Core, LTE_5GCN_connect-Core

R2-2110942	Correction to nas-Container	Sequans Communications	CR	Rel-16	36.331	16.6.0	4742	-	A	NR_newRAT-Core, LTE_5GCN_connect-Core

Question 7: Do company agree with the changes proposed in R2-2110939 and R2-2110942?

| Company | Agree (y/n) | Comments |
| --- | --- | --- |
| Nokia | Yes, but | The problem statement is correct though not sure UEs in the field misunderstood this and there is a real field issue. Would support the clarification though as it makes sense if companies do agree that there is a potential for misunderstanding. |

# Conclusion

Based on the discussion in the previous sections we propose the following: