S3GPP TSG-RAN WG2 Meeting #116-e	R2-210xxxx

# Online, November 1-12, 2021

Agenda Item: 	5.4.3

Source: 			Huawei, HiSilicon

Title: 	Summary of [AT116-e][003][NR15] UE Capabilities I

Document for: 	Discussion and decision

- Introduction

This document summarizes the following offline discussion.

[AT116-e][003][NR15] UE Capabilities I (Huawei)

Scope: Determine agreeable parts in a first phase, for agreeable parts agree on CRs. Treat R2-2109310, R2-2110969, R2-2110970, R2-2110971, R2-2110972,

Intended outcome: Report, agreed CRs if applicable

Deadline: Schedule 1

- Contact from companies

| Company | Email |
| --- | --- |
| Ericsson | lian.araujo@ericsson.com |
| Nokia | amaanat.ali@nokia.com |
| Apple | naveen.palle@apple.com |
| OPPO | qianxi.lu@oppo.com, duzhongda@oppo.com |

- Discussion

  - Part 1: Intended to determine agreeable parts

Reply LS on the Intra-band and Inter-band (NG)EN-DC/NE-DC Capabilties

R2-2109310	Reply LS on the Intra-band and Inter-band (NG)EN-DC/NE-DC Capabilties (R1-2108378; contact: ZTE)	RAN1	LS in	Rel-15	NR_newRAT-Core	To:RAN2	Cc:RAN4

Rapporteur understands this LS has already been taken into account in the last meeting in [AT115-e][017][NR15] UE Capabilties III (ZTE), so no further discussion is needed in this meeting.

Clarification on intraAndInterF-MeasAndReport capability

R2-2110969	Clarification on intraAndInterF-MeasAndReport capability	Huawei, HiSilicon	CR	Rel-15	38.306	15.15.0	0655	-	F	NR_newRAT-Core

R2-2110970	Clarification on intraAndInterF-MeasAndReport capability	Huawei, HiSilicon	CR	Rel-16	38.306	16.6.0	0656	-	A	NR_newRAT-Core

According to current TS 38.306, the capability of intraAndInterF-MeasAndReport is mandatory supported by UE for NR MCG. The network could see this feature as mandatory supported without checking with UE capabilities in NR SA, NR-DC and NE-DC. However, it is also described that this capability applies to NE-DC when configured, which should be interpreted as mandatory with capability signalling. Thus the requirement for intraAndInterF-MeasAndReport capability in NE-DC is not clear in the current 38.306, it should be clarified to avoid possible misunderstanding between the UE and the network.

Q1 Do companies agree with the intention of the CRs above?

| Company | Yes or No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes (proponent) |  |
| Ericsson | Yes but | We should clarify also the NR-DC case, which does not seem clear at the moment either. Hence we propose the following: Indicates whether the UE supports NR intra-frequency and inter-frequency measurements and at least periodical reporting. This field only applies to NE-DC and SN configured measurement when (NG)EN-DC is configured. For NR MCG, For NR SA, and MN and SN configured measurement when NR-DC or NE-DC is configured, this feature is mandatory supported. |
| Nokia | Yes | Agree but the formulation from Ericsson should be corrected as in NE-DC only the NR MCG part is relevant. |
| Apple | Yes |  |
| OPPO (Qianxi) | Yes | Same view as Nokia |

Miscellaneous corrections for Rel-15 UE capabilities

R2-2110971	Miscellaneous corrections for Rel-15 UE capabilities	Huawei, HiSilicon	CR	Rel-15	38.306	15.15.0	0657	-	F	NR_newRAT-Core

R2-2110972	Miscellaneous corrections for Rel-15 UE capabilities	Huawei, HiSilicon	CR	Rel-16	38.306	16.6.0	0658	-	A	NR_newRAT-Core

1) pdsch-ProcessingType1-DifferentTB-PerSlot defines the UE supported number of unicast PDSCH TB(s) multiplexed in time domain within the same slot per CC per serving cell for processing time capability 1, in current TS38.306, the counted unicast PDSCH include the PDSCH scrambled with C-RNTI, TC-RNTI or CS-RNTI. However, MCS-C-RNTI, which is also used for unicast should be included as well. Otherwise, the PDSCH TB scrambled with MCS-C-RNTI (if supported by UE) scheduled in a slot may beyond UE capability.

2) timeDurationForQCL defines minimum number of OFDM symbols required by the UE to perform PDCCH reception and applying spatial QCL information received in DCI for PDSCH processing as described in TS 38.214 [12] clause 5.1.5. It is unclear about the starting and ending point for the above highlight time durations and how to count the time durations. For example, whether the last symbol or/and the first symbol are counted in the duration. Therefore, we propose to clarify the boundary of time duration.

beamReportTiming indicates the number of OFDM symbols between the last symbol of SSB/CSI-RS and the first symbol of the transmission channel containing beam report.

beamSwitchTiming indicates the minimum number of OFDM symbols between the DCI triggering of aperiodic CSI-RS and aperiodic CSI-RS transmission. The number of OFDM symbols is measured from the last symbol containing the indication to the first symbol of CSI-RS.

Q2 Do companies agree with the intention of the CRs above?

| Company | Yes or No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes (proponent) |  |
| Ericsson | Yes |  |
| Nokia | Yes | The changes are non-essential and are purely editorial. These are editorial changes so could be merged to the rapporteur’s CR? |
| Apple | Yes |  |
| OPPO (Zhongda) | Yes |  |

- Conclusions

- References

R2-2109310	Reply LS on the Intra-band and Inter-band (NG)EN-DC/NE-DC Capabilties (R1-2108378; contact: ZTE)	RAN1	LS in	Rel-15	NR_newRAT-Core	To:RAN2	Cc:RAN4

R2-2110969	Clarification on intraAndInterF-MeasAndReport capability	Huawei, HiSilicon	CR	Rel-15	38.306	15.15.0	0655	-	F	NR_newRAT-Core

R2-2110970	Clarification on intraAndInterF-MeasAndReport capability	Huawei, HiSilicon	CR	Rel-16	38.306	16.6.0	0656	-	A	NR_newRAT-Core

R2-2110971	Miscellaneous corrections for Rel-15 UE capabilities	Huawei, HiSilicon	CR	Rel-15	38.306	15.15.0	0657	-	F	NR_newRAT-Core

R2-2110972	Miscellaneous corrections for Rel-15 UE capabilities	Huawei, HiSilicon	CR	Rel-16	38.306	16.6.0	0658	-	A	NR_newRAT-Core