3GPP TSG-RAN WG2 Meeting #116 electronic						   	R2-21xxxxx

Online, 1 – 12 Nov 2021

Agenda Item:	6.1.4.1.1	Connection control

Source: 	Huawei

Title:  	Report of [AT116-e][008][NR16] Connection Control I (Huawei)

Document for:	Discussion and decision

# Introduction

This document is to kick off the following email discussion:

[AT116-e][008][NR16] Connection Control I (Huawei)

Scope: Determine agreeable parts in a first phase, for agreeable parts agree on CRs. Treat R2-2110879, R2-2109314, R2-2110626, R2-2109864, R2-2110421, R2-2110423, R2-2111173, R2-2110631, R2-2110632, R2-2111080, R2-2111070, R2-2111071

Intended outcome: Report, Agreed CRs if applicable

Deadline: Schedule 1

Discussions with Deadline Schedule 1:

A first round with Deadline for comments Thursday W1 Nov 4 1200 UTC to settle scope what is agreeable etc

A Final round with Final deadline Thursday W2 Nov 11 1200 UTC to settle details / agree CRs etc.

Contact Information

| Company | Email |
| --- | --- |
| Ericsson | cecilia.eklof@ericsson.com |
| MediaTek | li-chuan.tseng@mediatek.com |
| Samsung | sy0123.jung@samsung.com |
| Huawei, HiSilicon | jun.chen@huawei.com |
| ZTE | zhang.mengjie@zte.com.cn |
| Apple | fangli_xu@apple.com |
| Qualcomm | (Mouaffac) mambriss@qti.qualcomm.com |
| OPPO | lihaitao@oppo.com |
| Xiaomi | xiongyi3@xiaomi.com |
| Lenovo | Wulh5@lenovo.com |
| CATT | chandrika@catt.cn |

# Discussion

Companies could add comments for contributions below.

## L1 eMIMO

[1] R2-2110879	Correction on pucch-SpatialRelationInfoId-v1610	Huawei, HiSilicon	CR	Rel-16	38.331	16.6.0	2858	-	F	NR_eMIMO-Core

In [1], the issues of the presence condition of pucch-SpatialRelationInfoId-v1610 is discussed, and it is proposed to replace the presence condition by Need S (as in the example in A.4.3.6).

Q1: Do companies agree the changes of the CR [1]?

| Company | Agree? (Yes or No) | Comments |
| --- | --- | --- |
| Ericsson | Yes | This Cond Setup does not make any sense in this context, and has likely been left from some early draft ASN.1. |
| MediaTek | Yes |  |
| Nokia | OK for problem NOK for solution | The issue is real but the solution makes it worse by introducing more ambiguities. First, indeed the condition is wrong and should say "the field is *optional* upon creation…" instead of mandatory. That alone would help a lot. However, the point was that this is ONLY used for the size extension cases, NOT for the original spatial relations. That's why it was mandatory. Second, Need S without conditions on absence just makes the field badly defined - was the intent that this would be Need R in that case? Third, while this is NBC CR, it's unlikely anyone has implemented it yet so that's not the main problem. Fourth, saying "it's not possible to modify the ID" is true but misses the point: These are used via AddModRelease-lists. Hence, it's always possible to release and add the spatial relation in the same message and provide a new ID. Therefore, there's no real need to have a release mechanism for the extended ID as there are other ways to change it (as always, we should NOT create multiple parallel ways for this). Also note that this is the "size-extended" list, not the original list. But we do get the point that maybe network would want to modify PUCCH spatial relation at some point, so no strong view. Hence we think at minimum, we should fix the following: 1) Need M --> Need R in the condition. The rest is not required. |
| Samsung | Yes |  |
| Huawei, HiSilicon | Yes (Proponent) |  |
| ZTE | Yes for the intention | Agree with the intention. But the change is NBC, if the majority thinks the CR is agreeable, we can also accept it. |
| Apple | Yes |  |
| QCOM | Yes | The change is needed |
| OPPO | Yes |  |
| Xiaomi | Yes |  |
| Lenovo&MM | Yes |  |
| CATT | Yes for the intention | Agree with the intention, but the change is NBC. |

Summary: TBD

## L1 NR-U

[2] R2-2109314	LS to RAN2 on default value for rb-Offset (R1-2108436; contact: Ericsson)	RAN1	LS in	Rel-16	NR_unlic-Core	To:RAN2

[3] R2-2110626	Clarification of default value for rb-Offset	Nokia, Nokia Shanghai Bell	CR	Rel-16	38.331	16.6.0	2840	-	F	NR_unlic

[4] R2-2109864	Correction of default value of rb-offset	Ericsson	CR	Rel-16	38.331	16.6.0	2819	-	F	NR_unlic-Core

In [2], RAN1 discussed the issue of rb-Offset and has the following conclusions:

RAN1 has discussed both RAN1 and RAN2-centric solutions, and has agreed that it would be more straightforward for RAN2 to make a change to 38.331 to resolve the incompatibility. The RAN2-centric solution that RAN1 discussed is the following; however, RAN1 acknowledges that the final decision is up to RAN2.

rb-Offset Indicates the RB level offset in units of RB from the first RB of the first 6RB group to the first RB of BWP (see 38.213 [13], clause 10.1). When the field is absent, the UE applies the value 0.

And then the action to RAN2 is as below:

To TSG RAN WG2

ACTION: 	RAN1 respectfully requests RAN2 to implement a suitable correction to 38.331 to remove incompatibility between RAN1 and RAN2 specs with respect to the Rel-16 parameter rb-Offset.

The CR [3] is related to the incoming LS [2] and the proposed changes are as below:

rb-Offset

Indicates the RB level offset in units of RB from the first RB of the first 6RB group to the first RB of BWP (see 38.213 [13], clause 10.1). When the field is absent, the UE applies the value specified in 38.213 [13], clause 10.1.

The CR [4] is related to the incoming LS [2] and the proposed changes are as below:

rb-Offset-r16                       INTEGER (0..5)                                            OPTIONAL,

rb-Offset

Indicates the RB level offset in units of RB from the first RB of the first 6RB group to the first RB of BWP (see 38.213 [13], clause 10.1).

In general, three types of changes are provided ([2][3][4]), so it is proposed to collect companies’ opinions on these changes.

Q2: In order to solve the issue mentioned in the LS [2], which of changes do companies prefer?

| Company | [2], or [3], or [4], or others? | Comments |
| --- | --- | --- |
| Ericsson | [4] | [4] simply proposes to remove the second sentence and the Need -S as the first sentence already refers to 38.213, clause 10.1. The proposed change in [3] is not accurate as 38.213 does not always assign a value, see the following example: “the first common RB of the first group of 6 PRBs has common RB index 6⋅NBWPstart/6 if rb-Offset-r16 is not provided“ Thus, no value is set there for rb-Offset. |
| MediaTek | [4] |  |
| Nokia | [4] | Okay we get the point that if Need S is removed then things should work but removing the Need S will make this NBC? |
| Samsung | [4] |  |
| Huawei, HiSilicon | [3], [4] | We prefer first [3] as not to change ASN.1 however if what Ericsson stated about 38.213 configuration flaw can be confirmed then [4]. |
| ZTE | [4] |  |
| Apple | [4] |  |
| QCOM | [4] |  |
| OPPO | [4] |  |
| Sharp | [4] |  |
| Xiaomi | [4] |  |
| Lenovo&MM | [4] |  |
| CATT | [3] | We think [3] is clear, but if Ericsson statement about 38.213 configuration is confirmed, it can be modified like “When the field is absent, the UE applies the valuecalculate the first common RB index of the first group of 6 PRBs as 0 specified in 38.213 [13], clause 10.1.” For [4] we wonder whether a need code is needed for this field due to it is an optional field. |
| Ericsson2 | [4] | After check with the RRC rapporteur, we agree the Need code is better not changed. In such case, we could have an absent statement like this: “When the field is absent, the UE determines the CORESET configuration related to this field as specified in TS 38.213 [13], clause 10.1.” |

Summary: TBD

## Conditional Reconfiguration

[5] R2-2110421	CPC handling during recovery procedure	Lenovo, Motorola Mobility	CR	Rel-16	38.331	16.6.0	2828	-	F	NR_Mob_enh-Core

[6] R2-2110423	CPC handling during recovery procedure	Lenovo, Motorola Mobility	CR	Rel-16	36.331	16.6.0	4731	-	F	LTE_feMob-Core

In [5][6], it mentions whether to stop conditional reconfiguration evaluation for CPC when UE initiates re-establishment procedure is not specified, so it is proposed that in the procedure for initiation of RRC connection re-establishment in 5.3.7.2, the UE shall stop conditional reconfiguration evaluation for CPC.

Q3: Do companies agree the changes of the CRs [5][6]?

| Company | Agree? (Yes or No) | Comments |
| --- | --- | --- |
| Ericsson | Agree with modification | We agree that the specification currently does not correctly capture the UE behaviour as the UE should stop monitor the conditions when the re-establishment is triggered. However, we think the proposal we had to RAN2#115 in R2-2108102 is better as it proposes the same behaviour for CPC and CHO when handover is not attempted. There is also no reason for the UE to keep the whole configuration during the cell reselection procedure. |
| MediaTek | (Neutral) | If the change is needed, we think that both CHO and CPC cases should be considered, as mentioned by Ericsson. However, we concluded in R2#115-e that R2-2108102 was not pursued, implying that this can be handled by proper UE implementation. |
| Nokia | (Neutral) | No strong view, the behavior proposed is correct. On the other hand, we think the UE will try to send SCG Failure Information beforehand and will stop CPC monitoring before sending this message. So in most cases this CPC monitoring may be stopped already before reestablishment is attempted. |
| Samsung | See comments | As MediTek indicated, RAN2 made a decision not to pursue former Ericsson's paper. But, we are ok with the modifications and think CHO case also needs to be considered. |
| Huawei, HiSilicon | (Neutral) | Share similar views as Ericsson. |
| ZTE | (Neutral) | No strong view. If the majority wants this, we are also fine with the change and think both CHO and CPC cases should be considered. |
| Apple | Neutral | Same view as MediaTek. We think it can be handled by proper UE implementation. |
| QCOM | Neutral | It seems Ericsson’s CR is more adequate … I don’t see why not RAN2#116 can revive the CR (if there is consensus). |
| OPPO | Neutral | Same view as MediaTek. We think it can be handled by proper UE implementation. |
| Sharp | No strong view | As commented by Ericsson, the issue is similar to that in R2-2108102. As it has already agreed to not pursue the issue in R2-2108102, we slightly prefer not to have such changes. |
| Xiaomi | (Neutral) | Agree with MTK. Both CHO and CPC cases should be considered. We think RAN2 can discuss whether it has been solved by UE implementation. When UE initiates re-establishment, it is useless for UE to perform conditional reconfiguration evaluation for CPC or CHO. Maybe in most case CPC and CHO evaluations have been stopped before RRC-reestablishment by UE implementation. |
| Lenovo&MM | See comments | CR for 38.331 UE shall stop conditional reconfiguration evaluation when UE performs fast MCG link recovery and SCG failure information procedure. It is better to align with all failure case. If majority think it can be UE implementation, we are also fine. CR for 36.331. Comparing to CR for 38.331, LTE CR propose to stop conditional reconfiguration evaluation in SCG failure information procedure besides re-establishment. |
| CATT | No | For changes of [5], we think the current TS38.331 already specifies that for R16 CPC, upon initiation of the RRC re-establishment, the SCG and the corresponding configurations should be released (as highlighted in green), thus the CPC evaluation will be stopped too. Note that the intention of the following description highlighted as yellow is only for CHO case 1> if UE is not configured with conditionalReconfiguration: 2> reset MAC; 2> release spCellConfig, if configured; 2> suspend all RBs, and BH RLC channels for IAB-MT, except SRB0 ; 2> release the MCG SCell(s), if configured; 2> if MR-DC is configured: 3> perform MR-DC release, as specified in clause 5.3.5.10; As for the changes of [6], note that in general, the R16 CPC related behavior will not be specified in TS36.331, since the R16 CPC can only applies to NR SCG and the MN is not aware of whether the SN configures the R16 CPC. And In particular, For the 1st change of [6], similar descriptions above is also present, and in TS36.331 (which specifies the E-UTRAN related behavior), UE is not aware of the R16 CPC configured, i.e., UE determines that there is not any conditionalReconfiguration configured, thus UE will delete the SCG and SCG related configuration. As a consequence, the CPC evaluation will be stopped. Similar reasons above also apply for the 2nd change of [6], in TS36.331, UE is not aware of the R16 CPC configured, how can UE decide to stop the CPC evaluation? Moreover, anyway the stage 2 of TS37.340 already captured that upon SCGFailureInformation, the CPC evaluation should be stopped. |

Summary: TBD

[7] R2-2111173	Conditional Handover with Two Triggering Events	MediaTek Inc.	CR	Rel-16	38.306	16.6.0	0663	-	F	NR_Mob_enh-Core

In [7], it mentions that some UEs may not be able to support evaluation of two measIds and CHO execution when both events are satisifed, so it is proposed to change the “manadatory supported” to “optionally supported” for condHandoverTwoTriggerEvents-r16.

The following CR is moved from email [AT116-e][205] because the CR has similar changes as [7].

[7a]	R2-2111178	Conditional Handover with Two Trigger Events	MediaTek Inc.	CR	Rel-16	36.306	16.6.0	1832	-	F	LTE_feMob-Core

Q4: Do companies agree the changes of the CR [7][7a]?

| Company | Agree? (Yes or No) | Comments |
| --- | --- | --- |
| Ericsson | No | We prefer the existing text. |
| MediaTek | Yes | Current text basically invalidates this capability indication. However, requiring two conditions to be both satisfied for CHO execution may not be a good idea in many situations, and we do think that UE should be allowed to support only one triggering event. |
| Nokia | Yes | Agree to have this converted to optional. |
| Samsung | No | Considering the network's logic to command HO discussed in earlier stage of CHO discussion, it is enough to handle only UEs having these mandatory capability. |
| Huawei, HiSilicon | No | The support of cho-TwoTriggerEvents-r16 (and also condPSCellChangeTwoTriggerEvents-r16) was agreed in Rel-16. RAN2 captured both features in TS 38.822 v16.1.0. In the past, RAN2 discussed the use case and benefits of the features, and then both features are conditionally mandatory. We suggest to keep the current definition (for both LTE and NR). |
| ZTE | No | We have discussed this and made the agreement that “For CHO, introduce additional IOT bit (i.e. mandatory with capability) on the support of 2 trigger events for same execution condition. This feature is mandatory for UEs supporting CHO (as per definition of IOT bits).” at RAN2#110e meeting. |
| Apple | Yes |  |
| QCOM | No | Agree with comments above. We note that there are IoT bits for this feature in both LTE and NR, so we do not see a need to modify the description, for both LTE and NR. |
| OPPO | Yes |  |
| Sharp | No | Current text is fine. |
| Xiaomi | Yes | If it is “optionally supported”, some UEs only supporting one triggering event can also support condHandover-r16. It is beneficial for these UEs to improve their handover performance. |
| Lenovo&MM | No | Prefer to keep the existing description. |
| CATT | No | Not clear for the motivation, anyway nothing is wrong. |

Summary: TBD

[8] R2-2110631	Correction on condRRCReconfig field description	Huawei, HiSilicon	CR	Rel-16	38.331	16.6.0	2842	-	F	NR_Mob_enh-Core

[9] R2-2110632	Correction on condReconfigurationToApply field description	Huawei, HiSilicon	CR	Rel-16	36.331	16.6.0	4736	-	F	LTE_feMob-Core

In [8][9], it mentions that the condRRCReconfig field can be used for CHO or CPC, and if the field is applied for CPC, it means condRRCReconfiguration can contain the configuration for target SCG, which conflicts with the definition of the condRRCReconfig field. So it is proposed to clarify the field description of condRRCReconfig that “the configuration for target SCG” is only for CHO.

Q5: Do companies agree the changes of the CRs [8][9]?

| Company | Agree? (Yes or No) | Comments |
| --- | --- | --- |
| Ericsson | Maybe | The update is a bit unclear. Perhaps better to say “except if configured for CPC”. |
| MediaTek | Yes |  |
| Nokia | Yes | The same IEs are used for CHO and CPC, so it may be clarified this is only for CHO. Some rewording can be considered (current change is a bit chaotic but some rewording is good). |
| Samsung | Yes | We can simply say "for CHO" rather than "(only for CHO)". |
| Huawei, HiSilicon | Yes | We are ok with Samsung’s suggestion: or the configuration for target SCG for CHO. |
| ZTE | Yes |  |
| Apple | Yes |  |
| QCOM | Agree with intention | May be better wording as the current proposed change is a bit confusing. E.g. condRRCReconfig The RRCReconfiguration message to be applied when the condition(s) are fulfilled. The RRCReconfiguration message contained in condRRCReconfig cannot contain the field conditionalReconfiguration, the field daps-Config and in case of CHO, it cannot contain the configuration for target SCG. |
| OPPO | Yes |  |
| Sharp | Yes | Prefer wording suggested by Ericsson. |
| Xiaomi | Yes | "for CHO" may be better. Agree with Samsung |
| Lenovo&MM | Yes | Agree with Samsung |
| CATT | Yes |  |

Summary: TBD

[10] R2-2111080	Conditional reconfiguration issues for modification of measId	Xiaomi Communications	discussion

Moved from 6.1.4.1.2

In [10], it mentions that currently a reconfigured measId lead to a reset of the fulfillment state to non-fulfilled, so UE need at least a TTT to consider the reconfigured event as fulfilled based on the spec, which increases the time for CHO trigger and UE cannot be handed over faster to target cell. It may conflict with NW's purpose of modifying measId or associated reportConfig when the NW’s reconfiguration is for faster handover. Proposal 1 is suggested to solve the problem

Proposal 1: When receiving conditional reconfiguration signaling from network to modify a measId or associated reportConfig associated with the condReconfigurationId, UE shall compare the modified event and the previous event to determine whether UE need to reset the fulfillment state to non-fulfilled, including comparing event conditions, triggering quantity, time to trigger, and triggering threshold.

Q6: Do companies agree with proposal 1 in [10]?

| Company | Agree? (Yes or No) | Comments |
| --- | --- | --- |
| Ericsson | No | Disagree. The current solution, where the UE resets the state is so much simpler, this does not make sense. In addition, these changes should not happen very often, so not worth optimizing. |
| MediaTek | No |  |
| Nokia | No | We don’t see the value of this corner case optimization and think this complicates things more than what is really necessary. |
| Samsung | No |  |
| Huawei, HiSilicon | No | P1 is more like an optimization, and it puts extra complexity to UE side and the value is not so clear. |
| ZTE | No |  |
| Apple | No |  |
| OPPO | No |  |
| Sharp | No |  |
| Xiaomi | Yes | When network find that UE has not triggered CHO and the RSRP/RSRQ of serving cell UE measured has become lower, NW may modify the configuration of CHO and make the associated CHO execution condition easy to be fulfilled for faster handover. In the case, when NW configured two trigger events for the CHO execution condition, one of previous event may has been fulfilled and NW don’t know because measurement report will not be triggered when CHO conditions fulfilled. So, NW probably modifies the event which has been fulfilled, which need extra time for UE to consider it as fulfilled again and increase the time for CHO trigger. It conflict with NW's purpose. |
| Lenovo&MM | No |  |
| CATT | No | It introduce additional complexity for the UE. |

Summary: TBD

[11] R2-2111070	Modification of reportConfig for conditional reconfiguration	Xiaomi Communications	CR	Rel-16	38.331	16.6.0	2860	-	F	NR_Mob_enh-Core

Moved from 6.1.4.1.2

[12] R2-2111071	Modification of reportConfig for conditional reconfiguration	Xiaomi Communications	CR	Rel-16	36.331	16.6.0	4743	-	F	LTE_feMob-Core

In [11][12], it mentions that when the associated reportConfig of the measId for conditional reconfiguration is modified, the fulfillment state of the event associated to that reportConfig should also be reset to non-fulfilled. This is similar to modification of measId. So it is proposed that in the procedure for reportConfig modification, the fulfilment of a condition for a measId associated with this reportConfig is reset when the reportConfig is reconfigured.

Q7: Do companies agree the changes of the CRs [11][12]?

| Company | Agree? (Yes or No) | Comments |
| --- | --- | --- |
| Ericsson | Yes | In addition, we should also have something for the measObject. Final text could be: 3> if the measId or the associated reportConfig or the associated measObject for this event associated with the condReconfigId has been modified; or 3> if the leaving condition(s) applicable for this event associated with the condReconfigId, i.e. the event corresponding with the condEventId(s) of the corresponding condTriggerConfig within VarConditionalReconfig, is fulfilled for the applicable cells for all measurements after layer 3 filtering taken during the corresponding timeToTrigger defined for this event within the VarConditionalReconfig: 4> consider the event associated to that measId to be not fulfilled; |
| MediaTek | Yes | Agree with Ericsson |
| Nokia | No | Does not seem to be essential and correct. The change to measID (which is directly associated with CHO execution condition) should lead to the change of status to non-fulfilled (the evaluation may start from scratch), but why the change to reportConfig shall lead to the same? reportConfig comes into play only after CHO execution condition is met. So we see no big justification and would propose not to agree the CR. |
| Samsung | Yes | Ericsson's suggestion looks fine for us. |
| Huawei, HiSilicon | No | We understand that reportConfig mentioned in both CRs is about configuration parameters for condEventA3 and condEventA5, e.g. a3-offset, hysteresis, timeToTrigger. It is our understanding that the update of these parameters once configured is infrequent, so there is no need to clarify UE behaviours. For Ericsson’s change, we wonder the motivation of adding “or the associated measObject”. |
| ZTE | No | In our understanding, if the associated reportConfig is changed, e.g. TTT, threshold, offset value is changed, the UE can continue to evaluate whether the triggering/leaving condition is still met based on the updated parameters value. It seems that no strong need to restrict that the event is not fulfilled anymore. |
| Apple | No |  |
| OPPO | Yes | Ok for the original CR. For the part added by Ericsson, we don’t see the need because for a certain candidate cell for which serving frequency is fixed, there is no use case for network to change the measurement object. |
| Sharp | Yes |  |
| Xiaomi | Yes | We are fine with Ericsson’s views. 3> if the measId or the associated reportConfig or the associated measObject for this event associated with the condReconfigId has been modified; or The configuration of CHO execution condition is included in reportConfig IE. It is also directly associated with CHO execution condition, just like measID modification. ReportConfigNR ::= SEQUENCE { reportType CHOICE { periodical PeriodicalReportConfig, eventTriggered EventTriggerConfig, ..., reportCGI ReportCGI, reportSFTD ReportSFTD-NR, condTriggerConfig-r16 CondTriggerConfig-r16, cli-Periodical-r16 CLI-PeriodicalReportConfig-r16, cli-EventTriggered-r16 CLI-EventTriggerConfig-r16 } } There are three ways to modify CHO execution condition, including modification of measId, modification of reportConfig and modification of measObject. A reconfigured measId for conditional reconfiguration lead to a reset of the fulfillment state to non-fulfilled in CHO, which has been agreed in RAN2#115e. And relevant UE behaviours has been clarified in spec. So, the fulfillment state of CHO event should also be reset to non-fulfilled when associated reportConfig or measObject has been modified, and it is necessary to clarify UE behaviours. |
| Lenovo&MM | Yes |  |
| CATT | No, but see comment | Understand the intention, but we think it may not only related to the reportConfig / measObject, if the other parameters of the MeasConfig is reconfigured e.g. quantityConfig, does the UE need to re-evaluate the execution condition? Considering it was agreed supporting to re-evaluate the execution condition when the MeasID is reconfigured, we think it can leave to NW implementation. If the NW hope the UE re-evaluate the execution condition due to the reconfiguration of reportConfig/measObject even the other parameters of the measConfig, the NW cannot reconfigure the MeasID(e.g. via release the old measID and add the new measID, and reconfigure the new measID for the execution condition) of the execution condition. |

Summary: TBD

# Conclusions

[To be added]