3GPP TSG-RAN WG2 #116-e	R2-211xxxx

Electronic meeting, 1st November – 12th November 2021

Agenda Item:	5.4.1.2

Source:	Ericsson

Title:	Summary of [AT116-e][009][NR16] Connection Control II

Document for:	Discussion, Decision

# 1	Introduction

This document is to handle the following email discussion:

[AT116-e][009][NR16] Connection Control II (Ericsson)

Scope: Determine agreeable parts in a first phase, for agreeable parts agree on CRs. Treat R2-2109340, R2-2109887, R2-2109888, R2-2110682, R2-2110683, R2-2110684, R2-2111036, R2-2110945, R2-2110012, R2-2110756,

Intended outcome: Report, Agreed CRs if applicable

Deadline: Schedule 1

Regarding the deadlines, I would like to set the following 2 deadlines:

1) First deadline on Thursday Nov 4 1200 UTC to settle scope what is agreeable.

2) Second deadline on Thursday Nov 11 1200 UTC to agree the CRs (where applicable) and final check.

# 2	Contact information

| Company (Name) | Email |
| --- | --- |
| Samsung (Sangyeob Jung) | sy0123.jung@samsung.com |
| Nokia | amaanat.ali@nokia.com |
| Huawei (Jun Chen) | jun.chen@huawei.com |
| ZTE | liu.jing30@zte.com.cn |
| Ericsson | antonino.orsino@ericsson.com |

# 3	Discussion

## 3.2	Inter-MN RRC resume without SN change

The following documents have been submitted regarding the LS received in R2-2109340 on the support of the inter-MN RRC resume without the SN change:

R2-2109340	LS on inter-MN RRC resume without SN change (R3-214360; contact: Qualcomm)	RAN3	LS in	Rel-17	LTE_NR_DC_enh2-Core	To:RAN2

Moved from 8.2.1

R2-2109887	Discussion on inter-MN RRC resume without SN change	ZTE Corporation, Sanechips	discussion	Rel-16	LTE_NR_DC_CA_enh-Core

R2-2109888	Reply LS on inter-MN RRC resume without SN change	ZTE Corporation, Sanechips	LS out	Rel-16	LTE_NR_DC_CA_enh-Core	To:RAN3

R2-2110682	Support of inter-MN RRC resume without SN change	Ericsson	discussion	Rel-16	TEI16, LTE_NR_DC_enh2-Core

R2-2110683	[Draft] Reply LS on inter-MN RRC resume without SN change	Ericsson	LS out	Rel-16	TEI16, LTE_NR_DC_enh2-Core	To:RAN3

R2-2110684	Clarification on restore MCG and SCG in case of RRC resume	Ericsson	CR	Rel-16	37.340	16.7.0	0289	-	F	TEI16, LTE_NR_DC_CA_enh-Core

R2-2111036	Discussion on LS on Inter-MN RRC resume without SN change	vivo	discussion	Rel-16	TEI16

R2-2110945	Inter-MN RRC resume without SN change - RAN2 aspects	Qualcomm Incorporated, Nokia, Nokia Shanghai Bell	discussion	Rel-17

Moved from 8.2.1

R2-2110012	Reply LS on Inter-MN RRC resume without SN change 	Qualcomm Incorporated	LS out	Rel-17	To:RAN3

Moved from 8.2.1

Based on the submitted contributions, there are basically two questions that RAN2 needs to answer to RAN3. The first one is whether current RRC signalling support the inter-MN RRC resume without SN change from a RAN2 perspective, and the second one is whether to have the support of this in Rel-16 or in Rel-17.

Question 1: Do company agree that current RRC signalling can already support the use case of inter-MN RRC resume without SN change?

| Company | Agree (y/n) | Comments |
| --- | --- | --- |
| Samsung | Yes with comments | Our understanding is that there is no restriction not to support it from a RAN2 perspective for now. On the other hand, we wonder whether the concerned use case is practically valid or not i.e. early measurement can not be received by the target MN before target MN makes the decision i.e. only blind decision can be allowed unlike inter-MN mobility without SN change. |
| Nokia | Yes | As proponent we think this should be clarified based on R2-2110945. |
| Huawei, HiSilicon | Yes | Based on contributions from companies, the common understanding is that there are no new RAN2 impacts in supporting inter-MN RRC resume without SN change. |
| ZTE | Yes | We agree this scenario is supported in RRC signalling. RRC resume can be triggered in a different Pcell, and when network sends SCG configuration in RRCResume message, the SCG configuration can be either old SCG or a new SCG. |
| Ericsson | Yes but | Even if RAN2 signaling support this scenario, the RAN3 signalling is missing and thus we can say that the inter-MN RRC resume within SN change is currently not supported by 3GPP. This should be clear, unless RAN3 decides to support this scenario already from Rel-16. |

Question 2: If the support of inter-MN RRC resume without SN change is to be introduced by RAN3, RAN2 would like to suggest to have this additional use case in Rel-16 or Rel-17?

| Company | Rel16/Rel-17 | Comments |
| --- | --- | --- |
| Samsung | Rel-17 | There seems no RAN2 specification impact except minor clarification i.e. R2-2110684. On the other hand, we think it is anyway optimization so it makes more sence to introduce Xn message enhancement in R17 so we slightly prefer to have this use case in R17. But we are OK to have it in R16 without any Xn message enhancement. |
| Nokia | Rel-16 | We prefer to have this from Rel-16 |
| Huawei, HiSilicon | Left to RAN3 discussion | Firstly, we think RAN3 LS is to ask whether RAN2 has supported the functionality (related to Q1), and our answer is Yes. Secondly, the only work is in RAN3, so it can be left to RAN3 discussion, i.e. whether they want to introduce necessary signalling parts in Rel-16 or Rel-17. |
| ZTE | Rel-17 | We don’t think the enhancement discussed in RAN3 is essential for Rel-16 because: - As we mentioned in R2-2109887, due to lack of measurement results before RRCResume, target MN can only add SN blindly (e.g. when the coverage of stored PSCell is overlapped with target Pcell), so In real deployment, inter-MN RRC resume without SN addition is a rare case. - From the perspective of target MN, it can trigger SN addition and send stored SCG context to SN for delta configuration, although from SN perspective, it may be regarded as “inter-MN RRC resume with SN change“, but it works without any problem. We think RAN3 is trying to figure out the urgency of this scenario, so we(RAN2) should provide clear guidance to them. And we think it is sufficient to consider the enhancements in Rel-17. |
| Ericsson | Rel-17 | Given that Rel-16 is fronzen for quite some time and that there is no real urgency to support this scenario, we prefer to introduce it from Rel-17. |

Finally, a company submitted a CR in order to clarify that even if the SCG/MCG restore feature has been introduced already in Rel-16, the use case of inter-MN RRC resume is not supported (meaning that the restore of the SCG/MCG is done only in case the UE resumes in the same MN).

R2-2110684	Clarification on restore MCG and SCG in case of RRC resume	Ericsson	CR	Rel-16	37.340	16.7.0	0289	-	F	TEI16, LTE_NR_DC_CA_enh-Core

Question 3: Do companies agree with the changes proposed in R2-2110684?

| Company | Agree (y/n) | Comments |
| --- | --- | --- |
| Samsung | See comments | It depends on the outcome of Q1 i.e. whether to confirm the support of inter-MN RRC resume without SN change in Rel-16. If agreed not to be supported, we are OK with the CR. |
| Huawei, HiSilicon | No | As pointed out by R2-2110945, from the UE point of view, Inter-MN or Intra-MN RRC resume is transparent to the UE, so that the Rel-16 SCG restore also applies to the Inter-MN RRC resume case. In the CR cover page, one reason for change is as below: However, this feature can only be used in case the UE resume in the same MN whereas if the UE resumes to a new MN the stored MCG and SCG configuration needs to be released and cannot be restore. We do not think the above text is correct as the current RAN2 specifications have not defined such UE behaviours. In general, we do not think the CR is needed. |
| ZTE | No | Similar view as HW, UE does not need to care whether MN or SN is changed, the UE just applies the configuration when received from network. From network perspective, during RRC Resume procedure, target MN can obtain the source MCG and SCG context based on legacy context retrieve procedure, so the network(target MN) is able to resume MCG and SCG during RRCResume procedure. Just as we mentioned, network needs to trigger it blindly. But the spec should not prevent network from doing it. |
| Ericsson | Yes with comment | If RAN3 decides to introduce the scenario of inter-MN RRC resume without SN change in Rel-17, a clarification is needed since there is no current support on RAN3 for it. Also, it is true that the network can keep the SN blindly but does it really work for inter-vendor deployment? Our understanding is that this may create IODT issues. |

## 3.1	IIOT – Mobility

R2-2110756	Correction to need code for drb-ContinueEHC-DL and drb-ContinueEHC-UL	MediaTek Inc.	CR	Rel-16	38.331	16.6.0	2845	-	F	NR_IIOT-Core

Question 1: Do company agree with the changes proposed in the CR in R2-2110756?

| Company | Agree (y/n) | Comments |
| --- | --- | --- |
| Samsung | No strong view | We have some sympathy with this motivation. However, no critical problem would be foreseen in the current specification. |
| Nokia | Neutral | Sounds logical if those fields are really one shot and there is no impact upon absence |
| Huawei, HiSilicon | No | We prefer not change ANSI.1 at this stage, the said issue could be handled by sensible network configuration. |
| ZTE | Neutral | We are fine with the change if there is no NBC concern. |
| Ericsson | Tend to agree | We are not sure if there would be any critical issue even if the UE keeps the configuration. On the other hand, since drb-ContinueROHC-DL/drb-ContinueROHC-UL (from Rel-15) have need code as need N, it is okay to align them. |

# Conclusion

Based on the discussion in the previous sections we propose the following: