3GPP TSG-RAN WG2#116-e	R2-21xxxxx

Electronic meeting, 1st November – 12th November 2021

Agenda Item:	6.1.4.1.2

Source:	Ericsson

Title:	Report of [Offline-011][NR16]RRC Measurements Other and LTE (Ericsson)

Document for:	Discussion, Decision

# 1	Introduction

This contribution provides the summary of the following offline discussion.

[AT116-e][011][NR16] RRC Measurements Other and LTE (Ericsson)

Scope: Determine agreeable parts in a first phase, for agreeable parts agree on CRs. Treat R2-2110982, R2-2109445, R2-2110579, R2-2110580, R2-2110697, R2-2110794, R2-2110878, R2-2111079, R2-2110725,

Intended outcome: Report, Agreed CRs if applicable

Deadline: Schedule 1

Discussions with Deadline Schedule 1:

A first round with Deadline for comments Thursday W1 Nov 4 1200 UTC to settle scope what is agreeable etc

A Final round with Final deadline Thursday W2 Nov 11 1200 UTC to settle details / agree CRs etc.

# 2	Contact Information

To make it easier to find the correct contact delegate in each company for potential follow-up questions, the rapporteur encourages the delegates who provide input to provide their contact information in this table:

| Company | Contact: Name (E-mail) |
| --- | --- |
| Nokia | amaanat.ali@nokia.com |

# 3	Discussion

## 3.1	RRM and Measurements

- R2-2110982	Discussion on inter-frequency no gap measurement in NR-DC	Huawei, HiSilicon	discussion	Rel-16	NR_newRAT-Core

In [1], Huawei brings up the aspect of configuration of interFrequencyConfig-NoGap-r16 in the NR-DC scenario. In the contribution, Huawei highlights that the current specification is not clear about whether interFrequencyConfig-NoGap-r16 can be configured by both the MN and the SN. The contributions further mentions that the associated UE behaviour is also ambiguous.

Observation 1: Based on the current specifications, it is not clear whether the measurement configurations from both the MN and the SN can contain the interFrequencyConfig-NoGap-r16 filed in the NR-DC scenario. Besides, UE’s corresponding behaviour for performing inter-frequency no gap measurement is also ambiguous.

Question-1: Do you agree with the Observation-1?

| Company name | Agree? (Yes/No) | Comments |
| --- | --- | --- |
| Nokia | Yes |  |

Rapporteur Summary:

To be added later

If the Observation-1 in Question-1 is agreeable, then the solutions can be discussed and as part of the solution, Huawei has proposed two options.

- Option 1: only MN controls the inter-frequency measurement without gaps feature. The configuration flag (interFrequencyConfig-NoGap-r16) provided by MN applies to all the inter-frequency measurements configured by MN and SN.

- Option 2: MN and SN independently control the inter-frequency measurement without gaps feature. The configuration flag (interFrequencyConfig-NoGap-r16) provided by one node applies to the inter-frequency measurements configured by this node.

Pros of Option-1:

- Only allowing MN to control the inter-frequency measurement without gaps feature for UE in NR-DC is simple for UE implementation with minor spec impacts

Pros of Option-2:

- Allowing MN and SN to independently control the inter-frequency measurement without gaps feature for UE in NR-DC is more favoured for utilizing the feature properly.

Cons of these options are the opposite of pros of the other option.

Based on this, rapporteur would like to ask the following question.

Question-2: If the answer to Question-1 is YES, then which of the following option is preferrable?

- Option 1: only MN controls the inter-frequency measurement without gaps feature. The configuration flag (interFrequencyConfig-NoGap-r16) provided by MN applies to all the inter-frequency measurements configured by MN and SN.

- Option 2: MN and SN independently control the inter-frequency measurement without gaps feature. The configuration flag (interFrequencyConfig-NoGap-r16) provided by one node applies to the inter-frequency measurements configured by this node.

| Company name | Option-1/ Option-2 | Comments |
| --- | --- | --- |
| Nokia | Option 1 | The issue seems valid as no specification on how to configure it in NR-DC. We would prefer Option1 for simplity (i.e. only MN controls the inter-frequency measurement without gaps feature) |

Rapporteur Summary:

To be added later

The outcome fo the first phase of this email discussion can be used to decide on whether to start CR discussions assocaited to this topic and which specific option related CR needs to be taken as baseline.

## 3.2	Other

- R2-2109445	Correction on msgA-SubcarrierSpacing	vivo, Samsung	CR	Rel-16	38.331	16.6.0	2814	-	F	NR_2step_RACH-Core

In [2], the proponents brings up the issue of not being able to configure the PRACH root sequence index separately for 2-step RACH in separate ROs. The CR proposes to update the field description of msgA-SubcarrierSpacing.

msgA-SubcarrierSpacing Subcarrier spacing of PRACH (see TS 38.211 [16], clause 5.3.2). Only the values 15 or 30 kHz (FR1), and 60 or 120 kHz (FR2) are applicable. If the field is absent, the UE applies the SCS as derived from the msg1-SubcarrierSpacing in RACH-ConfigCommon in case of msgA-PRACH-RootSequenceIndex L=139, otherwise, the UE applies the SCS as derived from the msgA-PRACH-ConfigurationIndex in RACH-ConfigGenericTwoStepRA (see tables Table 6.3.3.1-1, Table 6.3.3.1-2, Table 6.3.3.2-2 and Table 6.3.3.2-3, TS 38.211 [16]). The value also applies to contention free 2-step random access type (RACH-ConfigDedicated).

Question-3: Do you agree with the changes in CR R2-2109445?

| Company name | Yes/No | Comments |
| --- | --- | --- |
| Nokia | Yes |  |

Rapporteur Summary:

To be added later

- R2-2110579	Correction on description of absoluteFrequencySSB	ZTE Corporation, Sanechips	CR	Rel-16	38.331	16.6.0	2837	-	F	NR_unlic-Core

In [3], ZTE brings up the issue of how the UE obtains the time and frequency sync for a serving cell that does not transmit any SSB. The CR proposes to update the field description of absoluteFrequencySSB.

absoluteFrequencySSB Frequency of the SSB to be used for this serving cell. SSB related parameters (e.g. SSB index) provided for a serving cell refer to this SSB frequency unless mentioned otherwise. The cell-defining SSB of the PCell is always on the sync raster. Frequencies are considered to be on the sync raster if they are also identifiable with a GSCN value (see TS 38.101-1 [15]). If the field is absent, the SSB related parameters should be absent, e.g. ssb-PositionsInBurst, ssb-periodicityServingCell and subcarrierSpacing in ServingCellConfigCommon IE. If the field is absent, the UE obtains timing reference from the SpCell or an SCell if applicable as described in [14, TS 38.133]. This is only supported in case the SCell is in the same frequency band as the SpCell.

Question-4: Do you agree with the changes in CR R2-2110579?

| Company name | Yes/No | Comments |
| --- | --- | --- |
| Nokia | Yes |  |

Rapporteur Summary:

To be added later

- R2-2110580	Correction on description of cp-ExtensionC2 and cp-ExtensionC3	ZTE Corporation, Sanechips	CR	Rel-16	38.331	16.6.0	2838	-	F	NR_unlic-Core

In [4], ZTE brings up a correction based on a past agreement.

cp-ExtensionC2, cp-ExtensionC3 Configures the cyclic prefix (CP) extension (see TS 38.211 [16], clause 5.3.1). For 15 kHz SCS, {1..28} are valid for both cp-ExtensionC2 and cp-ExtensionC3. For 30 kHz SCS, {1..28} are valid for cp-ExtensionC2 and {2..28} are valid for cp-ExtensionC3. For 60 kHz SCS, {2..28} are valid for cp-ExtensionC2 and {3..28} are valid for cp-ExtensionC3.

Question-5: Do you agree with the changes in CR R2-2110580?

| Company name | Yes/No | Comments |
| --- | --- | --- |
| Nokia | Yes, but | We can merge this to rapporteur CR |

Rapporteur Summary:

To be added later

- R2-2110697	Miscellaneous non-controversial corrections Set XII	Ericsson	CR	Rel-16	38.331	16.6.0	2844	-	F	NR_newRAT-Core, TEI16

In [5], Ericsson brings up some non-controversial changes

Question-6: Do you agree with the changes in CR R2-2110697?

| Company name | Yes/No | Comments |
| --- | --- | --- |
| Nokia | Yes |  |

Rapporteur Summary:

To be added later

- R2-2110794	Extension of pathlossReferenceRSs	MediaTek Inc.	CR	Rel-16	38.331	16.6.0	2849	-	F	TEI16

In [6], MediaTek brings up the issue of handling the pathLossReferenceRSs pathlossReferenceRSs-v1610 in PUCCH-PowerControl. They propose changes similar to the ones agreed for the extension of candidateBeamRSList in the previous meeting. The proposed changes are to the field descriptions of

pathlossReferenceRSs, pathlossReferenceRSs-v1610 A set of Reference Signals (e.g. a CSI-RS config or a SS block) to be used for PUCCH pathloss estimation. Up to maxNrofPUCCH-PathlossReference-RSs may be configured. If the field is not configured, the UE uses the SSB as reference signal (see TS 38.213 [13], clause 7.2). The set includes Reference Signals indicated in pathlossReferenceRSs (without suffix) and in pathlossReferenceRSs-v1610. The UE maintains pathlossReferenceRSs and pathlossReferenceRSs-v1610 separately: Receiving pathlossReferenceRSs-v1610 set to release releases only the entries that were configured by pathlossReferenceRSs-v1610, and receiving pathlossReferenceRSs-v1610 set to setup replaces only the entries that were configured by pathlossReferenceRSs-v1610 with the newly signalled entries.

Question-7: Do you agree with the changes in CR R2-2110794?

| Company name | Yes/No | Comments |
| --- | --- | --- |
| Nokia | Yes |  |

Rapporteur Summary:

To be added later

- R2-2110878	Correction on supportNewDMRS-Port-r16 capability	Huawei, HiSilicon	CR	Rel-16	38.331	16.6.0	2857	-	F	NR_eMIMO-Core

In [7], Huawei proposes the alignment of description in TS 38.306 and TS 38.331 for the supportNewDMRS-Port-r16 capability. The associated changes captured in [7] are;

- Change n0, n2 and n3 to supported, supported1n supported2.

- Clarify that whatever the value reported for supportNewDMRS-Port-r16 indicates UE supports the new DMRS port entry {0, 2, 3}.

Question-8: Do you agree with the changes in CR R2-2110878?

| Company name | Yes/No | Comments |
| --- | --- | --- |
| Nokia | No | The change doesn't make sense as the original text looks fine with values in the enumeration. The other change can be merged to rapporteur CR |

Rapporteur Summary:

To be added later

## 3.3	LTE changes

- R2-2111079	SCG Overheating termination indication in EN-DC	Qualcomm Incorporated, Ericsson	CR	Rel-16	36.331	16.6.0	4744	-	F	TEI16

In [8], the proponents bring up the issue of SCG overheating indication termination in EN-DC. The CR proposes to not to include overheatingAssistance-v1610 instead of overheatingAssistanceForSCG when the UE no longer experiences overheating condition.

2>	else (if the UE no longer experiences an overheating condition):

3>	do not include reducedUE-Category, reducedMaxCCs and overheatingAssistance-v1610 (if configured to provide overheating assistance indication for NR SCG) in OverheatingAssistance IE;

- R2-2110725	Correction on sending SCG Overheating in EN-DC	Nokia, Nokia Shanghai Bell	CR	Rel-16	36.331	16.6.0	4737	-	F	TEI16, NR_newRAT-Core

In [9], Nokia discusses the same issue but proposes to include an empty IE.

2>	else (if the UE no longer experiences an overheating condition):

3>	do not include reducedUE-Category, reducedMaxCCs and provide an empty overheatingAssistanceForSCG (if configured to provide overheating assistance indication for NR SCG) in OverheatingAssistance IE;

Question-9: Which of the following changes are agreeable with respect to SCG overheating indication termination in EN-DC?

- Changes in CR R2-2111079

- Changes in CR R2-2110725

- None (none of the changes as in R2-2111079 or R2-2110725)

| Company name | R2-2111079 / R2-2110725 / None | Comments |
| --- | --- | --- |
| Nokia | No strong view | We have also proposal in R2-2110725, but this change is also acceptable to us in R2-2111079. |

Rapporteur Summary:

To be added later

# 3	Conclusion

To be added later.