3GPP TSG RAN WG2 Meeting #116bis-e                                                        	   R2-220xxxx

E-Meeting, 17th – 25th January 2022

Agenda Item:	8.16.3

Source: 	Intel Corporation

Title:	Report of Offline on Rel-17 NPN UE capability for Rel-17 NPN

Document for: 	Discussion/Decision

# Introduction

his email discussion is to discuss the following offline topic:

[AT116bis-e][032][eNPN] UE capabilities (Intel)

Scope: Initial discussion on UE caps. Identify agreements (for offline agreement), and Open issues, to be addressed at next meeting. If need is high, e.g. if LS out is needed, can also identify some point for online CB W2.

Intended outcome: Report

Deadline: EOM (or earlier for CB point if needed).

This document aims to summarize all the papers that have been submitted to agenda item 8.16.3 of RAN2#116bis-e and provides agreeable proposals and open issues on the following UE capabilities.

- Onboarding over SNPN

- Third party credential holder over SNPN

- IMS emergency services over SNPN

# Companies’ point of contact

| Company | Point of contact | Email address |
| --- | --- | --- |
| Intel Corporation | Seau Sian Lim | seau.s.lim@intel.com |

# Need of UE capabilities for onboarding and external credential holder over SNPN

These are the proposals from the different companies associated with UE capabilities for onboarding and external credential holder over SNPN:

| Companies | Proposals |
| --- | --- |
| OPPO [1] | Proposal 1: Introduce an independent UE capability bit (1 optional per UE bit without xDD/FRx differentiation) for supporting third party credential feature in eNPN. Proposal 2: Introduce an independent UE capability bit (1 optional per UE bit without xDD/FRx differentiation) for supporting onboarding feature in eNPN. |
| Huawei [2] | - Proposal 1: The UE capability information on the support of eNPN features is not needed. |
| Intel, Nokia [3] | Proposal#1: No UE AS capabilities for onboarding and external credential holder need to be specified in TS38.306. Proposal#2: If RAN2 agreed to specified UE AS capabilities for both onboarding and external credential holders access, RAN2 needs to decide whether these UE AS capabilities are ‘Optional without UE capability signalling’ specified in Clause 5.4 or ‘Conditional mandatory without UE capability signalling’. |
| China Telecom [5] | Proposal 1: No UE capability is needed to indicated whether external credential, onboarding and emergency service are supported. Proposal 2: No UE capability is needed to indicated whether GINs are supported. Proposal 3: No UE capability is needed to indicated whether UE is in SNPN AM. |
| CMCC [6] | Proposal 1: There is no need to introduce a separate AS UE capability for supporting SNPN with subscription or credentials by a separate entity. Proposal 2: There is no need to introduce a separate AS UE capability for onboarding and provisioning for NPN. |
| ZTE [7] | Proposal 1: No UE capabilities for the CH and On-boarding feature. |
| Vivo [8] | Proposal 1: Introduce a UE capability on whether UE supports to access SNPN by using subscriptions/credentials owned by an entity separate from the SNPN. It is an optional feature without UE radio access capability parameters. Proposal 2: Introduce a UE capability on whether the UE supports onboarding. It is an optional feature without UE radio access capability parameters. |
| LG [9] | Proposal 1: The support for access with external credential and support for onboarding access is purely optional without capability signaling. Proposal 2: GIN related capabilities including interpretation of a new SIB are mandatory for UEs supporting access with external credential holder or onboarding access. No stage-3 capability description in 38.306 on GIN-related capability signaling is needed. |
| Samsung [10] | Proposal 1: No new UE capability bit is introduce to indicate UE’s support for external CH access via SNPN Proposal 2: No new UE capability bit is introduce to indicate UE’s support of onboarding and provisioning over SNPN. |
| Ericsson [11] | - Existing UE capabilities are sufficient to address new Rel-17 NPN functionalities. |

On whether UE AS capability signalling is needed for CH and onboarding, only [1] think there is a need while others [2-11] think that no capability signalling is needed as gNB does not need to know. Since it is the majority view, a straight proposal as follow:

Proposal#1: No UE AS capability signalling is needed for CH and onboarding.

1. Do companies agree to the above proposal 1?

| Companies | Yes or No | Comments |
| --- | --- | --- |
| Intel | Yes |  |

On whether UE AS capabilities need to be included in TS38.306 either as “Optional without UE capability signalling” or “Conditional mandatory without UE capability signalling”:

[3] thinks that there is no need to specify neither as “Optional without capability signalling” nor as “Conditional mandatory without capability signalling” in TS38.306 since both CH and onboarding are NAS features and if UE supports the features in NAS, the associated AS functions (just forwarding the SIBs) have to be mandatorily supported and the link is quite obvious.

On the other hand, [8] and [9] thinks that CH and onboarding should be specified as “Optional without UE capability signalling” since some AS functions are specified for onboarding and external CHs (e.g. support of the onboarding indication in the SIB1 etc.) and these functions do not have to be mandatory.

2. Do companies think that there is a need to specify CH and onboarding AS capabilities (without capability signalling) in TS38.306? If there is a need to specify in TS38.306, should it be specified as (a) “Optional without capability signalling” or as (b) “Conditional mandatory without capability signalling”?

| Companies | Yes or No | (a) or (b) | Comments |
| --- | --- | --- | --- |
| Intel | No | (a), if companies feel that it is necessary to add | We do not think either optional is needed or conditional mandatory is suitable to link AS with corresponding NAS feature since both CH and onboarding are NAS features and if UE supports the features in NAS, the associated AS functions (just forwarding the SIBs etc.) have to be mandatorily supported and the link is quite obvious. |

## Need of UE capability for CGI report

The following are the relevant proposals on this.

| Companies | Proposals |
| --- | --- |
| Huawei [2] | - Proposal 1: The UE capability information on eNPN relevant CGI report is not needed. |
| China Telecom [5] | Proposal 4: UE capability for CGI reporting is needed in eNPN. |

[2] thinks that it is not needed for CH since the support of external credentials is uniform across the whole SNPN and for onboarding, the onboarding related indicators do not affect the mobility management functions, referring to TS 23.501 [1] as follows:

When the SNPN supports Onboarding of UEs for SNPNs (i.e. the SNPN can be used as ON-SNPN), the NG-RAN node additionally broadcasts the following information: - An onboarding enabled indication that indicates whether onboarding is currently enabled for the SNPN. The onboarding enabled indication is broadcasted per cell e.g. to allow start of the onboarding procedure only in parts of the SNPN. NOTE: Onboarding enabled indication per cell does not affect mobility management functions, i.e. once the UE selects the ON-SNPN as described in clause 5.30.2.10.2.5 and successfully registers within ON-SNPN as described in clause 5.30.2.10.2.6, the UE can move to a cell of the ON-SNPN not indicating onboarding support and continue with the remote provisioning as described in clause 5.30.2.10.4.

On the hand, [5] thinks that it is needed to align with Rel-16 CGI reporting UE capability introduced for ANR function in NPN.

3. Do companies think that there is a need to specify UE AS capability signalling for CGI reporting for CH and onboarding?

| Companies | Yes or No | Comments |
| --- | --- | --- |
| Intel | No |  |

# UE capability for IMS emergency services over SNPN

| Companies | Proposals |
| --- | --- |
| OPPO [1] | Proposal 3: Introduce an independent UE capability bit (1 optional per UE bit without xDD/FRx differentiation) for supporting IMS voice and emergency services for SNPN in eNPN. |
| Intel, Nokia [3] | Proposal#3: For IMS emergency services over SNPN in limited-service state, similar to Rel-15 IMS emergency call, it should be conditional mandatory to that the UE operating in SNPN access mode is IMS voice capable with the following update to TS38.306 in Clause 6. IMS emergency service over SNPN; It is mandatory for UEs operating in SNPN access mode which are IMS voice capable to support IMS emergency service over SNPN in limited service state, according to ims-SNPN-EmergencySupport as specified in TS38.331 [9].. |
| CMCC [6] | Proposal 3: There is no need to introduce a separate AS UE capability for SNPN for IMS voice and emergency services. |
| ZTE [7] | Proposal 2: No new UE capabilities for the eNPN IMS feature, the legacy capabilities (e.g. VoiceOverNR/IMS emergency call) can be reused. |
| Vivo [8] | Proposal 3: Introduce a new UE capability on whether the UE supports IMS emergency call through SNPN cell. It is a conditionally mandatory feature without UE radio access capability parameters. |
| LG [9] | Proposal 3: To specify in 38.306 that the support for emergency services in SNPN is mandatory for Rel-17 and onward UEs supporting IMS voice in NR. Proposal 4: To allow early implementation of emergency services support by Rel-16 SNPN capable UEs supporting IMS voice services. |
| Ericsson [11] | - Proposal 1 Existing UE capabilities are sufficient to address new Rel-17 NPN functionalities. |

On whether UE AS capability signalling is needed for IMS emergency services, 6 companies think that there is no need except for one. Since it is the majority view, a straight proposal as follow:

Proposal#2: No UE AS capability signalling is needed for IMS emergency services.

4. Do companies agree to the above proposal 2?

| Companies | Yes or No | Comments |
| --- | --- | --- |
| Intel | Yes |  |

On whether UE AS capabilities need to be included in TS38.306 as AS capabilities without capability signalling, [3], [8] and [9] think that there a need to specify that the support for emergency services in SNPN is mandatory on the condition that the UE is IMS voice capable. [7] thinks that the existing ISM emergency call can be reused.

5. Do companies think that there is a need to specify IMS emergency call over SNPN AS capability without signalling in TS38.306? If there is a need to specify in TS38.306, can (a) the existing conditional mandatory without capability signalling for IMS emergency call be reused or (b) a new “Conditional mandatory without capability signalling” be introduced?

| Companies | Yes or No | (a) or (b) | Comments |
| --- | --- | --- | --- |
| Intel | Yes | (b) | It should be conditional mandatory to UE supporting IMS voice over SNPN. |

On the early implementation [9], the rapporteur thinks that this can be postponed to the next meeting when the capability signalling for the eNPN is decided, noting that early implementation also depends on CT1 NAS support.

# Conclusion

To be included later.

# References

[1] R2-2200233	UE Capabilities for eNPN	OPPO	discussion	Rel-17	NG_RAN_PRN_enh-Core

[2] R2-2200293	Discussion on UE capability for eNPN	Huawei, HiSilicon	discussion	Rel-17	NG_RAN_PRN_enh-Core

[3] R2-2200508	UE capability for Rel-17 NPN	Intel Corporation, Nokia, Nokia Shanghai Bell	discussion	Rel-17	NG_RAN_PRN_enh-Core

[4] R2-2200509	UE capability for Rel-17 NPN	Intel Corporation, Nokia, Nokia Shanghai Bell	draftCR	Rel-17	38.306	16.7.0	NG_RAN_PRN_enh-Core

[5] R2-2200521	Discussion of UE capability of eNPN	China Telecom	discussion	Rel-17	NG_RAN_PRN_enh-Core

[6] R2-2200849	Discussion on UE capability for NPN	CMCC	discussion	Rel-17	NG_RAN_PRN_enh

[7] R2-2201236	Consideration on the eNPN UE Capability	ZTE Corporation, Sanechips	discussion	Rel-17	NG_RAN_PRN_enh-Core

[8] R2-2201266	Discussion on UE capabilities for R17 NPN	vivo	discussion	Rel-17	NG_RAN_PRN_enh-Core

[9] R2-2201469	UE capabilities	LG Electronics	discussion	Rel-17

[10] R2-2201524	Discussion on UE capabilities relating to Rel17 eNPN features	Samsung R&D Institute India	discussion	Rel-17	NG_RAN_PRN_enh-Core

[11] R2-2201566	UE capabilities for eNPN	Ericsson	discussion	Rel-17	NG_RAN_PRN_enh-Core