3GPP TSG-RAN WG2 Meeting #116bis-e     			          	                         R2-22XXXX

Electronic Meeting, 17 – 25 January 2022

Agenda item:	8.8.1

Source:	CMCC

Title:	Report for [AT116bis-e][240][Slicing] Remaining details for slice groups (CMCC)

WID/SID:	FS_NR_slice

Document for:	Discussion and Decision

# Introduction

This document aims at address the remaining details for slice groups

[AT116bis-e][240][Slicing] Remaining details for slice groups (CMCC)

Scope: Discuss the slice group aspects: 1) discuss what should be the definition of slice group (based on latest RAN2 and SA2 agreements)? 2) how to resolve the TA boundary aspects? 3) does UE select different slice group if no cell supporting that slice group is available?

Intended outcome: Discussion summary in R2-2201708.

Deadline: Deadline 3

Comment deadline: Thursday W1, 1600 UTC (for collecting views)

Rapporteur proposals: Friday W1, 0900 UTC (proposed resolution of issues)

Document deadline: Monday W2, 1200 UTC (report or agreed CRs)

Contact List

| Company | Name | Email |
| --- | --- | --- |
| Qualcomm | Peng Cheng | chengp@qti.qualcomm.com |

# Discussion

Open issue 1: Definition of slice grouping

Previous agreements in RAN2#116-e:

A network slice can be associated to none or only one slice group.

Since SA2 hasn’t reached agreement on the definition of slice grouping, RAN2 can try to define one based our understanding. A candidate definition is shown below:

Slice group: A group which is associated with one or multiple slices. And a slice is associated to none or only one slice group.

Q1.1: Do you agree with the above definition for slice group? Other suggestions are also welcome.

| Company | Comments |
| --- | --- |
| Qualcomm | Agree. |

Regarding to the maximum number of slice groups, the contributions [19, 30, 47] suggested the maximum number of slice groups as at most 16 slice groups, or as less as possible (e.g. 3 slice groups representing for high/medium/low degree of importance). While, [5] suggested to postpone the decision on maximum number of slice grouping configured by Network after it is clear what dedicate configurations can be configured for one slice group.

Q1.2: What is the maximum number of slice group?

Option 1: 16

Option 2: other numbers.

| Company | Which option do you prefer | comments |
| --- | --- | --- |
| Qualcomm | See comments | We still see more than 1 companies are proposing to modify previous RAN2 WA on per-TA slice grouping to per-PLMN with some technique justifications. If it is per-PLMN, it is obviously up to 16 groups is not sufficient. That is why we think it seems still not a good time to determine the maximum number of slice group. We think RAN2 can agree max slice grouping number only after: - The definition of slice grouping is finalized (including its definition and granularity) - What configuration (and its payload size) can be configured to one slice group |

Open Issue 2: TA boundary

Previous agreements in RAN2#116-e

3: Working assumption: The granularities of the slice groups for cell reselection are per TA. FFS on the details (e.g. how to resolve TA boundaries).

A number of contributions [4, 16, 19, 25, 30, 49] see issues for TA boundary and suggested to resolve the issues, while the contributions [22, 48] thought that there is no issue and no spec impacts on TA boundary.

The first open issue is that, when UE is checking whether the highest ranked cell support the highest priority slice, how the UE is aware of the slice supported by the neighbouring cell belonging to other TA?

According to the contributions, there are several potential solutions for resolving the TA boundary issue.

Q2.1: How UE can know the supported slice for neighbouring cell at TA boundary?

Option A: The gNBs exchange the supported slices (S-NSSAI/NSSAI) through Xn interface, then serving gNB can map the slices supported by neighbour cells to slice groups based on the slice group mapping relationship in current TA and broadcast it to the UEs. [19]

Option B: As assistance information, an optional PCI list is introduced to indicate the cells supporting one slice group in a new SIB. And if NW don’t provide such info on the best ranked cell, the UE may skip the checking on slice support in best ranked cell. [4]

Option C: Add the association of slice group IDs and their valid TACs in the slice-specific cell reselection info for inter-TA cell reselection. The UE can determine whether to camp on the highest ranked cell based on the TAC received in its SIB1.[30]

| Company | Which option do you prefer | comments |
| --- | --- | --- |
| Qualcomm | Option B in RAN2 | We are proponent of Option B. It is also one solution to reduce signalling overhead of slice info in SIB. In this way, the UE behaviour is clearly specified, and no RAN3 and SA2 impacts are foreseen (i.e., it is up to source cell implementation how to provide the optional PCI list) For Option A and C, it obviously has at least RAN3 impacts. And we don’t see they are mutual exclusive to Option B. Because they are not RAN2 expertise, we suggest proponents to propose these solution in RAN3 and SA2 first. |

The contribution [19, 12] pointed out the case that the neighbour cell supports a slice which cannot be mapped into any current slice group and suggested to handle this issue. There are several potential solutions for this issue:

Q2.2: How to handle the case if the gNB doesn’t support the slice group mapping for the slice of the neighbouring cell? Do we need to send LS to RAN3/SA2?

Option 1: The gNB can request CN to update the mapping to involve the new slice.[19]

Option 2:  The gNB can request RAN OAM to update the mapping to involve the new slice.

Option 3: Restrict that one slice is mapped to only one slice group. [12]

| Company | Which option do you prefer | Send LS to RAN3/SA2? | comments |
| --- | --- | --- | --- |
| Qualcomm | See comments | No | As we replied in Q2.1, Option 1 and Option 2 are RAN3 expertise. We suggest proponents to propose these solutions in RAN3 first. We don’t think any issue to resolve in RAN2, and also we don’t think RAN2 should trigger such discussion to RAN3. |

Open issue 3: Consider low priority slice or not

The contribution [8, 19] have proposed that low priority slice is considered with iteration but needs to some enhancements, for example, reuse the immediate past measurements, or set the maximum number of iterations, or set a timer for iteration, etc. Another contribution [30] proposed that RAN can indicate the UE whether to perform Step 7 or the limit times of iterations.

On the other hand, the contribution [35, 37] proposed new algorithm on frequency priority handling, in which the low priority slice is considered without iteration.

Some contributions [4, 16, 11, 33, 41] suggested to only consider the highest priority slice.

Q3: Does UE select different slice if no cell supporting that slice is available, which option do the companies prefer?

Option A1: Low priority slice is considered with iteration. [8, 19, 44, 45]

Option A2: Low priority slice is considered, but without iteration. [35, 37]

Option B: Only highest priority slice considered. [4, 16, 11, 33, 41]

| Company | Which option do you prefer | Comments |
| --- | --- | --- |
| Qualcomm | Option B with comments | We support a clearer Option B: Option B-1: Only highest priority slice considered, then legacy priorities considered. [4, 16, 11, 33, 41] For Option A1 and A2: - We do not support them as we commented before that it will increase UE’s latency and power consumptions during cell reselection, especially if number of slice group is large. It is conflicted with the intention to introduce slice specific cell reselection. - At such late stage of Rel-17, we have to emphasize that RAN2 need to focus on how to close open issues in the remaining two meetings. In our understanding, there is even not a converged baseline solution on how to consider the low priority slice (e.g., how to converge Option A1 and Option A2?). And more and more enhancements are being proposed as Rapporteur mentioned (e.g., set the maximum number of iterations, or set a timer for iteration), which we are not sure whether they are on top of Option A1 or Option A2? We don’t think how RAN2 can make progress for these on-fly proposals in the remaining 2 meetings. In all, although we understand Option B-1 has some performance restriction, we think it is a reasonable way forward to finalize Rel-17 RAN slicing enhancement, given the current situation. |

# Summary

# References

- R2-2200043	[Post116-e][242][Slicing] Slice-based cell re-selection algorithm	Ericsson

- R2-2200044	Running 38.304 CR for RAN slicing	Ericsson

- R2-2200055	List of open issues for RAN slicing WI	CMCC

- R2-2200179	Remaining issues on slice specific cell reselection	Qualcomm Incorporated

- R2-2200180	Remaining issues on slice specific RACH	Qualcomm Incorporated

- R2-2200181	Further discussion on UE capability related to RAN slicing enhancement	Qualcomm Incorporated

- R2-2200406	Optimizations for signalling Slice Information	Lenovo, Motorola Mobility

- R2-2200407	RAN Slicing CR to 38.304	Lenovo, Motorola Mobility

- R2-2200408	Triggers for initiating RAN slicing based cell reselections	Lenovo, Motorola Mobility

- R2-2200409	Principles of Slice based reselection	Lenovo, Motorola Mobility

- R2-2200416	Discussion on Slice based Cell Reselection	CATT

- R2-2200417	Analysis on issues of slice groups at TA boundaries	CATT

- R2-2200418	Analysis on UE capability for RAN slicing enhancement	CATT

- R2-2200510	Further considerations of slice based cell reselection	Intel Corporation

- R2-2200511	UE capability for Slicing enhancement	Intel Corporation

- R2-2200636	Consideration on slice based cell reselection	Spreadtrum Communications

- R2-2200697	Considerations on UE capability for RAN slicing	Beijing Xiaomi Software Tech

- R2-2200844	Open issues list for RAN Slicing	CMCC

- R2-2200845	Discussion on open issues for slice based cell reselection	CMCC

- R2-2200846	Discussion on open issues for slice based RACH configuration	CMCC

- R2-2200847	Discussion on UE capability for RAN slicing enhancement	CMCC

- R2-2200929	Consideration on slice-specific cell reselection	OPPO

- R2-2200930	Consideration on slice-specific RACH	OPPO

- R2-2200931	Consideration on UE capability for Slicing	OPPO

- R2-2200947	Considerations on slice groups	Nokia, Nokia Shanghai Bell

- R2-2200948	Text Proposals for the draft 38.304 PCR	Nokia, Nokia Shanghai Bell

- R2-2200949	Cell reselection delay for option B and option C 	Samsung R&D Institute India

- R2-2200972	Report of [Post116-e][243][Slicing] Running NR RRC CR for RAN slicing (Huawei)	Huawei

- R2-2200973	Running NR RRC CR for RAN slicing	Huawei, HiSilicon

- R2-2200974	Discussion on slice based cell reselection under network control	Huawei, HiSilicon

- R2-2200975	Discussion on slice based RACH configuration	Huawei, HiSilicon

- R2-2200976	Discussion on UE capabilities for RAN slicing	Huawei, HiSilicon

- R2-2201005	Leftover issues in slice based cell reselection	ZTE corporation, Sanechips

- R2-2201050	Detailed RRC signalling for RACH prioritization configuration	Nokia, Nokia Shanghai Bell

- R2-2201110	Text proposal for slice based cell reselection under NW control	Apple

- R2-2201111	Slice based RACH configuration	Apple

- R2-2201169	On slice-based cell re-selection TP for 38.304	Ericsson

- R2-2201170	RACH for RAN slicing enhancement	Ericsson

- R2-2201171	UE Capabilities  for Slice- based Cell re-selection	Ericsson

- R2-2201190	Slice-Info provision	NEC Telecom MODUS Ltd.

- R2-2201192	Slice-based cell re-selection TP for solution 4C	NEC Telecom MODUS Ltd.

- R2-2201200	Slice information provided by RRCRelease	Sharp

- R2-2201208	Discussion on signalling slice information	LG Electronics UK

- R2-2201209	Discussion on slice based cell reselection	LG Electronics UK

- R2-2201389	A couple of FFS for Cell Reselection 	Kyocera

- R2-2201406	Discussion on Slice Aware UL BSR	RadiSys, Reliance JIO

- R2-2201409	Considerations on remaining issues for slice based RACH	Beijing Xiaomi Software Tech

- R2-2201410	Resolving the common issues in slice based cell reselection	Beijing Xiaomi Software Tech

- R2-2201417	Further consideration on slice specific RACH	ZTE corporation, Sanechips

- R2-2201418	TP for system information and slice based reselection priority handling	ZTE corporation, Sanechips

- R2-2201422	On selection of Solution 4 Option A, B and C	Samsung R&D Institute UK

- R2-2201443	Remaining Issues on Slice Information	Samsung R&D Institute UK

- R2-2201475	Remaining issues on slice based RACH prioritization	LG Electronics Inc.

- R2-2201536	38.321 running CR for RAN Slicing	OPPO