3GPP TSG RAN WG2 Meeting #116bis-e		R2-21xxxx

Electronic meeting, 17th-25th January 2022

Title:	[draft] LS on RAN3 impacts for CCCH based solution for non-SDT handling

Release:	Rel-17

Work Item:	NR_SmallData_INACTIVE-Core

Source:	Intel Corporation (to be RAN2)

To:	RAN3

Cc:	SA3, CT1

Contact person:	Marta Martinez Tarradell

Contact email:	marta.m.tarradell <at> intel <dot> com

1	Overall description

During the ongoing SDT session with anchor relocation, the UE context is transferred from the anchor gNB to the serving gNB.  For the CCCH solution, when there is data for a non-SDT RB, the UE aborts the ongoing SDT session and the network would not have sent a RRCRelease message with new I-RNTI or security key information.  The UE will send a second RRCResumeRequest message using the I-RNTI that was issued by the old anchor gNB and performs horizontal key derivation.  The ResumeMAC-I is expected to be updated as discussed in [Reference to SA3 LS].

RAN2 would like to ask RAN3:

- Q1: Which node (old anchor gNB or serving gNB) will process the second RRCResumeRequest message with I-RNTI associated to the old anchor gNB and will perform ResumeMAC-I verification and key derivation?

- Q2: When UE autonomously terminates the ongoing SDT session and sends second RRCResumeRequest message, can the buffered DL data from the previous ongoing SDT session in the serving gNB be sent without loss after the UE gets RRC_CONNECTED?

2	Actions

To RAN3

ACTION:

RAN3 is respectfully requested to provide a response and any additional feedback on the proposed CCCH based solution, above questions (Q1 and Q2) and any network handling of the second RRCResumeRequest message.

3	Dates of next TSG RAN WG2 meetings

RAN2#117-e	21st February to 03 March 2022, E-meeting

RAN2#118-e	16th to 27th May 2022, E-meeting