3GPP TSG RAN WG2#116bis-e	R2-2200080

e-Meeting, 17th - 25th January, 2022

3GPP TSG-RAN WG1 Meeting #107-e	R1-2112834

e-Meeting, November 11–19, 2021

Title:	LS on propagation delay compensation

Response to:	-

Release:	Rel-17

Work Item:	NR_IIOT_URLLC_enh

Source:	TSG RAN WG1

To:	TSG RAN WG2, TSG RAN WG4

Contact Person:

#### Name:	Yan Cheng

E-mail Address:	chengyan.cheng@huawei.com

1. Overall Description:

RAN1 further discussed propagation delay compensation enhancements in RAN1#107-e, and the following additional agreements are achieved on top of the ones included in LS R1-2110647 sent to RAN2 and RAN4 in RAN1#106bis-e:

Agreement If RTT-based PDC is supported, a single granularity 32Tc (i.e. k=5) is supported for Rx-Tx measurement report. Agreement For Rel-17 - Support RTT-based PDC method - Support PDC method based on legacy TA-based mechanism - No RAN1/RAN4 specification impact expected Agreement For RTT-based PDC, existing definitions of UE Rx – Tx time difference (i.e. section 5.1.30 in TS 38.215) and gNB Rx – Tx time difference (i.e. section 5.2.3 in TS 38.215) are reused, with updates at least to reflect the single pair of TRS/PRS and SRS configured for RTT-based PDC. Agreement Send an LS to RAN2 and RAN4 with the content including: - The agreements made in RAN1#107-e for propagation delay compensation. - Ask RAN4 to define the following for RTT-based propagation delay compensation: - UE Rx-Tx time difference measurement accuracy based on CSI-RS for tracking - UE Rx-Tx time difference measurement accuracy based on PRS (including reuse existing spec if appropriate) - gNB Rx-Tx time difference absolute accuracy based on SRS (including reuse existing spec if appropriate) - Inform RAN4 that enhanced TA-based PDC with reduced Te and enhanced TA command granularity is precluded in RAN1. Conclusion For RTT-based PDC, it is assumed that the transmission of DL TRS/PRS, UL SRS and reference time information are associated with a same TRP. - Note: No RAN1 specification impact is expected for this conclusion Agreement For RTT-based propagation delay compensation, the Rx-Tx time difference is reported via RRC signaling. Conclusion The reporting range of Rx-Tx time difference measurement for RTT-based PDC is up to RAN4.

Based on the agreements achieved in RAN1, RTT-based PDC is supported in Rel-17. RAN1 respectfully asks RAN4 to at least define the following for RTT-based PDC in PCell:

- UE Rx-Tx time difference measurement accuracy based on CSI-RS for tracking

- UE Rx-Tx time difference measurement accuracy based on PRS (including reuse existing spec if appropriate)

- gNB Rx-Tx time difference absolute accuracyerrorgNB,RxTxDiff based on SRS (including reuse existing spec if appropriate)

RAN1 also would like to inform RAN2 and RAN4 that enhanced TA-based PDC with reduced Te and enhanced TA command granularity is precluded in RAN1.

2. Actions:

To RAN2

ACTION: 	RAN1 respectfully asks RAN2 and RAN4 to take the above information into account in their future work.

3. Date of Next TSG-RAN WG1 Meetings:

TSG-RAN WG1 Meeting #107bis-e	17 – 25 January 2022	E-Meeting

TSG-RAN WG1 Meeting #108-e	21 February – 03 March 2022	E-Meeting