3GPP TSG-RAN WG2 Meeting #117 Electronic	R2-2202331

Elbonia, 21 February – 03 March 2022

Agenda item:	8.1.5.2

Source:	Nokia, Nokia Shanghai Bell

Title:	MBS User Plane Issues

WID/SID:	NR_MBS-Core - Release 17

Document for:	Discussion and Decision

# 1	Introduction

This contribution discusses a few user plane issues that remain open for the provision of MBS in NR.

# 2	Stalling of Receive Window

In NR, reordering of PDUs takes place at PDCP only and RLC limits itself to reorder possible segments. To prevent the receiver from stalling forever in case of gaps, a reordering timer (t-Reordering) was introduced. This means that any missing PDCP PDU will stall the re-ordering window. This differs from LTE where when the UE receives a PDCP SDU, it delivers it to higher layer together with all PDCP SDUs with lower SNs regardless of possible gaps and in ascending order of the SNs.

Observation 1: unlike in LTE, any gap in in the receive SN sequence of PDCP PDUs will stall the reordering window until a reordering timer expires.

The corresponding behaviour specified in 38.323 is quoted below for reference:

When t-Reordering expires, the receiving PDCP entity shall: - deliver to upper layers in ascending order of the associated COUNT value after performing header decompression, if not decompressed before: - all stored PDCP SDU(s) with associated COUNT value(s) < RX_REORD; - all stored PDCP SDU(s) with consecutively associated COUNT value(s) starting from RX_REORD; - update RX_DELIV to the COUNT value of the first PDCP SDU which has not been delivered to upper layers, with COUNT value >= RX_REORD; - if RX_DELIV < RX_NEXT: - update RX_REORD to RX_NEXT; - start t-Reordering.

When it comes to the handling of possible losses, while we should aim at minimising them, it may not always be possible or even desired for instance during mobility events, or when a UE lacks too far behind to catch up with an ongoing PTM transmission. To avoid stalling the re-ordering window when losses occur, the LTE behaviour should be followed i.e. deliver to upper layers:

-	all stored PDCP SDU(s) with an associated COUNT value less than the COUNT value associated with the received PDCP SDU;

-    all stored PDCP SDU(s) with consecutively associated COUNT value(s) starting from the COUNT value associated with the received PDCP SDU.

This would be a one-shot-operation, triggered by the gNB, and once completed, the window would then switch back to the current behaviour for NR.

Proposal 1: PDCP receive window should temporarily follow the LTE behaviour when ordered to do so by the gNB i.e. deliver to upper layers 1) all stored PDCP SDU(s) with an associated COUNT value less than the COUNT value associated with the received PDCP SDU; and 2) all stored PDCP SDU(s) with consecutively associated COUNT value(s) starting from the COUNT value associated with the received PDCP SDU.

# 3	Measurement Gaps

Measurement gaps are provided to UE in RRC_CONNECTED to measure neighbouring cells. Whether a measurement is non-gap-assisted or gap-assisted depends on the capability of the UE, the active BWP of the UE, and the current operating frequency. This baseline mechanism was further enhanced in Rel-16 by introducing NeedForGap as the UE capability to perform gapless measurement not only depends on the current band combination but also other L1 parameters that may occupy some baseband resource (e.g. number of MIMO layers).

When multiple UEs requiring measurement gaps receive the same MBS service via group scheduling (i.e. PTM transmission though G-RNTI) arise the question of how to handle these measurement gaps. We can think of three different alternatives:

1.	Common gap without MBS transmission: the measurement gaps of all UEs are configured to overlap and MBS data transmission stops during that one gap.

2.	Common gap with MBS transmission: the measurement gaps of all UEs are configured to overlap but MBS data transmission does not stop during that one gap.

3.	Dedicated gap with MBS transmission: gaps are individually configured and MBS data transmission does not stop for each individual gap.

NOTE:	Dedicated gaps without transmission is obviously not an option as it would leave very little time available for MBS data transmission, if any.

The first alternative is feasible but unfair as the worst UE will govern the throughput and latency of all UEs. The second and third alternatives do not exhibit the same drawback but UEs having to apply measurement gaps will suffer losses unless retransmissions are separately scheduled. So let us investigate this point.

For the retransmissions of PDUs originally sent via group scheduling and targeting multiple UEs (the ones having used a measurement gap), we believe that group scheduling should also be used. Indeed, the very same reasons and benefits justifying the use of group scheduling for the first transmission also apply to retransmissions as soon as multiple UEs need to receive that retransmission. This favours the 2nd alternative with retransmissions being typically needed for multiple UEs. Such retransmissions would naturally be blind PDCP retransmissions not relying on any UE feedback: the network only assumes that the gap was used and schedules retransmissions to compensate for the corresponding losses.

Proposal 2: Blind PDCP retransmissions to compensate for missed PTM transmissions due to measurement gaps also relies on group scheduling.

This could be achieved by the configuration of a second G-RNTI, which would then schedule retransmissions of MBS transmission overlapping with a common measurement gap. Only UEs having used the gap would decode these retransmissions.

# 4	Conclusion

Two proposals were made to conclude the MBS work from a user plane perspective:

Proposal 1: PDCP receive window should temporarily follow the LTE behaviour when ordered to do so by the gNB i.e. deliver to upper layers 1) all stored PDCP SDU(s) with an associated COUNT value less than the COUNT value associated with the received PDCP SDU; and 2) all stored PDCP SDU(s) with consecutively associated COUNT value(s) starting from the COUNT value associated with the received PDCP SDU.

Proposal 2: Blind PDCP retransmissions to compensate for missed PTM transmissions due to measurement gaps also relies on group scheduling.

Possible changes to incorporate Proposal 1 into PDCP are given below:

First Modified Subclause

#### 5.2.2.1	Actions when a PDCP Data PDU is received from lower layers

In this section, following definitions are used:

-	HFN(State Variable): the HFN part (i.e. the number of most significant bits equal to HFN length) of the State Variable;

-	SN(State Variable): the SN part (i.e. the number of least significant bits equal to PDCP SN length) of the State Variable;

-	RCVD_SN: the PDCP SN of the received PDCP Data PDU, included in the PDU header;

-	RCVD_HFN: the HFN of the received PDCP Data PDU, calculated by the receiving PDCP entity;

-	RCVD_COUNT: the COUNT of the received PDCP Data PDU = [RCVD_HFN, RCVD_SN].

At reception of a PDCP Data PDU from lower layers, the receiving PDCP entity shall determine the COUNT value of the received PDCP Data PDU, i.e. RCVD_COUNT, as follows:

-	if RCVD_SN < SN(RX_DELIV) – Window_Size:

-	RCVD_HFN = HFN(RX_DELIV) + 1.

-	else if RCVD_SN >= SN(RX_DELIV) + Window_Size:

-	RCVD_HFN = HFN(RX_DELIV) – 1.

-	else:

-	RCVD_HFN = HFN(RX_DELIV);

-	RCVD_COUNT = [RCVD_HFN, RCVD_SN].

After determining the COUNT value of the received PDCP Data PDU = RCVD_COUNT, the receiving PDCP entity shall:

-	perform deciphering and integrity verification of the PDCP Data PDU using COUNT = RCVD_COUNT;

-	if integrity verification fails:

-	indicate the integrity verification failure to upper layer;

-	discard the PDCP Data PDU;

-	if RCVD_COUNT < RX_DELIV; or

-	if the PDCP Data PDU with COUNT = RCVD_COUNT has been received before:

-	discard the PDCP Data PDU;

-	if RCVD_COUNT was flagged for selective reordering:

-	deliver to upper layers in ascending order of the associated COUNT value after performing header decompression, if not decompressed before;

-	all stored PDCP SDU(s) with an associated COUNT value less than the COUNT = RCVD_COUNT;

-	all stored PDCP SDU(s) with consecutively associated COUNT value(s) starting from COUNT = RCVD_COUNT;

-	update RX_DELIV to the COUNT value of the first PDCP SDU which has not been delivered to upper layers, with COUNT value > RCVD_COUNT;

If the received PDCP Data PDU with COUNT value = RCVD_COUNT is not discarded above, the receiving PDCP entity shall:

-	store the resulting PDCP SDU in the reception buffer;

-	if RCVD_COUNT >= RX_NEXT:

-	update RX_NEXT to RCVD_COUNT + 1.

-	if outOfOrderDelivery is configured:

-	deliver the resulting PDCP SDU to upper layers.

-	if RCVD_COUNT = RX_DELIV:

-	deliver to upper layers in ascending order of the associated COUNT value after performing header decompression, if not decompressed before;

-	all stored PDCP SDU(s) with consecutively associated COUNT value(s) starting from COUNT = RX_DELIV;

-	update RX_DELIV to the COUNT value of the first PDCP SDU which has not been delivered to upper layers, with COUNT value > RX_DELIV;

-	if t-Reordering is running, and if RX_DELIV >= RX_REORD:

-	stop and reset t-Reordering.

-	if t-Reordering is not running (includes the case when t-Reordering is stopped due to actions above), and RX_DELIV < RX_NEXT:

-	update RX_REORD to RX_NEXT;

-	start t-Reordering.

Next Modified Subclause

#### 6.2.2.2	Data PDU for DRBs with 12 bits PDCP SN

Figure 6.2.2.2-1 shows the format of the PDCP Data PDU with 12 bits PDCP SN. This format is applicable for UM DRBs and AM DRBs.

Figure 6.2.2.2-1: PDCP Data PDU format with 12 bits PDCP SN

#### 6.2.2.3	Data PDU for DRBs with 18 bits PDCP SN

Figure 6.2.2.3-1 shows the format of the PDCP Data PDU with 18 bits PDCP SN. This format is applicable for UM DRBs and AM DRBs.

Figure 6.2.2.3-1: PDCP Data PDU format for DRBs with 18 bits PDCP SN

Next Modified Subclause

### 6.3.12	SRF

Length: 1 bit

This field indicates whether the corresponding PDCP Data PDU is flagged for selective reordering.

Table 6.3.12-1: SRF field

| Bit | Description |
| --- | --- |
| 0 | PDCP Data PDU is not flagged for selective reordering. |
| 1 | PDCP Data PDU is flagged for selective reordering. |

End of Modified Subclauses