3GPP TSG-RAN WG2 #117	R2-2203429

Electronical meeting, 21 February– 3 March 2022

Agenda Item:	8.14.4 UE capabilities

Source:	Ericsson

Title:	UE capabilities for QoE measurements

Document for:	Discussion, Decision

# 1	Introduction

The following is listed as an outstanding issue in the outstanding issue document for QoE, R2-2202043:

Proposal 14: RAN2 can discuss how AS layer obtains application capability.

# 2	Discussion

## 2.1	Application layer capabilities

A question came up in RAN2 whether there needs to be any communication between the AS layer and the application layer regarding UE capabilities for QoE. The AS layer reports the capabilities to the network in UE capability signalling. It is not clear if the capabilities signalled include also support in the application.

There can be several different applications for the same service type. UE capabilities for QoE are defined per service type. If the UE e.g. has three applications for streaming, but only one of the applications support QoE, can the UE indicate support for QoE streaming or do all three applications need to support QoE? Or alternatively, the UE may not take the support in the application into account when reporting the AS UE capability for QoE.

Discuss if there is any requirement on application layer support if the UE reports support for QoE for a certain service type.

If the UE should take the application layer capability into account when reporting the AS capabilities, there needs to be some communication within the UE to exchange the information. The communication within the UE could be based on UE implementation or it could be based on AT-commands. If RAN2 concludes that the application layer support will be taken into account in the AS layer capability, it might be good to ask SA4 and CT1 whether any AT-command is needed to indicate support for QoE in the application to the AS layer.

Ask SA4 and CT1 whether an AT-command is needed for transferring of UE capabilities from the application to the AS layer.

# Conclusion

Based on the discussion in the previous sections the following is proposed:

Proposal 1	Discuss if there is any requirement on application layer support if the UE reports support for QoE for a certain service type.

Proposal 2	Ask SA4 and CT1 whether an AT-command is needed for transferring of UE capabilities from the application to the AS layer.

# References

TR 38.890, Study on NR QoE management and optimizations for diverse services, Ericsson

RP-210913, NR QoE management and optimizations for diverse services, China Unicom