- 3GPP TSG RAN WG2 Meeting #117-e	      	               R2-220xxxx
E-Conference, 21st Feb – 3rd Mar, 2022

Agenda item: 	8.22.4

Source:	Intel Corporation

Title: 	[AT117-e][020][MGE] UE capabilites (Intel)

WID/SID:	NR_MG_enh-Core – Release 17

Document for:	Discussion and Decision

# Introduction

Here is the RAN4 feature list (some column is omitted):

| Features | Index | Feature group | Components | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- |
| 19. NR_MG_enh | 19-1 | Network controlled small gap (NCSG) | Support of NCSG |  | Optional with capability signalling |
| 19. NR_MG_enh | 19-2 | Concurrent measurement gaps | o Support of more than 1 per-UE measurement gap configurations o Support of more than 1 per-FR gap measurement gap configurations in an FR, or simultaneous 1 per-UE measurement gap plus 1 per-FR measurement gap configurations in an FR, or more than 1 per-UE measurement gap configurations for UE capable of Rel-15 per-FR gap (independentGapConfig) Note: The above 2 bullets are not 2 separate indications but a single indication with different interpretations, depending on the support of independentGapConfig. | This is the baseline capability is to indicate UE support multiple concurrent gaps. | Optional with capability signalling |

And RAN4 last meeting agreed (some column is omitted):

| Features | Index | Feature group | Components | Mandatory/ Optional |
| --- | --- | --- | --- | --- |
| X. Measurement gap enhancement | X-1-1 | Pre-configured measurement gap with network-controlled activation and deactivation mechanism | Capability of supporting preconfigured measurement gap with network-controlled mechanism for activation and deactivation | Optional with capability signalling |
|  | X-1-2 | Pre-configured measurement gap with UE autonomous activation and deactivation mechanism | Capability of supporting preconfigured measurement gap with UE autonomous mechanism for activation and deactivation | Optional with capability signalling |

This is to start offline discussion below:

- [AT117-e][020][MGE] UE capabilites (Intel)

Scope: Based on R2-2203522. Determine agreeable parts, points for discussion, open issues if needed. Converge as far as possible to reduce the need for on-line discussion. Treat R2-2202462 and R2-2202463, collect comments and update accordingly, in preparation to endorse for merge revisions at EOM. (i.e. the time to make the last changes, review and endorse the draft CRs will be very short)

Intended outcome: Report (revised draft CRs may be provided for W2 Tuesday if there is some discussion point that needs the CRs).

Deadline: In time for on-line CB W2 Tuesday

The offline discussion will have 2 phases, the deadline for input are as follow:

- Phase one, offline discussion comment deadline: 2/25/2022 13:00UTC

- Phase two, CR review deadline: 2/28/2022 08:00UTC

# Discussion

## UE capability for NCSG measurement gap

|  | Source | Related proposals |
| --- | --- | --- |
| One bit capability for NCSG gap | Vivo, R2-2202324 | Proposal 3: Introduce a per-UE capability for support reporting NCSG requirement information. Proposal 4: Introduce a UE capability on supported NCSG patterns. |
|  | Intel, R2-2202462 | Introduce one per UE optional capability for NCSG as descripted in RAN4 feature table 19-1. |
|  | MT, R2-2202879 | Proposal 3: Introduce one per UE optional capability for reporting of NCSG gap requirement information. Wait further R4 input on capability of NCSG gap patterns. Rapp: agree to wait for RAN4 input |
|  | HW, R2-2202892 | Proposal 3: A general capability for the support of NCSG feature is introduced based on RAN4 feature list. Proposal 4: Reuse nr-NeedForGap-Reporting-r16 to indicate whether the UE supports dynamically reporting the NCSG requirement information for NR target in the UE response to a network configuration RRC message. |
|  | E///, R2-2203450 | Proposal 3 Introduce a new general/single UE capability to indicate support of the Rel-17 NCSG feature. |

In current RAN4 feature list, there is 1 bit UE capability to support NCSG measurement gap. In addition, RAN2 has agreed the following:

- RAN2 confirms that reporting of NCSG for E-UTRA target bands is supported. RAN2 assumes that support for EUTRA target bands can be a separate UE cap

Therefore, do you agree to have 1 bit UE capability to support NCSG in addition to the UE capability to support for EUTRA target band? From companies contribution, some companies would like to add UE capability on supporting NCSG pattern, do you prefer to have UE capability to support NCSG pattern?

Q1: Please indicate which of the follow do you agree to support in addition to the UE capability to support for EUTRA target band?

- A) 1 bit UE capability to support NCSG

- B) UE capability to support NCSG pattern

| Company | A/B/both | Comments |
| --- | --- | --- |
| MediaTek | See comment | We think that UE capacity for NCSG including the following <1> A single per UE bit to indicate dynamic reporting of NCSG gap requirement information for NR target bands. <2> A single per UE bit to indicate dynamic reporting of NCSG gap requirement information for E-UTRA target bands. <3> Supported NCSG gap patterns. Some of them will be mandatory if NCSG is supported. Details are still waiting for RAN4 conclusion. |

Summary:

## UE capability for concurrent measurement gap

|  | Source | Related proposals |
| --- | --- | --- |
| One bit capability for concurrent gap | Vivo, R2-2202324 | Proposal 2: Introduce a UE capability on the support of multiple concurrent and independent gap patterns, the UE capability is defined per UE. |
|  | Intel, R2-2202462 | Introduce 1 bit per UE optional capability as descripted in RAN4 feature table 19-2 |
|  | MT, R2-2202879 | Proposal 2: Introduce one per UE optional capability for concurrent gap as descripted in RAN4 feature table 19-2. |
|  | HW, R2-2202892 | Proposal 2: A general capability for the support of Concurrent measurement gaps feature is introduced based on RAN4 feature list. |
|  | Xiaomi, R2-2203065 | Proposal 1: A UE capability to indicate that the UE supports concurrent gap can be introduced. Proposal 2: RAN2 can discussed whether the following UE capabilities can be introduce to indicate the simultaneous MG configuration supported by UE capable of per-FR gap and concurrent gap. Proposal 3: Send an LS to RAN4 on UE capabilities for concurrent gap. |
|  | E///, R2-2203450 | Proposal 2 Introduce a general UE capability to indicate support of Rel-17’s concurrent measurement gaps. |

In current RAN4 feature list, there is 1 bit UE capability to support concurrent measurement gap. Do you agree to introduce 1 bit UE capability to support concurrent gap? Please indicate any addition requirement in comments.

Q2: Do you agree to introduce 1 bit UE capability to support concurrent gap?

| Company | Agree/ disagree | Comments |
| --- | --- | --- |
| MediaTek | Agree |  |

Summary:

## UE capability for pre-configured measurement gap

|  | Source | Related proposals |
| --- | --- | --- |
| One general capability for pre-configured gap | E///, R2-2203450 | Proposal 1 As done for Rel-16’s ‘inter-frequency no gap’, introduce a new (single) UE capability to indicate whether the UE can perform measurements according to Rel-17’s pre-configured MG. |
| Separate capability for each activation/ deactivation: - One for UE autonomous - One for network controlled | Vivo, R2-2202324 | Proposal 1: Introduce separate UE capabilities of network-controlled and UE autonomous activation and deactivation mechanism for pre-configured measurement gap. |
|  | Intel, R2-2202462 | Introduce separate capability for each activation/ deactivation or further wait for RAN4 input. |
|  | MT, R2-2202879 | Proposal 1: Introduce two optional per UE capability for pre-configured MG. One for pre-configured MG with NW-Controlled activation/deactivation mechanism. The other for pre-configured MG with UE autonomous activation/deactivation mechanism. |
|  | HW, R2-2202892 | Proposal 1: Separate UE capabilities for different pre-MG activation/deactivation mechanisms (i.e. NW-Controlled activation/deactivation mechanism and UE autonomous activation/deactivation mechanism) are introduced based on RAN4 feature list. |
|  | Xiaomi, R2-2203065 | Proposal 4: Two capabilities for UE autonomous and NW-Controlled pre-configured MG activation/deactivation mechanism can be introduced. Proposal 5: The UE capability to indicate that the UE supports pre-configured gap is not needed. Proposal 6: Send an LS to RAN4 on UE capabilities for pre-configured gap. |

Base on the RAN4 latest agreement to add to feature list and companies input, do you agree to introduce separate UE capability for pre-configured measurement gap as follow:

- Pre-configured measurement gap with network-controlled activation and deactivation mechanism

- Pre-configured measurement gap with UE autonomous activation and deactivation mechanism

Q3: Do you agree to introduce separate UE capability for pre-configured measurement gap as follow:

- Pre-configured measurement gap with network-controlled activation and deactivation mechanism

- Pre-configured measurement gap with UE autonomous activation and deactivation mechanism

| Company | Agree/ disagree | Comments |
| --- | --- | --- |
| MediaTek | Agree |  |

Summary:

# Conclusion

TBD

# References

[1] R2-2202324, Discussion on capability for MG enhancement, vivo

[2] R2-2202462, UE capability for NR and MR-DC measurement gap enhancements, Intel

[3] R2-2202463, UE capability for NR and MR-DC measurement gap enhancements, Intel

[4] R2-2202879, Discussion on UE capabilities of MGE, MediaTek Inc.

[5] R2-2202892, Discussion on UE capability for MGE, Huawei, HiSilicon

[6] R2-2203065, Discussion on UE capabilities for gap enhancement, Xiaomi Communications

[7] R2-2203450, UE capabilities for MGE, Ericsson

[8] R2-2203522, [Pre117-e][020][MGE] AI summary of 8.22.4 UE capabilities (Intel), Intel

[9] R2-2202462, CR 38.306, UE capability for NR and MR-DC measurement gap enhancements, Intel

[10] R2-2202563, CR 38.331, UE capability for NR and MR-DC measurement gap enhancements, Intel