3GPP TSG-RAN WG2 Meeting #117-e 													_R2-2203934

Online, 21st February–3rd March, 2022

Agenda Item:	8.4.3.2

Source: 	Huawei, HiSilicon

Title:	Report of [AT117-e][021][eIAB] BAP

Document for:	Discussion and Decision

# Introduction

This paper aims at capturing the summary of email discussion.

- [AT117-e][021][eIAB] BAP (Huawei)

Scope: Based on R2-2203527, progress remaining proposals. Treat also R2-2202373. Determine agreeable parts, points for discussion if needed, open issues if needed. Aim for offline agreement, if not possible then pave the way for efficient on-line. This discussion will continue as post meeting discussion for BAP CR, and updated BAP CR (taking into acc this meetings agreements) can also be reviewed as part of this discussion.

Intended outcome: Report (assume that CR revision is not needed for CB).

Deadline: In time for on-line CB W2 Wednesday

- Type-2/3 indication MAY be propagated, if the situation in the node doing the propagation is such that all BAP links are affected by the condition (e.g. single connected) (additional decision if to propagate or not can be left for implementation). - Type-2/3 indication is not propagated if the situation in the node doing the propagation is such that some BAP links are un-affected by the condition (e.g. dual connected). - For the 2 above agreements, no stage-3 impact is foreseen. - For Type-2/3 indication in any case there is no routing information included. - The Rel-16 term “BH RLF indication” is used for type-4 indication in Rel-17. - We go with Option c (if we find that some config is needed we include also Option b), where Option c = Rewriting mapping for inter-donor-DU re-routing is based on the BAP routing IDs included in the routing entries configured for each parent, and Option b = Rewriting mapping for inter-donor-DU re-routing is based on a default egress BAP routing ID(s) configured for each parent link.

Contact information

| Company | Name (email) |
| --- | --- |
| Qualcomm | ghampel@qti.qualcomm.com |
| Lenovo | zhuoyb1@lenovo.com |
| Kyocera | Masato Fujishiro (masato.fujishiro.fj@kyocera.jp) |
| LG Electronics | Gyeong-Cheol LEE (gyeongcheol.lee@lge.com) |
| Fujitsu | Sue Yi (yisu@fujitsu.com) |
| Samsung | Milos Tesanovic (m.tesanovic@samsung.com) |
| Apple | Ralf Rossbach (rrossbach@apple.com) |
| ZTE | Lin Chen(chen.lin@zte.com.cn) |
| vivo | Boubacar Kimba kimba@vivo.com |
| Intel | Ziyi Li (ziyi.li@intel.com) |
| NEC | Sidong Li(lisidong@labs.nec.cn) |
| Nokia | Esa Malkamäki (esa.malkamaki@nokia-bell-labs.com) |

# Discussion

- 2.1	Option c & b

For the configuration of header rewriting mappings for UL inter-donor-DU re-routing, RAN2 to discuss:

Option b: Rewriting mapping for inter-donor-DU re-routing is based on a default egress BAP routing ID(s) configured for each parent link.

Option c: Rewriting mapping for inter-donor-DU re-routing is based on the BAP routing IDs included in the routing entries configured for each parent.

With the agreement below

- We go with Option c (if we find that some config is needed we include also Option b), where Option c = Rewriting mapping for inter-donor-DU re-routing is based on the BAP routing IDs included in the routing entries configured for each parent, and Option b = Rewriting mapping for inter-donor-DU re-routing is based on a default egress BAP routing ID(s) configured for each parent link.

Rapporteur interprets the option C as below:

Upon inter-donor-DU re-routing is triggered, IAB-node with dual parent nodes checks the alternative path: - If the alternative egress link is available, and - If the BH Routing Configuration, associated with the topology of the alternative egress link, has at least one entry, whose Next Hop BAP Address corresponds to this alternative egress link: - Select this egress link and replace the BAP header with BAP routing ID as the one in the entry. - else (no matched entry found), not allow to perform the BAP header rewriting based re-routing. NOTE: it is implementation to select any entry if there are multiple entries matched.

Question1: Do you confirm the above understanding on option C? [This is only to achieve the common understanding on option C to implement the running CR. If the rapporteur’s understanding is generally correct, we will not need any further agreement.]

| Companies | Yes or No | Comments |
| --- | --- | --- |
| Qualcomm | Yes |  |
| Lenovo | Yes |  |
| Kyocera | Yes, but… | On the first sentence, we just wonder how (i.e., what condition) the IAB-node triggers inter-donor-DU re-routing, and we assume Option C is processed after Rel-16 local rerouting fails. Also, on the first sentence as well, we assume “the alternative path” is equal to “the alternative link” in the second sentence. On the fifth sentence, we assume “the BAP header rewriting based re-routing” only means inter-donor-DU re-routing. Additionally, we’re wondering how to prevent processing this procedure for the downstream packets. [Rapp]: - Yes, we assume R16 local rerouting failure will result in this inter-donor-DU re-routing checking. - Yes, alternative path means alternative link. 3) Thanks. My plan is to limit the operation to IAB-MT, and it will be discussed in the running CR. |
| LGE | Yes |  |
| Fujitsu | Yes |  |
| Samsung | Yes |  |
| Ericsson | Yes |  |
| Apple | Yes |  |
| ZTE | Yes |  |
| vivo | Yes |  |
| Intel | Yes | We think Q1 has some dependency on Q2. If Q2 is agreed (i.e. default UL BH RLC channel can be used for re-routing if no entry is found), some updates is needed in ‘else’. |
| NEC | Yes |  |
| Nokia | Yes, but… | We agree with the principle but this assumes multiple BH Routing configurations associated with multiple topologies. We propose to reuse R16 BH Routing Configuration. Then “associated with the topology of the alternative egress link” is not needed. |

Summary:

It seems companies share the same view. So, no proposal is needed.

One issue on option c is that, in case CU wants all the traffics originally to be routed to SCG, which means there is no routing entry needed associated with MCG in the BH Routing Configuration. Then, in order to support the inter-donor-DU re-routing, it seems CU has to configure one invalid routing entry with “BAP routing IDx=> MCG”. And this “BAP routing IDx” should not be same with any BAP routing ID in header of the traffic originally to be routed to SCG. In this case, this invalid “BAP routing IDx=> MCG” is only for re-routing purpose, and can be considered as the option B default configuration.

Another issue on option c is that: there may be multiple routing entries matched when performing re-routing, which means the new routing ID can be selected by IAB-node implementation from any of those. In that case, CU cannot anticipate/control the target donor-DU to be routed, since different routing ID goes to different target donor-DU. Then, CU needs to ensure the tunnels between donor-DUs are all established for each pair of source and target donor-DU, to avoid the source IP filter issue. There seems no way to disable the inter-donor-DU re-routing from CU (e.g. even if there is no tunnel established), if there are traffics originally forward to both MCG and SCG.

Question2: Do you think there is any need to also support option b? (Or provide comment if you see any critical issue with option C, or any need to re-consider the agreement to option b)

| Companies | Yes or No | Comments |
| --- | --- | --- |
| Qualcomm | No | We agreed to support option C. We did NOT agree to support option B or A. Obviously, B allows limiting re-routing to a single donor-DU. Obviously, option A allows selective re-routing for each ingress BAP routing ID to a different donor-DU. These trade-offs are well known. There is no need for re-discussion. On the points by the rapporteur: - If the CU wants to ensure that re-routing can be supported on a parent link, it needs to configure at least one routing entry for this parent link. Obviously, it selects a path ID that is not used anywhere else. This is business as usual. - Inter-donor-DU re-routing should be configurable. This allows the CU to turn it on or off based on the support of this functionality on the wireline network. [Rapp]: The question is how to achieve “Inter-donor-DU re-routing should be configurable”. If we don’t revert back to option b, we need to introduce new RRC signaling to disable the whole inter-donor-DU re-routing. |
| Lenovo | Yes | To support option c, we need to guarantee that tunnels have been setup among all the source-donor-DUs and target-donor-DUs, if not, the UL traffic may be rerouted to a wrong target-donor-DU and lead to failure of UL transmission. With option b (or a), the rerouting configuration is implemented by CU, and it can limit the rerouting path to some specific target-donor-DUs which have setup tunnel with source-donor-DUs. In this case, CU can enable the inter-donor-DU re-routing only when some of the target-donor-DUs have tunnels with source-donor-DUs. |
| Kyocera | Yes | According to the rapporteur’s analysis, we think the default Routing ID, i.e., Option B, can make the network configuration simple, e.g., by less number of tunnels, in case e.g., the donor-CU has more than two donor-DUs. We assume if the default Routing ID is configured, the IAB-node always selects it for inter-donor-DU re-routing. |
| LGE | No | Same view as QC |
| Fujitsu | No | In option c, CU implementation can configure routing IDs that go to the correct target donor-DU that has tunnel to the source donor-DU. In this sense, option b is no better than option c. CU configuration can also disable inter-donor-DU rerouting, e.g., by removing relevant routing IDs in the routing entries. [Rapp]: The scenario is that the descendant node traffics are originally supposed to be routed via both MCG and SCG. In this scenario, how to disable the inter-donor-DU rerouting if CU is not ready for the tunnels between donor-DUs? It means CU has to enable the inter-donor-DU re-routing if it wants to support the scenario of routing traffic via both MCG and SCG. |
| Samsung | Yes | Agree with issues pertaining to Option c as detailed by the rapporteur above. Both issues can be fixed by falling back on Option b. In fact, Option c seems to create a lot of work for RAN2, and the only benefit that we can see is better QoS management, which we think is irrelevant for this use-case of essentially irregular/infrequent operation. Please note that, in Rel-16, we already configure the default BAP routing ID for non-UP traffic, and this can be reused in Rel-17 for non-UP traffic. So, the only step needed to support Option c is to define a default BAP routing ID for UP traffic. In particular, the relevant RRC message can additionally include the default BAP routing ID for UP traffic. Another thing to consider is whether we also need to configure the default BH RLC CH for re-routing: - Option 1: If RAN2 agree to have default BH RLC CH, the Rel-16 design can be reused for non-UP traffic, and Rel-17 only needs to include the default BH RLC CH for UP traffic. - Option 2: If RAN2 do not agree to have default BH RLC CH for re-routing, then the boundary node can select it by implementation. We slightly prefer to Option 1, which can control well the BAP path of the re-routed traffic. However, Option 2 is also fine for us. [Rapp]: It seems “no agreement” means option 2, as in R16 case when default BH RLC is not configured. We are not sure about the Qualcomm claim that we did not agree to support Option b – agreement is pretty clear that we should discuss Option b further, which is what we are doing here. |
| Ericsson | No | Same view as QC. It is just enough to focus on solution C, since the NW implementation can make sure that the inter-donor DU routing works by proper routing configuration. [Rapp]: “inter-donor DU routing works” is optionally supported from NW side, because it requires the tunnels setup between source and target donor-DUs. This is definitely implementation, on which we should have no restriction. |
| Apple |  | No strong view. Ok with a fall back to option b under certain conditions, but only to mitigate the issues in option c. Otherwise RAN2 should stay with option c. |
| ZTE | No | We think it is possible for the CU to configure at least one routing entry in SCG/MCG for inter-donor DU re-routing purpose if there is tunnel between donor DUs. For example, CU may acquire the tunnel info between donor DUs from OAM, then CU configure the routing entry which may be used for the potential inter-donor DU re-routing even if there is no real traffic go through with this BAP routing ID. It can be up to CU’s implementation. |
| vivo | No | Let’s go with Option c. Agree with the comments from QC. With Option c, the network can ensure the desired performance by proper tunnel configuration for each candidate entry. |
| Intel | Yes with comment | First, we think some clarification is needed for the above scenario. We think IAB-donor CU still has the control of which target donor-DU to be routed based on Routing Configuration for the normal routing case. It is only when one BH link is unavailable and local re-routing happens, the tunnel between two donor DUs is required to be established. However, due to local re-routing is not controlled by IAB-donor CU, the IAB-donor CU is not aware of when the boundary IAB-node performs local re-routing and require a tunnel to be established for re-routed traffic. It should be configurable by IAB-donor CU whether inter-donor re-routing is supported or not at the boundary IAB-node based on the corresponding UE capability. The routing entries towards another topology/destination can be considered for inter-donor re-routing if it’s configured by IAB-donor CU, e.g. via RRC signaling. However, in Rel-16, defaultUL-BH-RLC-Channel is used for routing when Routing Configuration is not configured by F1AP. For dual-connected IAB, two defaultUL-BH-RLC-Channel can be configured for MCG and SCG, respectively. Therefore, when there’s no available entry configured in Routing Configuration for the available egress link (e.g. all traffic are routed to one of the CGs), defaultUL-BH-RLC-Channel of the alternative CG is used for re-routing (if configured). That is, if option c cannot find re-routing path, fall back to option b if re-routing is configured. By supporting option b, it still requires IAB-donor CU to configure whether BAP-header based re-routing is supported or not to the boundary IAB-node via RRC, as the tunnel for default UL channels also doesn’t require always being established for re-routing. |
| Nokia | No | Same view as QC. We can make the inter-donor-DU re-routing configurable. |

Summary:

Yes, needed: Lenovo, Kyocera, Samsung, Intel, Huawei

No issue: Qualcomm, LGE, Fujitsu, Ericsson, ZTE, vivo, Nokia

There seems enough/some sympathy to support that:

- CU should be able to disable the whole inter-donor-DU re-routing;

- The tunnels need to be setup among the source-donor-DUs and target-donor-DUs, to which the data may be re-routed based on BH Routing Configuration.

- In case the descendant node traffics are originally supposed to be routed via both MCG and SCG (i.e. the routing entries have to be configured for both MCG and SCG), CU cannot disable the inter-donor-DU re-routing, even if the tunnels are not setup.

To address the above, we have:

Atl.1: Revert back to option b: Rewriting mapping for inter-donor-DU re-routing is based on a default egress BAP routing ID(s) configured for each parent link.

Alt2: Add new RRC signaling to directly disable the inter-donor-DU re-routing.

Proposal 1: [Online] RAN2 to discuss below options: to allow CU to disable the inter-donor-DU re-routing, e.g. in case the descendant node traffics are originally supposed to be routed via both MCG and SCG.

Atl.1: Revert back to option b: Rewriting mapping for inter-donor-DU re-routing is based on a default egress BAP routing ID(s) configured for each parent link.

Alt2: Add new RRC signaling to directly disable the inter-donor-DU re-routing.

- 2.2 BAP#3: inter-topology routing indicator

For inter-topology routing, the header rewriting configuration to include information that allows the boundary node to determine either the egress topology, or the ingress topology, or the traffic direction of a header-rewriting entry (selection of one of these expected):

With option C agreed, the header rewriting configuration is only used for inter-topology routing. For “inter-topology routing”, it only includes the 2 cases “CU1 to CU2” and “CU2 to CU1”. So, all those 3 options are same.

Option 1: egress topology

Option 2: ingress topology

Option 3: traffic direction

Question3: which option do you prefer? [NOTE, rapporteur suggests to directly go with the majority option, since there is no significant difference.]

| Companies | Option | Comments |
| --- | --- | --- |
| Qualcomm |  | This is a ST3 detail we can leave to RAN3. |
| Lenovo |  | Option 1 is preferred, but this can be left to RAN3. |
| Kyocera | Option 3 | We’re fine to go with majority, while we still assume Option 3 is more robust and future proofing. |
| LGE | Option 2 |  |
| Fujitsu |  | Leave it to RAN3. |
| Samsung | Option 2 | We would like to use the ingress topology. The reason is that the header writing operation at BAP layer is: - BAP RX part determines to send the packet to BAP TX part or upper layer, and at RX part, it can only know the ingress topology. - BAP TX part performs the rewriting based on the ingress topology indicated by the RX part. |
| Ericsson | Option 1 | Option 1 is preferred, but it can be left also to RAN3 to decide it. |
| Apple |  | Same view as QC |
| ZTE | Option 1 | Since both UL mapping configuration and routing configuration use egress topology indicator, it is suggested that the egress topology indicator is used for header rewriting as well. We are also fine to leave it to RAN3. |
| vivo | Option 2 | Either Option 1 or 2 is fine. OK to leave it for RAN3. |
| Intel | Option 3 | Stage 3 on how to capture traffic direction can be left to RAN3, however, we need to explain that traffic direction means both ingress topology and egress link topology is included in header rewriting configuration. |
| NEC | Option 1/2 | Both 1 and 2 are fine, ok to wait for RAN3. |
| Nokia | Option 1/2 |  |

Summary:

No consensus. We can leave it to RAN3, which is under discussion.

- 2.3 BAP#2: RAN3 signaling on the “information”

The RAN3 signalling on how to include/configure the “information” in below:

- The BH RLC CH mapping configuration of the boundary node includes information for the boundary node to differentiate mappings based on ingress topology and egress topology.

- The UL mapping configuration to include information for the boundary node to determine the egress topology of each UL mapping entry.

- The routing configuration to include information that allows the boundary node to determine the topology each routing entry applies to. RAN3 to decide on St3-related aspects.

QC (R2-2202330): Proposal 6: BAP#2 to be handled by RAN3’s ST3 signaling design. Lenovo (R2- 2202583): The issues for BAP#02 can be directly resolved by RAN3 via F1AP signaling design. LG (R2-2203054): Proposal 2. For RAN3 related signalling/configuration, wait for further RAN3 agreements. Samsung (R2-2203105): Proposal 2: The details of boundary node configuration which implement the relevant existing RAN2 agreements are left to RAN3. Nokia (R2-2203402): [Assuming option 5 in BAP#1 somehow change the previous agreement] - Proposal 6: (BAP#02) The BH RLC CH mapping configuration of the boundary node need not include information on ingress or egress topology, i.e., reuse Rel16. - Proposal 7: (BAP#02) UL mapping configuration need not include information on egress topology, i.e., reuse Rel16. - Proposal 8: (BAP#02) Routing configuration to indicate whether an egress link (Next Hop node) belongs to non-F1-terminating donor’s topology. Summary: It seems majority are fine to leave the details to RAN3. Proposal 5: RAN2 leave the signalling details to RAN3 on BAP#2.

Question4: Do you confirm that RAN2 leave the signalling details to RAN3 on BAP#2.

| Companies | Option | Comments |
| --- | --- | --- |
| Qualcomm |  | Yes. |
| Lenovo |  | Yes |
| Kyocera |  | Yes |
| LGE |  | Yes. |
| Fujitsu |  | Yes |
| Samsung |  | Yes |
| Ericsson |  | Yes |
| Apple |  | Yes |
| ZTE |  | Yes |
| vivo |  | Yes |
| Intel |  | Yes |
| NEC |  | Yes |
| Nokia |  | Yes, once RAN2 has agreed the details of Routing ID mapping, routing and BH RLC channel mapping. |

Summary:

RAN3 agreed the FFS point of the above issues in their session

3-1: To enable the boundary node determine the topology each routing entry/UL mapping config applies to, a non-F1-terminating topology indicator is included only if the configuration applies to the non-F1-terminating CU’s topology, the absent of this indicator indicates the configuration applies to the F1-terminating CU’s topology.

3-3: A non-F1 terminating topology indicator is introduced in BAP layer BH RLC channel mapping Information List IE for the boundary node’s BH RLC CH mapping configuration, to indicate if the ingress topology (of the prior-hop node) and/or the egress topology (of the next-hop node) is the non-F1-terminating CU’s topology.

5-1: For each BAP header rewriting configuration, include an egress topology indicator to indicate the traffic direction.

Together with Q3, we have the below proposal:

Proposal 2: [Easy] RAN2 leave the signalling details to RAN3 on open issue BAP#2 and #3.

- 2.4 R2-2202373 on BAP#5

For the agreed local re-routing triggered by flow control feedback, rapporteur understand there are two candidate granularity of local re-routing. The first one is routing ID level re-routing, which has been captured and endorsed in the running CR. The second one is BH RLC level re-routing, which is still FFS. Considering the limited time, it is proposed to not support the BH RLC channel level re-routing.

Proposal 1: [BAP#5] The granularity of flow control feedback triggered local re-routing is only per routing ID level, but no per BH RLC level.

Question5: Do you agree the above proposal?

| Companies | Yes or No | Comments |
| --- | --- | --- |
| Qualcomm | Disagree | We can easily re-routing for BH RLC-level congestion: In section 5.2.1.1 General, replace: - perform routing to determine the egress link in accordance with clause 5.2.1.3; - determine the egress BH RLC channel in accordance with clause 5.2.1.4; with: - perform routing to determine the egress link and selection of the egress BH RLC channel in accordance with clause 5.2.1.3; - In section 5.2.1.3 Routing, add at the end: - After selection of an egress link, determine the egress BH RLC channel in accordance with clause 5.2.1.4; Move NOTEs 1, 2, and 3 to the end and modify Note 3: NOTE 3: An egress link may be not considered to be available for a BAP routing ID, if it is determined as congested for this BAP routing ID or for the BH RLC Channel (derived from clause 5.2.1.4) based on the received flow control feedback, as defined in sub-clause 5.3.1. - Finally, in 5.3.1.y Receiving operation, modify: For a link, the BAP entity at the IAB-DU or IAB-donor-DU may: - if the available buffer size as indicated by the received BAP Control PDU for flow control feedback per BAP routing ID is less than the [congestedThreshold-r17], if configured: - consider the BH link as congested for this BAP routing ID (for rerouting purpose defined in accordance with clause 5.2.1.3). - if the available buffer size as indicated by the received BAP Control PDU for flow control feedback per BH RLC Channel is less than the [congestedThreshold-r17], if configured: - consider the BH link as congested for this BAP routing ID (for rerouting purpose defined in accordance with clause 5.2.1.3). Editor’s Note: FFS if the per BH RLC channel level link congestion should also be determined for local rerouting. |
| Lenovo | Yes, but | Based on the per BH RLC CH level HbH flow control feedback, if the available buffer size for a BH RLC CH is less than the threshold, the BH RLC CH can be considered as congestion. However, other BH RLC CHs on this BH link are still available and we can select one of them to transmit the congested DL traffic. In summary, we can also support per BH RLC CH feedback to trigger packet remapped to other available BH RLC CH in the same BH link. |
| Kyocera | Yes |  |
| LGE | Yes | per BH RLC level is not needed. The BAP routing is performed based on BAP routing ID, not BH RLC channel. Considering that the current BAP routing operation is to determine the next egress link first and then select a BH RLC channel based on the determined egress link, if per BH RLC channel granularity is applied, this would increase complexity of local re-routing operation because the IAB node may need to perform local re-routing operation after determining the BH RLC channel and then again to select the BH RLC channel on the re-routed link. |
| Fujitsu | No | We have two levels of flow control feedback, so why not have two levels of local re-routing? There is no critical issue about supporting BH RLC CH level local re-routing. We can work it out with running CR. |
| Samsung | Yes |  |
| Ericsson | Yes | It can beneficial to consider the BH RLC channel as well, but given the limited time left, we are ok to only focus on the BAP routing ID. |
| Apple | No | We would prefer to consider local rerouting per BH RLC channel in Rel-17, it can be worked out in stage 3. |
| ZTE | No | We think it is better to support both per BAP routing ID and per BH RLC channel level local-rerouting triggered by the flow control feedbacks. |
| vivo | No | As per BH RLC CH feedback is already in R16 and local rerouting based on HbH flow control feedback was agreed in R17, it is better also support BH RLC CH based local rerouting is R17. |
| Intel | No | We think there’s no difficulty to implement granularity of per BH RLC level. When receiving a BH RLC level flow control feedback, the IAB-node is aware of which mapped routing ID is not available by implementation. Thus, it can still be re-routed during routing phase. |
| NEC | No | We have two level flow control feedback, and we see no difficult to support per RLC channel level flow control feedback. Thus, why we only support one of the two flow control feedback triggered local re-routing. |
| Nokia | Yes | Qualcomm text proposal has this undefined reference in the text proposal: - if the available buffer size as indicated by the received BAP Control PDU for flow control feedback per BH RLC Channel is less than the [congestedThreshold-r17], if configured: - consider the BH link as congested for this BAP routing ID (for rerouting purpose defined in accordance with clause 5.2.1.3). To Lenovo: we disagree that RLC CH remapping in the same link counts as re-routing. We think re-routing implies selection of another link. Given the above, there seems to be too many open issues to close within the limited time left. |

Summary:

Support BH RLC level: Qualcomm, Fujitsu, Apple, ZTE, vivo, Intel, NEC, Nokia

No: Lenovo, Kyocera, LGE, Samsung, Ericsson, Huawei

There is no clear majority view on this.

The concern from rapporteur to support this per BH RLC level local re-routing is that it will change the modeling of “routing first and then BH RLC determination”.

Routing try first time to select BH link->BH RLC determination->check if the select BH RLC is congested on the selected BH link->go back routing function to local re-route-> BH RLC determination.

It seems some companies are fine with the QC’ solution/TP to implement the BH RLC level re-routing. Rapporteur understand the solution may not work, because the bearer mapping is configured at intermediate node as ingress BH RLC to egress BH RLC mapping. It means the BH RLC is not associated with some routing ID at intermediate node, rather it is associated with both “routing ID+ ingress BH RLC”.

Proposal 3a: [Easy] For the flow control feedback triggered local re-routing, the re-routing is only performed on routing IDs level.

Proposal 3b: [Online] FFS on whether this can also be triggered by BH RLC level flow control feedback (i.e. the BH link of a routing ID is not considered to be available, if its intended BH RLC is congested).

- 2.4 R2-2202373 on BAP#9

In R16, the type4 trigger condition is captured in BAP specification, rather than RRC, using general description.

“When a BH RLF recovery failure is detected at the IAB-MT, for each egress link associated with the IAB-DU, the transmitting part of the collocated BAP entity at the IAB-DU may:”

Since the IAB-DU operation is more like the NW side behaviors (we used ‘may’ in R16), it is not necessary to capture too much accurate trigger conditions also for type2/3 in BAP specification.

Proposal 6: [BAP#9] As in R16, the trigger conditions for type 2/3 will be captured in BAP spec. rather than in RRC spec., with just some general descriptions.

Question6: Do you agree the above proposal?

| Companies | Yes or No | Comments |
| --- | --- | --- |
| Qualcomm | Yes |  |
| Lenovo | Yes |  |
| Kyocera | Yes | If some clarification for “recovery” (or “recovered”) is needed, we think it can be captured in Stage-2 specification. |
| LGE | Yes |  |
| Fujitsu | Yes |  |
| Samsung | Yes but… | With regards to Type-2/3 indication propagation, we agreed not to capture anything in stage-3 specs and this includes the BAP spec. So we assume the rapporteur is only referring here to capturing in the BAP spec descriptions of Type-2/3 indications, and triggering conditions on a high level. [Rapp]: Yes, propagation is another story. |
| Ericsson | Yes | Agree with Samsung observation, i.e. it should be clarified that this does not apply to propagation |
| Apple | Yes |  |
| ZTE | Yes |  |
| vivo | Yes |  |
| Intel | Yes |  |
| NEC | Yes |  |
| Nokia | Yes, and | Given the agreement on stage-3 impact, the agreements regarding type-2/3 propagation need to be captured in stage 2. |

Summary:

There is clear majority to support the proposal.

Proposal 4: [Easy] As in R16, the trigger conditions (not the propagation) for type 2/3 will be captured in BAP spec. rather than in RRC spec., with just some general descriptions.

# Conclusion and proposals

Based on the above summary, following proposals are given

Proposal 1: [Online] RAN2 to discuss below options: to allow CU to disable the inter-donor-DU re-routing, e.g. in case the descendant node traffics are originally supposed to be routed via both MCG and SCG.

Atl.1: Revert back to option b: Rewriting mapping for inter-donor-DU re-routing is based on a default egress BAP routing ID(s) configured for each parent link.

Alt2: Add new RRC signaling to directly disable the inter-donor-DU re-routing.

Proposal 2: [Easy] RAN2 leave the signalling details to RAN3 on open issue BAP#2 and #3.

Proposal 3a: [Easy] For the flow control feedback triggered local re-routing, the re-routing is only performed on routing IDs level.

Proposal 3b: [Online] FFS on whether this can also be triggered by BH RLC level flow control feedback (i.e. the BH link of a routing ID is not considered to be available, if its intended BH RLC is congested).

Proposal 4: [Easy] As in R16, the trigger conditions (not the propagation) for type 2/3 will be captured in BAP spec. rather than in RRC spec., with just some general descriptions.

# Reference

- R2-2202255	BAP re-writing mapping confirguration	NEC

- R2-2202330	Remaining BAP issues for eIAB	Qualcomm Incorporated

- R2-2202346	Discussion on the BAP open issues	Fujitsu

- R2-2202374	BAP open issues on option a to d and issues BAP#1, #3 and #4	Huawei, HiSilicon

- R2-2202382	Further considerations on local re-routing	ZTE, Sanechips

- R2-2202383	Discussion on re-routing and header rewriting configuration	ZTE, Sanechips

- R2-2202583	Discussion on remaining issues for BAP routing	Lenovo, Motorola Mobility

- R2-2202643	Discussion on remaining BAP open issues	Intel Corporation

- R2-2202761	Remaining issues regarding BH RLF indications	InterDigital, Inc.

- R2-2202908	BAP open issues on BAP#01, BAP#03 and BAP#04 	Kyocera

- R2-2202968	Remaining Issues of Inter-donor DU Rerouting	vivo

- R2-2202969	Remaining Issues of Inter-Topology Routing and Rerouting	vivo

- R2-2203053	Discussion on BAP re-writing mapping configurations for UL inter-donor-DU re-routing	LG Electronics Inc.

- R2-2203054	Discussion on identified BAP open issues (BAP#1, BAP#2, BAP#3, BAP#4)	LG Electronics Inc.

- R2-2203105	BAP open issues	Samsung Electronics GmbH

- R2-2203402	BAP header rewriting and inter-donor-DU re-routing	Nokia, Nokia Shanghai Bell

- R2-2203403	Simplified text proposal for BAP routing and header rewriting	Nokia, Nokia Shanghai Bell

- R2-2203469	BAP open issues	Ericsson

- R2-2203507	Header Rewriting for Inter-to-intra topology re-routing 	Futurewei Technologies