3GPP TSG-RAN WG2 Meeting #117-e		R2-220xxxx

Electronic, 21st Feb. – 3rd Mar. 2022

Source:	vivo

Title:	Summary of [Pre117-e][006][ePowSav] RLM/BFD relaxation Open Issues Input (vivo)

Agenda Item:	8.9.3.1

Document for:	Discussion and Decision

- Introduction

The document summarizes the following pre-meeting offline discussion:

[Pre117-e][006][ePowSav] BFR-BFD relaxation Open Issues Input (vivo) RLM/BFD relaxation OI 3.1: Can UE start/stop RLM/BFD relaxation by itself if it meets/fails the relaxation criteria? OI 3.2: Should UE report fulfilment or not (entry/exit) to network for RLM/BFD relaxation? OI 3.3: Should NW be able to enable/disable RLM/BFD relaxation with explicit indication irrespective if the RLM/BFD relaxation criteria is configured or not?”

In order for rapporteur to have sufficient time to provide the summary, your comments before Thursday 2022-02-13 1200 UTC is appreciated.

- Contact information

| Company | Name and email address |
| --- | --- |
| vivo | Chenli (chenli5g@vivo.com) |

- Discussion

The open issue list for ePowSav was discussed and summarized in [1] with the following open issues on RLM/BFD relaxation:

- RLM/BFD relaxation Company input into Pre117-e-offline OI 3.1: Can UE start/stop RLM/BFD relaxation by itself if it meets/fails the relaxation criteria? OI 3.2: Should UE report fulfilment or not (entry/exit) to network for RLM/BFD relaxation? OI 3.3: Should NW be able to enable/disable RLM/BFD relaxation with explicit indication irrespective if the RLM/BFD relaxation criteria is configured or not?” Other (wait for RAN4) OI 3.4: Granularity for RLM/BFD relaxation enable/disable (e.g. per-UE/CG/Serving cell) // Rapporteur: As we clarified in offline [Post116bis-e][000] Session Reports Approval, this Open issue should be: how to enable/disable RLM relaxation per-CG, and how to enable/disable BFD relaxation per-serving cell. OI 3.5: How to provide the criteria configuration for RLM relaxation and BFD relaxation for low mobility criterion? OI 3.6: How to provide the criteria configuration for RLM relaxation and BFD relaxation for serving cell quality criterion? OI 3.7: How to evaluate the low mobility criterion for RLM/BFD relaxation? OI 3.8: How to evaluate the serving cell quality criterion for RLM/BFD relaxation?

Besides, an LS from RAN4 on RLM/BFD relaxation was received in [2] with some further RAN4 progresses.

  - Mechanism for RLM/BFD relaxation

Regarding OI 3.1: Can UE start/stop RLM/BFD relaxation by itself if it meets/fails the relaxation criteria. This issue was discussed in RAN2#116bis-e meeting [3], the discussion was summarized as below:

Companies are invited to show your preference among the following options on how to perform RLM/BFD relaxation: - Option 1: UE can start/stop RLM/BFD relaxation by itself if it meets/fails the relaxation criteria. Note: UE reporting will be discussed later as separate issue. - Option 2: Whether UE performs RLM/BFD relaxation is based on network indication. FFS irrespective if the RLM/BFD relaxation criteria is configured or if only apply to RLM/BFD relaxation criteria is not configured. - Option 3: Wait for RAN4 progress - Option 4: Others, please specify Summary: 19 companies provided views on whether the criteria configuration for RLM relaxation and BFD relaxation are configured separately. 13 companies support option 1: UE can start/stop RLM/BFD relaxation by itself if it meets/fails the relaxation criteria. Companies think presence of relaxation criteria is sufficient that network allows UE to perform relaxation when relaxation criteria are met, introducing indication on relaxation will increase the signaling overhead. Besides, some proponents for option 1 think network configures the UE with RLM/BFD relaxation criteria is some form of network control, hence the explicit network indication isn’t needed. 3 companies prefer option 3: wait for RAN4 progress. 3 companies support option 2: Whether UE performs RLM/BFD relaxation is based on network indication. 2 companies think UE enters the RLM/BFD relaxation based on network indication while UE can stop RLM/BFD relaxation without further network control. It seems that most companies support or can accept option 1, rapporteur suggests to follow the clear majority, i.e., UE can start/stop RLM/BFD relaxation by itself if it meets/fails the relaxation criteria. Regarding the suggestion to wait for RAN4 progress, rapporteur suggests we could make the assumption from RAN2 point of view, and further comeback may happen if RAN4 has progress on this issue. Proposal 8: [To discuss][13/19] RAN2 assume UE can start/stop RLM/BFD relaxation by itself if it meets/fails the relaxation criteria. RAN2 can come back on this based on RAN4 decisions.

According to the discussion, it seems that majority companies support or can accept option 1: UE can start/stop RLM/BFD relaxation by itself if it meets/fails the relaxation criteria. In this way, rapporteur would like to check with companies whether companies’ views have changed, and whether the above option 1 (UE can start/stop RLM/BFD relaxation by itself if it meets/fails the relaxation criteria.) is acceptable. If not, please provide technique reason.

- Companies are invited to show your views on whether UE can start/stop RLM/BFD relaxation by itself if it meets/fails the relaxation criteria. RAN2 can come back on this based on RAN4 decisions.

Note: UE reporting will be discussed later as separate issue below.

- Option 1: Yes.

- Option 2: No, please provide technique reason.

- Option 3: Others, please specify.

| Company’s name | Option | Comments, if any |
| --- | --- | --- |

Regarding OI 3.2: Should UE report fulfilment or not (entry/exit) to network for RLM/BFD relaxation? This issue was also discussed in RAN2#116bis-e meeting [3], the discussion was summarized as below:

Companies are invited to show your preference among the following options on whether/how/what need to report fulfillment or not (entry/exit) to network: - Option 1: Yes, please provide what information, e.g. fulfilled or not (entry/exit) - Option 2: No, please provide your reason. - Option 3: Wait for RAN4 progress Summary: 19 companies provided views on whether/how/what need to report fulfillment or not (entry/exit) to network for RLM/BFD relaxation. 9 companies prefer option 1, i.e. UE should report fulfillment or not (entry/exit) to network. Companies think for UEs in RRC_CONNECTED, network should be aware of UE’s situation for a better implementation. Besides, most companies think entry/exit is sufficient while 1 company thinks the exit doesn’t need to be reported. 8 companies prefer option 2, i.e. such a report is NOT needed. 3 companies can accept wait for RAN4. Unfortunately, it seems companies’ views are split on this issue and it’s hard to reach consensus. Hence, rapporteur suggests to discuss this issue online: Proposal 10: [To discuss][9 vs. 8 vs. 3] FFS whether/how/what need to report fulfillment or not (entry/exit) to network for RLM/BFD relaxation.

According to the discussion, companies’ views are diverse (9 vs. 8 vs.3). As far as rapporteur knows, whether/how/what need to report for RLM/BFD relaxation is not likely to be decided in current RAN4 discussion. Companies are also invited to check with RAN4 colleagues. Thus, rapporteur thinks we could make progress from RAN2 point of view, and further comeback may happen if RAN4 has progress on this issue.

Please note that it was agreed in RAN2#116bis-e meeting:

- [056] if UE report on fulfillment or not (entry/exit) to network for RLM/BFD relaxation is agreeable, UAI is used to provide the report.

During the discussion below, it is appreciated if companies could provide more technical justifications on you reply.

- Companies are invited to show your preference among the following options on whether/what need to report fulfillment or not (entry/exit) to network:

- Option 1: Yes, please provide your reason and what information, e.g. fulfilled or not (entry/exit)

- Option 2: No, please provide your reason.

| Company’s name | Option | Comments, if any |
| --- | --- | --- |

Regarding OI 3.3: Should NW be able to enable/disable RLM/BFD relaxation with explicit indication irrespective if the RLM/BFD relaxation criteria is configured or not. This issue was not discussed in RAN2#116bis-e meeting, but this solution was mentioned in the pre-meeting summary [3]. The justification provided in company’s contribution [4] is:

UE autonomous measurement relaxation may degrade mobility performance and therefore it should be avoided. We think that the network should be able to control the RLM/BFD relaxation based on internal evaluation. As this WI is for connected mode, the network has more information relevant about the UE e.g. device capability, service type. It would be convenient and beneficial to allow the network to command the UE that it is considered from network perspective to be in low mobility state, hence is allowed to perform relaxed RLM/BFD measurements right now. How the network evaluates the relaxation potential is up to network implementation, but such explicit command/signalling can be used as another solution to enable or disable the RLM/BFD relaxation. Network can monitor regular measurement reports (periodic or event based) from the UE and take those into account when allowing the RRM measurement relaxations. In our views, RRC signalling shall be used for the explicit relaxation command. If such explicit relaxation command is agreed, the question comes if and how the explicit command can be used while the UE is meanwhile configured with some relaxation criteria. In our view, the explicit relaxation command can be used irrespective of the relaxation criteria is configured or not. It should override the evaluation result at the UE side of the relaxation criteria if there is any inconsistence between them.

During the discussion below, it is appreciated if companies could provide more technical justifications on you reply.

- Companies are invited to show your preference among the following options on whether NW should be able to enable/disable RLM/BFD relaxation with explicit indication irrespective if the RLM/BFD relaxation criteria is configured or not:

- Option 1: Yes, please provide your reason.

- Option 2: No, please provide your reason.

| Company’s name | Option | Comments, if any |
| --- | --- | --- |

- Conclusion

This contribution summarizes the pre-meeting offline discussion: [Pre117-e][006][ePowSav] BFR-BFD relaxation Open Issues Input (vivo), and achieves the following proposals:

Aiming to help with the meeting discussion/progress, the proposals are categorized starting with:

- [To agree] when there is large support and hence proposed for easy agreement.

- [To discuss] when there is substantial level of support and agreement may be possible.

- [FFS] when there is low support or companies propose new solutions or options to possibly consider further e.g. if there is sufficient support (understanding that these topic have not been discussed by all companies when providing their views in the different discussion points).

The proposals also start with a number: for the format [x], ‘x’ represents the number of supportive companies (i.e. these solutions are marked as FFS as the proposed solutions were not discussed by all companies) and, for the format [x/y], ‘x’ represents the number of supportive companies, and (y-x) the number of companies with different view.

The following list shows the proposals above organized based on the suggested priority aiming to help during its meeting discussion:

Proposals for easy agreement

Proposals have chance for agreement (need online):

Proposals need further discussion:

- Reference

- R2-2201785, Summary of [Post116bis-e][080][ePowSav] Open Issues, MTK

- R4-2202769, LS on signalings for enabling RLM and BFD relaxation in R17 UE power saving, RAN4/vivo

- R2-2201941, Summary of offline#056 on RLM BFD relaxation, vivo

- R2-2201684, Summary of 8.9.2.3 RLM BFD relaxation, vivo

- R2-2201614, On RLM/BFD relaxation, Nokia, Nokia Shanghai Bell