3GPP TSG-RAN WG2 Meeting #117-e		R2-2202317

Electronic, 21st Feb. – 3rd Mar. 2022

Source:	vivo

Title:	Report of [Pre117-e][106][RedCap] MAC open issues (vivo)

Agenda Item:	8.12.3.1.1

Document for:	Discussion and Decision

# Introduction

The document summarizes the following pre-meeting offline discussion:

[Pre117-e][106][RedCap] MAC open issues (vivo) Regarding the MAC mode open issues listed in R2-2201891: - Issues "1-3", "2-3" and "2-4" will be handled in offline discussion [Pre117-e][106][RedCap] MAC open issues - Issue "1-1" can be discussed as part of the RACH indication and partitioning discussion (AI 8.18) - Issue "1-2" will be handled in offline discussion [Pre117-e][107][RedCap] - Issues "2-1" and "2-2" can be handled via company contributions in AI 8.12.4 Other MAC issues can be handled via company contributions in AI 8.12.3.1.2

In order for rapporteur to have sufficient time to provide the summary, your comments before Monday 2022-2-14 23:59 UTC is appreciated.

- Contact information

| Company | Name and email address |
| --- | --- |
| vivo | Chenli (chenli5g@vivo.com) |
| Huawei, HiSilicon | Yulong (shiyulong5@huawei.com) |
| Samsung | Jaehyuk JANG (jack.jang@samsung.com) |
| LGE | SunYoung LEE (ssunyoung.lee@lge.com) |
| Nokia, Nokia Shanghai Bell | Samuli Turtinen (samuli.turtinen@nokia.com) |
| Apple | Naveen Palle (naveen.palle@apple.com) |
| Qualcomm | Linhai He (linhaihe@qti.qualcomm.com) |
| Intel | Yi Guo (yi.guo@intel.com) |
| ZTE | LiuJing (liu.jing30@zte.com.cn) |
| CATT | Xiangdong zhang ( zhangxiangdong@catt.cn ) |
| Xiaomi | Liyanhua (liyanhua1@xiaomi.com) |
| Futurewei | Yunsong Yang (yyang1@futurewei.com) |
| CMCC | Xiaoman Liu(liuxiaoman@chinamobile.com) |
| Fujitsu | Guorong Li (liguorong@fujitsu.com) |
| NEC | Hisashi Futaki (hisashi.futaki@ nec.com) |
| OPPO | Haitao Li (lihaitao@oppo.com) |
| Lenovo&MM | Lianhai(wulh5@Lenovo.com) |
| DENSO | Hideaki Takahashi (hideaki.takahashi.j6e@jp.denso.com) |
| Spreadtrum | Lifeng Han(Lifeng.Han@unisoc.com) |
| MediaTek | Pradeep Jose (pradeep dot jose at mediatek dot com) |

# Discussion

The open issue list on MAC aspects for RedCap was discussed and summarized in [1] with the following open issues:

Regarding early identification:

| OI Index | Open issue | Rapporteur comment | OI type |
| --- | --- | --- | --- |
| 1-1 | How to apply the dedicated RACH resource for RedCap or how to define the applicability priority (e.g. whether RedCap type is prioritized) | This OI will be handled in RACH partitioning session. | Type 2 |
| 1-2 | Confirm Working assumption or not on: Working assumption: - Msg3 early identification is mandatorily supported by RedCap UE | This OI will be handled in RAN2 also considering MsgA early identification. | Type 1 |
| 1-3 | Whether LCID for RedCap is always indicated when MsgA CCCH is sent by a RedCap UE (i.e. no other precondition) | This OI will be handled in RAN2 | Type 1 |

Regarding NCD-SSB:

| OI Index | Open issue | Rapporteur comment | OI type |
| --- | --- | --- | --- |
| 2-1 | Whether/how NCD-SSB could be applied for Non-RedCap Ues | This OI will be handled by both RAN1 and RAN2 | Type 2 or Type 1? |
| 2-2 | Any other impacts on BWP operation in RRC_CONNECTED for the behavior for NCD-SSB, e.g. RRM, RLM, etc. | This OI will be handled by RAN2 | Type 2 or Type 1? |
| 2-3 | How to implement the redcap specific initial BWP in MAC, including RACH procedure and BWP operation | This OI will be handled by RAN2 | Type 3 |
| 2-4 | FFS whether to have CD-SSB/NCD-SSB concept in MAC specification |  | Type 3 |

Per Chair guidance, we only need to discuss these issues:

Issues “1-3”, “2-3” and “2-4” will be handled in offline discussion [Pre117-e][106][RedCap] MAC open issues

## Open issue for early identification

In RAN2#116bis-e, it was agreed that:

Agreements online: 1. In MAC perspective, RedCap UE uses the dedicated LCID for Msg3 early identification, when the Msg3 includes the CCCH data (no other precondition) 2. Also when msg1 early identification is configured, new dedicated LCID is used for CCCH identification

For 2-step RACH, Rapporteur thinks similar conclusions should be made accordingly.

- Companies are invited to show your views on whether LCID for RedCap is always indicated when MsgA CCCH is sent by a RedCap UE (i.e. no other precondition). Otherwise, please provide your reason.

| Company’s name | Yes/No | Comments, if any |
| --- | --- | --- |
| Huawei, HiSilicon | Yes | Same as Msg3. |
| Samsung | Yes | To add precondition would only add unnecessary complexity to the specification/implementation. |
| LGE | Yes |  |
| Nokia | Yes | No precondition needed. |
| Apple | Yes | While we were not favorable to earlier agreement, it make senses to keep 2-step RACH the same as 4-step |
| Qualcomm | Yes |  |
| Intel | Yes |  |
| ZTE | Yes |  |
| CATT | Yes | Keep consistent with 4-step RACH. |
| Xiaomi | Yes |  |
| Vivo | Yes |  |
| Futurewei | Yes |  |
| CMCC | Yes |  |
| Fujitsu | Yes |  |
| NEC | Yes | Given the previous agreement was made for Msg3, it would be reasonable to assume the same. |
| OPPO | Yes |  |
| Lenovo&MM | Yes | A common solution is preferred. |
| DENSO | Yes | Should be the same as for 4-step RACH |
| Spreadtrum | Yes | Keep align with Msg3 solution. |
| MediaTek | Yes |  |

Summary: 20 companies provided their views on this discussion point.

All companies agree that LCID for RedCap is always indicated when MsgA CCCH is sent by a RedCap UE (i.e. no other precondition). Hence, rapporteur suggests to have an easy agreement on this issue.

Proposal 1: [To agree][20/20] Dedicated LCID for RedCap is always indicated when MsgA CCCH is sent by a RedCap UE (i.e. no other precondition).

## Whether/how to capture separate initial BWP in MAC

In RAN2#116bis-e, it was agreed that:

1. If a RedCap UE in idle/inactive mode is configured with a separate initial BWP associated with no SSB (CD or NCD) for RACH, measurements are based on CD-SSB for initial RACH resource selection. 3. From RAN2 perspective, if a RedCap UE in idle/inactive mode is configured with a separate initial BWP associated with no SSB (CD or NCD) for RACH, it is up to UE implementation to perform new RSRP measurement in a DL BWP associated with CD-SSB before Msg1/A retransmission.

Rapporteur thinks this is related to RACH procedure in idle/inactive mode, and it could be captured in MAC specification as a Note. During the running MAC CR discussion, some companies provided some rewording suggestions as below [2]:

NOTE X1: If a RedCap UE in RRC_IDLE or RRC_INACTIVE mode is configured with a separate initial BWP this is not associated with any SSB for RACH, measurements are based on CD-SSB for initial RACH resource selection, while it is up to UE implementation whether to perform new RSRP measurements in the DL BWP associated with CD-SSB for Random Access Preamble retransmission.

- Companies are invited to show your preference among the following options on whether/how to capture the above conclusions:

- Option 1: Agree.

- Option 2: Not agree, please provide your suggestions.

- Option 3: No need to capture this in MAC specification.

| Company’s name | Option | Comments, if any |
| --- | --- | --- |
| Huawei, HiSilicon | Option 3 for agreement 1; FFS/Wait for agreement 3; | There is no differentiation between CD-SSB and NCD-SSB in MAC specification for RA procedure. It can be clarified in RRC that the SSB to be used is NCD-SSB. The agreement 3, “it is up to UE implementation to perform new RSRP measurement in a DL BWP associated with CD-SSB before Msg1/A retransmission”, we can wait for the R1/4 discussion and see how they capture/clarify this in their specification. |
| Samsung | Option 1 | - |
| LGE | Option 2 for Agreement 1; Option 3 for Agreement 3; | The intention of the agreement is to allow using normal initial BWP for a RedCap UE if there is no SSB on the RedCap specific initial BWP. There is no need to differentiate CD-SSB or NCD-SSB in MAC. More importantly, if we introduce CD-SSB/NCD-SSB, it becomes unclear what the legacy SSB in MAC specification means. Thus, we have a concern on having CD/NCD-SSB in MAC specification only for RedCap. The only one thing we need in MAC is that, for RedCap UE, there is a RedCap specific initial BWP. In this regards, our suggestion is: NOTE X1: If a RedCap UE is configured with an [initial BWP for RedCap] not associated with any SSB, SS-RSRP is measured for the SSB associated with initial BWP. |
| Nokia | No strong view | It would need to be captured somewhere the CD-SSB is used. |
| Apple |  | Share similar view as Nokia |
| Qualcomm | Option 1 |  |
| Intel | Option 3 for agreement 1; FFS/Wait for agreement 3; | For agreements 3, RAN2 has sent LS in R2- 2201760, asked RAN1/RAN4 to consider if any update is required in their specifications. Therefore we could wait for their discussion. |
| ZTE | Option 1 for agreement 1; Wait for agreement 3 | For agreement 1, since there is no NCD-SSB for idle/inactive UEs, SSB selection can only be performed based on CD-SSB. For agreement 3, we think the proposed wording is different from RAN2 agreement, the agreement says “it is up to UE implementation to perform new RSRP”, it cannot be simply interpreted as “up to UE implementation whether to perform…”, only using old measurement results can harm RACH performance, the RAN2 agreement can imply “up to UE implementation how to perform…”. Considering LS is sent to RAN4/1, we prefer to wait for their feedback first. |
| CATT | Option 3 for agreement 1; Wait for agreement 3; | For agreement 1, there is no other option for UE implementation. For agreement e, we can wait the feedback of our LS. |
| Xiaomi | Option 3 for agreement 1; Wait for agreement 3; | We do not need to differentiate CD-SSB or NCD-SSB in MAC. It can be clarified in RRC that the SSB is NCD-SSB from the initial BWP. |
| Vivo | Option 1 with comments | For agreement 1: If a separate initial BWP is configured for RACH, we think it is better to clarify how to perform measurement for RACH resource selection. As there is no associated SSB for the separate initial BWP, the SSB associated with initial BWP should be used. If companies donot want to mention CD-SSB/NCD-SSB in RACH procedure, we are also fine with the suggested wording provided by LG. For agreement 3: With the similar reason, it is better to clarify the measurement on CD-SSB for RACH preamble retransmission. |
| Futurewei | Option 3 for agreement 1 and can wait for agreement 3. | Same view as Huawei and CATT. |
| CMCC | Option 3 for agreement 1; FFS/Wait for agreement 3; | Same view with Huawei. |
| Fujitsu | Option 1 |  |
| NEC | Option 1 for agreement 1, Wait for agreement 3 | For 1, This is important aspect and it’s good to capture as a NOTE. If other companies have another suggestion on wording, we are open for further discussion. For 3, we are fine to wait for response from RAN1/4. |
| OPPO | Option 3 for agreement 1; FFS/Wait for agreement 3; | Same view as Huawei. |
| Lenovo&MM | Option 3 for agreement 1; FFS/Wait for agreement 3; | Same view with Huawei. |
| DENSO | Option 1 for agreement 1; Wait for agreement 3 | Agree that the agreement 1 should be captured in the spec to clarify what is used for SSB-based measurements in this case. |
| Spreadtrum | Option 1 |  |
| MediaTek | Option 1 | We prefer something along the lines of LGE’s response to be captured to have clear procedures. Regarding agreement 3, we are also ok to wait for response from RAN4/1 |

Summary: 20 companies provided their preference on whether/how to capture the above RAN2 agreement.

For agreement#1:

1. If a RedCap UE in idle/inactive mode is configured with a separate initial BWP associated with no SSB (CD or NCD) for RACH, measurements are based on CD-SSB for initial RACH resource selection.

10 companies think the agreement need to be captured in MAC specification:

- 9 companies among them agree with the text provided in current running MAC CR [2]. The proponent companies think it is better to clarify how to perform measurement for RACH resource selection, which is important aspect..

- 1 company suggests that there is no need to differentiate CD-SSB or NCD-SSB in MAC, and provides a suggestion (2 more companies are fine with this suggestion) as: NOTE X1: If a RedCap UE is configured with an [initial BWP for RedCap] not associated with any SSB, SS-RSRP is measured for the SSB associated with initial BWP.

8 companies think there is no need to capture this agreement in MAC specification with the following reasons:

- there is no differentiation between CD-SSB and NCD-SSB in MAC specification for RA procedure. It can be clarified in RRC that the SSB to be used is NCD-SSB;

- there is no other option for UE implementation.

2 companies have no strong view.

Based on inputs from companies, there are split views on this issue. Considering more companies (including MAC rapporteur Jaehyuk) prefer to capture this agreement, and many companies are open for the wording on how to capture it as a note, while the concern from other companies is about CD-SSB/NCD-SSB in MAC specification. Rapporteur thinks we could try to figure out an acceptable wording on how to capture this agreement, e.g. the wording suggested from LG could be the starting point.

Proposal 2: [To discuss][10 vs. 8] Capture the below Note in MAC specification as the starting point:

NOTE X1: If a RedCap UE in RRC_IDLE or RRC_INACTIVE mode is configured with an [separate initial BWP for RedCap] which is not associated with any SSB for RACH, SS-RSRP is measured for the SSB associated with initial BWP.

For agreement#3:

From RAN2 perspective, if a RedCap UE in idle/inactive mode is configured with a separate initial BWP associated with no SSB (CD or NCD) for RACH, it is up to UE implementation to perform new RSRP measurement in a DL BWP associated with CD-SSB before Msg1/A retransmission.

12 companies prefer to wait for the RAN1/RAN4 discussion and seeing how they capture/clarify this in their specification.

5 company agree with the text provided in current running MAC CR [2].

1 company thinks there is no need to capture this agreement in MAC specification.

2 companies have no strong view

Based on inputs from companies, there are clearly majority to wait for RAN1 and RAN4 progress and response. It is reasonable. Rapporteur suggests to wait and further discuss it later based on RAN1/RAN4 progress. A corresponding EN will be added in the updated MAC CR and there is no proposal on this issue.

In RAN2#116bis-e, it was agreed that:

2. If a RedCap-specific initial UL BWP is configured for RACH, RedCap UEs shall use only the RedCap-specific initial UL BWP to perform RACH.

Rapporteur thinks this conclusion has impacts on MAC specification on BWP switching (i.e. clause 5.15).

In idle/inactive mode, there was no BWP switching before, as there is only initial BWP, on which UE performs SI/Paging reception, RACH, etc. For RedCap, if a RedCap-specific initial UL BWP is configured for RACH, RedCap UEs shall use only the RedCap-specific initial UL BWP to perform RACH. In this way, the corresponding UE behaviour should be captured. During the running MAC CR discussion, some companies provided some rewording suggestions as below [2]:

[A RedCap UE in RRC_IDLE or RRC_INACTIVE mode may be configured with a RedCap-specific initial UL BWP, as specified in TS 38.213 [6]. If the UE is a RedCap UE in RRC_IDLE or RRC_INACTIVE mode, the MAC entity shall: - if the RedCap-specific initial UL BWP is configured with RACH: 2> perform RACH procedure as specified in clause 5.1 by using the RedCap-specific initial UL BWP; 3> if the RedCap-specific initial DL BWP is configured: 4> monitor the PDCCH on the RedCap-specific initial DL BWP.] Editor’s NOTE: FFS any other impacts on BWP operation in RRC_CONNECTED for the behavior for NCD-SSB, e.g. RRM, RLM, etc. Editor’s NOTE: How separate initial UL/DL BWP impacts MAC specification will be discussed and determined further. Editor’s NOTE: The behaviour on RedCap specific initial BWP need to be updated based on further progress.

- Companies are invited to show your preference among the following options on whether/how to capture the above conclusions:

- Option 1: Agree.

- Option 2: Not agree, please provide your suggestions.

- Option 3: No need to capture this in MAC specification.

- Option 4: Others, e.g., wait for more progress on separate initial BWP.

| Company’s name | Option | Comments, if any |
| --- | --- | --- |
| Huawei, HiSilicon | Option 2 | The above procedure text is only to clarify one thing that the RACH is on RedCap-specific initial UL BWP. Also, There is no definition in MAC on “RedCap-specific initial UL BWP”. We suggest to just capture one NOTE in the beginning of 5.1.1 to clarify all those parameters for RedCap: “NOTE: If configured, RedCap UE should use the configurations provided in [RedCap-specific initial UL BWP configuration], as specified in TS 38.331[x].” |
| Samsung | Option 1 | 3> and 4> can move one level up (i.e., to 2> and 3> respectively). |
| LGE | Option 4 | As indicated by EN, this may be impacted by the discussion under RACH partitioning, e.g., feature priority based BWP selection. Therefore, we propose to wait for more progress. |
| Nokia | Option 4 | Let’s capture once we have all the details how things should work. |
| Apple | Op 4 | RACH partitioning disc needs to be factored in. |
| Qualcomm | Option 1 with change | The same comment as Samsung. In addition, unless the term “RedCap-specific initial UL BWP” is defined elsewhere (e.g. 38.331), we suggest using the following wording instead: - if an initial UL BWP is configured specifically for RedCap UEs: and 2> if an initial DL BWP is configured specifically for RedCap UEs: Otherwise, we can directly use the field name for RedCap-specific initial BWPs defined in 38.331 |
| Intel | Option 1 |  |
| ZTE | Option 4 | Same comment as above companies, the discussion in RACH partitioning session also cover this, we prefer to wait for the progress of common RACH session, then to check if anything needs to be captured in RedCap MAC CR. |
| CATT | Option 4 |  |
| Xiaomi | Option 4 |  |
| Vivo | Option 1 | We think it is better to clarify the BWP switching behaviour, which should be captured in MAC specification. Considering potential future proof, there may be other case for BWP change behaviour in idle mode. Option 4 is also acceptable for us, if other companies prefer it, as the corresponding function has not been completely figured out. |
| Futurewei | Option 4 |  |
| CMCC | Option 4 |  |
| Fujitsu | Option 1 | Agree with Samsung. |
| NEC | Option 1/2 | Agree to capture the agreement, while how to capture this can be discussed further, e.g. “IDLE, INACTIVE” can be captured in the normative text of MAC spec? Probably it will be better to capture that without saying RRC state, if possible. |
| OPPO | Option 1 | Agree with Samsung. |
| Lenovo&MM | Option 1 |  |
| DENSO | Option 1 | Agree with Samsung and Qualcomm |
| Spreadtrum | Option 1 |  |
| MediaTek | Option 1 | Agree with Samsung |

Summary: 20 companies provided their preference on whether/how to capture the RAN2 agreement:

2. If a RedCap-specific initial UL BWP is configured for RACH, RedCap UEs shall use only the RedCap-specific initial UL BWP to perform RACH.

12 companies support to capture the above agreement in MAC specification:

- 11 companies agree with the text provided in current running MAC CR [2], with some suggstions:

  - 6 companies agree with suggestion from Samsung: 3> and 4> can move one level up (i.e., to 2> and 3> respectively).

  - 3 companies suggest not use “RedCap-specific initial UL/DL BWP” unless the terms are defined.

  - 1 company thinks it will be better to capture that without the RRC state.

- 2 companies don’t agree with the text provided in current running MAC CR [2]

  - 1 company suggests to just capture one NOTE in the beginning of 5.1.1 to clarify all those parameters for RedCap: NOTE: If configured, RedCap UE should use the configurations provided in [RedCap-specific initial UL BWP configuration], as specified in TS 38.331[x].

8 companies prefer to wait for more progress on separate initial BWP:

- companies think this may be impacted by the discussion under RACH partitioning and want to wait for the progress of common RACH session to determine whether/how to capture the conclusion in MAC specification.

Based on inputs from companies, many companies (12/20) prefer to capture this agreement in MAC specification with some wording suggestions, while less companies (8/20) prefer to wait for more progress on separate initial BWP. Rapporteur thinks we anyway need to further discuss the wording how to capture the corresponding conclusions. So we could try to figure out an acceptable way considering the text in current running MAC CR as the baseline. Further update could be considered according to further progress.

Proposal 3: [To discuss][12/20] Capture the following agreement in MAC specification for idle/inactive mode, using the text in the current MAC running CR as the start point:

Agreement: If a RedCap-specific initial UL BWP is configured for RACH, RedCap UEs shall use only the RedCap-specific initial UL BWP to perform RACH.

During the running MAC CR discussion, some companies commented that connected mode should be also considered. Then, the corresponding TP was provided in MAC running CR as below [2]:

[1> if for a RedCap UE, PRACH occasions for the Redcap UE are not configured for the active UL BWP, and if the RedCap-specific initial UL BWP is configured with RACH: - switch the active UL BWP to the RedCap-specific initial UL BWP; - if the RedCap-specific initial DL BWP is configured: - switch the active DL BWP to the RedCap-specific initial DL BWP; - else: - switch the active DL BWP to BWP indicated by initialDownlinkBWP. - else if, for a RedCap UE, the RedCap-specific initial UL BWP is not configured with RACH: or,] - [except for RedCap UEs,] if PRACH occasions are not configured for the active UL BWP: - switch the active UL BWP to BWP indicated by initialUplinkBWP; - if the Serving Cell is an SpCell: 3> switch the active DL BWP to BWP indicated by initialDownlinkBWP.

- Companies are invited to show your preference among the following options on whether/how to capture the above conclusions:

- Option 1: Agree.

- Option 2: Not agree, please provide your suggestions.

- Option 3: No need to capture this in MAC specification.

- Option 4: Others, e.g., wait for more progress on separate initial BWP.

| Company’s name | Option | Comments, if any |
| --- | --- | --- |
| Huawei, HiSilicon | Option 2 | See our comment in DP3, and our suggestion can cover both idle/inactive and connected. Also, existing spec already supports the BWP switching to the one with RACH resource. We only need to clarify in RRC what’s the RedCap-specific initial BWP. |
| Samsung | Option 1 | The text change can be slightly updated (merely for better readability) as follows: - if PRACH occasions are not configured for the active UL BWP: - if UE is a RedCap UE and if initialUplinkBWPRedcap is configured: - switch the active UL BWP to BWP indicated by initialUplinkBWPRedcap; - else: - switch the active UL BWP to BWP indicated by initialUplinkBWP; - if the Serving Cell is an SpCell: - if UE is RedCap and if initialDownlinkBWPRedcap is configured: 4> switch the active DL BWP to BWP indicated by initialDownlinkBWPRedcap; - else: 4> switch the active DL BWP to BWP indicated by initialDownlinkBWP. |
| LGE | Option 4 | As indicated by EN, this may be impacted by the discussion under RACH partitioning, e.g., considering CE. Therefore, we propose to wait for more progress. |
| Nokia | Option 4 | Agree with the intention but let’s wait for progress before capturing. |
| Apple | Op4 |  |
| Qualcomm | Option 1 with change | The same comment as for Discussion Point 3 |
| Intel | Option 1 |  |
| ZTE | Option 4 | Same comment as for previous question. |
| CATT | Option 4 |  |
| Xiaomi | Option 4 |  |
| Vivo | Option 1 | Same comment as previous one. |
| Futurewei | Option 4 |  |
| CMCC | Option 4 |  |
| Fujitsu | Option 1 | Agree with Samsung. |
| NEC | Option 1 |  |
| OPPO | Option 1 |  |
| Lenovo&MM | Option 1 |  |
| DENSO | Option 1 |  |
| Spreadtrum | Option 1 |  |
| MediaTek | Option 1 | Agree with Samsung |

Summary: 20 companies provide their views on connected mode for RedCap UE in current running MAC CR [2]:

12 companies support capture the BWP switch behaviour in connected mode for RedCap UE in MAC specification:

- 11 companies agree with the text provided in current running MAC CR [2]: with same suggestion as idle/inactive mode

- 1 company suggests to just capture one NOTE in the beginning of 5.1.1 to clarify all those parameters for RedCap as in idle/inactive mode.

8 companies prefer to wait for more progress on separate initial BWP:

- companies think this may be impacted by the discussion under RACH partitioning and want to wait for the progress of common RACH session to determine whether/how to capture the conclusion in MAC specification

Similar as idle/inactive mode, rapporteur suggests to take the text in current running MAC CR as the starting point.

Proposal 4: [To discuss][12/20] Capture the following agreement in MAC specification for connected mode, using the text in the current MAC running CR as the start point:

Agreement: If a RedCap-specific initial UL BWP is configured for RACH, RedCap UEs shall use only the RedCap-specific initial UL BWP to perform RACH.

In RAN2#116bis-e, it was agreed that:

8. For connected mode operation if NCD-SSB is configured in a dedicated DL BWP whose paired UL BWP is configured with RACH-ConfigDedicated, RACH-ConfigCommon or BeamFailureRecovery Config, SSB in that RACH configuration (e.g., in CFRA-SSB-Resource IE or in PRACH-ResourceDedicatedBFR IE) refers to the NCD-SSB configured in that DL BWP.

This conclusion concern RACH procedure in connected mode. Some companies think there is no need to capture this conclusion in RACH procedure or no need to capture it in MAC specification.

- Companies are invited to show your preference among the following options on whether/how to capture the above conclusion:

- Option 1: No need in MAC, clarification in other specification (e.g. in RRC) is enough.

- Option 2: Yes in MAC, please provide where/how to capture it (e.g. as a Note).

| Company’s name | Option | Comments, if any |
| --- | --- | --- |
| Huawei, HiSilicon | Option 1 | Apparently, the SSB clarification on CFRA-SSB-Resource IE or in PRACH-ResourceDedicatedBFR IE should be captured in RRC. |
| Samsung | Option 1 | - |
| LGE | Option 1 |  |
| Nokia | Option 1 |  |
| Apple | Op1 RRC is clear |  |
| Qualcomm | Option 1 |  |
| Intel | Option 1 |  |
| ZTE | Option 1 |  |
| CATT | Option 1 |  |
| Xiaomi | Option1 |  |
| Vivo | Option 1 | OK, we are also fine to clarify this in RRC specification. |
| Futurewei | Option 1 |  |
| CMCC | Option 1 |  |
| Fujitsu | Option 1 |  |
| NEC | Option 1 | RRC or Stage 2 with only high level concept (e.g. if NCD-SSB is configured in a dedicated DL BWP whose paired UL BWP is configured with RACH configurations, SSB in that RACH configuration refers to the NCD-SSB configured in that DL BWP.) |
| OPPO | Option 1 |  |
| Lenovo&MM | Option 1 |  |
| DENSO | Option 1 |  |
| Spreadtrum | Option 1 |  |
| MediaTek | Option 1 | Prefer to clarify this in the RRC field description |

Summary: 20 companies provided their views on whether capture the agreement concern RACH procedure in connected mode in MAC specification.

8. For connected mode operation if NCD-SSB is configured in a dedicated DL BWP whose paired UL BWP is configured with RACH-ConfigDedicated, RACH-ConfigCommon or BeamFailureRecovery Config, SSB in that RACH configuration (e.g., in CFRA-SSB-Resource IE or in PRACH-ResourceDedicatedBFR IE) refers to the NCD-SSB configured in that DL BWP.

All companies agree not capture this agreement in MAC specification and think clarification in other specification (e.g. in RRC) is enough.

Based on inputs from companies, Rapporteur suggests to follow companies’ view. The corresponding EN in the running MAC CR will be removed.

## Others

- Companies are invited to provide your views on any other open issues not included above which has impacts on MAC specification:

| Company’s name | Comments, if any |
| --- | --- |
| DENSO | For the case where the UE uses the RedCap-specific initial DL/UL BWP for RACH, what happens if the number of preamble transmission is reached to the maximum value and a random access problem is indicated to the upper layer? Should the UE reside in the RedCap specific DL/UL BWP? Or should the UE go back to the default initial DL BWP where SSB is present? |

Summary: 1 company provide comments on others, and several open issues are identified on RACH for RedCap UEs when UE fails to perform RACH on RedCap-specific initial DL/UL BWP. Rapporteur thinks these open issues could be considered in further discussion on NCD-SSB part or MAC part. Companies are also invited to raise similar issues during further discussion.

# Conclusion

This contribution summarizes the pre-meeting offline discussion: [Pre117-e][106][RedCap] MAC open issues (vivo), and achieves the following proposals: Please note that some discussion points bave no proposal, while the conclusions will be reflected in the next version of running MAC CR.

Proposals for easy agreement

Proposal 1: [To agree][20/20] Dedicated LCID for RedCap is always indicated when MsgA CCCH is sent by a RedCap UE (i.e. no other precondition).

Proposals need further discussion:

Proposal 2: [To discuss][10 vs. 8] Capture the below Note in MAC specification as the starting point:

NOTE X1: If a RedCap UE in RRC_IDLE or RRC_INACTIVE mode is configured with an [separate initial BWP for RedCap] which is not associated with any SSB for RACH, SS-RSRP is measured for the SSB associated with initial BWP.

Proposal 3: [To discuss][12/20] Capture the following agreement in MAC specification for idle/inactive mode, using the text in the current MAC running CR as the start point:

Agreement: If a RedCap-specific initial UL BWP is configured for RACH, RedCap UEs shall use only the RedCap-specific initial UL BWP to perform RACH.

Proposal 4: [To discuss][12/20] Capture the following agreement in MAC specification for connected mode, using the text in the current MAC running CR as the start point:

Agreement: If a RedCap-specific initial UL BWP is configured for RACH, RedCap UEs shall use only the RedCap-specific initial UL BWP to perform RACH.

# References

- R2-2201891, Open issue list on MAC for RedCap, vivo

- R2-2201890, Running MAC CR for RedCap, vivo