3GPP TSG-RAN WG2 Meeting #118 Electronic	R2-2205020

# Elbonia, 09 – 20 May 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | Num | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on PDCP duplication activation triggered by retransmission grant |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IIOT_URLLC_enh-Core |  |  |  |  |  | Date: |  |  | 2022-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Currently upon survival time state entry all RLC entities configured for the DRB are activated by the UE for duplication, which might be problematic if for some RLC entities there is no activated serving cell to transmit the duplicated data. While NW should be able decide which serving cells it wants to keep activated for the UE depending on the cell load and radio quality etc., other than being enforced to keep serving cells activated for all the duplication legs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - 5.4.1, upon receiption of UL retransmission grant for a logical channel associated with a DRB configured with survivalTimeStateSupport is multiplexed in the MAC PDU, change “trigger activation of PDCP duplication for all configured RLC entities of the DRB.” to “trigger activation of PDCP duplication for all configured RLC entities of the DRB with at least one serving cell configured in the allowedServingCells activated” - 5.10, only indicate activation of the correpsonding RLC entity with activated serving cell to upper layer |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RLC entities without any activated serving cell are activated and the duplicated data is stuck in the buffer. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.1, 5.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS38.300 CR0450 |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Modified Subclause

### 5.4.1	UL Grant reception

Uplink grant is either received dynamically on the PDCCH, in a Random Access Response, configured semi-persistently by RRC or determined to be associated with the PUSCH resource of MSGA as specified in clause 5.1.2a. The MAC entity shall have an uplink grant to transmit on the UL-SCH. To perform the requested transmissions, the MAC layer receives HARQ information from lower layers. An uplink grant addressed to CS-RNTI with NDI = 0 is considered as a configured uplink grant. An uplink grant addressed to CS-RNTI with NDI = 1 is considered as a dynamic uplink grant.

If the MAC entity has a C-RNTI, a Temporary C-RNTI, or CS-RNTI, the MAC entity shall for each PDCCH occasion and for each Serving Cell belonging to a TAG that has a running timeAlignmentTimer or a running cg-SDT-TimeAlignmentTimer and for each grant received for this PDCCH occasion:

1>	if an uplink grant for this Serving Cell has been received on the PDCCH for the MAC entity's C-RNTI or Temporary C-RNTI; or

1>	if an uplink grant has been received in a Random Access Response:

2>	if the uplink grant is for MAC entity's C-RNTI and if the previous uplink grant delivered to the HARQ entity for the same HARQ process was either an uplink grant received for the MAC entity's CS-RNTI or a configured uplink grant:

3>	consider the NDI to have been toggled for the corresponding HARQ process regardless of the value of the NDI.

2>	if the uplink grant is for MAC entity's C-RNTI, and the identified HARQ process is configured for a configured uplink grant:

3>	start or restart the configuredGrantTimer for the corresponding HARQ process, if configured.

3>	stop the cg-SDT-RetransmissionTimer for the corresponding HARQ process, if running.

3>	stop the cg-RetransmissionTimer for the corresponding HARQ process, if running.

2>	deliver the uplink grant and the associated HARQ information to the HARQ entity.

1>	else if an uplink grant for this PDCCH occasion has been received for this Serving Cell on the PDCCH for the MAC entity's CS-RNTI:

2>	if the NDI in the received HARQ information is 1:

3>	consider the NDI for the corresponding HARQ process not to have been toggled;

3>	start or restart the configuredGrantTimer for the corresponding HARQ process, if configured;

3>	stop the cg-RetransmissionTimer for the corresponding HARQ process, if running;

3>	stop the cg-SDT-RetransmissionTimer for the corresponding HARQ process, if running;

3>	deliver the uplink grant and the associated HARQ information to the HARQ entity;

3>	if a logical channel associated with a DRB configured with survivalTimeStateSupport is multiplexed in the MAC PDU stored in the HARQ buffer for the corresponding HARQ process:

4>	trigger activation of PDCP duplication for all configured RLC entities of the DRB with at least one serving cell configured in the allowedServingCells activated.

2>	else if the NDI in the received HARQ information is 0:

3>	if PDCCH contents indicate configured grant Type 2 deactivation:

4>	trigger configured uplink grant confirmation.

3>	else if PDCCH contents indicate configured grant Type 2 activation:

4>	trigger configured uplink grant confirmation;

4>	store the uplink grant for this Serving Cell and the associated HARQ information as configured uplink grant;

4>	initialise or re-initialise the configured uplink grant for this Serving Cell to start in the associated PUSCH duration and to recur according to rules in clause 5.8.2;

4>	stop the configuredGrantTimer for the corresponding HARQ process, if running;

4>	stop the cg-RetransmissionTimer for the corresponding HARQ process, if running.

For each Serving Cell and each configured uplink grant, if configured and activated, the MAC entity shall:

1>	if the MAC entity is configured with lch-basedPrioritization, and the PUSCH duration of the configured uplink grant does not overlap with the PUSCH duration of an uplink grant received in a Random Access Response or with the PUSCH duration of an uplink grant addressed to Temporary C-RNTI or the PUSCH duration of a MSGA payload for this Serving Cell; or

1>	if the MAC entity is not configured with lch-basedPrioritization, and the PUSCH duration of the configured uplink grant does not overlap with the PUSCH duration of an uplink grant received on the PDCCH or in a Random Access Response or the PUSCH duration of a MSGA payload for this Serving Cell:

2>	set the HARQ Process ID to the HARQ Process ID associated with this PUSCH duration;

2>	if, for the corresponding HARQ process, the configuredGrantTimer is not running and cg-RetransmissionTimer is not configured and cg-SDT-RetransmissionTimer is not configured (i.e. new transmission):

3>	consider the NDI bit for the corresponding HARQ process to have been toggled;

3>	deliver the configured uplink grant and the associated HARQ information to the HARQ entity.

2>	else if the cg-RetransmissionTimer for the corresponding HARQ process is configured and not running, then for the corresponding HARQ process:

3>	if the configuredGrantTimer is not running, and the HARQ process is not pending (i.e. new transmission):

4>	consider the NDI bit to have been toggled;

4>	deliver the configured uplink grant and the associated HARQ information to the HARQ entity.

3>	else if the previous uplink grant delivered to the HARQ entity for the same HARQ process was a configured uplink grant (i.e. retransmission on configured grant):

4>	deliver the configured uplink grant and the associated HARQ information to the HARQ entity.

2>	else if the cg-SDT-RetransmissionTimer is configured and not running for the corresponding HARQ process;

3>	if the configured uplink grant is for the initial transmission for the CG-SDT with CCCH message (i.e., initial new transmission); or

3>	if the configuredGrantTimer is not running or not configured, and PDCCH addressed to the MAC entity’s C-RNTI has been received after the initial transmission of the CG-SDT with CCCH message (i.e., subsequent new transmission):

4>	consider the NDI bit to have been toggled.

3>	else if the previous uplink grant delivered to the HARQ entity for the same HARQ process was a configured uplink grant for initial transmission of CG-SDT with CCCH message or for its retransmission; and

3>	if PDCCH addressed to the MAC entity’s C-RNTI has not been received (i.e., retransmission for initial CG-SDT transmission):

4>	consider the NDI bit to have not been toggled.

3>	deliver the configured uplink grant and the associated HARQ information to the HARQ entity.

For configured uplink grants neither configured with harq-ProcID-Offset2 nor with cg-RetransmissionTimer, the HARQ Process ID associated with the first symbol of a UL transmission is derived from the following equation:

HARQ Process ID = [floor(CURRENT_symbol/periodicity)] modulo nrofHARQ-Processes

For configured uplink grants with harq-ProcID-Offset2, the HARQ Process ID associated with the first symbol of a UL transmission is derived from the following equation:

HARQ Process ID = [floor(CURRENT_symbol / periodicity)] modulo nrofHARQ-Processes + harq-ProcID-Offset2

where CURRENT_symbol = (SFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot + slot number in the frame × numberOfSymbolsPerSlot + symbol number in the slot), and numberOfSlotsPerFrame and numberOfSymbolsPerSlot refer to the number of consecutive slots per frame and the number of consecutive symbols per slot, respectively as specified in TS 38.211 [8].

For configured uplink grants configured with cg-RetransmissionTimer, the UE implementation selects an HARQ Process ID among the HARQ process IDs available for the configured grant configuration. If the MAC entity is configured with intraCG-Prioritization, for HARQ Process ID selection, the UE shall prioritize the HARQ Process ID with the highest priority, where the priority of HARQ process is determined by the highest priority among priorities of the logical channels that are multiplexed (i.e. the MAC PDU to transmit is already stored in the HARQ buffer) or have data available that can be multiplexed (i.e. the MAC PDU to transmit is not stored in the HARQ buffer) in the MAC PDU, according to the mapping restrictions as described in clause 5.4.3.1.2. If the MAC entity is configured with intraCG-Prioritization, for HARQ Process ID selection among initial transmission and retransmission with equal priority, the UE shall prioritize retransmissions before initial transmissions. The priority of a HARQ Process for which no data for logical channels is multiplexed or can be multiplexed in the MAC PDU is lower than the priority of a HARQ Process for which data for any logical channels is multiplexed or can be multiplexed in the MAC PDU. If the MAC entity is not configured with intraCG-Prioritization, for HARQ Process ID selection, the UE shall prioritize retransmissions before initial transmissions. The UE shall toggle the NDI in the CG-UCI for new transmissions and not toggle the NDI in the CG-UCI in retransmissions.

NOTE 1:	CURRENT_symbol refers to the symbol index of the first transmission occasion of a bundle of configured uplink grant.

NOTE 2:	A HARQ process is configured for a configured uplink grant where neither harq-ProcID-Offset nor harq-ProcID-Offset2 is configured, if the configured uplink grant is activated and the associated HARQ process ID is less than nrofHARQ-Processes. A HARQ process is configured for a configured uplink grant where harq-ProcID-Offset2 is configured, if the configured uplink grant is activated and the associated HARQ process ID is greater than or equal to harq-ProcID-Offset2 and less than sum of harq-ProcID-Offset2 and nrofHARQ-Processes for the configured grant configuration.

NOTE 3:	If the MAC entity receives a grant in a Random Access Response (i.e. MAC RAR or fallbackRAR), or addressed to Temporary C-RNTI or determines a grant as specified in clause 5.1.2a for MSGA payload and if the MAC entity also receives an overlapping grant for its C-RNTI or CS-RNTI, requiring concurrent transmissions on the SpCell, the MAC entity may choose to continue with either the grant for its RA-RNTI/Temporary C-RNTI/MSGB-RNTI/the MSGA payload transmission or the grant for its C-RNTI or CS-RNTI.

NOTE 4:	In case of unaligned SFN across carriers in a cell group, the SFN of the concerned Serving Cell is used to calculate the HARQ Process ID used for configured uplink grants.

NOTE 5:	If cg-RetransmissionTimer is not configured, a HARQ process is not shared between different configured grant configurations in the same BWP.

For the MAC entity configured with lch-basedPrioritization, priority of an uplink grant is determined by the highest priority among priorities of the logical channels that are multiplexed (i.e. the MAC PDU to transmit is already stored in the HARQ buffer) or have data available that can be multiplexed (i.e. the MAC PDU to transmit is not stored in the HARQ buffer) in the MAC PDU, according to the mapping restrictions as described in clause 5.4.3.1.2. The priority of an uplink grant for which no data for logical channels is multiplexed or can be multiplexed in the MAC PDU is lower than either the priority of an uplink grant for which data for any logical channels is multiplexed or can be multiplexed in the MAC PDU or the priority of the logical channel triggering an SR.

For the MAC entity configured with lch-basedPrioritization, if the corresponding PUSCH transmission of a configured uplink grant is cancelled by CI-RNTI as specified in clause 11.2A of TS 38.213 [6] or cancelled by a high PHY-priority PUCCH transmission as specified in clause 9 of TS 38.213 [6], this configured uplink grant is considered as a de-prioritized uplink grant. If this deprioritized uplink grant is configured with autonomousTx, the configuredGrantTimer for the corresponding HARQ process of this de-prioritized uplink grant shall be stopped if it is running. If this de-prioritized uplink grant is configured with autonomousTx, the cg-RetransmissionTimer for the corresponding HARQ process of this de-prioritized uplink grant shall be stopped if it is running.

When the MAC entity is configured with lch-basedPrioritization, for each uplink grant delivered to the HARQ entity and whose associated PUSCH can be transmitted by lower layers, the MAC entity shall:

1>	if this uplink grant is received in a Random Access Response (i.e. in a MAC RAR or fallback RAR), or addressed to Temporary C-RNTI, or is determined as specified in clause 5.1.2a for the transmission of the MSGA payload:

2>	consider this uplink grant as a prioritized uplink grant.

1>	else if this uplink grant is addressed to CS-RNTI with NDI = 1 or C-RNTI:

2>	if there is no overlapping PUSCH duration of a configured uplink grant which was not already de-prioritized and the simultaneous transmission of the SR and the uplink grant is not allowed by configuration of simultaneousPUCCH-PUSCH, in the same BWP whose priority is higher than the priority of the uplink grant; and

2>	if there is no overlapping PUCCH resource with an SR transmission which was not already de-prioritized and the priority of the logical channel that triggered the SR is higher than the priority of the uplink grant:

3>	consider this uplink grant as a prioritized uplink grant;

3>	consider the other overlapping uplink grant(s), if any, as a de-prioritized uplink grant(s);

3>	consider the other overlapping SR transmission(s), if any, as a de-prioritized SR transmission(s);

3>	if the de-prioritized uplink grant(s) is a configured uplink grant configured with autonomousTx whose PUSCH has already started:

4>	stop the configuredGrantTimer for the corresponding HARQ process of the de-prioritized uplink grant(s);

4>	stop the cg-RetransmissionTimer for the corresponding HARQ process of the de-prioritized uplink grant(s).

1>	else if this uplink grant is a configured uplink grant:

2>	if there is no overlapping PUSCH duration of another configured uplink grant which was not already de-prioritized, in the same BWP, whose priority is higher than the priority of the uplink grant; and

2>	if there is no overlapping PUSCH duration of an uplink grant addressed to CS-RNTI with NDI = 1 or C-RNTI which was not already de-prioritized, in the same BWP, whose priority is higher than or equal to the priority of the uplink grant; and

2>	if there is no overlapping PUCCH resource with an SR transmission which was not already de-prioritized and the simultaneous transmission of the SR and the uplink grant is not allowed by configuration of simultaneousPUCCH-PUSCH and the priority of the logical channel that triggered the SR is higher than the priority of the uplink grant:

3>	consider this uplink grant as a prioritized uplink grant;

3>	consider the other overlapping uplink grant(s), if any, as a de-prioritized uplink grant(s);

3>	if the de-prioritized uplink grant(s) is a configured uplink grant configured with autonomousTx whose PUSCH has already started:

4>	stop the configuredGrantTimer for the corresponding HARQ process of the de-prioritized uplink grant(s);

4>	stop the cg-RetransmissionTimer for the corresponding HARQ process of the de-prioritized uplink grant(s).

3>	consider the other overlapping SR transmission(s), if any, as a de-prioritized SR transmission(s).

NOTE 6:	If the MAC entity is configured with lch-basedPrioritization and if there is overlapping PUSCH duration of at least two configured uplink grants whose priorities are equal, the prioritized uplink grant is determined by UE implementation.

NOTE 7:	If the MAC entity is not configured with lch-basedPrioritization and if there is overlapping PUSCH duration of at least two configured uplink grants, it is up to UE implementation to choose one of the configured uplink grants.

NOTE 8:	If the MAC entity is configured with lch-basedPrioritization, the MAC entity does not take UCI multiplexing according to the procedure specified in TS 38.213 [6] into account when determining whether the PUSCH duration of an uplink grant overlaps with the PUCCH resource for an SR transmission.

Next Modified Subclause

## 5.10	Activation/Deactivation of PDCP duplication

If one or more DRBs are configured with PDCP duplication, the network may activate and deactivate the PDCP duplication for all or a subset of associated RLC entities for the configured DRB(s).

The PDCP duplication for the configured DRB(s) is activated and deactivated by:

-	receiving the Duplication Activation/Deactivation MAC CE described in clause 6.1.3.11;

-	receiving the Duplication RLC Activation/Deactivation MAC CE described in clause 6.1.3.32;

-	indication by RRC;

-	receiving an uplink grant addressed to CS-RNTI with NDI=1 described in 5.4.1 (for activation only).

The PDCP duplication for all or a subset of associated RLC entities for the configured DRB(s) is activated and deactivated by:

-	receiving the Duplication RLC Activation/Deactivation MAC CE described in clause 6.1.3.32;

-	indication by RRC;

-	receiving an uplink grant addressed to CS-RNTI with NDI=1 described in 5.4.1 (for activation only).

The MAC entity shall for each DRB configured with PDCP duplication:

1>	if a Duplication Activation/Deactivation MAC CE is received activating the PDCP duplication of the DRB:

2>	indicate the activation of PDCP duplication of the DRB to upper layers.

1>	if a Duplication Activation/Deactivation MAC CE is received deactivating the PDCP duplication of the DRB:

2>	indicate the deactivation of PDCP duplication of the DRB to upper layers.

1>	if a Duplication RLC Activation/Deactivation MAC CE is received activating PDCP duplication for associated RLC entities of a DRB configured with PDCP duplication:

2>	indicate the activation of PDCP duplication for the indicated secondary RLC entity(ies) of the DRB to upper layers.

1>	if a Duplication RLC Activation/Deactivation MAC CE is received deactivating PDCP duplication for associated RLC entities of a DRB configured with PDCP duplication:

2>	indicate the deactivation of PDCP duplication for the indicated secondary RLC entity(ies) of the DRB to upper layers.

1>	if activation of a PDCP duplication for all configured RLC entities with at least one serving cell configured in the allowedServingCells activated is triggered for the DRB as specified in clause 5.4.1:

2>	indicate the activation of PDCP duplication for the corresponding RLC entities of the DRB to upper layers.

End of Changes