3GPP TSG-RAN WG2 Meeting #118 Electronic	R2-2205601

Elbonia, 9th May- 19th May 2022

Agenda item:	7.2.4

Source:	Nokia, Nokia Shanghai Bell

Title:	On Capability Indication of existing IoT-Features for NTN connectivity

WID/SID:	LTE_NBIOT_eMTC_NTN - Release 17

Document for:	Discussion and Decision

# 1	Introduction

In RAN2-118e meeting need for duplicating the capability indications of existing IoT features for NTN connectivity was discussed over AT-meeting e-mail discussion. Different solution options for NB-IoT and eMTC to address this issue were discussed without consensus on the final option. Moreover, the need for separate capability and features which requires such indication, and the reasons were not discussed. In this paper we further analyse this open issue.

# 2	Discussion

During the study phase of IoT-NTN WI it was concluded that all the features specified until Rel-16 are applicable for NTN connectivity unless specific issues are identified.  For some features such as PUR the additional changes required are identified and additional capabilities are specified for Rel-17.  During RAN2-118e meetings, many companies indicated that eventhough these features does not have any specification impacts there could be some additional UE implementation impacts and testing efforts to make these features applicable for NTN connectivity. Hence some companies prefer to indicate the capability for IoT features using new capability indication towards NTN. But the scenarios where such additional indications needed were not discussed in detail.

Observation 1: Device implementation may not support all the IoT-features supported for TN for NTN connectivity due to additional implementation and testing efforts required for the NTN connectivity.

When some capabilities supported for TN are not supported for NTN connectivity, UE can still make use of existing capability indication to indicate the same with values of capability field varies depending on the connected network. But this will require the UE to trigger NAS level signalling procedure on switching between TN and NTN cells to trigger the capability enquiry procedure from new serving cell. This option is already possible where UE can trigger NAS procedure if its E-UTRA capability has been changed  [TS36.331. Clause 5.6.3].

Observation 2: It is possible to indicate different values for same capability indication parameter towards TN and NTN cell for idle mode mobility. In this case, UE should trigger NAS signalling procedure to indicate the change of capability indication.

In case of connected mode mobility between TN and NTN cell use of same capability indication with different values for TN and NTN may lead to some impacts after the mobility procedure. To provide the updated capability UE may need to trigger NAS signalling procedure after connected mode mobility. This will require some additional specification changes.  But connected mode mobility between TN and NTN is not in the scope for Rel-17 IoT-NTN. Moreover NB-IoT does not support connected mode mobility.

Observation 3: Separate capability indication for IoT-Features will be required for connected mode mobility between TN and NTN.

Connected mode mobility between TN and NTN cells is not in the scope of Rel-17 functionality. Support for TN and NTN mobility will require some additional features to be introduced. But introducing separate capability for the features which are not supported for NTN in later release may lead to backward compatibility issues. Introducing the separate capability for specific features which requires additional UE implementation or testing efforts for NTN in Rel-17 is preferred for devices which supports connected mode mobility.

If separate capability indication is needed for NTN connectivity, the set of features which requires duplicate capability for NTN needs to be identified instead of introducing the separate capability indication for all the features.  We have considered NB-IoT feature capability list as example and analyse the need for duplication based on whether the feature is essential for NTN connectivity and the changes needed in UE implementation or testing for the specific feature in the below table.  The highlighted rows are potential candidate to consider for separate capability indication for NTN connectivity.

|  | Essential for Initial IoT-NTN deployments | Need for Additional Testing specific to NTN scenario | Separate capability for NTN connectivity |
| --- | --- | --- | --- |
| ue-Category-NB in NB-IoT (clause 4.1C) |  |  |  |
| supportedROHC-Profiles-r13 (clause 4.3.1.1A) | Non Critical | No NTN specific impacts | No |
| maxNumberROHC-ContextSessions-r13 (clause 4.3.1.2A) | Non Critical | No NTN specific impacts | No |
| rlc-UM-r15 (clause 4.3.2.5) | Essential | No NTN specific impacts | No |
| multiTone-r13 (clause 4.3.4.55) | Essential | No NTN specific impacts | No |
| multiCarrier-r13 (clause 4.3.4.56) | Essential | No NTN specific impacts | No |
| twoHARQ-Processes-r14 (clause 4.3.4.62) | Essential | No NTN specific impacts | No |
| multiCarrier-NPRACH-r14 (clause 4.3.4.75) | Essential due to large device density per cell | No NTN specific impacts | No |
| multiCarrierPaging-r14 (clause 4.3.4.76) | Essential for large NTN cell and higher UE density | No NTN specific impacts | No |
| interferenceRandomisation-r14 (clause 4.3.4.80) | Non Critical | No NTN specific impacts | No |
| wakeUpSignal-r15 (clause 4.3.4.113) | Essential | Reception of WUS and the offset to Paging occasion may have some impact due to longer delay. Also Feature effectiveness considering large number of UE per cell to be analysed. | Can be considered |
| wakeUpSignalMinGap-eDRX-r15 (clause 4.3.4.114) | Essential | Reception of WUS and the offset to Paging occasion may have some impact due to longer delay. Also Feature effectiveness considering large number of UE per cell to be analysed. | Can be considered |
| mixedOperationMode-r15 (clause 4.3.4.115) | Non Critical | No NTN specific impacts. No TDD spectrum in NTN band | No |
| sr-WithHARQ-ACK-r15 (clause 4.3.4.117) | Non Critical | No NTN specific impacts. No TDD spectrum in NTN band | No |
| sr-WithoutHARQ-ACK-r15 (clause 4.3.4.118) | Non Critical | No NTN specific impacts. No TDD spectrum in NTN band | No |
| nprach-Format2-r15 (clause 4.3.4.119) | Non Critical | No NTN specific impacts. No TDD spectrum in NTN band | Not needed for NTN |
| multiCarrierPagingTDD-r15 (clause 4.3.4.134) | Non Critical | No NTN specific impacts. No TDD spectrum in NTN band | Not needed for NTN |
| additionalTransmissionSIB1-r15 (clause 4.3.4.137) |  |  |  |
| npusch-3dot75kHz-SCS-TDD-r15 (clause 4.3.4.177) | Non Critical | No NTN specific impacts. No TDD spectrum in NTN band | Not needed for NTN |
| npusch-MultiTB-r16 (clause 4.3.4.182) | Essential | Considering long HARQ related delay multi-TB will improve the overall throughput /delay performance |  |
| npdsch-MultiTB-r16 (clause 4.3.4.183) | Essential | Considering long HARQ related delay multi-TB will improve the overall throughput /delay performance |  |
| npusch-MultiTB-Interleaving-r16 (clause 4.3.4.192) | Essential | Considering long HARQ related delay multi-TB will improve the overall throughput /delay performance |  |
| npdsch-MultiTB-Interleaving-r16 (clause 4.3.4.193) | Essential | Considering long HARQ related delay multi-TB will improve the overall throughput /delay performance |  |
| multiTB-HARQ-AckBundling-r16 (clause 4.3.4.194) | Essential | Considering long HARQ related delay multi-TB will improve the overall throughput /delay performance |  |
| groupWakeUpSignal-r16 (clause 4.3.4.195) | Essential | Reception of WUS and the offset to Paging occasion may have some impact due to longer delay. Also Feature effectiveness considering large number of UE per cell to be analysed. | Can be considered |
| groupWakeUpSignalAlternation-r16 (clause 4.3.4.196) | Essential | Reception of WUS and the offset to Paging occasion may have some impact due to longer delay. Also Feature effectiveness considering large number of UE per cell to be analysed. | Can be considered |
| subframeResourceResvUL-r16 (clause 4.3.4.197) | Non Critical | The functionality does not have impact due to NTN connectivity |  |
| subframeResourceResvDL-r16 (clause 4.3.4.198) | Non Critical | The functionality does not have impact due to NTN connectivity |  |
| slotSymbolResourceResvUL-r16 (clause 4.3.4.199) | Non Critical | The functionality does not have impact due to NTN connectivity |  |
| slotSymbolResourceResvDL-r16 (clause 4.3.4.200) | Non Critical | The functionality does not have impact due to NTN connectivity |  |
| supportedBandList-r13 (clause 4.3.5.1A) |  |  |  |
| multiNS-Pmax-r13 (clause 4.3.5.16A) | Non Critical |  |  |
| powerClassNB-20dBm-r13 (clause 4.3.5.1A.1) | Non Critical |  |  |
| powerClassNB-14dBm-r14 (clause 4.3.5.1A.2) | Non Critical |  |  |
| dl-ChannelQualityReporting-r16 (clause 4.3.6.37) | Non Critical |  |  |
| accessStratumRelease-r13 (clause 4.3.8.1A) |  |  |  |
| multipleDRB-r13 (clause 4.3.8.5) | Non Critical |  |  |
| earlyData-UP-r15 (clause 4.3.8.7) | Essential |  |  |
| earlySecurityReactivation-r16 (clause 4.3.8.11) |  |  |  |
| anr-Report-r16 (clause 4.3.12.2) | Non Critical | May require some changes to ANR aspects considering NTN deployments. | Can be considered |
| rach-Report-r16 (clause 4.3.12.3) | Non Critical |  |  |
| logicalChannelSR-ProhibitTimer (clause 4.3.19.2) |  |  |  |
| dataInactMon-r14 (clause 4.3.19.9) |  |  |  |
| rai-Support-r14 (clause 4.3.19.10) | Non Critical | Functionality is not impacted due to NTN connectivity |  |
| earlyContentionResolution-r14 (clause 4.3.19.14) | Non Critical | May have some additional impacts or testing required for NTN connectivity. | Can be considered |
| sr-SPS-BSR-r15 (clause 4.3.19.15) | Non Critical |  |  |
| rai-SupportEnh-r16 (clause 4.3.19.22) | Non Critical |  |  |
| earlyData-UP-5GC-r16 (clause 4.3.36.9) | Not applicable |  |  |
| pur-CP-EPC-r16 (clause 4.3.37.1) | Not Critical | Feature not applicable for LEO deployment in Rel-17. For GEO case, the feature will work along with additional NTN specific capability. As there is already NTN specific capability for NTN specific functionality, these capability bits need not be repeated for NTN |  |
| pur-UP-EPC-r16 (clause 4.3.37.2) | Not Uritical | Feature not applicable for LEO deployment. For GEO case, the feature will work along with additional NTN specific capability. As there is already NTN specific capability for NTN specific functionality, these capability bits need not be repeated for NTN |  |
| pur-CP-5GC-r16 (clause 4.3.37.3) | Not Critical | Feature not applicable for LEO deployment. For GEO case, the feature will work along with additional NTN specific capability. As there is already NTN specific capability for NTN specific functionality, these capability bits need not be repeated for NTN |  |
| pur-UP-5GC-r16 (clause 4.3.37.4) | Not Uritical | Feature not applicable for LEO deployment. For GEO case, the feature will work along with additional NTN specific capability. As there is already NTN specific capability for NTN specific functionality, these capability bits need not be repeated for NTN |  |
| pur-CP-L1Ack-r16 (clause 4.3.37.5) | Not Critical | Feature not applicable for LEO deployment. For GEO case, the feature will work along with additional NTN specific capability. As there is already NTN specific capability for NTN specific functionality, these capability bits need not be repeated for NTN |  |
| pur-NRSRP-Validation-r16 (clause 4.3.37.6) | Not Critical | Feature not applicable for LEO deployment. For GEO case, the feature will work along with additional NTN specific capability. As there is already NTN specific capability for NTN specific functionality, these capability bits need not be repeated for NTN |  |

Similar analysis should be done to identify the features which requires separate capability indication for eMTC also should be carried out if RAN2 decides to introduce separate capability indication for NTN for the existing features.

Proposal 1: RAN2 to conclude on the features which requires separate capability indication for eMTC based on Feature-wise analysis on the relevance and additional UE implementation impacts for NTN connectivity.

Based on the above analysis, it is possible to use same capability indication with different value for NTN and TN during idle mode mobility, separate indication is not needed for NB-IoT. Because NB-IoT does not support connected mode mobility.

Proposal 2: For NB-IoT, same capability indication parameter can be used with different values for TN and NTN connectivity.

# 3	Conclusion

Observation 1: Device implementation may not support all the IoT-features supported for TN for NTN connectivity due to additional implementation and testing efforts required for the NTN connectivity.

Observation 2: It is possible to indicate different values for same capability indication parameter towards TN and NTN cell for idle mode mobility. In this case, UE should trigger NAS signalling procedure to indicate the change of capability indication.

Observation 3: Separate capability indication for IoT-Features will be required for connected mode mobility scenario between TN and NTN.

Proposal 1: RAN2 to conclude on the features which requires separate capability indication for eMTC based on Feature-wise analysis on the relevance and additional UE implementation impacts for NTN connectivity.

Proposal 2: For NB-IoT, same capability indication parameter can be used with different values for TN and NTN connectivity.